3GPP TSG-RAN WG4 Meeting # 98-e 	                                                                      R4-2102962

Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda item:	8

Source: 	Moderator (CMCC)

Title: 	Email discussion summary for [98e][114] R16_UE_ feature

Document for:	Information

- Introduction

This is a document for email discussion on Rel-16 LTE and NR UE features in RAN4#98e. Companies are encouraged to discuss the open issues and provide comments during 98e meeting if any.

The document in RAN4#97e is R4-2016961, and the UE feature list agreed in RAN4#97e is R4-2016850.

- Topic#1: R16 NR UE feature list

  - Companies’ contributions summery

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2101155 | Discussion on UE feature list | MediaTek inc. |
| R4-2101659 | Discussion on per-FR gap capability | Huawei, HiSilicon |
| R4-2102891 | UE feature on Simultaneous dormant BWP switching | Qualcomm Incorporated |

  - Open issues summery

    - MR DC/CA enhancement

Issue 1-1: New feature simultaneous dormant BWP switching

Option 1 (MTK, R4-2101155): Introduce a new UE capability for the incremental BWP switch delay on additional CCs for dormancy indication involving multiple CCs. The candidate values for the incremental delay D' are

- D’ = 100us or 200us for UE indicating type1 in bwp-SwitchingDelay

- D’ = 200us or 400us for UE indicating type2 in bwp-SwitchingDelay

Option 2 (Qualcomm, R4-2102891): Add the following UE capability for simultaneous dormant BWP switching to the feature list.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6. LTE_NR_DC_CA_enh | 6-[X] | Dormant BWP switching on multiple CCs RRM requirements | Incremental delay for BWP switch processing on additional SCells in timer/DCI based simultaneous dormant BWP switching on multiple SCells | RAN1 feature 18-4 and 18-4a | Yes | N/A | There may be additional unclear BWP switching delay if network trigger dormant BWP switching on multiple SCells simultaneously. | Per UE | No | No | N/A | For component 2), the candidate values are: ● {100us, 200us} for UE indicates type1 in bwp-SwitchingDelay ● {200us, 400us, 800us, 1000us} for UE indicates type 2 in bwp-SwitchingDelay The total BWP switching delay will be captured in TS38.133 UE needs to indicate either of the candidate values in case it supports dormant BWP | Optional with capability signalling |

Recommended WF:

The technical discussion on the set of D’ values are in [211].

Focus on the discussion on the rest of the aspects in this email thread (e.g. pre-requisite features, feature description, etc)

    - Others

Issue 2-1: New per BC indication of the per-FR gap

Option 1 (Huawei, R4-2101155): The per-BC indication of the per-FR gap to be introduced and the original per-UE indication to be kept.

Recommended WF:

More discussion is needed.

  - Companies views’ collection for 1st round

    - Open issues

| Company | Comments |
| --- | --- |
| MTK | Issue 1-1 New feature simultaneous dormant BWP switching - It should be fine to put it as a feature under LTE_NR_DC_CA_enh rather than RRM enh - We are fine with the feature description suggested by Qualcomm - Prerequisite feature groups should be 18-4 or 18-4a in RAN1 feature least - Agree to leave technical discussion in RRM session Issue 2-1: New per BC indication of the per-FR gap - No. Not necessary. - During any measurement gap occasions, all data receptions and transmissions in carriers in the same FR are stopped. UE only has to receive one single frequency layer per FR during one gap occasion. It is strange to link this capability to the band combination. - Regarding relation to interruption requirement, it was introduced by pure considering whether FR1 and FR2 RFs have mutual influence to each other, which does not seem to be changed with band combination. - RRM session already agreed that a baseline UE should at least implement 2 searchers for measurement. The 2 searchers can directly be used for per-FR gap. Therefore, we do not see this feature demands a UE complexity in baseband side. |
| Ericsson | Issue 1-1: We are also fine to discuss this feature. But the values of D’ should be discussed and decided under LTE_NR_DC_CA_enh WI in the RRM session. Issue 2-1: We also agree with MTK that there is the need to define any additional BC capability within per FR gap capability. If the UE can support per FR gaps for certain FR1+FR2 band combination then there is no reason not to support per FR gaps for another FR1+FR2 band combination. |
| Verizon | Issue 2-1: Yes, it is reasonable to define the concept of per-FR gap in Rel-16 UE. Besides of reasons in R4-2101659, two more major benefits for us are that UE could enlarge more CCs in CA combination (for spectrum efficiency) and define more consistence behaviors of CC configurations in both UE and NW. |
| Qualcomm | Issue 1-1: New feature simultaneous dormant BWP switching Agree to the comment from MTK “Prerequisite feature groups should be 18-4 or 18-4a in RAN1 feature least”. FG 18-4 and 18-4a are “SCell dormancy indication within active time” and “SCell dormancy indication outside active time”, respectively. Agree to Recommended WF “leave technical discussion on the set of D’ values in [211]”. Issue 2-1: New per BC indication of the per-FR gap We fully support the proposal from Huawei, this is inline with our proposal from the last meeting. As we stated in the last meeting, there are also baseband constraints, especially considering measurements in FR2 which are a lot more demanding from a hardware point of view. If the signaling is done in a backwards compatible manner, the added overhead is very small. Without this capability, per FR gaps will see very limited use in real UEs. |
| Huawei | Issue 2-1: Yes. It is necessary to introduce the per-BC indicated Per-FR gap capability. UE may only needs to perform measurement on one frequency layer within the gap occasion within the certain FR even there are numerous CCs configured within the same FR, but the data transmission and other procedure shall not be interrupted in the other FR, which means the hardware resource is required to guarantee the active procedures in one FR and the measurement within gap in the other one. Thus, the resources demanded to support this feature varies from different CA combinations. Another constraint is the coupling issue pointed out during the GTW session. We don’t believe it is a feasible way to investigate the issue case by case. The related requirements/features are from different features and different releases, which may cause tremendous efforts and leads to compatibility issues. It is not possible to reopen the discussion for all requirements and features from R15 and revert the related discussion case by case if we really want to fix this issue. Most companies agreed that there is no compatibility issue by combining the original per-UE indication and the per-BC indication. And there are clear demand from UE side and operator to make full usage of the feature and enlarge the CC combinations. We didn’t see any convinced reason not to have this per-BC indication but turn to tremendous discussion about implementations design and reverting the previous requirements in a less feasible way |

  - Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1: New feature simultaneous dormant BWP switching | Agreement in RRM session: • Session chair: Agreed capability needs to be captured in the LS to RAN2/1 on UE feature list. |
| Issue 2-1: New per BC indication of the per-FR gap | 5 companies discuss this issue, 3 companies support the new per BC indication of per-FR gap, and 2 companies don’t agree to introduce this. This issue was also discussed on the GTW RRM session, no consensus was reached. Recommended WF for 2nd round discussion: Continue discussion. Identify the implementation constraints which may come from per-UE per-FR gap capability. Further capture the conclusion in the WF in the 2nd round. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on new per BC indication of the per-FR gap | CMCC |

  - Discussion on 2nd round (if applicable)

    - Open issues summary

Issue 2-1: New per BC indication of the per-FR gap

Option 1 (Huawei, R4-2101155): The per-BC indication of the per-FR gap to be introduced and the original per-UE indication to be kept.

Recommended WF:

Further discuss in 2nd round on the following aspects:

- Is there any compatibility issue by combining the original per-UE indication and the per-BC indication?

- What are the implementation constraints for per-UE per-FR gap capability?

    - Open issues

| Company | Comments |
| --- | --- |

    - Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |