3GPP TSG-RAN WG4 Meeting # 98-e	R4-2103106

# Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda item:	10.1.2

Source: 	Huawei

Title: 	TP to TR 38.921: remaining BS parameters

Document for:	Approval

- Introduction

The latest WF on on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz was approved in [1].  There are still some open issues such as spectral mask and out of band boundary.

This paper provides a TP for TR 38.921 to capture our proposal on remaining requirements.

References

[1] R4-2016905, WF on BS and UE parameters for 6.425-7.125 and 10.0-10.5 GHz, Nokia, CATT, Ericsson, ZTE, Huawei, Qualcomm

[2] R4-2100489	further discussion on BS parameters for  6.425-7.125GHz, 7.025-7.125GHz and 10.0-10.5GHz	CATT

[3] R4-2100823	Discussion on remaining Tx requirements of BS for 6425-7125MHz	CMCC

[4] R4-2101496	TP to TR 38.921: BS remaining parameters	Huawei, HiSilicon

[5] R4-2101792	Proposals of BS Parameters for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz	Nokia, Nokia Shanghai Bell

[6] R4-2101949	TP to TR 38.921 BS requirements	ZTE Corporation

[7] R4-2102156	SI on IMT parameters - Remaining BS parameters	Ericsson

TP to TR 38.921

< START OF CHANGE>

### 6.1.2	Spectral mask

Existing FR1 operating band unwanted emission mask is the same as for LTE (5MHz~20MHz CBW). For both 6.425-7.125GHz and 10.0-10.5GHz band, it is foreseen the smaller channel bandwidth such as less than 50 MHz CBW is less attractive and will not be used in reality. Hence the basic limits for OOB emission mask should be updated as below. It is agreed that it does not mean that 50 MHz will be the minimum channel bandwidth when RAN4 specified the channel bandwidth and emission requirements.

Table 6.1.2-1: Wide Area BS operating band unwanted emission limits for 6.425-7.125GHz for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -13 dBm | 1MHz |

Table 6.1.2-2: Wide Area BS operating band unwanted emission limits for 6.425-7.125GHz for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -15 dBm | 1MHz |

Table 6.1.2-3: Wide Area BS operating band unwanted emission limits for 10.0-10.5GHz for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-650foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | [-14 dBm] | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -13 dBm | 1MHz |

Table 6.1.2-4: Wide Area BS operating band unwanted emission limits for 10.0-10.5GHz for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-650foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | [-14 dBm] | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -15 dBm | 1MHz |

### 6.1.3	ACLR

According to the simulation results in clause 4.3, it is agreed to specify 38 dB ACLR for 6.425-7.125GHz and 37 dB ACLR for 10.0-10.5GHz.

For 6425-7125MHz and 10-10.5GHz, the ACLR shall be higher than the value specified in table 6.1.3-1.

Table 6.1.3-1: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 40, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 38 dB for 6425-7125MHz 37 for 10-10.5GHz |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 38 dB for 6425-7125MHz 37 for 10-10.5GHz |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

### 6.1.4	Spurious emissions

Requirements defined in TS 38.104 clause 6.6.5.2.1 and 9.7.5.2 for FR1 BS spurious emissions can be reused for both 6.425-7.125GHz and 10.0-10.5GHz. From the Rel-15 discussion, the offset ΔfOBUE for the boundary between OBUE and spurious emissions is the most demanding parameters for OTA or Hybrid limits AAS BS. We propose to further extent the boundary to consider the wide operating bandwidth and large number of antenna arrays. Based on the implementation evaluation and regulatory consideration, it is agreed to adoptΔfOBUE = 100 MHz for both 6.425-7.125GHz and 10.0-10.5GHz.

< Next CHANGE>

### 6.2.3	Blocking response

The in-band blocking requirement shall apply from FUL,low - ΔfOOB to FUL,high + ΔfOOB, excluding the downlink frequency range of the FDD operating band. It is agreed to adopt ΔfOOB = 100 MHz for both 6.425-7.125GHz and 10.0-10.5GHz. The in-band blocking levels are reused from existing FR1 requirements.

Table 6.2.3-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20,25, 30, 40, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±30 | 20 MHz DFT-s-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE: PREFSENS depends on the RAT. |  |  |  |  |

[The out-of-band blocking requirement apply from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz for low frequency bands. -15 dBm CW interfering signal is rarely present close to the 6GHz and 10 GHz operating band. For UE specification for n77 and n78, the -15 dBm CW interfering signal applies at least 200 MHz outside the operating band.

Hence it is proposed to align with UE specification, the -15 dBm CW interfering signal applies from 1 MHz to FUL,low – 200MHz and from FUL,high + 200MHz up to 12750 MHz.]

### 6.2.4	ACS

According to the simulation results in clause 4.3, it is agreed to specify 42 dB ACS for 6.425-7.125GHz and 40 dB ACLR for 10.0-10.5GHz.

ACS requirement for 6425-7125MHz and 10-10.5GHz is defined in Table 6.2.4-1.

Table 6.2.4-1: Base station ACS requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 (Note 1) | PREFSENS + 6 dB | Wide Area BS: [-52] Medium Range BS: [-47] Local Area BS: [-44] |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the BS for that bandwidth. NOTE 2: PREFSENS depends on the RAT. |  |  |

< END OF CHANGE>