3GPP TSG-RAN4 Meeting #98-e	R4-2103364

# Online, , 25th Jan 2020 – 5th Feb 2021

# Title:	Reply LS on technical feasibilities for frequency arrangements for IMT in 470 – 703 MHz band

# Response to:	LS (APT_LS200918, RP-202934, R4-2016598) on Frequency arrangements for IMT in the band 470-703MHz from APT Wireless Group

# Release:	Rel-17

# Work Item:	FS_NR_600MHz_ext

Source:	3GPP RAN WG4

To:	Asia-Pacific Telecommunity Wireless Group (AWG)

Cc:	3GPP TSG RAN

Contact Person:

Name:	Michal Szydelko, Shuang Li

Tel. Number:

E-mail Address:	michal.szydelko@huawei.com, lishuang@cbn.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	Status Report to TSG#91-e

1. Overall Description:

3GPP RAN WG4 would like to thank APT Wireless Group for the LS in RP-202934 / R4-2016598.

In response to a request from APT Wireless Group on 3GPP RAN WG4 feedback on technical feasibility of B1 and B2 frequency arrangements in 470 – 703 MHz band, including views on suggested sizes of the second duplexer from a feasibility and cost-efficient perspective, 3GPP RAN WG4 would like to inform on the progress of the discussion during RAN4#98-e meeting.

Based on the discussion the following agreements on bands plans and duplex filter options were reached during RAN4#98-e meeting, considering both option B1, B2, as well as potential alternatives based on B2:

1. Band plan

- Option B1 or B2 is further studied as baseline to respond to the request of APT/AWG.

- Other options can be studied as well, if such options are considered to better address the device performance, cost and ecosystem issues, etc.

- Companies are encouraged to study how to benefit from band n71 ecosystem with these options.

2. Duplex filter options

- Companies are encouraged to provide further analysis whether single duplexer more than 35 MHz pass band can be ruled out.

- Companies are encouraged to provide further analysis whether the following split duplexer assumptions can be used.

  - One of the duplexers supports band n71, another duplexer covers (at least) the extended frequency range for APT.

  - Other split-duplexer arrangements that facilitate support of 71/n71 (and hence the requirements for 71/n71)

- Companies are encouraged to further analyze the feasibility of duplexers w.r.t.

  - Insertion loss and isolation for different center gap and pass band sizes.

  - Coexistence with own band, band 28, DTV, RAS etc., e.g., by means of filter isolation, uplink RB restriction, or A-MPR, etc.

  - Compatibility with band n71

As background information on the progress of the discussions, the Status Report to TSG#91-e is attached for information.

RAN4 will continue technical discussion and plans to inform APT Wireless Group on its final conclusion by the September 2021, as requested.

2. Actions:

To APT Wireless Group:

ACTION: 	RAN WG4 asks AWG to take the above information into account.

3. Date of Next RAN WG4 Meetings:

RAN WG4 Meeting #98-bis-e 	12th – 20th April 2021		Online meeting

RAN WG4 Meeting #99-e		19th – 27th May 2021		Online meeting