3GPP TSG-RAN WG4 Meeting # 98-e												   R4-2103454

Electronic Meeting, Jan 25-Feb 05, 2021

Agenda item:			7.7.2.3

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [98e][215] NR_pos_RRM_Part_3

Document for:	Information

# Introduction

The documents in agenda item 7.7.2.3 contains the following main topics for discussion:

- Topic #1: gNB requirements: general aspects

- Topic #2: SRS-RSRP requirements

- Topic #3: gNB Rx-Tx time difference requirements

- Topic #4: UL RTOA requirements

# Topic #1: gNB requirements: general aspects

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100048 | ZTE Corporation | Proposal 1: Accuracy requirements don’t depend on beam configuration. Proposal 2: Further discuss how to specify beam directions used in the tests. |
| R4-2100353 | Ericsson | Proposal 1: UL RTOA measurement accuracy requirements cannot be reused from gNB Rx-Tx accuracy requirements Proposal 3: Set of proposed values [Es/Iot1; Es/Iot2] = [-13; +3] dB Proposal 4: Antenna beam configuration assumption for gNB positioning measurement accuracy follows the approach used in BS specifications: TS 38.104 and TS 38.141-1/2 (support option 1 and following candidate option 1) Proposal 5: Fix and agree on set of Ês/Iot values as input for link level simulations in RAN4#98-e for better alignment of the accuracy results. Proposal 6: Option 1 from Table 1 regarding SRS bandwidth, SCS, comb size, no. of symbols and TSRS Proposal 7.1 Calibration error should be considered as implementation dependent Proposal 7.2 Calibration for antenna reference point recommended, since measurement at connector incorporates unknown delays Proposal 7.3 All impairments impacting gNB timing measurement accuracy should be included in Proposal 7.1 and 7.2 Proposal 8.1 Impact of RF calibration error on SRS-RSRP measurement accuracy is XdB Proposal 8.2 RF calibration error differs for different gNB types, XdB for all type C measurements and YdB for all type O measurements, where Y>X Proposal 8.3 All impairments impacting SRS-RSRP measurement accuracy should be included in Proposal 8.1 and 8.2, namely X and Y |
| R4-2101729 | Ericsson | System simulation results not yet available |
| R4-2102690 | Nokia, Nokia Shanghai Bell | Proposal 1: For link simulations to determine gNB measurement accuracy for the considered timing offset and power measurement types: 1a) Select Ês/Iot = +3 dB dB as side condition for high Ês/Iot (i.e. for serving cell) in FR1 and FR2. 1b) Select Ês/Iot = -13 dB as side condition for low Ês/Iot (i.e. for neighbor cell) in FR1 and FR2. Proposal 2: Specify performance requirements for the high Ês/Iot side condition, associated with UE in serving cell, as mandatory for BS supporting a given SRS configuration, and performance requirements for the low Ês/Iot side condition, associated with UE in neighbor cell, as optional for BS for the same SRS configuration. Proposal 3: Additional SRS bandwidths as depicted in R4-2012142, i.e. 10 MHz for 2 GHz range, 50 MHz for 2 and 4 GHz ranges and 100 MHz for 40 GHz range, are added to candidate SRS configurations for deriving accuracy gNB requirements. This will enable to better cover the entire UE CBW range. |
| R4-2102779 | Huawei, HiSilicon | Proposal 1: Define the gNB accuracy requirements based on single shot measurement assumption, and requirements apply provided that the SRS transmission occasion is not dropped. Proposal 2: Define two sets of Es/Iot conditions for gNB positioning requirements at -15dB and 3dB, respectively. Proposal 3: The positioning measurement requirements apply for the same RoAoA as OTA reference sensitivity requirements for 1-O and 2-O BS. Accuracy requirements do not depend on antenna beam configuration in gNB. |

## Open issues summary

### Sub-topic 1-1: Side conditions for accuracy for all gNB measurements

Issue 1-1-1: Side conditions (Ês/Iot) for applicability of gNB accuracy

| Company | Ês/Iot1 [dB] (High Ês/Iot) |  | Ês/Iot2 [dB] (Low Ês/Iot) |  |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 | FR1 | FR2 |
| HW | 3 | 3 | -15 | -15 |
| Nokia | 3 | 3 | -13 | -13 |
| Ericsson | 3 | 3 | -13 | -13 |

- Recommended WF

  - Ês/Iot1 [dB] (High Ês/Iot) = 3 dB for FR1 and FR2

  - Ês/Iot2 [dB] (Low Ês/Iot) need further discussion.

Issue 1-1-2: Number of accuracies per set of side conditions

- Option 1: Huawei

  - Use Table 1-1-2-1 as template to form gNB SRS-RSRP accuracy requirements. The exact accuracy number and min SRS BW (BWmin) is to be checked from link level results, once RAN4 has agreed on the side conditions. Only one set of accuracy requirements is defined for each side condition, based on the min SRS BW.

Table 1-1-2-1: suggested SRS-RSRP accuracy requirements

| Accuracy (dB) | SINR condition (dB) | BW (PRB) |
| --- | --- | --- |
|  |  | >= BWmin1 |
|  |  | >= BWmin2 |

  - Use Table 1-1-2-2 as template to form gNB Rx-Tx time difference and UL-RTOA accuracy requirements. The exact accuracy number and min SRS BW (BWmin) is to be checked from link level results, once RAN4 has agreed on the side conditions. One set of accuracy requirements is defined for each combination of {SINR, SCS, SRS BW range}.

- Table 1-1-2-2: Template for gNB TOA estimation accuracy requirements

| Accuracy (Tc) | SCS (kHz) | PRB num |
| --- | --- | --- |
|  | 15/30/60/120 | BWmin-40 |
|  |  | 44-84 |
|  |  | 88-168 |
|  |  | 172-max |

- Other options not precluded

- Recommended WF

  - Collect companies’ feedback on the above proposals

### Sub-topic 1-2: Antenna/beam configuration

Issue 1-2-1: Antenna configuration in accuracy requirement

- Option 1: E///

  - Fixed antenna beams are assumed in gNB for deriving accuracy

- Option 2:

  - Proposal 1: ZTE, HW

    - Accuracy does not depend on antenna beam configuration in gNB, i.e. do not assume fixed gNB antenna beams.

  - Proposal 2: HW

    - Positioning measurement requirements apply for the same RoAoA as OTA reference sensitivity requirements for 1-O and 2-O BS

- Recommended WF

  - Collect companies’ feedback on the above proposals

### Sub-topic 1-3: SRS configuration

Issue 1-3-1: Additional/remaining parameters for suitable SRS configurations

Any subset of suitable SRS configurations declared by the manufacturer for meeting accuracy.

- Option 1: E///

Table 1-3-1-1: Option 1: Additional SRS parameters for suitable SRS configurations

| FR | Bandwidth SRS [MHz] / SCS [kHz] | SRS comb size | No. of SRS symbols | SRS resource periodicity TSRS [slots] |
| --- | --- | --- | --- | --- |
| FR1 | 5 / 15 | Com4 | 4 | 160 |
| FR1 | 20 / 15 | Com2 | 2 | 160 |
| FR1 | 20 / 30 | Com4 | 4 | 160 |
| FR1 | 20 / 30 | Com8 | 8 | 40 |
| FR2 | 50 / 60 | Com8 | 8 | 40 |
| FR2 | 50 / 120 | Com8 | 8 | 40 |
| FR2 | 200 / 60 | Com8 | 8 | 40 |
| FR2 | 400 / 120 | Com8 | 8 | 40 |

- Option 2: Nokia

Table 1-3-1-2: Option 2: Additional SRS parameters for suitable SRS configurations

| FR | Bandwidth SRS [MHz] |
| --- | --- |
| FR1 | 5 MHz |
| FR1 | 10 MHz |
| FR1 | 20 MHz |
| FR1 | 50 MHz |
| FR1 | 100 MHz |
| FR2 | 50 MHz |
| FR2 | 100 MHz |
| FR2 | 200 MHz |

- Other options not precluded

- Recommended WF:

  - Agree on table 1-3-1-3 below.

  - Collect feedback on other SRS BWs and SRS parameters

Table 1-3-1-3: SRS BW for suitable SRS configurations

| FR | SRS BW (MHz) |
| --- | --- |
| FR1 | 5 |
| FR1 | 20 |
| FR2 | 50 |
| FR2 | 200 |

### Sub-topic 1-4: RF calibration errors for gNB accuracy requirements

Issue 1-4-1: RF calibration error for timing measurement accuracy for different gNB types (1-C, 1-H, 1-O, 2-O)

- Option 1: E///

  - RF calibration error depends on gNB type

- Other options not precluded

- Recommended WF

  - Collect companies’ feedback on the above proposal

Issue 1-4-2: RF calibration error for SRS-RSRP accuracy for different gNB types (1-C, 1-H, 1-O, 2-O)

- Option 1: E///

  - RF calibration error for gNB type 1-C (X) is small than that for gNB types 1-O/2-O (Y) i.e. Y>X.

- Other options not precluded

- Recommended WF

  - Collect companies’ feedback on the above proposal

### Sub-topic 1-5: Samples for gNB accuracy requirements

Issue 1-5-1: Number of samples for gNB accuracy requirements

- Option 1: HW

  - Define the gNB accuracy requirements based on single shot measurement assumption

- Other options not precluded

- Recommended WF

  - Collect companies’ feedback on the above proposal

### Sub-topic 1-6: gNB accuracy requirements

Issue 1-6-1: Optional accuracy for low side condition

- Option 1: Nokia

  - gNB accuracy defined only for high Ês/Iot side condition (i.e. corresponding to serving cell) is mandatory for gNB supporting that measurement.

  - gNB accuracy defined for low Ês/Iot side condition (i.e. corresponding to neighbor cell) is optional even for gNB supporting that measurement.

- Other options not precluded

- Recommended WF:

  - Need further discussion.

## Companies views’ collection for 1st round

### Open issues

Issue 1-1-1: Side conditions (Ês/Iot) for applicability of gNB accuracys

| Company | Comments |
| --- | --- |
| Ericsson | Support -13dB for low Ês/Iot value side condition |
| Huawei | We can compromise to adopt -13dB as the low Es/Iot side condition |
| Nokia | We support -13 dB for low Ês/Iot for FR1 and FR2. |
| Intel | Support -13dB for low Ês/Iot value side condition |

Issue 1-1-2: Number of accuracies per set of side conditions

| Company | Comments |
| --- | --- |
| Ericsson | The actual accuracy requirements could be grouped, and SRS configs can be reduced in permutations to minimize the number of different accuracy requirements, once we have a clear picture of SRS configuration dependency. Since the side conditions for link level simulations are not agreed on and we do not have all the link results so it would be premature to conclude on number of accuracies per side condition at this time. |
| Huawei | We support option 1. Option 1 means we would have requirements for all SRS BW above a lower bound (BWmin), and as such we would not need to select a subset of SRS BWs for defining requirements. gNB can declare the supported SRS BWs, and meet the corresponding applicable requirements. The lower bound for the SRS BW is selected to ensure a reasonable accuracy performance, and can be further checked via link level simulations. For SRS-RSRP, we suggest to define one set of accuracy requirements for each Es/Iot side condition, and it is based on the min SRS BW (worst case). For gNB Rx-Tx and UL-RTOA, we suggest to define one set of accuracy requirements for each combination of {SINR, SCS, SRS BW range}. The SRS BW range is determined by the FFT size, e.g. 44-84 PRB corresponds to FFT size of 1024, 88-168 PRB corresponds to FFT size of 2048. The FFT size together with SCS will determine the resolution of the timing measurement, and further determine the achievable accuracy. |
| CATT | Fine with option 1 and the requirements need to be checked based on link level simulation. |
| Nokia | We principally agree to define accuracy requirements based on SRS BW grouping to reduce the number of requirements. The template for SRS-RSRP, based on different minimum SRS BW, can serve well with number of different SRS BW’s and settings for minimum SRS BW to be carried out after link simulations. For gNB TOA estimation accuracy, the SRS BW grouping (number of BW groups and BW range) can only be done based on link simulation results. Further, the dependency on SCS and PRB number is to be seen based on link simulation results. Then, regarding Huawei’s proposal to split BW into ranges up to 40 PRB’s, up to 84 PRB’s, up to 168 PRB’s and up to max PRB’s we would like to understand the rationale for this approach. With this approach, final link simulations would need to be done for the lower bounds of the PRB num intervals rather than for BW’s agreed in R4-2012142. |
| Qualcomm | Defining requirements for multiple bins (intervals) covering the complete range of configurable SRS BW makes sense and would be consistent with proposals being made at the UE side (for PRS). For each bin, the accuracy requirement would be determined by the smallest SRS BW in that bin. How to partition into bins can be guided by simulation results. |

1-2-1: Antenna configuration in accuracy requirement

| Company | Comments |
| --- | --- |
| Ericsson | Support option 1. As stated, the measurement accuracy does not depend on beam configuration, but rather fixed beams are assumed. The actual accuracy is dependent on the beam configuration, but it is unfeasible to assume every possible beam configuration. Therefore, our proposal is as advertised in the contribution: Antenna beam configuration assumption for gNB positioning measurement accuracy follows the approach used in BS specifications: TS 38.104 and TS 38.141-1/2 (support option 1 and following candidate option 1) |
| ZTE | The measurement accuracy shouldn’t depend on beam configurations, no fixed beam shall be assumed. In the test cases, we can further study how to configure the beam directions. For performance requirements we don’t see why fixed beams shall be assumed. |
| Huawei | Support both proposals in option 2. Option 1 is still unclear to us, e.g. what does ‘fixed beams’ mean, is there a definition for the tem in 38.104 or 38.141? What are the applicable directions (AoAs) for the accuracy requirements fi we follow option 1? |
| CATT | It has been agreed that antenna beam configuration assumption for gNB positioning measurement accuracy follows the approach used in BS specifications: TS 38.104 and TS 38.141-1/2. Further assumption is not needed. |
| Ericsson | To clarify our previous comment, with fixed beams it is assumed that the beam and its defining parameters for RoAoA do not change within the measurement period, i.e. moving beams should be neglected for deriving positioning accuracy requirements since this is not covered in TS38.104. Deriving gNB measurement accuracy requirements shall not be dependent on declared beam configurations and RoAoA assumptions for OTA receiver characteristics shall apply as per earlier agreement to follow approach used in BS specifications: TS 38.104 and TS 38.141-1/2. gNB positioning measurement accuracy shall apply provided that the SRS beam configuration does not change within the positioning measurement period |
| Nokia | We support option 1. Accuracy requirements should be derived from fixed beams as depicted in our contribution to last meeting in R4-2016157 (Proposal 4: For deriving accuracy requirements and for defining requirements for conformance testing, static gNB antenna beams with peak / main lobe directed towards the UE will be assumed for FR1 and FR2.). |
| Qualcomm | From Ericsson’s second comment it seems that this issue is about whether antenna beams can change during a measurement. Isn’t it expected that the gNB could try to measure the same SRS resource with different beams during the measurement period? It could be clarified whether the condition is that each individual measurement should be made with a single beam. To us this seems more like a question of applicability rather than definition of requirements. |

Issue 1-3-1: Additional/remaining parameters for suitable SRS configurations

| Company | Comments |
| --- | --- |
| Ericsson | We can agree on proposed table 1-3-1-3 and furthermore can agree to incorporate Nokia’s proposed additional bandwidth values (table 1-3-1-2) into our proposal option 1. Furthermore, we can agree to exclude the 400MHz bandwidth option for FR2, if needed. |
| Huawei | If we adopt the principle of option 1 in Issue 1-1-2, i.e. requirements are defined for all SRS BW above a lower bound (BWmin), we would not need to select a subset of SRS BWs for defining requirements. If a gNB declares to support 20MHz with 15k SCSW, then it meets the requirements defined for 88-168 PRB with 15k SCS. We see it difficult to select a subset of SRS BW for defining requirements, e.g. what happens if a gNB uses 40MHz for positioning? On comb size and symbol number, based on our simulation results, they do not have much impact on the accuracy performance, so we suggest to define the requirements agnostic to the comb size and symbol number. This can be done by deriving the accuracy numbers from the worst case of {comb size, symbol number} e.g. {2,2} or {4,4}. It is also difficult to select a subset of {comb size, symbol number}, e.g. if we follow option 1, there would be no requirement for {2,2} for FR2. Do we have any particular reason to exclude it? |
| Nokia | We support option 2. In our view, proposed table 1-3-1-3 is not sufficient as e.g. SRS BW of 5 MHz will not provide sufficient performance. Thus, we should target higher number of suitable SRS configurations. |
| Intel | . Is there any justification on the proper number of requirements set? In our understanding, the number of sets is up to the performance gap among the different sets. So it is better to provide some simulation results on these parameter combinations. |
| Qualcomm | All the SRS configurations for FR2 in option 1 have periodicity of 40 ms, which seems very dense in time. Would that be a typical configuration? Consider adding configurations with T_SRS = 160 ms for FR2 as well. |

Issue 1-4-1: RF calibration error for timing measurement accuracy for different gNB types (1-C, 1-H, 1-O, 2-O)

| Company | Comments |
| --- | --- |
| Ericsson | Support option 1 |
| Huawei | We are fine with option 1, provided that it does not force RAN4 to use different margins for different gNB types. We can further check the margins for different gNB types, and they can be same or different. |
| CATT | Fine with option 1. |
| Nokia | We support option 1. |
| Intel | Support Option 1. |

Issue 1-4-2: RF calibration error for SRS-RSRP accuracy for different gNB types (1-C, 1-H, 1-O, 2-O)

| Company | Comments |
| --- | --- |
| Ericsson | Support option 1 |
| Huawei | We are fine with option 1. |
| Nokia | We support option 1. |
| Intel | Support Option 1. |

Issue 1-5-1: Number of samples for gNB accuracy requirements

| Company | Comments |
| --- | --- |
| Ericsson | Since a single shot measurement assumption is unreliable due to time variance of the underlying conditions and sometimes even rendered impossible by other limiting factors such as small bandwidth, single shot measurement assumption cannot be used for accuracy definition. The number of measurement samples for which measurement accuracy requirements will be defined should also be defined and is FFS. Therefore, at this stage we cannot agree on option 1. |
| Huawei | We support option 1. RAN4 only defines min requirements and it should be based on baseline/worst case. Of course, this does not prevent a gNB to use more than one samples to derive the results. On the SRS BW, we can check via link level simulations and determine the min PRB number such that one shot measurement can work. |
| CATT | Can be checked based on the link level simulation. |
| Nokia | We do not agree to option 1. The single shot will not provide good performance, especially for fading channels. For PRS accuracy requirements, 4 samples are taken, hence it needs to be investigated if this number can be reused for SRS-P. |
| Intel | Single shot could represent the worst case. So we can support Option 1. |
| Qualcomm | Single shot means the measurement is based on all SRS resources in one slot? Please clarify. |

Issue 1-6-1: Optional accuracy for low side condition

| Company | Comments |
| --- | --- |
| Ericsson | As agreed in RAN4#97-e WF, the optionality is for SRS subsets declared by manufacturer, for which the manufacturer must satisfy the measurement accuracy requirements, i.e. for both side conditions. Optionality regarding side conditions was not discussed but positively included in the agreement. |
| Huawei | We do not agree to associate the side condition with the serving cell or neighbor cell. This has already been discussed and agreed in last meeting. - all positioning methods based on gNB measurements, including multi-RTT and UL-TDOA, require gNB to measure UE in a neighbor cell - what impacts the accuracy performance is the Es/Iot, a gNB should achieve same accuracy for a certain Es/Iot, no matter the measured UE is in the serving cell or neighbor cell On the other hand, we are open to discuss the requirements optionality regarding high and low Es/Iot. |
| Nokia | We support option 1. As mentioned in our contribution (R4-2102690), the high Ês/Iot side condition refers to serving cell reception conditions and accuracy will be considerably higher than for low Ês/Iot side condition. Then complexity level at gNB for reception of SRS-P from UE in neighbor cell is different to serving cell due to unknown TA. Thus, high Ês/Iot side condition should be specified as mandatory requirement and low Ês/Iot side condition as optional requirement. |
| Qualcomm | In our view, requirements should be defined at least for a side condition that is representative of typical deployment scenarios. If additional looser (more relaxed) side conditions (with correspondingly stricter requirements) are desired, they could be added. We think all the requirements should be mandatory, otherwise the benefit of defining requirements is questionable. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

Issue 1-1-1: Side conditions (Ês/Iot) for applicability of gNB accuracy

|  | Status summary |
| --- | --- |
| Sub-topic#1-1; issue 1-1-1 | Tentative agreements: - Ês/Iot1 [dB] (High Ês/Iot) = 3 dB for FR1 and FR2 - Ês/Iot2 [dB] (Low Ês/Iot) = -13 dB for FR1 and FR2. Recommendations for 2nd round: No further discussion is needed |

Issue 1-1-2: Number of accuracies per set of side conditions

|  | Status summary |
| --- | --- |
| Sub-topic#1-1; issue 1-1-2 | Tentative agreements: - gNB accuracy requirements shall be defined for group of SRS BWs - grouping of SRS BWs will be decided based on link simulation results - Whether gNB accuracy requirements may also be based on grouping of other SRS parameters (e.g. SCS) is FFS. - grouping of other parameters (e.g. SCS) will be decided based on link simulation results Recommendations for 2nd round: Further discuss group of SRS BW and other SRS parameters |

Issue 1-2-1: Antenna configuration in accuracy requirement

|  | Status summary |
| --- | --- |
| Sub-topic#1-2; issue 1-2-1 | Tentative agreements from GTW on 28th Jan: Tentative agreements - gNB accuracy requirements do not mandate gNB RX beam sweeping Candidate options: Option 1: - gNB accuracy requirements do not mandate gNB RX beam sweeping is captured only in the WF. Option 2: - gNB accuracy requirements do not mandate gNB RX beam sweeping is included in the accuracy side conditions. Other options not precluded Recommendations for 2nd round: Need further discussion |

Issue 1-3-1: Additional/remaining parameters for suitable SRS configurations

|  | Status summary |
| --- | --- |
| Sub-topic#1-3; issue 1-3-1 | Tentative agreements: - List of SRS BWs for which requirements will apply is FFS; SRS BWs can be grouped for defining accuracy (issue 1-1-2), - SRS BWs and SRS BW grouping can be determined based on link simulations - Other SRS parameters are FFS - Other SRS parameters (if needed) can be determined based on link simulations Recommendations for 2nd round: Further SRS BW and other SRS parameters |

Issue 1-4-1: RF calibration error for timing measurement accuracy for different gNB types (1-C, 1-H, 1-O, 2-O)

|  | Status summary |
| --- | --- |
| Sub-topic#1-4; issue 1-4-1 | Tentative agreements: - RF calibration error depends on gNB type Recommendations for 2nd round: None |

Issue 1-4-2: RF calibration error for SRS-RSRP accuracy for different gNB types (1-C, 1-H, 1-O, 2-O)

|  | Status summary |
| --- | --- |
| Sub-topic#1-4; issue 1-4-2 | Tentative agreements: - RF calibration error for gNB type 1-C (X) is small than that for gNB types 1-O/2-O (Y) i.e. Y>X. Recommendations for 2nd round: None |

Issue 1-5-1: Number of samples for gNB accuracy requirements

|  | Status summary |
| --- | --- |
| Sub-topic#1-5; issue 1-5-1 | Tentative agreements: None Candidate options: - Option 1: - Define the gNB accuracy requirements based on single shot measurement assumption - Option 2: - Define the gNB accuracy requirements based on multiple shots (Ns) - Ns is FFS - Other options not precluded Recommendations for 2nd round: Discuss further including interpretation of “shot”. |

Issue 1-6-1: Optional accuracy for low side condition

|  | Status summary |
| --- | --- |
| Sub-topic#1-6; issue 1-6-1 | Agreements from GTW on 28th Jan: Agreement - The gNB, which declares the support of positioning measurements, needs to declare the support of measurements for at least either high or low Ês/Iot side conditions Recommendations for 2nd round: None |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on gNB positioning measurement requirements | Ericsson |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: SRS-RSRP requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100450 | CATT | Not available |
| R4-2101737 | Ericsson | Proposal 1: Obtain baseline gNB SRS-RSRP measurement accuracy with link level simulation RSRP evaluation. Implementation and RF margins shall be added to the baseline accuracy. Proposal 2: gNB SRS-RSRP measurement absolute accuracy in FR1 Proposal 3: gNB SRS-RSRP measurement absolute accuracy in FR2 |
| R4-2101760 | Ericsson | DraftCR |
| R4-2102783 | Huawei, HiSilicon | Proposal 1: Use Table 1 as template to form gNB SRS-RSRP accuracy requirements. The exact accuracy number and min SRS BW (BWmin) is to be checked from link level results, once RAN4 has agreed on the side conditions. Table 1: suggested SRS-RSRP accuracy requirements Accuracy (dB); SINR condition (dB); BW (PRB). >= BWmin1. >= BWmin2. Proposal 2: RAN4 to define the accuracy requirements agnostic to comb and symbols size. Proposal 3: Re-use the UE RF calibration margin for gNB SRS-RSRP accuracy. |
| R4-2102784 | Huawei, HiSilicon | Observation 1: The performance is very dependent on SNR conditions. Observation 2: The performance difference between different comb and symbol size is small. Observation 3: The performance are quite dependent on the channel profile. Observation 4: The accuracy improves in proportion with BW in RB and the impact of SCS is small. |
| R4-2102785 | Huawei, HiSilicon | DraftCR |

### Sub-topic 2-1: SRS-RSRP requirements

Issue 2-1-1: Baseline SRS-RSRP accuracy without margin

- Option 1: E///

  - Baseline accuracy without margin are based on link simulation results

- Other options not precluded

- Recommended WF:

  - Agree on option 1.

Issue 2-1-2: RF and implementation margins for final SRS-RSRP accuracy

What margin is needed?

- Option 1: HW

  - Reuse UE RF calibration margin for SRS-RSRP accuracy

- Option 2: E///

  - Implementation and RF margins specific to SRS-RSRP

- Other options not precluded

- Recommended WF:

  - Need further discussion.

Issue 2-1-3: SRS-RSRP accuracy agnostic to comb and symbols size

- Option 1: HW

  - Define SRS-RSRP accuracy agnostic to comb and symbols size

- Other options not precluded

- Recommended WF:

  - Need further discussion.

Issue 2-1-4: SRS-RSRP accuracy defined based on which side conditions?

- Option 1: HW, E///

  - Define SRS-RSRP accuracy based on SINR and SRS BW (RBs)

- Other options not precluded

- Recommended WF:

  - Need further discussion.

### Sub-topic 2-2: CRs for SRS-RSRP

•	Directly provide comments on the cat B draft CRs, if any, in section 2.2.2.

## Companies views’ collection for 1st round

### Open issues

Issue 2-1-1: Baseline SRS-RSRP accuracy without margin

| Company | Comments |
| --- | --- |
| Ericsson | Agree with the recommended WF |
| Huawei | Support the recommended WF |
| CATT | Fine with the recommended WF. |
| Nokia | We agree to the recommended WF. |
| Intel | We agree to the recommended WF |
| Qualcomm | Agree with the recommended WF. |

Issue 2-1-2: RF and implementation margins for final SRS-RSRP accuracy

| Company | Comments |
| --- | --- |
| Ericsson | Support option 2, RF and implementation margins should be analyzed specifically for gNB and not to be reused from UE side |
| ZTE | We prefer Option 2. The implementation of gNB can be different than the UE so don’t see strong technical reasons why the UE margins shall be re-used. |
| Huawei | We are also fine with option 2, and we look forward to other companies’ views. |
| CATT | Fine with option 2. |
| Nokia | We support option 2. |
| Intel | The RF margin in gNB side can be smaller than that of UE sides. |
| Qualcomm | Regarding option 1, could HW point to the existing margins to be leveraged? Both the UE and the gNB should make their best effort to reduce calibration error. The burden should not be disproportionately placed on either side. |

Issue 2-1-3: SRS-RSRP accuracy agnostic to comb and symbols size

| Company | Comments |
| --- | --- |
| Ericsson | If 2-1-1 option 1 is agreed, link level simulations shall be carried out with the agreed Ês/Iot side conditions, to check if dependency can be neglected or is non-existent. Ranges of values with same range of measurement accuracy should of course be summarized for the measurement accuracy requirements table for simplicity. If all comb size and symbol size values lead to the same accuracy result, they can be considered negligible for requirement definition i.e. same accuracy can be defined for all comb size and symbol size values in that case. |
| Huawei | We are fine to further study. Option 1 is based on our simulation results, and we can further discuss when more results are available. |
| CATT | Further study based on the link level simulation. |
| Nokia | We cannot agree to option 1 at this point in time. Dependency on comb and symbol size should be further investigated for both Ês/Iot side conditions, before a conclusion is drawn. |
| Intel | This is similar as PRS based requirements in principle. |
| Qualcomm | We support the objective of making the requirements agnostic to comb size. But the requirements cannot be agnostic to both comb size and number of symbols. It doesn’t make sense. Maybe we are misunderstanding the proposal in option 1. Further clarification is welcome. |

Issue 2-1-4: SRS-RSRP accuracy defined based on which side conditions?

| Company | Comments |
| --- | --- |
| Ericsson | Support option 1, but other dependencies shall not be excluded and are FFS. |
| Huawei | We support option 1. |
| CATT | Fine with option 1. |
| Nokia | We cannot agree to option 1 at this point in time. We also observe a dependency on the channel model and the dependency on SRS configuration parameters, as being discussed in Issue 2-1-3, needs further investigation, before a conclusion is drawn. |
| Intel | These two parameters can be agreed firstly. The others can be FFS. |
| Qualcomm | The parameters in option 1 should be considered in the definition of accuracy requirements. Number of intra-slot repetitions (of the comb pattern) may also be taken into account. This is related to issue 1-5-1 (“single shot”). |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2101760 | Ericsson: Note 6: Measurement accuracy dependencies other than SRS Ês/Iot and SRS bandwidth are still FFS |
|  | Huawei: need to wait for technical conclusions. |
|  | Nokia: The proposed template cannot be endorsed, as inclusion of SCS=60 kHz for FR1 should be discussed and the dependency of SRS symbols / comb-size needs to be further investigated. Table numbering is wrong. |
| R4-2102785 | Ericsson: side condition values for Ês/Iot are not agreed upon yet and have to be defined. |
|  | Huawei: need to wait for technical conclusions. |
|  | Nokia: The proposed template cannot be endorsed, as Ês/Iot side conditions are not agreed. Rationale for splitting into these SRS BW groups should be provided. On the conditions, the case of BS type 1-O is missing. |

## Summary for 1st round

### Open issues

Issue 2-1-1: Baseline SRS-RSRP accuracy without margin

|  | Status summary |
| --- | --- |
| Sub-topic#2-1; issue 2-1-1 | Tentative agreements: - Baseline accuracy without margin are based on link simulation results Recommendations for 2nd round: None |

Issue 2-1-2: RF and implementation margins for final SRS-RSRP accuracy

|  | Status summary |
| --- | --- |
| Sub-topic#2-1; issue 2-1-2 | Tentative agreements: - Implementation and RF margins specific to SRS-RSRP. Values are FFS. Recommendations for 2nd round: None |

Issue 2-1-3: SRS-RSRP accuracy agnostic to comb and symbols size

|  | Status summary |
| --- | --- |
| Sub-topic#2-1; issue 2-1-3 | Tentative agreements: - FFS whether SRS-RSRP accuracy is agnostic to or depends on comb and symbols size - Decision will be based on link simulation results Recommendations for 2nd round: None |

Issue 2-1-4: SRS-RSRP accuracy defined based on which side conditions?

|  | Status summary |
| --- | --- |
| Sub-topic#2-1; issue 2-1-4 | Tentative agreements: - Agreements related to side conditions: - Agreement on Ês/Iot1 and Ês/Iot2 (Issues 1-1-1 and 1-6-1) - Agreement on SRS BW grouping and SRS BWs (issues 1-1-2, 1-3-1 and WF in R4-2012140) - Other SRS parameters as part of side conditions are FFS Recommendations for 2nd round: Need further discussion for other SRS parameters |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2101760 | Postponed |
| R4-2102785 | Postponed |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: gNB Rx-Tx time difference requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100451 | CATT | Proposal 1: The gNB antenna beam configuration is not needed in the requirement definition. Proposal 2: When gNB Rx-Tx time difference measurement requirements are defined, except the simulation error of measurement, at least two times of calibration error is needed as the margin. |
| R4-2101761 | Ericsson | Proposal 1: gNB Rx-Tx time difference measurement absolute accuracy in FR1 Proposal 2: gNB Rx-Tx time difference measurement absolute accuracy in FR2 Proposal 3: Fix and agree on set of Ês/Iot values as input for link level simulations in RAN4#98-e for better alignment of the accuracy results. |
| R4-2101790 | Ericsson | Draft CR |
| R4-2102691 | Nokia, Nokia Shanghai Bell | Observation 1: The gNB Rx-Tx time difference accuracy for all SRS configurations depends majorly on the radio channel type, SRS bandwidth and Ês/Iot ratio. In our view the investigated range for Ês/Iot is sufficient for specifying performance requirements. We propose to select Ês/Iot = +3 dB for the high Ês/Iot ratio and Ês/Iot = -13 dB for the low Ês/Iot ratio. Following proposal for agreement is made: Proposal 1: For link simulations to determine gNB Rx-Tx time difference accuracy at gNB: 1a) Select Ês/Iot = +3 dB dB as side condition for high Ês/Iot (i.e. for serving cell) in FR1 and FR2. 1b) Select Ês/Iot = -13 dB as side condition for low Ês/Iot (i.e. for neighbor cell) in FR1 and FR2. Regarding time shifts defined in [2], e.g. 0 for UE in serving cell and 3 us for UE in neighbor cell, respectively, for the synchronous case, our understanding is that no additional propagation delay is configured on top and this time shift is fixed throughout the simulation. Hence regarding modelling of transmitter chain and radio channel, i.e. to keep impact of modelling differences low, it is proposed to agree few assumptions on sampling rates / FFT sizes for them or at least require a description as part of the reported simulation assumptions. Following proposal for agreement is made: Proposal 2: No additional propagation delay is configured on top of the defined time shifts in R4-2012142. Some assumptions on sampling rates / FFT sizes used in the transmitter chain and radio channel should be defined or at least be described as part of the reported simulation assumptions to reduce impact of modelling differences. |
| R4-2102780 | Huawei, HiSilicon | Proposal 1: Use Table 2 as template to form gNB TOA estimation accuracy requirements. The exact accuracy number and min SRS BW (BWmin) is to be checked from link level results, once RAN4 has agreed on the side conditions. Table 2: Template for gNB TOA estimation accuracy requirements Accuracy (Tc); SCS (kHz); PRB num. 15/30/60/120; BWmin-40. 44-84. 88-168. 172-max. Proposal 2: RAN4 to define the accuracy requirements agnostic to comb and symbols size. Proposal 3: Use 8Tc as the group delay calibration margin for gNB Rx-Tx and UL-RTOA accuracy. Proposal 4: Captured in the specification the propagation channel models based on which the accuracy requirements are derived. |
| R4-2102781 | Huawei, HiSilicon | Observation 1: The performance is very dependent on SNR conditions. Observation 2: The performance difference between different comb and symbol size is mainly seen at low SNR and small BW. Observation 3: The performance are quite dependent on the channel profile. Observation 4: The accuracy improves in proportion with BW in Hz due to better resolution. |
| R4-2102782 | Huawei, HiSilicon | Draft CR |

### Sub-topic 3-1: gNB Rx-Tx requirements

Issue 3-1-1: Baseline gNB Rx-Tx accuracy without margin

- Option 1: E///

  - Baseline accuracy without margin are based on link simulation results

- Other options not precluded

- Recommended WF:

  - Agree on option 1.

Issue 3-1-2: RF and implementation margins for final gNB Rx-Tx accuracy

What margin is needed?

- Option 1: CATT

  - 2 times calibration error

- Option 2: HW

  - group delay calibration margin = 8 Tc

- Other options not precluded

- Recommended WF:

  - Need further discussion.

Issue 3-1-3: gNB Rx-Tx accuracy agnostic to comb and symbols size

- Option 1: HW

  - Define gNB Rx-Tx accuracy agnostic to comb and symbols size

- Other options not precluded

- Recommended WF:

  - Need further discussion.

Issue 3-1-4: gNB Rx-Tx accuracy defined based on which side conditions?

- Option 1: E///

  - Define gNB Rx-Tx accuracy based on SINR and SRS BW

- Option 2: HW

  - Define gNB Rx-Tx accuracy based on SINR, SRS SCS and SRS BW

- Other options not precluded

- Recommended WF:

  - Need further discussion.

Issue 3-1-5: Applicable propagation channel for gNB Rx-Tx accuracy

- Option 1: HW

  - Specify propagation channel model in spec for which gNB Rx-Tx accuracy shall apply

- Other options not precluded

- Recommended WF:

  - Need further discussion.

### Sub-topic 3-2: CRs for gNB Rx-Tx

•	Directly provide comments on the cat B draft CRs, if any, in section 3.2.2.

## Companies views’ collection for 1st round

### Open issues

Issue 3-1-1: Baseline gNB Rx-Tx accuracy without margin

| Company | Comments |
| --- | --- |
| Ericsson | Agree with the recommended WF |
| Huawei | Support the recommended WF |
| CATT | Fine with the recommended WF. |
| Nokia | We agree that baseline performance can be evaluated without margin in order to determine dependency on BW, SCS, Ês/Iot, SRS configuration parameters and identify the final SRS configurations for deriving accuracy requirements. As we point out in our contribution, further link simulation assumptions should be aligned or at least reported such as simulation length, modelling of transmitter and radio channel in terms of sampling rates / FFT sizes and number of SRS samples per measurement. Hence we propose to add Option 1a: Baseline accuracy without margin are based on link simulation results. Simulation assumptions on simulation length, modelling of transmitter and radio channel in terms of sampling rates / FFT sizes and number of SRS samples per measurement shall be described by each proponent. |
| Intel | Fine with the recommended WF. |
| Qualcomm | Agree with the recommended WF. |

Issue 3-1-2: RF and implementation margins for final gNB Rx-Tx accuracy

| Company | Comments |
| --- | --- |
| Ericsson | Group delay calibration margin is implementation dependent and varies across frequency range, CBW, antenna topology and more factors. Therefore, these factors leading to a group delay calibration error difference shall be analyzed. |
| Huawei | We are fine to further study the group delay calibration error, and we look forward to other companies’ views. |
| CATT | Option 1 in principle and the value of calibration error can be further studied. |
| Nokia | We can neither agree to option 1 nor option 2. The group delay calibration margin is implementation dependent and depends as well on the final SRS configurations for deriving accuracy requirements. |
| Intel | Same comments for Issue 2-1-2 |
| Qualcomm | Needs to be studied and analyzed further. |

Issue 3-1-3: gNB Rx-Tx accuracy agnostic to comb and symbols size

| Company | Comments |
| --- | --- |
| Ericsson | If 3-1-1 option 1 is agreed, link level simulations shall be carried out with the agreed Ês/Iot side conditions, to check if dependency can be neglected or is non-existent. Ranges of values with same range of measurement accuracy should of course be summarized for the measurement accuracy requirements table for simplicity. If all comb size and symbol size values lead to the same accuracy result, they can be considered negligible for requirement definition. |
| Huawei | We are fine to further study. Option 1 is based on our simulation results, and we can further discuss when more results are available. |
| CATT | Further study based on the link level simulation. |
| Nokia | Regarding option 1, as commented above for general part, we cannot yet conclude, so we don’t support it. Further investigation is needed to identify, if for some SRS configurations there is a dependency. |
| Qualcomm | Same comment as for issue 2-1-3. |

Issue 3-1-4: gNB Rx-Tx accuracy defined based on which side conditions?

| Company | Comments |
| --- | --- |
| Ericsson | If there are link level simulation results which concur on that there can be different SRS BW groups which need different measurement accuracy values for different SRS SCS settings, we can reevaluate. At the moment, only the grouping into different SRS BW groups is clearly needed. Support option1, other dependencies shall not be excluded and are FFS. |
| Huawei | We support option 2. gNB Rx-Tx is timing measurement, and the resolution of the TOA estimation is determined by PRB number of the SCS. The resolution will further determine the achievable accuracy, e.g. if the resolution is 32 Tc, then error of one sample in the TOA estimation corresponds to an inaccuracy of 32 Tc. Therefore, unlike SRS-RSRP, the SCS has to be included. |
| CATT | Fine with SINR and SRS BW. Dependency of SCS can be further study based on the link level simulation. |
| Nokia | As for issue 3-1-3, we can determine the dependency on Ês/Iot, BW and SCS upon further investigation. |
| Qualcomm | The parameters in option 2 should be considered in the definition of accuracy requirements. Number of intra-slot repetitions (of the comb pattern) may also be taken into account. This is related to issue 1-5-1 (“single shot”). |

Issue 3-1-5: Applicable propagation channel for gNB Rx-Tx accuracy

| Company | Comments |
| --- | --- |
| Ericsson | Measurement accuracy requirement should be a per hardware unit specified value (optionality as discussed and agreed, declaration by manufacturer) and not be dependent by external factors and different deployment environments. |
| ZTE | We don’t agree with Option 1. Same view as Ericsson and we don’t feel it realistic to specify propagation models in the spec and to set requirements according to them. The testability will also be an issue. |
| Huawei | We support option 1. The same issue is being discussed also for the UE side. The technical reason is that different propagation channel models have different power-delay profile, and it causes different challenges in TOA estimation, which is to find the timing of the first path. The performance difference in different propagation channel models is clearly observed in our simulation results, and we need to take in into account in the requirements. |
| CATT | Same as UE side, the accuracy requirements are defined considering fading channel but can capture the channel mode which is used for deriving accuracy requirements in the specification. |
| Nokia | Regarding option 1, we note that the proposal in R4-2102780 refers to several propagation channel models, to be captured in the specification, rather than one. In fact, we propose to align to methodology taken for DL, where the discussion is about whether to include or exclude TDL-C channel for applicability requirements. |
| Qualcomm | We don’t agree with Ericson’s comment. Accuracy is not impacted by external factors, e.g. propagation conditions? At the very least, for informative purposes, option 1 should be considered. We would recommend defining accuracy requirements (at least) for AWGN, at least for the timing measurements (e.g. gNB Rx-Tx time difference). |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2101790 | Ericsson: Note 6: Measurement accuracy dependencies other than SRS Ês/Iot and SRS bandwidth are still FFS |
|  | Huawei: need to wait for technical conclusions. |
|  | Nokia: The proposed template cannot be endorsed, as inclusion of SCS=60 kHz for FR1 should be discussed and the dependency of SRS symbols / comb-size needs to be further investigated. Table numbering in text is wrong. |
| R4-2102782 | Ericsson: side condition values for Ês/Iot are not agreed upon yet and have to be defined. |
|  | Huawei: need to wait for technical conclusions. |
|  | Nokia: The proposed template cannot be endorsed, as Ês/Iot side conditions are not agreed. Rationale for splitting into these SRS BW groups should be provided. On the conditions, the case of BS type 1-O is missing. |

## Summary for 1st round

### Open issues

Issue 3-1-1: Baseline gNB Rx-Tx accuracy without margin

|  | Status summary |
| --- | --- |
| Sub-topic#3-1, Issue 3-1-1 | Tentative agreements: - Baseline accuracy without margin are based on link simulation results Recommendations for 2nd round: None |

Issue 3-1-2: RF and implementation margins for final gNB Rx-Tx accuracy

|  | Status summary |
| --- | --- |
| Sub-topic#3-1; issue 3-1-2 | Tentative agreements: None Candidate options: - Option 1: - 2 times calibration error - Option 2: - group delay calibration margin = 8 Tc - Option 3: - Depends on frequency range, SRS configuration and implementation (e.g. antenna) - Other options not precluded Recommendations for 2nd round: Discuss further |

Issue 3-1-3: gNB Rx-Tx accuracy agnostic to comb and symbols size

|  | Status summary |
| --- | --- |
| Sub-topic#3-1; issue 3-1-3 | Tentative agreements: - FFS whether gNB Rx-Tx accuracy is agnostic to or depends on comb and symbols size - Decision will be based on link simulation results Recommendations for 2nd round: None |

Issue 3-1-4: gNB Rx-Tx accuracy defined based on which side conditions?

|  | Status summary |
| --- | --- |
| Sub-topic#3-1; issue 3-1-4 | Tentative agreements: - Agreements related to side conditions: - Agreement on Ês/Iot1 and Ês/Iot2 (Issues 1-1-1 and 1-6-1) - Agreement on SRS BW grouping and SRS BWs (issues 1-1-2, 1-3-1 and WF in R4-2012140) - Other SRS parameters as part of side conditions are FFS Recommendations for 2nd round: Need further discussion for other SRS parameters |

Issue 3-1-5: Applicable propagation channel for gNB Rx-Tx accuracy

|  | Status summary |
| --- | --- |
| Sub-topic#3-1; issue 3-1-5 | Tentative agreements: None Candidate options: - Option 1: - Specify propagation channel model in specification for which gNB Rx-Tx accuracy shall apply - Applicable channel model(s) (if specified) is FFS - Option 2: - gNB Rx-Tx accuracy are defined for AWGN - Other options not precluded Recommendations for 2nd round: Need further discussion |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2101790 | Postponed |
| R4-2102782 | Postponed |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4: UL RTOA requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100452 | CATT | Not available |
| R4-2100453 | CATT | Link simulation result not available |
| R4-2101801 | Ericsson | Observation 1: The implementation of a UL-RTOA positioning method vastly differs from the gNB Rx-Tx method. Observation 2: The positioning techniques using UL-RTOA or gNB-RxTx respectively have different closest common timing references Observation 3: The closest common timing reference for a positioning technique using UL-RTOA is dependent on many factors which can vary in a deployment used for positioning itself Observation 4: In any case, it can be assumed that the closest common timing reference for multiple gNBs used for UL-RTOA based positioning is always further away from the antenna reference point than for gNB-RxTx based positioning Observation 5: Actual positioning accuracy using UL-RTOA measurement method is unfeasible to be tested Observation 6: While the agreed optionality for gNB measurement accuracy requirements still should be valid for UL-RTOA measurements, the resulting impacts on positioning with an actual multi-vendor deployment using UL-RTOA measurement method would be unfeasible to describe since accuracy is described on a as declared by manufacturer basis Proposal 1: UL RTOA measurement accuracy requirements cannot be reused from gNB Rx-Tx accuracy requirements |
| R4-2102692 | Nokia, Nokia Shanghai Bell | Proposal 1: For gNB supporting UL RTOA, minimum accuracy requirements will be specified for high Ês/Iot side condition referring to UE in serving cell and are mandatory. Minimum accuracy requirements will be specified also for low Ês/Iot side condition referring to UE in neighbor cell and are optional. |
| R4-2102786 | Huawei, HiSilicon, CMCC | Observation 1: There is no issue in re-using the gNB Rx-Tx time difference requirements for UL-RTOA. Observation 2: gNB measurement requirements are necessary to guarantee the minimum performance of UL-based positioning techniques and UL-and-DL-based positioning techniques. Observation 3: Not defining requirements for a measurement type means the measurement performance cannot be tested even the measurement is implemented. Observation 4: UL-based positioning is an important use case, and it will not be supported by RAN4 requirements if RAN4 only defines gNB requirements for Rx-Tx time difference and SRS-RSRP. Proposal: RAN4 to define gNB measurement accuracy requirements for UL-RTOA in the Perf part of the WI. The requirements for gNB Rx-Tx time difference are re-used. |
| R4-2102787 | Huawei, HiSilicon |  |

### Sub-topic 4-1: UL RTOA requirements

Issue 4-1-1: Reuse of gNB-Rx-Tx time difference accuracy for UL RTOA accuracy

- Can gNB-Rx-Tx time difference accuracy be reused for defining UL RTOA accuracy?

  - Option 1: HW, Nokia

    - Yes

  - Option 2: E///

    - No

- Recommended WF:

  - Need further discussion

Issue 4-1-2: Side conditions for defining UL RTOA accuracy

In issue 4-1-1, if option 1 is agreeable, then side conditions under which UL RTOA accuracy is defined

  - Option 1: Nokia

    - UL RTOA accuracy defined only for high Ês/Iot side condition (i.e. corresponding to serving cell) is mandatory for gNB supporting UL RTOA.

    - UL RTOA accuracy defined for low Ês/Iot side condition (i.e. corresponding to neighbor cell) is optional even for gNB supporting UL RTOA.

  - Option 2: HW

    - UL RTOA accuracies defined for both high and low Ês/Iot side conditions (like for gNB Rx-Tx) are mandatory.

  - Other options not precluded

- Recommended WF:

  - Need further discussion.

### Sub-topic 4-2: CRs for UL RTOA

•	Directly provide comments on the cat B draft CRs, if any, in section 4.2.2.

## Companies views’ collection for 1st round

### Open issues

4-1-1: Reuse of gNB-Rx-Tx time difference accuracy for UL RTOA accuracy

| Company | Comments |
| --- | --- |
| Ericsson | Support option 2 |
| Huawei | We support option 1. In our understanding, the analysis in Ericsson paper R4-2101801 is mixing the UL-TDOA positioning accuracy and gNB UL-RTOA measurement accuracy. In our view, UL-RTOA is a single node measurement, the Rx time is measured from SRS, and the reference time is determined by the gNB based on LMF configuration. The measurement has nothing to do with other gNBs or the synchronization with other gNBs, and there is no "common timing reference" in the UL-RTOA measurement. How to calibrate the timing among different gNBs is up to NW implementation. What RAN4 is defining is the gNB measurement accuracy requirements, and the measurement in UL-RTOA is the SRS receive timing. This is same as gNB Rx-Tx measurement, where the accuracy is determined by SRS receive timing, so we suggest to re-use the gNB Rx-Tx requirements for UL-RTOA. To address Ericsson’s concern, we suggest to clarify that the reference time in the ideal UL-RTOA is determined by gNB’s own timing based on LMF configuration. |
| CATT | Option 1. |
| Nokia | We do not support option 1, as a reuse of the gNB Rx-Tx time difference accuracy was not discussed in our contribution. We agree with Ericsson’s observation in R4-2101801, that neither for high Ês/Iot side condition, nor for low Ês/Iot side condition a reuse of the accuracy requirements is possible, as the accuracy of the measurement for UL RTOA depends in both cases on external factors such as gNB timing difference versus a reference time, set by another network entity (LMF), and different gNB implementations, which is not the case for gNB Rx-Tx time difference being derived in the same network entity. Thus, we support option 2, and hence not to define UL RTOA accuracy requirements for gNB. |
| Intel | If the SINR side condition is same, Option 1 is fine. It seems up to the issue 4-1-2 |

Issue 4-1-2: Side conditions for defining UL RTOA accuracy

| Company | Comments |
| --- | --- |
| Huawei | Same comment as Issue 1-6-1. |
| Nokia | We withdraw option 1. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2102787 | Ericsson: the definition of UL RTOA measurement accuracy requirements is not agreed |
|  | Huawei: need to wait for technical conclusions. |
|  | Nokia: We are against introducing UL RTOA accuracy requirements. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

4-1-1: Reuse of gNB-Rx-Tx time difference accuracy for UL RTOA accuracy

|  | Status summary |
| --- | --- |
| Sub-topic#4-1; issue 4-1-1 | Agreements from GTW on 28th Jan: Agreement - Further identify how to derive reference time in the ideal UL-RTOA (e.g. whether it is determined by gNB’s own timing based on LMF configuration). - If no consensus reached in the 2nd round, then further send LS to RAN1 and RAN3 to clarify the procedure. Recommendations for 2nd round: Further discuss how reference time is derived in the ideal UL-RTOA |

4-1-2: Side conditions for defining UL RTOA accuracy

|  | Status summary |
| --- | --- |
| Sub-topic#4-1; issue 4-1-2 | Tentative agreements: - If UL RTOA is to be defined then follow agreements on side conditions under issue 1-1-1 and issue 1-6-1. Recommendations for 2nd round: None |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2102787 | Postponed |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |