3GPP TSG-RAN WG4 Meeting # 98-e 	R4-21xxxxx

# Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda item:			6.2.2

Source:	Moderator (Huawei)

Title:	Email discussion summary for [98e][226] NB_IOTenh3_RRM

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Discuss on the submitted CR and provide comments.

- 2nd round: Revise the CR and keep discussion if needed.

Topic #1: Core requirements maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

There are four CRs submitted for core requirements maintenance. Companies please provide comments in 1.1.1 directly.

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2101654 | Ericsson: This change is not necessary since this is already captured. In clause 4.6.2.1 and 4.6.2.3, state that UE may perform NRSRP and NRSRQ on anchor-carrier, or NRSRP on non-anchor carrier under certain conditions. Later in same section, NRSRP and NSRQ are defined using 2 and 4 measurements respectively. “The UE shall filter the NRSRP and NRSRQ measurements of the NB-IoT serving cell using at least 2 measurements.” “The UE shall filter the NRSRP and NRSRQ measurements of the serving NB-IoT cell using at least 4 measurements. “ In our understanding, these sentences apply to measurements performed on both anchor- and non-anchor. There is no need to explicitly state it for non-anchor carrier. Additional comments: In our view, the text is clear as it is. It is already specified that only NRSRP can be performed on non-anchor, and the cited text states the number of measurements used in the samples. If UE is operating in non-anchor, then it should be clear that NRSRP is filtered using 2 measurements since there is no NRSRQ. We have a similar interpretation also about the corresponding enhanced coverage requirements. |
|  | Huawei: The measurement behavior on anchor and non-anchor carrier is clear that UE perform NRSRP and NRSRQ on anchor carrier and only NRSRP on non-anchor carrier. And it is also obvious that UE will filter NRSRP and NRSRQ on anchor carrier only and filter NRSRP on non-anchor carrier only. But as commented by Ericsson, it is stated that “The UE shall filter the NRSRP and NRSRQ measurements of the NB-IoT serving cell using at least 2 measurements.” There could be ambiguity for UE performing measurement on anchor carrier and non-anchor carrier (NRSRP1 and NRSRQ 1 on anchor carrier and NRSRP2 on non-anchor carrier). Following the statement which is not updated for non-anchor carrier measurement, the UE’s behavior is not clear. One interpretation is UE shall using the measurement only from anchor carrier (abandon the measurement form non-anchor carrier and filter NRSRP and NRSRQ); another interpretation is that UE is allowed to filter the measurement from anchor and non-anchor carrier, then UE shall only filter NRSRP instead of NRSRP and NRSRQ. Further comment: To QC’s comment: There has been tremendous discussion about whether UE is allowed to filter the samples from anchor and non-anchor carrier at the end of the core part stage. The inequation in QC’s comments is just the conditions for UE to filter the samples from different carriers in normal coverage. And UE could just filter the samples from different carries differently in enhanced coverage without further validation. So the answer to the first question “whether it is allowed to do so” is yes. Therefore, without necessary clarifications, it will be interpreted as that UE is not allowed to do so when measurement from different carriers and UE may just filter the samples separately. It is not aligned with our previous agreement. Se we believe the clarification is needed. |
|  | Qualcomm : The reason for change in the CR is confusing. It says “the measurements for filtering may be obtained from both anchor carrier and non anchor carrier.” This could be interpreted as meaning that the UE would be allowed to filter together measurements from the anchor carrier and the non-anchor carrier. We are not sure if that is the intention but, anyway, we don’t think that would be allowed. Our understanding is that the UE is required to filter NRSRP measurements (separately) from the anchor carrier and the non-anchor carrier. This is clear from TS 36.133 v16.8.0 clause 4.6.2.1. “When UE measures the NRSRP on non-anchor carrier, UE shall compare the measurements from anchor carrier and non-anchor carrier at least once every one hour by the following inequality: ¦ NRSRPanchor – (NRSRPnon-anchor - nrs-PowerOffsetNonAnchor) ¦ ≤ 10 dB where NRSRPanchor is the NRSRP measurement on anchor carrier and NRSRPnon-anchor is the NRSRP measurement on non-anchor carrier. The measurement for comparison shall use at least 2 measurements for filtering.” We don’t see that any further clarification is needed. |
| R4-2101655 (Cat A CR of R4-2101654) | Ericsson: same comments as for R4-2101654. |
| R4-2102235 | Huawei: It seems that the CR is not based on the latest spec. The issue has already been fixed in g80 version. |
|  | Ericsson: This CR can be noted since the change was already agreed. |
|  | Qualcomm: Same comment as Huawei. |
| R4-2102234 (Cat A CR of R4-2102235) |  |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | N/A |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
|  | N/A |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2101654 | to be revised |
| R4-2102235 | not pursued |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |