3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2103488

# Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1566 | rev | 1 | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the time for Scell activation and CSI-report in R15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2021-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to TS38.133 core requirement, the delay requirement for activation time (Tactivation_time ) in A.5.5.3.1.2 and A.7.5.3.1.2 is incorrect. - The CSI reporting time (TCSI_Reporting) is incorrect because it does not consider the time of uncertainty in acquiring the first available downlink CSI reference resource and the UE processing time for CSI reporting |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct TSMTC_SCell to TFirstSSB in A.5.5.3.1.2 and A.7.5.3.1.2. - Remove the value of TCSI_Reporting and add the time of the uncertainty in acquiring the first available downlink CSI reference resource and the UE processing time for CSI reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The result of the testing may be wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.5.3.1.1, A.5.5.3.1.2, A.6.5.3.1.1, A.7.5.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>>>Start of Change 1<<<<<

A.4.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.4.5.3.1.1-1 below. The test parameters are given in Tables A.4.5.3.1.1-2 and cell-specific parameters in A.4.5.3.1.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, E-UTRA has one cell, NR has two cells. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on E-UTRA and Cell 2 (PSCell) on NR, but is not aware of Cell 3 (SCell) on NR. The UE is monitoring the PCell and PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 3) becomes configured on NR. The UE now starts monitoring the SCell. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2. The UE shall be able to report valid CSI in PSCell for the activated SCell at latest in slot m+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PSCell in slot (m+k) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PSCell interruption due to activation of SCell shall occur in the slot m+1+THARQNR slot length to slot m+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in section 8.2. Any E-UTRA PCell interruption due to activation of SCell shall occur in the subframe m1+1+THARQEUTRA slot length to subframe m2+1+THARQ+3ms+TXEUTRA slot length+Ninterruption, where m1 and m2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot m, and Ninterruption is the interruption length given in TS 36.133 [14] section 7.32.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot n+THARQ+3msNR slot length, as defined in clause 8.3. The starting point of any PSCell interruption due to the deactivation shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3. The starting point of any E-UTRA PCell interruption due to the deactivation shall occur in the subframe n1+1+THARQEUTRA subframe length to subframe n2+1+THARQ+3msEUTRA subframe length, where n1 and n2 are the index of the first and last subframe of E-UTRA PCell which overlaps with slot n.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PSCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CSI reporting for SCell is discontinued.

Table A.4.5.3.1.1-1: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.4.5.3.1.1-2: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | One E-UTRAN radio channel (1) and two NR radio channel (2,3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on E-UTRAN RF channel number 1. As specified in clause A.3.7.2.1 |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on NR RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every four slots |
| Cell-individual offset for cells on E-UTRA RF channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell3 timing offset to cell2 | s | 0 |  |
| Time alignment error between cell3 and cell2 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting | ms | - | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k | ms |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A. 4.5.3.1.1-3: Cell specific test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  |  | Unit | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| SSB ARFCN |  |  |  | freq1 |  |  | freq2 |  |  |
| Duplex mode | Config 1,4 |  |  | FDD |  |  |  |  |  |
|  | Config 2,3,5,6 |  |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1,4 |  |  | Not Applicable |  |  |  |  |  |
|  | Config 2,5 |  |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3,6 |  |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,4 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 2,5 |  |  | 10: NRB,c = 52 |  |  |  |  |  |
|  | Config 3,6 |  |  | 40: NRB,c = 106 |  |  |  |  |  |
| DL initial BWP configuration | Config 1, 2, 3, 4, 5, 6 |  |  | DLBWP.0.1 |  |  |  |  |  |
| DL dedicated BWP configuration | Config 1, 2, 3, 4, 5, 6 |  |  | DLBWP.1.1 |  |  |  |  |  |
| UL initial BWP configuration | Config 1, 2, 3, 4, 5, 6 |  |  | ULBWP.0.1 |  |  |  |  |  |
| UL dedicated BWP configuration | Config 1, 2, 3, 4, 5, 6 |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1,4 |  |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  | Config 2,5 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  | Config 3,6 |  |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel | Config 1,4 |  |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  | Config 2,5 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  | Config 3,6 |  |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel | Config 1,4 |  |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  | Config 2,5 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  | Config 3,6 |  |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| TRS configuration | Config 1,4 |  |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  | Config 2,5 |  |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  | Config 3,6 |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration | Config 1,2,4,5 |  |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3,6 |  |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4,5 |  | kHz | 15 kHz |  |  |  |  |  |
|  | Config 3,6 |  |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104 |  |  |  |  |  |
| Note2 |  | Config 1,2,4,5 | dBm/SCS | -104 |  |  |  |  |  |
|  |  | Config 3,6 |  | -101 |  |  |  |  |  |
|  |  |  | dB | 17 |  |  |  |  |  |
|  |  |  | dB | 17 |  |  |  |  |  |
| SS-RSRPNote3 |  | Config 1,2,4,5 | dBm/SCS | -87 |  |  |  |  |  |
|  |  | Config 3,6 |  | -84 |  |  |  |  |  |
| SCH_RP Note 3 |  |  | dBm/15 kHz | -87 |  |  |  |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2.] |  |  |  |  |  |  |  |  |  |

>>>>>End of Change 1<<<<<

>>>>>Start of Change 2<<<<<

A.5.5.3.1.2	Test Requirements

The test requirements defined in clause A.4.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB + 5ms as defined in clause 8.3.

>>>>>End of Change 2<<<<<

>>>>>Start of Change 3<<<<<

A.6.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.1.1-1 below. The test parameters are given in Tables A.6.5.3.1.1-2 and cell-specific parameters in A.6.5.3.1.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in section 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.3.1.1-1: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.3.1.1-2: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every four slots |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | ms | - | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.1.1-3: Cell specific test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  |  |  | Unit | T1 |  |  | T2 |  | T3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  | Not applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.1.2 |  |  |  |  |  |  |  |
| BWchannel |  | Config 1,2 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| Initial BWP configuration |  |  |  |  | DLBWP.0.2 |  |  |  |  |  |  |  |
| TCI state |  |  |  |  | TCI.State.0 |  |  |  |  |  |  |  |
| TRS Configuration |  |  |  |  | TRS.1.1 TDD |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD |  | - | SR.1.1 FDD | - | SR.1.1 FDD |  | - |
|  |  | Config 2 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD |  | - | CR.1.1 FDD | - | CR.1.1 FDD |  | - |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | CR2.1 TDD |  |  | CR2.1 TDD |  | CR2.1 TDD |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  | Config 3 |  |  |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| SMTC configuration |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | Config 1,2,4,5 | dBm/15kHz | -104 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  | -101 |  |  |  |  |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |  |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |  |  |  |  |
| SS-RSRPNote3 |  |  | Config 1,2,4,5 | dBm/SCS | -87 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  | -84 |  |  |  |  |  |  |  |
| SCH_RP Note 3 |  |  |  | dBm/15 kHz | -87 |  |  |  |  |  |  |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |  |  |  |  |

>>>>>End of Change 3<<<<<

>>>>>Start of Change 4<<<<<

A.7.5.3.1.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB + 5ms as defined in clause 8.3.

>>>>>End of Change 4<<<<<