3GPP TSG-RAN WG4 Meeting #98-e	R4-2103812

# Electronic Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Updated simulation assumptions for V2X single link test cases

Source: 	Huawei, HiSilicon

Agenda item:	7.3.6.2

Document for:	Discussion

Introduction

In this contribution, we provide the updated simulation assumptions for NR V2X single-link demodulation requirements for simulation results alignment purpose based on the latest agreements. Companies are encouraged to submit the simulation results for next meeting. The values with different options or [ ] need be updated in the next RAN4 meeting based on further discussions.

Simulation assumptions

PSSCH

- Common assumptions for reference receiver:

- CFO estimation: Cross-DMRS symbol CFO estimation.

- CE channel estimation:

  - Frequency domain interpolation: MMSE interpolation

  - Time domain interpolation: Linear interpolation

- RX window: CP/2 from GNSS

- Table 1: Simulation assumption for PSSCH demodulation performance

| Parameters |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| Test |  |  | SCH_Test1 | SCH_Test2 | SCH_Test3 |
| Synchronization source |  |  | GNSS | GNSS | GNSS |
| Propagation condition |  |  | TDLA30-180 | TDLA30-1400 | Option 1:TDLA30-2700 Option 2:TDLB100-2700 |
| Channel bandwidth |  | MHz | 20 | 20 | 20 |
| Allocated resource blocks |  | RB | Option 1: 10 Option 2: 20 | 20 | 20 |
| Subcarrier spacing |  | kHz | 30 | 30 | 30 |
| Timing offset |  |  | CP/2-12Ts | CP/2-12Ts | CP/2-12Ts |
| Frequency offset |  | Hz | 600 | 600 | 600 |
| CP-OFDM symbols for slot with PSFCH(Note2) |  |  | 10 | 10 | 10 |
| CP-OFDM symbols for slot without PSFCH (Note3) |  |  | 13 | 13 | 13 |
| DMRS symbols for slot with PSFCH |  |  | 2 | 2 | 3 |
| DMRS symbols for slot without PSFCH |  |  | 2 | 3 | 4 |
| Modulation order |  |  | 6 | 4 | 2 |
| MCS index |  |  | 17 | 11 | 4 |
| 2nd stage SCI format 2-A configuration | Payloads | Bits | 35 | 35 | 35 |
|  | α |  | 1 | 1 | 1 |
|  | βoffset |  | 5 | 5 | 3.5 |
| Transport Block Size for slot with PSFCH |  | Bits |  |  |  |
| Transport Block Size for slot without PSFCH |  | Bits |  |  |  |
| Transport block CRC |  | Bits | 24 | 24 | 24 |
| Maximum number of HARQ transmissions |  |  | 1 | 1 | 1 |
| Binary Channel Bits for slots with PSFCH |  |  |  |  |  |
| Binary Channel Bits for slots without PSFCH |  | Bits |  |  |  |
| PSFCH resource periodicity |  | Slot | 4 | 4 | 4 |
| MinTimeGapPSFCH |  | Slot | 3 | 3 | 3 |
| Antenna configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| Performance metric |  |  | SNR@10% BLER of PSSCH |  |  |
| Note 1: The first OFDM symbol of a PSSCH and its associated PSCCH is duplicated as described in clauses 8.3.1.5 and 8.3.2.3 of TS 38.211 and not used for demodulation. The first OFDM symbol of a PSFCH is duplicated as described in clause 8.3.4.2.2 of TS 38.211 Note 2: Guard OFDM symbols as described in clause 8.2.1 of TS 38.211 and OFDM symbols used for PSFCH are not included. Note 3: Last OFDM symbol used for guard symbol as described in clause 8.2.1 of TS 38.211 is not included. Note 4: 10 RBs and 3 OFDM symbols including symbol for AGC are used for PSCCH transmission. |  |  |  |  |  |

PSCCH

Table 2 Simulation assumption for PSCCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | CCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | TDLA30-1400 |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 10 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSCCH symbols |  | 3 |
| Modulation order |  | QPSK |
| Payload size (without CRC) | Bits | 26 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 540 |
| Antenna configuration |  | 1x2 Low |
| Performance metric (Note 1) |  | SNR@1% BLER of PSCCH |
| Note 1: OCC index i (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 2: The first OFDM symbol of a PSSCH and its associated PSCCH is duplicated as described in clauses 8.3.1.5 and 8.3.2.3 of TS 38.211 and not used for demodulation. |  |  |

PSBCH

Table 3 Simulation assumption for PSBCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | BCH_Test1 |
| SLID |  | 0 |
| Synchronization source |  | SLSS |
| Propagation condition |  | TDLA30-180 |
| Channel bandwidth | MHz | 20 |
| Number SSB per SL period |  | 1 |
| Allocated resource blocks |  | 11 |
| Subcarrier spacing | kHz | 30 |
| Timing offset (Note2) |  | 0 |
| Frequency offset (Note 3) | Hz | 0 |
| The number of symbols (Note 1) |  | 9 |
| Modulation order |  | QPSK |
| Transport Block Size (without CRC) | Bits | 32 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits | Bits | 1782 |
| Antenna configuration |  | 1x2 Low |
| Performance metric |  | SNR@1% BLER of PSBCH |
| Note 1: The first OFDM symbol is used for AGC, not included in PSBCH decoding. OFDM symbols used for PSS/SSS transmission and guard symbol are not included either. Note 2: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 3: Frequency offset of Sidelink UE with respect to GNSS reference frequency. |  |  |

PSFCH

- Note: This case is used to test PSFCH performance for UE supporting NACK only feedback mode.

Table 4 Simulation assumption for PSFCH demodulation performance

| Parameters | Unit | Value |
| --- | --- | --- |
| Test |  | FCH_Test1 |
| Synchronization source |  | GNSS |
| Propagation condition |  | TDLA30-180 |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks |  | 1 |
| Subcarrier spacing | kHz | 30 |
| Timing offset |  | CP/2-12Ts |
| Frequency offset | Hz | 600 |
| The number of PSFCH symbols(Note 1) |  | 2 |
| Number of information bits | bit | 1 |
| PSFCH periodicity | Slot | 1 |
| Number of cyclic shift pair |  | 1 |
| Antenna configuration |  | 1x2 Low |
| Performance metric |  | SNR@ Pr(NACK miss) ≤1% and Pr(DTX to NACK)=1% |
| Note 1: The first OFDM symbol of a PSFCH is duplicated as described in clause 8.3.4.2.2 of TS 38.211 and not used for demodulation. |  |  |