3GPP TSG-RAN WG4 Meeting # 98-e	R4-2103815

# Electronic Meeting, 25 Jan. - 5 Feb., 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for PSSCH demodulation requirements for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to Big draft CR approach, draft CR for PSSCH demodulation requirements were submitted based on RAN4#97-e meeting agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PSSCH demodulation requirements have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements will be incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (New clause) 11, A.6, A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----- << Start of Change 1>> -----

11	V2X Requirements

11.1	Demodulation performance requirements (conducted requirements)

11.1.1	General

11.1.2	PSSCH demodulation requirements

11.1.2.1	2Rx requirements

11.1.2.1.1	Minimum requirements

The purpose of the requirements in this subclause is to verify the PSSCH for V2X demodulation performance with a single active PSSCH link.

The minimum requirements are specified in Table 11.1.2.1.1-2 with the test parameters specified in Table 11.1.2.1.1-1. In this test scenario, GNSS or GNSS-equivalent synchronization source is used and sidelink UE 1 transmits PSCCH and PSSCH.

Table 11.1.2.1.1-1: Test parameters

| Parameter |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 | Test 2 | Test 3 |
| Resource pool configuration |  |  |  | As specified in Table A.7-1 (Configuration #1-V2X) |  |  |
| Active cell(s) |  |  |  | None |  |  |
| Sidelink UE 1 | Sidelink transmissions |  |  | PSCCH + PSSCH |  |  |
|  | 2nd stage SCI format 2-A configuration | Payloads | Bits | 35 | 35 | 35 |
|  |  | α |  | 1 | 1 | 1 |
|  |  | βoffset |  | 3.5 | 5 | 5 |
|  | Timing offset (Note 1) |  |  | CP/2-12*64*Tc |  |  |
|  | Frequency offset (Note 2) |  | Hz | +600 |  |  |
|  | Synchronization |  |  | GNSS or GNSS-equivalent |  |  |
|  | Antenna configuration |  |  | 1x2 |  |  |
| PSFCH resource period |  |  | Slot | 4 | 4 | 4 |
| MinTimeGapPSFCH |  |  | Slot | 3 | 3 | 3 |
| Note 1: Time offset of sidelink UE receive signal with respect to GNSS referring timing. Note 2: Frequency offset of sidelink UE with respect to GNSS reference frequency. |  |  |  |  |  |  |

Table 11.1.2.1.1-2: Minimum performance

| Test num. | Reference channel | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSSCH BLER (%) | SNR(dB) of PSSCH |
| 1 | R.PSSCH.2-1.1 | 20 / 30 | QPSK, 0.30 | [TDLA30-2700] | 10% | TBD |
| 2 | R.PSSCH.2-1.2 | 20 / 30 | 16QAM, 0.37 | TDLA-1400 |  | TBD |
| 3 | R.PSSCH.2-1.3 | 20 / 30 | 64QAM, 0.43 | TDLA-180 |  | TBD |

----- << End of Change 1>> -----

----- << Start of Change 2>> -----

A.6	SL reference measurement channels

A.6.1	General

The transport block size (TBS) determination procedure is described in clause 8.1.3 of TS 38.214 [12].

A.6.2	Reference measurement channels for PSSCH performance requirements

### A.6.2.1 Reference measurement channels for SCS 15 kHz FR1

### A.6.2.2 Reference measurement channels for SCS 30 kHz FR1

Table A.6.2.2-1: PSSCH Reference Channel for V2X

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PSSCH.2-1.1 | R.PSSCH.2-1.2 | R.PSSCH.2-1.3 |  |
| Channel bandwidth | MHz | 20 | 20 | 20 |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |
| Allocated resource blocks | RB | 20 | 20 | TBD |  |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  | 9 | 9 | 9 |  |
| CP-OFDM symbols for slot without PSFCH |  | 12 | 12 | 12 |  |
| DMRS symbols for slot with PSFCH |  | 3 | 2 | 2 |  |
| DMRS symbols for slot without PSFCH |  | 4 | 3 | 2 |  |
| Modulation order |  | QPSK | 16QAM | 64QAM |  |
| MCS index |  | 4 | 11 | 17 |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |
| Number of DMRS REs |  |  |  |  |  |
| Transport Block Size for slot with PSFCH | Bits | 704 | 1800 | TBD |  |
| Transport Block Size for slot without PSFCH | Bits | 1128 | 2856 | TBD |  |
| Transport block CRC | Bits | 24 | 24 | 24 |  |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |  |
| Binary Channel Bits for slots with PSFCH |  | 2304 | 5088 | TBD |  |
| Binary Channel Bits for slots without PSFCH | Bits | 3504 | 7728 | TBD |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC), PSFCH symbols, and guard symbols. |  |  |  |  |  |

----- << End of Change 2>> -----

----- << Start of Change 3>> -----

A.7	SL reference resource pool configurations

Table A.7-1 V2X sidelink communication resource pool for PSSCH/PSCCH tests (Configuration #1-V2X)

| Information Element |  |  | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| SL-ResourcePool-r16 | sl-PSCCH-Config-r16 | sl-TimeResourcePSCCH-r16 |  | n2 |  |  |
|  |  | sl-FreqResourcePSCCH-r16 |  | n10 |  |  |
|  | sl-SyncAllowed-r16 |  |  | gnss-Sync-r16 |  |  |
|  | sl-SubchannelSize-r16 |  |  | n10 |  |  |
|  | sl-TimeResource-r16 |  |  | 160 |  |  |
|  | sl-StartRB-Subchannel-r16 |  |  | 0 |  |  |
|  | sl-NumSubchannel-r16 |  |  |  | Depending on allocated resource blocks in A.6 |  |
|  | sl-Additional-MCS-Table-r16 |  |  | Not presented |  |  |
|  | sl-RB-Number-r16 |  |  | 51 |  |  |
|  | sl-X-Overhead-r16 |  |  | n0 |  |  |

----- << End of Change 3>> -----