3GPP TSG-WG RAN4 Meeting #98e	R4-2103839

# Electronic Meeting, 25 Jan - 5 Feb, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | 0043 | rev | 1 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.307 on UE demodulation performance requirements (Rel-15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2021-02-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Resubmission of endorsed Draft CR R4-2017564, with some additional changes to implement the following RAN4 #97e agreement: - PMI reporting requirements for Rel-15 type II codebook - Release independent from Rel-15 - Power imbalance requirements for CA and EN-DC - The CA/EN-DC power imbalance requirements only for those intra-band contiguous CA configurations, intra-band contiguous and non-contiguous EN-DC configurations defined as release independent from release 15 in TS 38.307 can be defined as release independent from Release 15. - CA CQI reporting requirements - The CA CQI reporting requirements for those CA configurations defined as release independent from release 15 in TS 38.307 can be defined as release independent from Release 15 New changes on top of the endorsed version are tracked with tracking ID “Additional Changes RAN4#98-e” and “China Telecom_0201”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce release independent features according to previous RAN4 agreements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The previous RAN4 agreements are not followed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, New section 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of first change >

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BW	Bandwidth

CA	Carrier Aggregation

CC	Component carrier

DL	Downlink

EN-DC	Dual connectivity between E-UTRA and NR

FDD	Frequency Division Duplex

FR1	Frequency range 1

FR2	Frequency range 2

NR	New radio

PMI	Precoding Matrix Indicator

REL	Release

SDL	Supplementary downlink

SUL	Supplementary uplink

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

< End of first change >

< Start of second change >

5.5	Other release independent requirements for NR frequency range 1

This clause covers requirements for a Rel-15 UE coming from all other release independent requirements with the corresponding features introduced in Rel-15.

Table 5.5-1: Release independent requirements with the corresponding features introduced in Rel-15

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| Precoding matrix indicator (PMI) reporting requirements for TypeI-SinglePanel and TypeII Codebooks with more than 8TX and up to 32TX | Rel-15 | Table D.1-1, Table D.2-1 | Rel-16 NR_perf_enh-Perf WI introduced band independent PMI reporting requirements for 16TX and 32TX TypeI-SinglePanel Codebook, and 16TX TypeII Codebook. |
| PDSCH demoulation requirements with LTE CRS rate matching for TDD band | Rel-15 | Table E.1-1 | Rel-16 NR_perf_enh-Perf WI introduced band independent PDSCH demoulation requirements with LTE CRS rate mataching for TDD band. |

< End of second change >