3GPP TSG-RAN WG4 Meeting #98e                                            R4-2103855

Electronic Meeting, 25th Jan– 5th Feb, 2021

Agenda Item:   7.4.3.2.3

Source:        Samsung

Title:           WF on manufacturer declarations

Document for:  Agreement

Introduction

According to 1st round discussion on [98e][306] NR_IAB_Conformance_Part1, the candidate agreement captured in moderator summary[1] is copied as below:

| Sub-topic#3-1 Manufacturer declarations | Tentative agreements: Issue 3-1-1: re-use of NR declarations Combination of options 2, 3 and 4 Option 2: Declaration table is copy-pasted to IAB conformance specifications and terminology is updated in the table Option 3: In case some declarations are not applicable for IAB, they shall be marked as void to keep declaration numbering align with NR BS. (Practice in R4-2102329) Option 4: In case some declarations are not applicable for only IAB-MT or only IAB-DU, the applicability shall be made clear with added applicability column (R4-2102016) Issue 3-1-2: Tentative agreements: type 1-C related declarations are not applicable for IAB declarations for bands not defined for IAB are not applicable Candidate options: Further look needed on TAE groups, D.25 and D.51, applicability of multiband declarations, Recommendations for 2nd round: WF lead to drive the discussion to confirm the tentative agreements and capture possible additional agreements |
| --- | --- |

This way forward is to confirm the tentative agreement and possible more agreement during 2nd round.

Discussion

2.1 Agreement on IAB declaration

Dedicated declaration tables will be created for IAB conformance specification for conducted and radiated test requirement respectively.

- Exiting declaration table structure of gNB will be copied as baseline with necessary update on terminology in declaration name and corresponding description.

- Regarding numbering for declaration item

  - In case some declarations originally defined for gNB but not applicable for IAB, they shall be marked as void in IAB declaration table to keep declaration numbering align with gNB.

  - In case additional declaration which never defined for gNB but newly introduced for IAB, the numbering should be considered differently (e.g. 4-digital identifier) to avoid potential conflicting with gNB declaration numbering.

- In case some declarations are applicable for IAB-MT only or IAB-DU only, the applicability shall be made clear with added applicability column

2.2 WF on how to handle gNB declarations not applicable for IAB

- BS Type 1-C dedicated declaration is not applicable for IAB

  - FFS whether D.1 can be voided in conducted conformance specification

  - D35 is not applicable for IAB and will be void in IAB type 1-H declaration on conducted requirement

  - NB-Iot related declaration is not applied for IAB type 1-H

    - D.41, D.42 , and D.43 shall be void in IAB declaration on conducted requirement

- For IAB-MT

  - For IAB-MT type 1-H declaration in (D.31 TAE group) is not applicable.

  - For IAB-MT type 1-O, FFS on whether declaration in (D.49 Ncells) can be applied to serve as scaling factor.

- Regarding supporting operating band it’s agreed that declarations for bands not defined for IAB are not applicable

  - Note is suggested to be added for D.3 in declaration for conducted requirement and D.4 in declaration for radiated requirement to clarify that declaration should be within operating band defined in TS38.174.

- Regarding the D.25 for conducted requirement and D.51 for radiated requirement for type 1-H, note is suggested to be added to clarify the difference for IAB-MT is at IAB output power with no dynamic range(backoff) taken into account.

2.3 WF on how to distinguish declaration dedicated for IAB-MT or IAB-DU only

- For conducted requirement declaration the table header is suggested as below as suggested in R4-2100908, R4-2102016 and R4-2102332.

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | IAB-DU type 1-H | IAB-MT type 1-H |
| D.X | xxx | Xxxxx | x | x |

- For radiated requirement declaration there are two options on table header as below: Option 2 gains more preference hence it is suggested to take this as starting point for TP editor consideration.

  - Option 1: suggested in R4-2102016 and R4-2102332

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | IAB-DU type 1-H (Note 2) | IAB-MT type 1-H (Note 2) | IAB-DU type 1-O (Note 2) | IAB-MT type 1-O (Note 2) | IAB-DU type 2-O (Note 2) | IAB-MT type 2-O (Note 2) |
| D.X | XXX | XXXXX | x | x | x | x | x | x |

  - Option 2: updated suggestion based R4-2100908 and R4-2102016

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | IAB type 1-H (Note 2) | IAB type 1-O | IAB type 2-O |
| D.X | XXX | XXX | x (only IAB-DU) | x (only IAB-DU) | x (only IAB-DU) |