3GPP TSG-RAN WG4 Meeting #98-e	R4-210xxxx

# Electronic meeting, January 25th – February 5th, 2021

Agenda item:			9.28

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [98e][126] NR_n67

Document for:	Information

# Introduction

The new WI to introduce NR band n67 re-farmed LTE band 67 was agreed in last RAN#90e meeting.

This agenda item will handle all contributions related to this WI:

- BS RF requirements discussion.

- UE RF requirements discussion

# Topic #1: BS RF

This topic is aiming to agree on the proposed updates of BS RF specifications when introducing band n67.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2102169 | Ericsson | Proposal: Agree with the changes listed in this contribution. |
| R4-2102491 | Huawei | Proposal 1: Operation band n67 is introduced: Proposal 2: Channel bandwidth is introduced: Proposal 3: Channel raster and NR-ARFCN are introduced: Proposal 4: Synchronization raster entries for n67: Proposal 5: Two antenna port reference sensitivity QPSK PREFSENS: |

## Open issues summary

### Sub-topic 1-1

Sub-topic description: n67 band system parameters.

Issue 1-1: n67 band system parameters.

- Proposals:

  - Option 1: Band n67 shall be defined as below (Huawei)

    - Band definition

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n67 | N/A | 758 MHz – 803 MHz | SDL |

    - CBW

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| n67 | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

    - Channel arrangement

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n67 | 100 | - | 147600– <20> – 151600 |

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n67 | 15 kHz | Case A | 1850 – <1> – 1888 |

  - Option 2: Band n67 shall be defined as below (Ericsson)

    - Band definition

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n67 | N/A | 758 MHz – 803 MHz | SDL |

    - CBW

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| n67 | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

    - Channel arrangement

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n67 | 100 | N/A | 151600 – <20> – 160600 |

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n67 | 15 kHz | Case A | 1901 – <1> – 2002 |

- Recommended WF

  - The difference between the 2 proposals is on the NREF and GSCN values, to be checked.

### Sub-topic 1-2

Sub-topic description: n67 band Tx requirements.

Issue 1-2: n67 band Tx requirements.

- Proposals:

  - Option 1: Tx requirements updates when introducing band n67, more details in R4-2102169: (Ericsson)

    - OBUE: n67 shall be added to the list of bands for Wide Area cat B option 1 limits for bands below 1GHz, in clause 6.6.4.2.2

    - Spurious: Band n67 shall be added to the coexistence spurious emissions limits, table 6.6.5.2.3-1.

- Recommended WF

  - Agree with this proposal.

### Sub-topic 1-3

Sub-topic description: n67 band Rx requirements.

Issue 1-3: n67 band Rx requirements.

- Proposals:

  - Option 1: No impact on Rx requirements when introducing band n67. (Ericsson)

- Recommended WF

  - Agree with this proposal.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: Sub topic 1-3: Others: |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
|  | NA |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: UE RF

This topic is aiming to agree on the proposed updates of UE RF specifications when introducing band n67.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2102170 | Ericsson | Proposal: Agree with the changes listed in this contribution. |
| R4-2102491 | Huawei | Proposal 1: Operation band n67 is introduced: Proposal 2: Channel bandwidth is introduced: Proposal 3: Channel raster and NR-ARFCN are introduced: Proposal 4: Synchronization raster entries for n67: Proposal 5: Two antenna port reference sensitivity QPSK PREFSENS: |

## Open issues summary

### Sub-topic 2-1

Sub-topic description: n67 band system parameters.

Issue 2-1: n67 band system parameters.

- Recommended WF

  - Shall be aligned with Issue 1-1.

### Sub-topic 2-2

Sub-topic description: n67 band Tx requirements.

Issue 2-2: n67 band Tx requirements.

- Proposals:

  - Option 1: As n67 is SDL band, no Tx requirements update is expected when introducing band n67, more details in R4-2102170: (Ericsson)

- Recommended WF

  - Agree with this proposal.

### Sub-topic 2-3

Sub-topic description: n67 band Rx requirements.

Issue 2-2: n67 band Rx requirements.

- Proposals:

  - Option 1: Rx requirements updates when introducing band n67 (Huawei):

    - REFSENS values and RB allocations:

|  | Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n67 |  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |  |  |  | SDL |
|  |  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |  |  |  |  |

  - Option 2: Rx requirements updates when introducing band n67 (Ericsson):

    - REFSENS values and RB allocations:

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n67 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  |  |  |  |  |  |  |  |  | SDL |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n67 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - Agree with one of the 2 options.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Sub topic 2-2: Sub topic 2-3: Others: |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
|  | NA |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |