3GPP TSG-RAN WG4 Meeting # 98-e	R4-2100121

# Electronic meeting,  25th Jan. - 5th Feb., 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0590 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-1 on optimization on delta TIB and RIB for band combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the UE which supports inter-band NR CA configurations, the maximum configured output power relaxation ΔTIB,c and reference sensitivity relaxation ΔRIB,c due to the configurations are too redundant with each frequency band arranged in sequence order, which makes the table too long in the spec. To simplify the ΔTIB,c and ΔRIB,c tables for band combinations, it is suggested to use the format of “NR Band / ΔTIB,c (dB)” and “NR Band / ΔRIB,c (dB)”, with which the ΔTIB,c and ΔRIB,c values can be listed within one row for each configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To simplify the ΔTIB,c and ΔRIB,c tables for inter-band CA and SUL by using the format of “NR Band / ΔTIB,c (dB)” and “NR Band / ΔRIB,c (dB)”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ΔTIB,c and ΔRIB,c tables due to inter-band CA and SUL will be redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.4.2.3, 6.2A.4.2.4, 6.2A.4.2.5, 6.2C.2, 7.3A.3.2.1, 7.3A.3.2.3, 7.3A.3.2.4, 7.3C.3.2.1, 7.3F.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

6.2A.4.2	ΔTIB,c for CA

For the UE which supports inter-band NR CA configuration, ΔTIB,c in tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

6.2A.4.2.1	Void

6.2A.4.2.2	Void

6.2A.4.2.3	ΔTIB,c for Inter-band CA (two bands)

Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination |  | NR Band / ΔTIB,c (dB) |  |  |
| --- | --- | --- | --- | --- |
| CA_n1-n3 | n1 / 0.3 | n3 / 0.3 |
| CA_n1-n7 | n1 / 0.5 | n7 / 0.6 |
| CA_n1-n8 | n1 / 0.3 | n8 / 0.3 |
| CA_n1-n28 | n1 / 0.3 | n28 / 0.6 |
| CA_n1-n40 | n1 / 0.5 | n40 / 0.5 |
| CA_n1-n41 | n1 / 0.5 | n41 / 0.5 |
| CA_n1-n77 | n1 / 0.6 | n77 / 0.8 |
| CA_n1-n78 | n1 / 0.3 | n78 / 0.8 |
| CA_n2-n5 | n2 / 0.3 | n5 / 0.3 |
| CA_n2-n48 | n2 / 0.6 | n48 / 0.8 |
| CA_n2-n66 | n2 / 0.5 | n66 / 0.5 |
| CA_n2-n77 | n2 / 0.6 | n77 / 0.8 |
| CA_n2-n78 | n2 / 0.6 | n78 / 0.8 |
| CA_n3-n7 | n3 / 0.5 | n7 / 0.5 |
| CA_n3-n8 | n3 / 0.3 | n8 / 0.3 |
| CA_n3-n28 | n3 / 0.3 | n28 / 0.3 |
| CA_n3-n38 | n3 / 0.5 | n38 / 0.5 |
| CA_n3-n40 | n3 / 0.5 | n40 / 0.5 |
| CA_n3-n41 | n3 / 0.5 | n41 / 0.34, 0.85 |
| CA_n3-n77 | n3 / 0.6 | n77 / 0.8 |
| CA_n3-n78 | n3 / 0.6 | n78 / 0.8 |
| CA_n3-n79 | n3 / 0.3 | n79 / 0.8 |
| CA_n5-n7 | n5 / 0.3 | n7 / 0.3 |
| CA_n5-n66 | n5 / 0.3 | n66 / 0.3 |
| CA_n5-n77 | n5 / 0.6 | n77 / 0.8 |
| CA_n5-n78 | n5 / 0.6 | n78 /0.8 |
| CA_n7-n25 | n7 / 0.5 | n25 / 0.5 |
| CA_n7-n28 | n7 / 0.3 | n28 / 0.3 |
| CA_n7-n66 | n7 / 0.5 | n66 / 0.5 |
| CA_n7-n78 | n7 / 0.5 | n78 / 0.8 |
| CA_n8-n39 | n8 / 0.3 | n39 / 0.3 |
| CA_n8-n40 | n8 / 0.3 | n40 / 0.3 |
| CA_n8-n41 | n8 / 0.6 | n41 / 0.3 |
| CA n8-n75 | n8 / 0.3 |  |
| CA n8-n78 | n8 / 0.6 | n78 / 0.8 |
| CA_n8-n79 | n8 / 0.3 | n79 / 0.8 |
| CA_n20-n28 | n20 / 0.5 | n28 / 0.5 |
| CA_n20-n75 | n20 / 0.3 | 0.3 |
| CA_n20-n78 | n20 / 0.6 | n78 / 0.8 |
| CA_n25-n41 | n25 / 0.5 | n41 / 0.46, 0.97 |
| CA_n25-n66 | n25 / 0.5 | n66 / 0.5 |
| CA_n25-n71 | n25 / 0.3 | n71 / 0.6 |
| CA_n28-n40 | n28 / 0.3 | n40 / 0.3 |
| CA_n28-n41 | n28 / 0.3 | n41 / 0.3 |
| CA_n28-n50 | n28 / 0.3 | n50 / 0.4 |
| CA_n28-n75 | n28 /0.3 |  |
| CA_n28-n77 | n28 / 0.5 | n77 / 0.8 |
| CA_n28-n78 | n28 / 0.5 | n78 / 0.8 |
| CA_n29-n66 | n66 / 0.3 |  |
| CA_n29-n70 | n70 / 0.3 |  |
| CA_n38-n66 | n38 / 0.5 | n66 / 0.5 |
| CA_n38-n78 | n38 / 0.3 | n78 / 0.8 |
| CA_n39-n41 | n39 / 02, 0.53 | n41 / 02, 0.53 |
| CA_n39-n79 | n39 / 0.3 | n79 / 0.8 |
| CA_n40-n41 | n40 / 0.53 | n41 / 0.53 |
| CA_n40-n78 | n40 / 0 | n78 / 0.5 |
| CA_n40-n79 | n40 / 0.3 | n79 / 0.8 |
| CA_n41-n50 | n41 / 0.3 | n50 / 0.4 |
| CA_n41-n66 | n41 / 0.86, 1.37 | n66 / 0.5 |
| CA_n41-n71 | n41 / 0.3 | n71 / 0.6 |
| CA_n41-n781 | n41 / 0.3 | n78 / 0.8 |
| CA_n41-n79 | n41 / 0.3 | n79 / 0.8 |
| CA_n48-n66 | n48 / 0.8 | n66 / 0.6 |
| CA_n50-n78 | n50 / 02, 0.53 | n78 / 02, 0.53 |
| CA_n66-n70 | n66 / 0.5 | n70 / 0.5 |
| CA_n66-n71 | n66 / 0.3 | n71 / 0.3 |
| CA_n66-n77 | n66 / 0.6 | n77 / 0.8 |
| CA_n66-n78 | n66 / 0.6 | n78 / 0.8 |
| CA_n70-n71 | n70 / 0.3 | n71 / 0.6 |
| CA_n75-n78 | n78 / 0.8 |  |
| CA_n76-n78 | n78 / 0.8 |  |
| CA n77-n79 | n77 / 0.5 | n79 / 0.5 |
| CA_n78-n79 | n78 / 0.5, 1.58 | n79 / 0.5, 1.58 |
| CA_n78-n92 | n78 / 0.8 | n92 / 0.6 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

Table 6.2A.4.2.3-2: Void

Table 6.2A.4.2.3-3: Void

6.2A.4.2.4	ΔTIB,c for Inter-band CA (three bands)

Table 6.2A.4.2.4-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination |  | NR Band / ΔTIB,c (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n3-n7 | n1 /0.6 | n3 / 0.6 |  | n7 / 0.6 |
| CA_n1-n3-n7 | n1 / 0.3 | n3 / 0.3 |  | n8 / 0.3 |
| CA_n1-n3-n28 | n1 / 0.3 | n3 / 0.3 |  | n28 / 0.6 |
| CA_n1-n3-n41 | n1 / 0.5 | n3 / 0.5 |  | n41 / 0.35, 0.86 |
| CA_n1-n3-n78 | n1 / 0.6 | n3 / 0.6 |  | n78 / 0.8 |
| CA_n1-n8-n78 | n1 / 0.3 | n8 / 0.6 |  | n78 / 0.8 |
| CA_n1-n28-n78 | n1 / 0.3 | n28 / 0.6 |  | n78 / 0.8 |
| CA_n3-n8-n78 | n3 / 0.6 | n8 / 0.6 |  | n78 / 0.8 |
| CA_n1-n7-n28 | n1 / 0.5 | n7 / 0.6 |  | n28 / 0.6 |
| CA_n1-n7-n78 | n1 / 0.6 | n7 / 0.6 |  | n78 / 0.8 |
| CA_n1-n40-n78 | n1 / 0.3 | n40 / 0.5 |  | n78 / 0.8 |
| CA_n3-n7-n28 | n3 / 0.5 | n7 / 0.5 |  | n28 / 0.3 |
| CA_n3-n7-n78 | n3 / 0.6 | n7 / 0.6 |  | n78 / 0.8 |
| CA_n3-n28-n77 | n3 / 0.6 | n28 / 0.5 |  | n77 / 0.8 |
| CA_n3-n28-n78 | n3 / 0.5 | n28 / 0.3 |  | n78 / 0.8 |
| CA_n3-n40-n41 | n3 / 0.5 | n40 / 0.5 |  | n41 / 0.51,3, 0.82,3 |
| CA_n3-n41-n79 | n3 / 0.3 | n41 / 0.31, 0.82 |  | n79 / 0.8 |
| CA_n5_n66-n78 | n5 / 0.6 | n66 / 0.6 |  | n78 / 0.8 |
| CA_n7_n25-n66 | n7 / 0.5 | n25 / 0.5 |  | n66 / 0.5 |
| CA_n7_n28-n78 | n7 / 0.3 | n28 / 0.3 |  | n78 / 0.8 |
| CA_n7_n66-n78 | n7 / 0.5 | n66 / 0.6 |  | n78 / 0.8 |
| CA_n8-n39-n41 | n8 / 0.6 | n39 / 0.54 |  | n41 / 0.54 |
| CA_n8-n41-n79 | n8 / 0.6 | n41 / 0.3 |  | n79 / 0.8 |
| CA_n20-n28-n78 | n20 / 0.6 | n28 / 0.5 |  | n78 / 0.8 |
| CA_n25-n41-n66 | n25 / 0.5 | n41 / 0.85, 1.36 |  | n66 / 0.5 |
| CA_n25-n41-n71 | n25 / 0.5 | n41 / 0.5 |  | n71 / 0.6 |
| CA_n25-n66-n71 | n25 / 0.5 | n66 / 0.5 |  | n71 / 0.6 |
| CA_n25-n66-n78 | n25 / 0.6 | n66 / 0.6 |  | n78 / 0.8 |
| CA_n28-n40-n78 | n28 / 0.5 | n40 / 0.3 |  | n78 / 0.8 |
| CA_n28-n41-n78 | n28 / 0.5 | n41 / 0.3 |  | n78 / 0.8 |
| CA_n29-n66-n70 | n29 / 0 | n66 / 0.5 |  | n70 / 0.5 |
| CA_n39-n41-n79 | n39 / 0.3 | n41 / 0.34 |  | n79 / 0.84 |
| CA_n40-n41-n79 | n40 / 0.53 | n41 / 0.53 |  | n79 / 0.8 |
| CA_n41-n66-n71 | n41 / 0.81, 1.32 | n66 / 0.5 |  | n71 / 0.3 |
| CA_n66-n70-n71 | n66 / 0.5 | n70 / 0.5 |  | n71 / 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. NOTE 4: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx between n39 and n41. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |  |  |

6.2A.4.2.5	ΔTIB,c for Inter-band CA (four bands)

Table 6.2A.4.2.5-1: ΔTIB,c due to NR CA (four bands)

| Inter-band CA combination |  | NR Band / ΔTIB,c (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n3-n7-n28 | n1 / 0.6 | n3 / 0.6 | n7 / 0.6 | n28 / 0.6 |
| CA_n1-n3-n7-n78 | n1 / 0.7 | n3 / 0.7 | n7 / 0.7 | n78/ 0.8 |
| CA_n1-n3-n8-n78 | n1 / 0.6 | n3 / 0.6 | n8 / 0.6 | n78 / 0.8 |
| CA_n1-n3-n28-n78 | n1 / 0.6 | n3 / 0.6 | n28 / 0.6 | n78 / 0.8 |
| CA_n3-n7-n28-n78 | n3 / 0.6 | n7 / 0.6 | n28 / 0.6 | n78 / 0.6 |
| CA_n7-n25-n66-n78 | n7 / 0.5 | n25 / 0.6 | n66 / 0.6 | n78 / 0.8 |

<< Unchanged sections omitted >>

6.2C.2	ΔTIB,c

For the UE which supports SUL band combination, ΔTIB,c in Tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

Table 6.2C.2-1: ΔTIB,c due to SUL

| Band combination for SUL | NR Band / ΔTIB,c (dB) |  |
| --- | --- | --- |
| SUL_n41-n80 | n41 / 0.31, 0.82 | n80 / 0.5 |
| SUL_n41-n81 | n41 / 0.3 | n81 / 0.3 |
| SUL_n77-n80 | n77 / 0.8 | n80 / 0.6 |
| SUL_n77-n84 | n77 / 0.8 | n84 / 0.6 |
| SUL_n78-n80 | n78 / 0.8 | n80 / 0.6 |
| SUL_n78-n81 | n78 / 0.8 | n81 / 0.6 |
| SUL_n78-n82 | n78 / 0.8 | n82 / 0.6 |
| SUL_n78-n83 | n78 / 0.8 | n83 / 0.5 |
| SUL_n78-n84 | n78 / 0.8 | n84 / 0.3 |
| SUL_n78-n86 | n78 / 0.8 | n86 / 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2515 MHz. |  |  |

<< Unchanged sections omitted >>

7.3A.3	ΔRIB,c for CA

7.3A.3.1	General

For a UE supporting a CA configuration, the ΔRIB,c applies for both SC and CA operation.

7.3A.3.2	ΔRIB,c for Inter-band CA

For the UE which supports inter-band carrier aggregation, the minimum requirement for reference sensitivity in clause 7.3A.2 shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

7.3A.3.2.1	ΔRIB,c for two bands

Table 7.3A.3.2.1-1: ΔRIB,c due to CA (two bands)

| Inter-band CA combination |  | NR Band / ΔRIB,c (dB) |  |  |
| --- | --- | --- | --- | --- |
| CA_n1-n28 | n28 / 0.2 |  |
| CA_n1-n77 | n1 / 0.2 | n77 / 0.5 |
| CA_n1-n78 | n78 / 0.5 |  |
| CA_n2-n48 | n2 / 0.2 | n48 / 0.5 |
| CA_n2-n66 | n2 / 0.3 | n66 / 0.3 |
| CA_n2-n77 | n2 / 0.2 | n77 / 0.5 |
| CA_n2-n78 | n2 / 0.2 | n78 / 0.5 |
| CA_n3-n41 | n41 / 04, 0.55 |  |
| CA_n3-n77 | n3 / 0.2 | n77 / 0.5 |
| CA_n3-n78 | n3 / 0.2 | n78 / 0.5 |
| CA_n3-n79 | n79 / 0.5 |  |
| CA_n5-n77 | n5 / 0.2 | n77 / 0.5 |
| CA_n5-n78 | n5 / 0.2 | n78 / 0.5 |
| CA_n7-n66 | n7 / 0.5 | n66 / 0.5 |
| CA_n7-n78 | n7 / 0.5 | n78 / 0.5 |
| CA_n25-n66 | n25 / 0.3 | n66 / 0.3 |
| CA_n8-n78 | n8 / 0.2 | n78 / 0.5 |
| CA_n25-n78 | n25 / 0.2 | n78 / 0.5 |
| CA_n8-n79 | n79 / 0.5 |  |
| CA_n20-n78 | n78 / 0.5 |  |
| CA_n25-n71 | n71 / 0.3 |  |
| CA_n28-n75 | n28 / 0.2 |  |
| CA_n28-n77 | n28 / 0.2 | n77 / 0.5 |
| CA_n28-n78 | n28 / 0.2 | n78 / 0.5 |
| CA_n38-n66 | n38 / 0.5 | n66 / 0.5 |
| CA_n38-n78 | n38 / 0.4 | n78 / 0.5 |
| CA_n39-n40 | n39 / 0.3 | n40 / 0.3 |
| CA_n39-n41 | n39 / 0.22, 0.23 | n41 / 0.22, 0.23 |
| CA_n39-n79 | n79 / 0.5 |  |
| CA_n40-n78 | n40 / 0.4 | n78 / 0.5 |
| CA_n40-n79 | n79 / 0.5 |  |
| CA_n41-n66 | n41 / 0.56, 17 | n66 / 0.5 |
| CA_n41-n71 | n71 / 0.2 |  |
| CA_n41-n781 | n78 / 0.5 |  |
| CA_n41-n79 | n41 / 0.5 | n79 / 0.5 |
| CA_n48-n66 | n48 / 0.5 | n66 / 0.2 |
| CA_n50-n78 | n50 / 0.22, 0.23 | n78 / 0.22, 0.23 |
| CA_n66-n77 | n66 / 0.2 | n77 / 0.5 |
| CA_n66-n78 | n66 / 0.2 | n78 / 0.5 |
| CA_n75-n78 | n78 / 0.5 |  |
| CA_n76-n78 | n78 / 0.5 |  |
| CA_n78-n92 | n78 / 0.5 |  |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz |  |  |

Table 7.3A.3.2.1-2: void

7.3A.3.2.2	Void

7.3A.3.2.3	ΔRIB,c for three bands

Table 7.3A.3.2.3-1: ΔRIB,c due to CA (three bands)

| Inter-band CA combination |  | NR Band / ΔRIB,c (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n3-n7 | n1 / 0 | n3 / 0 |  | n7 / 0 |
| CA_n1-n3-n28 | n28 / 0.2 |  |  |  |
| CA_n1-n3-n41 | n41 / 05, 0.56 |  |  |  |
| CA_n1-n3-n78 | n1 / 0.2 | n3 / 0.2 |  | n78 / 0.5 |
| CA_n1-n7-n28 | n1 / 0 | n7 / 0 |  | n28 / 0.2 |
| CA_n1-n7-n78 | n1 / 0.2 | n7 / 0.2 |  | n78 / 0.5 |
| CA_n1-n8-n78 | n1 / 0 | n8 / 0.2 |  | n78 / 0.5 |
| CA_n1-n28-n78 | n1 / 0 | n28 / 0.2 |  | n78 / 0.5 |
| CA_n1-n40-n78 | n1 / 0 | n40 / 0 |  | n78 / 0.5 |
| CA_n3-n7-n28 | n3 / 0 | n7 / 0 |  | n28 / 0 |
| CA_n3-n7-n78 | n3 / 0.2 | n7 / 0.2 |  | n78 / 0.5 |
| CA_n3-n8-n78 | n3 / 0.2 | n8 / 0.2 |  | n78 / 0.5 |
| CA_n3-n28-n77 | n3 / 0.2 | n28 / 0.2 |  | n77 / 0.5 |
| CA_n3-n28-n78 | n3 / 0 | n28 / 0.2 |  | n78 / 0.5 |
| CA_n3-n40-n41 | n41 / 01,3, 0.52,3 |  |  |  |
| CA_n3-n41-n79 | n41 / 0.5 | n79 / 0.5 |  |  |
| CA_n5_n66-n78 | n5 / 0.5 | n66 / 0.2 |  | n78 / 0.5 |
| CA_n7_n25-n66 | n7 / 0.5 | n25 / 0.3 |  | n66 / 0.5 |
| CA_n7_n28-n78 | n7 / 0 | n28 / 0 |  | n78 / 0.5 |
| CA_n7_n66-n78 | n7 / 0.5 | n66 / 0.5 |  | n78 / 0.5 |
| CA_n8-n39-n41 | n39 / 0.24 | n41 / 0.24 |  |  |
| CA_n8-n41-n79 | n41 / 0.5 | n79 / 0.5 |  |  |
| CA_n20-n28-n78 | n20 / 0 | n28 / 0.2 |  | n78 / 0.5 |
| CA_n25-n41-n66 | n25 / 0.3 | n41 / 0.55, 16 |  | n66 / 0.3 |
| CA_n25-n41-n71 | n25 / 0 | n41 / 0 |  | n71 / 0.2 |
| CA_n25-n66-n71 | n25 / 0.3 | n66 / 0.3 |  | n71 / 0.3 |
| CA_n25-n66-n78 | n25 / 0.3 | n66 / 0.3 |  | n78 / 0.5 |
| CA_n28-n40-n78 | n28 / 0 | n40 / 0 |  | n78 / 0.5 |
| CA_n28-n41-n78 | n28 / 0.2 | n41 / 0 |  | n78 / 0.5 |
| CA_n39-n41-n79 | n39 / 0.34 | n41 / 0.34 |  | n79 / 0.8 |
| CA_n40-n41-n79 | n40 / 08 | n41 / 0.58 |  | n79 / 0.5 |
| CA_n41-n66-n71 | n41 / 0.51, 12 | n66 / 0.5 |  | n71 / 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. NOTE 3: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among band 40 and 41. NOTE 4: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx between n39 and n41. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 7: Void. NOTE 8: Void. |  |  |  |  |

7.3A.3.2.4	ΔRIB,c for four bands

Table 7.3A.3.2.4-1: ΔRIB,c due to CA (four bands)

| Inter-band CA combination |  | NR Band / ΔRIB,c (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n3-n7-n28 | n28 / 0.2 |  |  |  |
| CA_n1-n3-n7-n78 | n1 / 0.3 | n3 / 0.3 | n7 / 0.3 | n78 / 0.5 |
| CA_n1-n3-n8-n78 | n1 / 0.2 | n3 / 0.2 | n8 / 0.2 | n78 / 0.5 |
| CA_n1-n3-n28-n78 | n1 / 0.2 | n3 / 0.2 | n28 / 0.2 | n78 / 0.5 |
| CA_n3-n7-n28-n78 | n3 / 0.2 | n7 / 0.2 | n28 / 0.2 | n78 / 0.5 |
| CA_n7-n25-n66-n78 | n7 / 0.5 | n25 / 0.6 | n66 / 0.6 | n78 / 0.8 |

<< Unchanged sections omitted >>

7.3C.3.2.1	ΔRIB,c  for two bands

Table 7.3C.3.2.1-1: ΔRIB,c due to SUL (two bands)

| Band combination for SUL | NR Band / ΔRIB,c (dB) |  |
| --- | --- | --- |
| SUL_n41-n80 | n41 / 0.51 |  |
| SUL_n41-n95 | n41 / 0.2 |  |
| SUL_n77-n80 | n77 / 0.5 |  |
| SUL_n77-n84 | n77 / 0.5 |  |
| SUL_n78-n80 | n78 / 0.5 |  |
| SUL_n78-n81 | n78 / 0.5 |  |
| SUL_n78-n82 | n78 / 0.5 |  |
| SUL_n78-n83 | n78 / 0.5 |  |
| SUL_n78-n84 | n78 / 0.5 |  |
| SUL_n78-n86 | n78 / 0.5 |  |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

<< Unchanged sections omitted >>

7.3F.3	ΔRIB,c

For a UE supporting CA or DC band combination, the minimum requirement for reference sensitivity in Table 7.3F.2-1 shall be increased by the amount given by ΔRIB,c defined in Table 7.3F.3-1.  Unless otherwise stated, ΔRIB,c is set to zero.

Table 7.3F.3-1: ΔRIB,c due to CA (two bands)

| Inter-band CA combination |  | Operating Band / ΔRIB,c (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CA_n46-n48 |  | n46 / 0 |  | n48 / 0.5 |  |

In case the UE supports more than one of band combinations for CA or DC, and an operating band belongs to more than one band combinations then the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A and 7.3F.3 in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

==============================================================

<< End of changes >>