3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100200

# Electronic Meeting, 25 Jan – 5 Feb, 2021

Agenda item:	7.8.1.2.1

Source:	Apple Inc.

Title:	UE demodulation requirements with higher BLER

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#97e the UE demodulation requirements for URLLC with higher BLER was discussed and way forward [1] was approved. In this contribution we present our views on the open issues related to UE demod requirements for higher BLER and also provide simulation results.

# 2. Discussion

## PDSCH Slot Aggregation in FR1

The agreements in [1] related to PDSCH slot aggregation in FR1:

- HARQ process number: 4 for FDD and TDD

- MCS 19

We provide simulation results for the agreed simulation assumptions below for FR1.

Table 1: Simulation results with PDSCH slot aggregation in FR1

| SNR @1% BLER |  | Ideal | Impairment |
| --- | --- | --- | --- |
| FDD | 2x2 | -1.0 | 1.0 |
|  | 2x4 | -4.9 | -2.9 |
| TDD | 2x2 | -1.2 | 0.8 |
|  | 2x4 | -5.0 | -3.0 |

Another open issue related to requirements with slot aggregation is how to specify the number of slots between PDSCH and corresponding HARQ-ACK information for FDD.

- Option 1: 4

- Option 2: 3 for initial transmission and 2 for repetition

- Option 3: The number of slots between final repetition PDSCH and corresponding HARQ-ACK information for FDD: 2

- Option 4: The number of slots between initial transmission PDSCH and corresponding HARQ-ACK information for FDD: 3

Since ACK/NACK is transmitted after all repetitions, the number of slots between PDSCH and HAR-ACK should be specified for the last transmission alone.

Proposal #1: The number of slots between final repetition PDSCH and corresponding HARQ-ACK information for FDD: 2.

For TDD we should also use a consistent approach and specify the HARQ-ACK delay from the last PDSCH repetition.

Proposal #2: In TDD define the number of slots between final repetition PDSCH and corresponding HARQ-ACK information

## PDSCH Slot Aggregation in FR2

The agreements in [1] related to PDSCH slot aggregation in FR2:

- SCS/BW: 120 kHz / 100 MHz - TDD pattern: DDSU, S=11:3:0 - PDSCH configuration - Frequency domain resource allocation: Full bandwidth - Mapping Type A, Start symbol 1, Length 13 - PDSCH aggregation factor: n2 - Scheduling: No scheduling in D slot i, where mod(i,160) = 0 and mod(i, 160) = 1, and S slots - MCS - Option 1: MCS19 from Table 3 - Option 2: MCS16 from Table 3 - Option 3: MCS13 from Table 3 - Higher or equal to -4 dB for final 2 Rx requirement definition (average ideal SNR alignment result + IM) - DM-RS configuration: Type 1, Single-symbol DM-RS, 1 additional DM-RS - PT-RS configuration: Frequency density 2, Time density 1, RE offset 2 - Overhead for TBS determination: 6 - HARQ process: 2 - Maximum number of HARQ re-transmission: 4 - Channel model: TDLA30-75 - Antenna configuration: 2x2, ULA low - Test metric: 1% BLER

Based on the simulation parameters agreed in [1], the simulation results with PDSCH slot aggregation for different MCS are provided in Table 2.

Table 2: Simulation results with PDSCH slot aggregation in FR2

| SNR @1% BLER | Ideal | Impairment |
| --- | --- | --- |
| MCS: 13 | -5.5 | -3.5 |
| MCS:16 | -3.5 | -1.5 |
| MCS:19 | -1.9 | 0.1 |

Based on the results in Table 2, MCS 13 or MCS 16 are suitable for defining requirement with PDSCH slot aggregation in FR2.

Proposal #3: Define requirements with PDSCH slot aggregation in FR2 with MCS 13 or MCS 16.

## PDSCH Mapping Type-B and Processing Capability 2 in FR1

For the agreed simulation assumptions, we provide simulation results in FR1 for PDSCH mapping Type B and processing capability 2 below.

Table 3: Simulation results for PDSCH Mapping Type B and Processing Capability 2 in FR1

| SNR @70% Max TP |  | Ideal | Impairment |
| --- | --- | --- | --- |
| FDD | 2x2 | -1.3 | 0.7 |
|  | 2x4 | -4.5 | -2.5 |
| TDD | 2x2 | -1.6 | 0.4 |
|  | 2x4 | -4.8 | -2.8 |

## PDSCH Mapping Type-B in FR2

In RAN4#97e for requirements in FR2 with PDSCH mapping Type B the following agreements were made for simulation parameters.

- Symbol length: 2os - DMRS configuration: Type 1, Single-symbol DM-RS, 0 additional DM-RS - PTRS configuration: Frequency density 2, Time density 1, RE offset 2 - MCS: MCS4 from Table 1. - Maximum number of HARQ re-transmission: 1 - Number of HARQ process: 4 - Overhead for TBS determination: 0

Based on the simulation assumptions in [1], the results with MCS 4 in FR2 are provided in table 3 below.

Table 4: Simulation results for PDSCH Mapping Type B in FR2

| SNR @70% Max TP | Ideal | Impairment |
| --- | --- | --- |
| 2x2 (MCS:4) | -0.2 | 1.8 |

## Pre-emption Indication

In RAN4#97e requirements for pre-emption indication were discussed but test parameters were not agreed. The following way forward was agreed for requirements with pre-emption indication:

      - Pre-emption probability

- Option 1: 10% within 1 radio frame

- Option 2: 20% within 1 radio frame

      - eMBB MCS

- Option 1: MCS13 in Table 1

- Option 2: MCS4 in Table 1

- Option 3: MCS 16 in Table 1

      - Test metric

- Option 1: 70% of max T-put

- Other options are not precluded

      - Select proper test parameters and test metric to discriminate UE behaviour and ensure proper UE processing i.e. the performance gap > 1dB

Companies were encouraged to bring simulation results comparing performance with and without HARQ buffer flushing in order to decide the test parameters. The table below shows the results with different options for parameters as discussed in [1].

Table 5: Simulation results for Pre-emption Indication

| SNR @ 70% Max TP |  | 10% Pre-emption |  | 20% Pre-emption |  | Delta with 10% Pre-emption | Delta with 20% Pre-emption |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Without buffer flush | With buffer flush | Without buffer flush | With buffer flush |  |  |
| 2x2 | MCS:4 | -2.6 | -2.6 | -1.8 | -1.8 | 0.0 | 0.0 |
|  | MCS:13 | 6.1 | 5.2 | 11.1 | 6.4 | -0.9 | -4.7 |
|  | MCS:16 | 9.6 | 8.0 | NaN | 9.2 | -1.6 | NaN |
| 2x4 | MCS:4 | -5.8 | -5.9 | -5.3 | -5.3 | -0.1 | 0.0 |
|  | MCS:13 | 2.0 | 1.5 | 3.8 | 2.3 | -0.5 | -1.5 |
|  | MCS:16 | 4.8 | 4.1 | NaN | 4.8 | -0.7 | NaN |

Based on the results in Table 7, we observe that for MCS 4 there is very small delta in performance with or without buffer flush. It would be easy for UE without correct buffer flush implementation to pass the requirement if introduced with low MCS. With MCS 13 and 10% pre-empted slots, the delta is close to 1 dB with and without correct UE implementation with 2RX, but 4RX the delts is still under 1dB. With 20% pre-empted slots and MCS 13, the delta is greater than 1dB for both 2RX and 4RX. For MCS 16, with 20% pre-empted slots, the UE doesn’t achieve 70% max TP without correct buffer flush. MCS 16 with 20% pre-emption would be suitable to introduce requirements for pre-emption indication. Hence, we propose to introduce requirements for pre-emption indication with 20% pre-empted slots with MCS of 16. Also, test metric of 70% of max throughput seems appropriate for this test.

Proposal #4: Introduce requirements for pre-emption indication with 20% pre-empted slots and MCS of 16.

Proposal #5: Define requirements for pre-emption indication at 70% of maximum throughput.

# 3. Conclusion

In this paper, we provide our views on the remaining open issues related to PDSCH demodulation requirements for URLLC with higher BLER. We also provide simulation results for consideration in defining requirements. Our proposals and results are captured below:

PDSCH Slot Aggregation in FR1

Simulation results with PDSCH slot aggregation in FR1

| SNR @1% BLER |  | Ideal | Impairment |
| --- | --- | --- | --- |
| FDD | 2x2 | -1.0 | 1.0 |
|  | 2x4 | -4.9 | -2.9 |
| TDD | 2x2 | -1.2 | 0.8 |
|  | 2x4 | -5.0 | -3.0 |

Proposal #1: The number of slots between final repetition PDSCH and corresponding HARQ-ACK information for FDD: 2.

Proposal #2: In TDD define the number of slots between final repetition PDSCH and corresponding HARQ-ACK information

PDSCH Slot Aggregation in FR2

Simulation results with PDSCH slot aggregation in FR2

| SNR @1% BLER | Ideal | Impairment |
| --- | --- | --- |
| MCS: 13 | -5.5 | -3.5 |
| MCS:16 | -3.5 | -1.5 |
| MCS:19 | -1.9 | 0.1 |

Proposal #3: Define requirements with PDSCH slot aggregation in FR2 with MCS 13 or MCS 16.

PDSCH Mapping Type-B and Processing Capability 2 in FR1

Simulation results for PDSCH Mapping Type B and Processing Capability 2 in FR1

| SNR @70% Max TP |  | Ideal | Impairment |
| --- | --- | --- | --- |
| FDD | 2x2 | -1.3 | 0.7 |
|  | 2x4 | -4.5 | -2.5 |
| TDD | 2x2 | -1.6 | 0.4 |
|  | 2x4 | -4.8 | -2.8 |

PDSCH Mapping Type-B in FR2

Simulation results for PDSCH Mapping Type B in FR2

| SNR @70% Max TP | Ideal | Impairment |
| --- | --- | --- |
| 2x2 (MCS:4) | -0.2 | 1.8 |

Pre-emption Indication

Simulation results for Pre-emption Indication

| SNR @ 70% Max TP |  | 10% Pre-emption |  | 20% Pre-emption |  | Delta with 10% Pre-emption | Delta with 20% Pre-emption |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Without buffer flush | With buffer flush | Without buffer flush | With buffer flush |  |  |
| 2x2 | MCS:4 | -2.6 | -2.6 | -1.8 | -1.8 | 0.0 | 0.0 |
|  | MCS:13 | 6.1 | 5.2 | 11.1 | 6.4 | -0.9 | -4.7 |
|  | MCS:16 | 9.6 | 8.0 | NaN | 9.2 | -1.6 | NaN |
| 2x4 | MCS:4 | -5.8 | -5.9 | -5.3 | -5.3 | -0.1 | 0.0 |
|  | MCS:13 | 2.0 | 1.5 | 3.8 | 2.3 | -0.5 | -1.5 |
|  | MCS:16 | 4.8 | 4.1 | NaN | 4.8 | -0.7 | NaN |

Proposal #4: Introduce requirements for pre-emption indication with 20% pre-empted slots and MCS of 16.

Proposal #5: Define requirements for pre-emption indication at 70% of maximum throughput.

# Reference

- R4-2017509, “Way forward for NR UE URLLC performance requirements with higher BLER”, Intel.