3GPP TSG-RAN WG4 Meeting # 98-e                	R4-2100584

Electronic Meeting, Jan. 25-Feb. 5, 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	2-step RACH simulation results

Agenda item: 	7.18.2

Document for: 	Discussion

Introduction

This paper presents simulation results for the definition of 2-step RACH performance parameters in 38.104 [1], 38.141-1 [2] and 38.141-2 [3]. The parameters for the simulations presented in this paper follow the previously agreed approach during the development of the 2-step RACH work item [4].

Simulation results

Table 1 Simulation parameters

| Parameter | Values for FR1 | Values for FR2 |
| --- | --- | --- |
| Preamble format | Free choice | Free choice |
| Waveform | CP-OFDM | CP-OFDM |
| Power offset between preamble and MsgA | Free choice | Free choice |
| Subcarrier spacing for PUSCH | 15kHz, 30kHz | 60kHz, 120 kHz |
| PUSCH Mapping Type | Type A and Type B | Type B |
| MCS level | 1 | 2 |
| Number of symbols | 14 | 10 |
| Number of PRBs | 2 | 2 |
| TBS | 72 bits | 72 bits |
| DMRS | 1+1+1 | 1+1 |
| Antenna configuration | 1T2R | 1T2R |
| Propagation channel | TDLC300-100 | TDLA30-300 |
| TO values | High level cycling values 15k SCS: [0:0.2:3.8], 30k SCS: [0:0.1:2] | High level cycling values: 60k SCS: [0:0.1,0.6], 120 SCS: [0:0.1,0.5] |
| Test metric (BLER of MsgA when preambles are correctly detected) | 0.01 | 0.01 |
| Re-transmission | No retransmission considered | No retransmission considered |

Table 2 Simulation results for FR1

| Mapping Type | SCS | Ideal [dB] | Impairment [dB] |
| --- | --- | --- | --- |
| TypeA | 15 kHz | 5.7 | 8.2 |
| TypeA | 30 kHz | 5.3 | 7.8 |
| TypeB | 15 kHz | 5.9 | 8.4 |
| TypeB | 30 kHz | 5.4 | 7.9 |

Table 3 Simulation results for FR2

| Mapping Type | SCS | Ideal [dB] | Impairment [dB] |
| --- | --- | --- | --- |
| TypeB | 60 kHz | 6.8 | 9.3 |
| TypeB | 120 kHz | 6.4 | 8.9 |

References

- TS 38.104 V16.6.0, NR; Base Station (BS) radio transmission and reception

- TS 38.141-1 V16.6.0, NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing

- TS 38.141-2 V16.6.0, NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing

- R4-2017580, “WF on BS demodulation requirements for 2-step RACH”, 3GPP TSG-RAN WG4 Meeting #97-e, ZTE