3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100620

# Electronic Meeting, 25 Jan. -5 Feb. 2021

Source: 	Qualcomm Incorporated

Title: 	RF specifications for DLCA n260A_n258A and n259A_n257A based on IBM

Agenda item:	11.3.2.1.2

Document for:	Approval

# Introduction

In RAN4#97e it was agreed [1] that companies would further study the EIS spherical coverage and reference sensitivity relaxations ΔRIB,S,n  and ΔRIB,P,n for DLCA CA_ n260A_n258A and CA_n259A_n257A power class 3 and determine whether the same values adopted for CA_ n260A_n261A could be used for these cases.  This paper presents ΔRIB,S,n  and ΔRIB,P,n proposals for these new CA combinations.

# Discussion

In [2] the reference sensitivity and spherical coverage relaxations for DLCA band combination CA_n260A_n261A is given in table 1 and table 2 respectively for power class 3.

| NR CA bands | NR band | ΔRIB,P,n (dB) |
| --- | --- | --- |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |

Table 1: ΔRIB,P,n reference sensitivity relaxation for inter-band CA for power class 3

| NR CA bands | NR band | ΔRIB,S,n (dB) |
| --- | --- | --- |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |

Table 2: ΔRIB,S,n EIS spherical coverage requirement relaxation for inter-band CA for power class 3

In RAN4#97-e some companies [3,4,5] proposed adopting the same ΔRIB,P,n and ΔRIB,S,n values for either one or both of the new DL CA band combinations under investigation as for CA_ n260A_n261A. In  [1] companies were requested to study ΔRIB,P,n and ΔRIB,S,n for the new DLCA band combinations CA_ n260A_n258A and CA_n259A_n257A based on IBM and determine whether the same values could be adopted as for CA_ n260A_n261A.

Our studies indicate that it is possible for DLCA band combinations CA_ n260A_n258A and CA_n259A_n257A to have the same ΔRIB,P,n and ΔRIB,S,n specifications as CA_ n260A_n261A for power class 3. Therefore, the following is proposed:

Proposal 1: Apply same ΔRIB,P,n and ΔRIB,S,n  for CA_ n260A_n258A as for CA_ n260A_n261A for power class 3

Proposal 2: Apply same ΔRIB,P,n and ΔRIB,S,n  for CA_ n259A_n257A as for CA_ n260A_n261A for power class 3

It is expected that the performance of the RF circuitry for these new CA combinations should be similar to that for CA_n260A_n261A. Therefore, it is proposed that the same requirements for maximum power level, ACS and in-band blocking as for CA_ n260A_n261A be used for CA_n260A_n258A & CA_n259A_n257A.

Proposal 3: Use same maximum input level, ACS and in-band blocking requirements for CA_ n260A_n258A as for CA_ n260A_n261A.

Proposal 4: Use same maximum input level, ACS and in-band blocking requirements for CA_ n259A_n257A as for CA_ n260A_n261A.

# Conclusion

In this paper the ΔRIB,S,n  and ΔRIB,P,n  were studied for  CA_ n260A_n258A and CA_n259A_n257A and it was found that the same relaxations as those used for  CA_ n260A_n261A  can be used.  The following proposals are made:

Proposal 1: Apply same ΔRIB,P,n and ΔRIB,S,n  for CA_ n260A_n258A as for CA_ n260A_n261A for power class 3

Proposal 2: Apply same ΔRIB,P,n and ΔRIB,S,n  for CA_ n259A_n257A as for CA_ n260A_n261A for power class 3

Proposal 3: Use same maximum input level, ACS and in-band blocking requirements for CA_ n260A_n258A as for CA_ n260A_n261A.

Proposal 4: Use same maximum input level, ACS and in-band blocking requirements for CA_ n259A_n257A as for CA_ n260A_n261A.

# Reference

- R4-2016916, “WF for UE requirements for CA configurations CA_n258A-n260A and CA_n257A-n259A based on IBM” RAN4#97-e, 2nd -13th  Nov 2020

- TS38.101-2, V16.5.0

- R4-2014589, “UE requirements for CA_258A-n260A and CA_257A-n259A based on IBM”, RAN4#97-e, 2nd -13th  Nov 2020

- R4-2014966, “DL Inter-band CA_n257-n259”, RAN4#97-e, 2nd -13th  Nov 2020

- R4-2015875, “Views on Rel-17 inter-band DL CA in FR2”, RAN4#97-e, 2nd -13th  Nov 2020