3GPP TSG RAN WG4 Meeting #98-e	R4-2100672

Electronic Meeting, 25 Jan. - 5 Feb., 2021

Agenda item:	9.6.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-41-11: EN-DC_1-3-8-11_n28

Document for:	Approval

1	Introduction

EN-DC of 4B LTE and 1B NR of DC_1-3-8-11_n28 was approved in RAN#88[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-8A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A DC_11A_n28A |

3.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows.

Table 2: ΔTIB of constituent 5 band combo.

| ΔTIB | B1+B3+B8+B11 | B1+B3+B8+n28 | B1+B3+B11+n28 | B1+B8+B11+n28 | B3+B8+B11+n28 | Proposed Value |
| --- | --- | --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 | 0.3 | 0.3 |  | 0.3 |
| B3 | 0.8 | 0.3 | 0.8 |  | 0.8 | 0.8 |
| B8 | 0.3 | 0.6 |  | 0.6 | 0.6 | 0.6 |
| B11 | 0.9 |  | 0.9 | 0.4 | 0.9 | 0.9 |
| n28 |  | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |

Table 3: ΔRIB of constituent 5 band combo.

| ΔRIB | B1+B3+B8+B11 | B1+B3+B8+n28 | B1+B3+B11+n28 | B1+B8+B11+n28 | B3+B8+B11+n28 | Proposed Value |
| --- | --- | --- | --- | --- | --- | --- |
| B1 | 0 | 0 | 0 | 0 |  | 0 |
| B3 | 0.3 | 0 | 0.3 |  | 0.3 | 0.3 |
| B8 | 0 | 0.2 |  | 0.2 | 0.2 | 0.2 |
| B11 | 0.5 |  | 0.5 | 0 | 0.5 | 0.5 |
| n28 |  | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 |

Then the proposal for this 4LTE-1NR EN-DC is:

Table 4: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-8-11_n28 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |

Table 5: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-8-11_n28 | 1 | 0 |
|  | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |

5.  REFSENS

MSD of this 5DL/2UL DC configuration are already covered by those of the 4DL/2UL fallback band combinations. Therefore, no additional MSD requirement is needed.

6.  Conclusion

This paper proposes study results of EN-DC 1-3-8-11_n28.

TP is shown in Annex.

7.  Reference

[1]  RP-201371  New WID on Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL), Nokia, Nokia Shanghai Bell

Annex.   Proposed text for TR37.717-41-11: LTE 4B -NR 1B DC

[Unchanged Parts Skipped]

5.1	Inter-band EN-DC within FR1

## 5.1.xx	DC_1-3-8-11_n28

### 5.1.xx.1	Configurations for EN-DC

5.2B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-8A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A DC_11A_n28A |

### 5.1.xx.2	∆TIB and ∆RIB values

For DC_1-3-8-11_n28, the TIB,c and RIB,c values are given in the tables below.

6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8-11_n28 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |

7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8-11_n28 | 1 | 0 |
|  | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |

### 5.1.xx.3	Reference sensitivity exceptions

Co-existence study for DC_1-3-8-11_n28 was covered by the studies for the fallback modes of DC_1-3-8_n28, DC_1-3-11_n28, DC_1-8-11_n28 and DC_3-8-11_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

[Unchanged Parts Skipped]