3GPP TSG-RAN WG4 meeting #98-e	R4-2101419

Electronic Meeting, 25 January – 5 February 2021

Agenda Item:	7.10.1.1

Source:	Ericsson

Title:	Simulation results of DL 256QAM in FR2

Document for:	Information

# 1	Introduction

RAN4#97-e agreed with the way forward on UE demodulation and CSI reporting for FR2 DL 256QAM [1]. This contribution provides our alignment and impairment simulation results according to the simulation parameters in the way forward.

# 2	Simulation results

Table 1 shows the simulation parameters according to [1]. We should note that that PDCCH aggregation level should be 1 or 2 because of 1 OFDM symbol and 32 PRB for CORESET, although it does not affect directly to the simulation results.

Figure 1 compares the ideal simulation results among AWGN, TDLA30-300 and TDLD-75. Table 2 summarizes the SNR levels to achieve 70% of the maximum throughput.

Table 1	Simulation parameters

| Parameters | Values |
| --- | --- |
| Tx EVM | 3% |
| SCS and CBW | 120kHz, 50MHz (32PRB) |
| Antenna configuration | 2x2 |
| Rank | 1 |
| MCS | 20 |
| TDD configuration | DDDSU, S=10D:2G:2U |
| PDCCH configuration | Symbol #0 (AL1 or AL2?) |
| PDSCH configuration | Mapping type A DL slots: start symbol 1, duration 13 Special slots: start symbol 1, duration 9 |
| PDSCH PRB bundling size and precoding | PRB bundling size: 2 Random precoding, per slot, WB granularity |
| PDSCH DMRS configuration | Type 1, 1+1 |
| TRS configuration | Periodicity: 20ms offset: 10ms |
| Overhead for TBS determination | 6 |
| HARQ RV sequence | {0, 2, 3, 1} |
| Propagation channel | Option 1: TDLD30-75 Option 2: TDLA30-300 |

Figure 1	Comparison of DL 256QAM in FR2 with different channel condition.

Table 2	Summary of simulation results for DL 255QAM in FR2.

| Channel condition | SNR (dB) to achieve 70% of maximum throughput (Alignment results) | SNR (dB) to achieve 70% of maximum throughput (Impairment results) |
| --- | --- | --- |
| TDLA30-300 | 17.5 | 19.5 |
| TDLD30-75 | 16.1 | 18.1 |
| AWGN (reference) | 15.5 | 17.5 |

# 3	Summary

This contribution provided our alignment simulation results for PDSCH with DL 256QAM in FR2. We would like to consider these results for the final PDSCH demodulation requirements.

# References

- R4-2017536, “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, China Telecom​.