3GPP TSG-RAN4 Meeting #98-e	R4-2101710

# Electronic Meeting, 25 Jan -05 Feb, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1659 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on test cases of DL interruptions at switching between two uplink carriers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Perf |  |  |  |  |  | Date: |  |  | 2021-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In some test cases, there are different SCS for different cells, however the unit of SS-RSRP is dbm/15kHz. Even in the test that there is only one configuration with 30kHz SCS, the unit of SS-RSRP is still dbm/15kHz. This is not intuitive and may lead to misunderstand. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the unit of SS-RSRP to dBm/SCS and accordling correct the RSRP value. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is vaue . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.5.8; A.6.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1 ------------------------------------------------------------

A.4.5.8	DL Interruptions at switching between two uplink carriers

A.4.5.8.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink carriers defined in clause 8.2.1.2.14. The test case is applicable for an uplink band pair of an inter-band EN-DC configuration when the capability uplinkTxSwitchingPeriod is present.

There are two cells: E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2). The test parameters for PSCell are given in Table A. 4.5.8.1-1, Table A. 4.5.8.1-2 and Table A. 4.5.8.1-3 below.

Aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the symbol#5 if UE capability uplinkTxSwitchingPeriod is 140us or symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us on the special slot on NR TDD carrier (Cell 2). The test parameters and applicability for E-UTRAN PCell are defined in A.3.7.2. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, uplinkTxSwitching is indicated to UE. This test verifies that the UE correctly report the L1-RSRP reporting.

Table A. 4.5.8.1-1: Supported test configurations

| Configuration | PSCell (Cell2) |
| --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A. 4.5.8.1-2: General test parameters for DL Interruptions at switching between two uplink carriers in EN-DC

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2 | Two radio channels are used for the test. |
| Active cell |  | Config 1 | Cell 1: E-UTRAN PCell Cell 2: FR1 PSCell | E-UTRAN PCell on RF channel number 1 FR1 PSCell on RF channel number 2 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A. 4.5.8.1-3: NR Cell specific test parameters for DL Interruptions at switching between two uplink carriers in EN-DC (Cell 2)

| Parameter |  | Unit | Cell2 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |
| Duplex mode | Config 1 |  | TDD |
| TDD configuration | Config 1 |  | TDDConf.2.1 except that: S=’10DL:2GP:2UL’; nrofDownlinkSymbols:10 nrofUplinkSymbols: 2 |
| BWchannel | Config 1 |  | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |
| UL dedicated BWP configuration |  |  | ULBWP.1.1 |
| SRS configuration |  |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.2.1 TDD |
| RMSI CORESET parameters | Confiq 1 |  | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 |
| SMTC Configuration |  |  | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 low |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -84 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| NocNote 2 | Config 1 | dBm/SCS | -101 |
| IoNote3 | Config 1 | dBm/ 38.16MHz | -52.86 |
| Time offset to Cell1 Note 5 |  | s | 0 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

A.4.5.8.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.1.2.14.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.1.

The rate of correct events observed during repeated tests shall be at least 90%.

------------------------------------------------------------- End of change 1 ------------------------------------------------------------

----------------------------------------------------- Beginning of Change 2 ------------------------------------------------------------

A.6.5.7	DL interruptions at switching between two uplink carriers

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA

A.6.5.7.1.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink carriers defined in clause 8.2.2.2.10. The test case is applicable for an uplink band pair of an inter-band FDD-TDD CA configuration when the capability uplinkTxSwitchingPeriod is present.

There are two cells: FR1 FDD PCell (Cell 1), FR1 TDD SCell (Cell 2). The test parameters for the two cells are given in Table A.6.5.7.1.1-1, Table A.6.5.7.1.1-2 and Table A.6.5.7.1.1-3 below.

For NR FDD carrier (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the symbol #8 if UE capability uplinkTxSwitchingPeriod is 210us or symbol #9 if UE capability uplinkTxSwitchingPeriod is 140us or symbol #10 if UE capability uplinkTxSwitchingPeriod is 35us in the slot overlapping with the special slot of the NR TDD carrier. For NR TDD carrier (Cell 2), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us in the special slot. This test verifies that the UE correctly report the L1-RSRP reporting. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, uplinkTxSwitching is indicated to UE.

Table A.6.5.7.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7.1.1-2: General test parameters for DL interruptions at switching between two uplink carriers in FDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2 | Two radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.1.5 FDD Cell 2: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7.1.1-3: Cell specific test parameters for DL interruptions at switching between two uplink carriers in FDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | TDD |
| TDD configuration | Config 1 |  | N/A | TDDConf.2.1 except that: S=’10DL:2GP:2UL’; nrofDownlinkSymbols:10 nrofUplinkSymbols: 2 |
| BWchannel | Config 1 |  | 10 MHz: NRB,c = 52 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0resourceMappingnrofSymbols: n2 | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.1 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -87 | -84 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - |
|  |  | dBm/ 38.16MHz | - | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - | 0 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |

A.6.5.7.1.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.1.

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA

A.6.5.7.2.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink carriers defined in clause 8.2.2.2.10. The test case is applicable for an uplink band pair of an inter-band TDD-TDD CA configuration when the capability uplinkTxSwitchingPeriod is present.

There are two cells: FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2). The test parameters for the two cells are given in Table A.6.5.7.2.1-1, Table A.6.5.7.2.1-2 and Table A.6.5.7.2.1-3 below.

For NR TDD PCell (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us on the special slot. For NR TDD SCell (Cell 2), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us on the 2nd special slot of every 8 slots. This test verifies that the UE correctly report the L1-RSRP reporting. The test case is only applicable to UE which supports simultaneousRxTxInterBandCA.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, uplinkTxSwitching is indicated to UE.

Table A.6.5.7.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7.2.1-2: General test parameters for DL interruptions at switching between two uplink carriers in TDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2 | Two radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.2.5 TDD Cell 2: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7.2.1-3: Cell specific test parameters for DL interruptions at switching between two uplink carriers in TDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | TDD | TDD |
| TDD configuration | Config 1 |  | TDDConf.2.1 except that S=’10DL:2GP:2UL’; nrofDownlinkSymbols:10 nrofUplinkSymbols: 2 | TDDConf.2.2 |
| BWchannel | Config 1 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -87 | -84 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - |
|  |  | dBm/ 38.16MHz | - | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - | 0 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |

A.6.5.7.2.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.1.

The rate of correct events observed during repeated tests shall be at least 90%.

------------------------------------------------------------- End of change 2 ------------------------------------------------------------