3GPP TSG-RAN WG4 Meeting #98-e	R4-2102077

Electronic Meeting, 25 January – 5 February 2021

Source:	Ericsson, Rogers

Title:	TP to TR 37.717.11-21: Addition of DC configurations for DC_2_n2-n66

Agenda item:	9.7.2

Document for:	Approval

# Background

This text proposal for TR 37.717-11-21 [1] to add DC_2_n2-n66 configurations as defined in WID [2].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_2_n2-n66

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n2A-n66A | DC_2A_ n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

Based on co-existence studies, IMD3 and IMD5 generated by dual uplink of 2_n66 may fall into own Rx of band n2.

### 6.x.4	∆TIB and ∆RIB values

For DC_2_n2-n66, the TIB,c and RIB,c values are derived from DC_2_n66 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n66 | 2 | 0.5 |
|  | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n66 | 2 | 0.3 |
|  | n2 | 0.3 |
|  | n66 | 0.3 |

### 6.x.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 and IMD5 generated by dual uplink of 2_n66 may fall into own Rx of band n2.

For IMD3 MSD of DC_2A_n66A is reused.

IMD5 from dual uplink of 2_n66 can only fall into own Rx of band n2 using the band edge frequencies 1780 (n66 UL) and 1850 (B2 UL). Therefore, test points for MSD for IMD5 cannot be defined.

The required MSD levels and test configuration are shown in the following Table.

Table 6.x.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n2A-n66A | B2 | IMD3 | ¦2*fB2 -fn66¦ | 1875 | 5 | 25 | 1955 | 5 | N/A |
|  | n2 |  |  | 1895 | 5 | 25 | 1975 | 5 | 20 |
|  | n66 |  |  | 1775 | 5 | 25 | 2175 | 5 | N/A |

##### ---End of changes---

# Reference

[1]		R4-2014304, TR37.717-11-21 v0.2.0 for DC LTE x Bands DL_1UL (x=1,2,3,4) and 2 NR bands DL_1UL basket WI_r2, LG Electronics

[2]		RP-202292, Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17, LG Electronics