3GPP TSG-RAN WG4 Meeting#98-bis             	R4-2104403

E-meeting, 12th – 20th  April, 2021

Title:	TP to TR 37.717-31-11: DC_1A-28A-40A_n78A

Source:	Nokia, Telefonica

Agenda item:	7.5.2

Document for:	Approval

# 1	Introduction

This is a TP into TR 37.717-31-11 to introduce DC_1A-28A-40A_n78A.

*********************** Start of TP ***************************************

## 5.1.X		DC_1-28-40_n78

### 5.1.X.1	Configuration for EN-DC

Table 5.1.X.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-28A-40A_n78A | DC_1A_n78A DC_28A_n78A DC_40A_n78A |

### 5.1.X.2	∆TIB and ∆RIB values

E-UTRA CA_1-28-40 has been used as a basis for proposed relaxations.

Table 5.1.X.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28-40_n78 | 1 | 0.6 |
|  | 28 | 0.3 |
|  | 40 | 0.5 |
|  | n78 | 0.8 |

Table 5.1.X.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-28-40_n78 | 1 | 0 |
|  | 28 | 0.2 |
|  | 40 | 0 |
|  | n78 | 0.5 |

### 5.1.X.3	REFSENS requirements

No additional MSD requirement need to be defined for this EN-DC configuration.

*********************** End of TP ***************************************