3GPP TSG-RAN WG4 #98bis-e	R4-2104439

e-Meeting, Apr 12 – 20, 2021

Agenda item:	8.12.6

Source: 	Qualcomm Incorporated

Title: 	Simulation results for NR coexistence study: indoor deployment at 60GHz

Document for:	Discussion

# Introduction

In this contribution we present preliminary simulation results for NR adjacent channel coexistence study at 60GHz. The paper addresses different indoor deployments where both uplink (UL) and downlink (DL) cases are analysed. The indoor simulation assumptions adopted in this study follow the ones considered in [1].

# Discussion

The simulation results presented in this paper refer to two different indoor layouts; namely scenario indoor-A and indoor-C, with a carrier frequency of 60GHz, as explained in detail in [1]. Both scenarios are based on the indoor 50m x 120m open space layout with a total of 12 base stations (BSs) per operator, 2 operators, with BS height at 3m (ceiling), and UE height of 1m. For scenario A, BSs are randomly deployed within a 10m x 10m virtual box with a minimum distance of 2m between BSs of the different operators. Additionally, x-axis inter-site distance (ISD) equals 20m and y-axis ISD equals 25m, where ISD is define by the distance between two the adjacent virtual boxes. A pictorial representation of scenario A is shown in Figure 1.

Figure 1. Indoor open office layout in scenario A.

On the other hand, scenario C considers a fixed BS location with ISD equals 20m as shown in Figure 2. In scenario C, we consider a coordinated deployment of the 2 operators (i.e., victim and aggressor BSs are spatially overlaid).

Figure 2. Indoor open office layout in scenario C.

The list of the simulation assumptions for the considered scenarios is reported in Table 1. In addition, we follow the simulation methodology as Section 5.3 in [4], where RF parameters are determined based on the degradation cause by adjacent channel interference (ACI).

Table 1 List of system level simulation parameters

| Deployment |  | Indoor Office A and C |
| --- | --- | --- |
| System Parameters | Carrier Frequency | 60 GHz |
|  | Channel BW | 400 and 2000 MHz |
|  | Number of active UEs | 1 |
|  | Channel model | InH open office specified in [2] |
| BS | (Mg, Ng, M, N, P) | (1,1,4,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-7 in [3] for ceiling mount |
|  | Max Tx Power | 20 dBm (≈EIRP = 43 dBm) |
|  | Noise Figure | 13 dB |
| UE | (Mg, Ng, M, N, P) | (1,2,2,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Table A.2.1-8 in [2] |
|  | Max Tx Power | 8 dBm (≈EIRP = 28 dBm) |
|  | Noise figure | 13 dB |
|  | LoS/ NLoS | LoS probability for indoor office defined in [4] |

## DL results

In this section we present the preliminary DL results for the two indoor deployments, under the 400 and 2 GHz channel bandwidth configurations, where the measurements are conducted at the victim UEs and the aggressor nodes are BSs operating in adjacent channel. First the signal to noise (SNR) and signal to interference and noise ratio (SINR) cumulative distribution function (CDF) curves are shown in Figure 3. We observe that as the bandwidth narrows, the SINR and SNR are improved due to the impact of higher thermal noise in the system (around 7 dB gap between the SNR curves for the 400 MHz and 2 GHz channel bandwidth). For the tail of the SINR curves, the bandwidth’s impact is negligible as the system is limited by the cochannel and adjacent interference.

Figure 3 Indoor scenario A and C SINR and SNR CDFs DL for 60GHz

Figure 4 shows the DL throughput (TP) degradation as a function of the adjacent channel interference ratio (ACIR), where the mean and the 5%-tile TP are plotted. The results show that the TP loss is insensitive to the channel bandwidth. Moreover, we observe around 6.7 % mean TP loss degradation between scenario A and C for ACIR = 10 dB. This is attributed to the fact that the interference is less aggressive in Scenario A deployment due to the randomized locations of the BSs. The DL ACIR requirements for the different deployments that ensure at most a 5% TP loss are listed in Table 2.

Figure 4. Indoor scenario A and C DL throughput degradation as a function of ACIR.

Table 2 DL ACIR requirement for 5% mean TP loss

| Case | ACIR requirement for 5% DL throughput loss |
| --- | --- |
| Scenario A/C with channel bandwidth = 400MHz | 15 dB |
| Scenario A/C with channel bandwidth = 2GHz | 15 dB |

## UL results

In this section we present the preliminary UL results, where the measurements are conducted at the victim BSs and the aggressor nodes are UEs operating in adjacent channel. Like the previous section, we start with the SNR and SINR curves as shown in Figure 5. We observe a nearly identical performance between the two scenarios as well as the bandwidth configurations. The convergence of the SINR at the 15 dB point is attributed to the employed power control mechanism, which follows the methodology presented in [5]. It is up to discussions if an SNR target of 15 dB is sufficient or should RAN 4 investigates higher values for the 60GHz coexistence studies.

Figure 5 Indoor scenario A and C SINR and SNR CDFs UL for 60GHz

Figure 6 shows the UL TP degradation as a function of ACIR, where mean and 5%-tile throughput are considered. Similar to the DL results, the results show that the mean TP loss is insensitive to the channel bandwidth. Moreover, we observe a round 1.64 % mean TP loss degradation between scenario A and C for ACIR of 10 dB. The UL ACIR requirements that ensure at most a 5% TP loss are listed in Table 2.

Figure 6. Indoor scenario A and C UL throughput degradation as a function of ACIR.

Table 2 UL ACIR requirement for 5% mean TP loss

| Case | ACIR requirement for 5% UL throughput loss |
| --- | --- |
| Scenario A (C) with channel bandwidth = 400MHz | 12 dB (13.7 dB) |
| Scenario A (C) with channel bandwidth = 2GHz | 12 dB (13.7 dB) |

# Conclusions

In this contribution we provided simulation results for NR coexistence study in two indoor scenario deployments for a carrier frequency of 60GHz for both DL and UL cases were analysed. Based on the results obtained so far, we have the following observations:

Observation 1:For DL case, an ACIR of 15 dB would be enough to keep degradation due to ACI within 5% loss region.

Observation 2: For UL case, an ACIR of 13.7 dB would be enough to keep degradation due to ACI within 5% loss region.

# Reference

- 3GPP TR 38.808: "Study on supporting NR from 52.6 GHz to 71 GHz".

- 3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

- 3GPP TR 38.802: "Study on New Radio Access Technology Physical Layer Aspects".

- 3GPP TR 38.803: "Study on new radio access technology: Radio Frequency (RF) and co-existence aspects ".

- R4-167751, “On the Uplink power control for NR coexistence study”, Qualcomm Incorporated.