3GPP TSG-RAN4 Meeting #98-bis-e			R4-2104440

Electronic Meeting, 12 April – 20 April, 2021

3GPP TSG RAN WG1 Meeting #102-e                                                                              R1-2007428

E-meeting, August 17 - 28, 2020

Title:	Reply LS on CSI-RS only beam correspondence

Response to: 	R1-2006952 (R4-2009167)

Release:	Rel-16

Work Item:	NR_RF_FR2_req_enh

Source:	RAN WG1, Huawei

To:	RAN WG4

CC:

Contact Person:

#### Name:	Xi Zhang

E-mail Address:	panda dot zhang {at} Huawei dot com

1. Overall Description:

RAN1 would like to thank RAN4 for the LS on CSI-RS only beam correspondence. RAN1 discussed these questions and provides the corresponding answers as below:

Q1: Does RAN1 consider it valid scenario(s) that P1 CSI-RS has no QCL relation for Rel-16? If valid, what are the corresponding usage scenarios? Has RAN1 analysed impact on UE mobility, scheduling restriction and overhead, UE power consumption, etc., with respect to number of active TCI states?

A1: P1 CSI-RS without QCL-TypeD source is supported by specifications. While there are multiple companies which consider there is valid scenarios(s) that P1 CSI-RS has no QCL relation for Rel-16 (such as P1 beam alignment, and inter-cell cooperation), there are a smaller number of companies which consider the scenario(s) as invalid. Consequently, RAN1 has no conclusion on valid scenarios(s) that P1 CSI-RS has no QCL relation for Rel-16. RAN1 did not analyze impacts on UE mobility, scheduling restriction and overhead, and UE power consumption, with respect to number of active TCI states.

Q2: Whether there is UE behaviour definition/expectation when P1 CSI-RS QCL relation is configured as ‘none’ and to SSB? For example: are resource prioritization rules or default QCL assumption rules when overlapped with other resources (e.g. PDCCH, DMRS) established for both scenarios?

A2: For P1 CSI-RS without QCL-TypeD source, there is no UE behavior/expectation explicitly defined in RAN1, and hence it is up to UE implementation. For P1 CSI-RS with SSB as QCL-TypeD source, the UE expectation is defined in RAN1, i.e., spatial Rx parameters for P1 CSI-RS can be inferred from those of the QCLed SSB.

Q3: Whether CSI-RS for beam management as the first level of beam measurement/indication in connected mode is specified if it has no QCL relation to SSB?

A3: CSI-RS for beam management without SSB as QCL-TypeD source can be used for beam measurement and indication.

2. Actions:

To RAN4

ACTION: RAN1 would like to respectfully ask RAN4 to take the above information into account.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #103-e	26 Oct - 13 Nov 2020, 	             Online

TSG-RAN WG1 Meeting #104	1 - 5 Mar 2021, 		Athens, GR