# 3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2104542

Electronic Meeting, April 12th -20th, 2021

Source: 	vivo

Title: 	Discussion and reply LS on Half-duplex FDD switching time for RedCap UE

Agenda Item:	12.1

Document for:	Discussion

Introduction

In RAN4#98-bis-e, an LS [1] was received from RAN1 to ask RAN4 group to feedback on RAN1’s working assumption on reusing current switching time definition for UE not capable of full duplex in TS 38.211 for HD-FDD.

In this contribution, we reviewed the background of the requirements for existing switching time, the scenario in RedCap, and provided a brief analysis for the applicability of the requirements for HD-FDD.

Discussion

Background of Current requirements

Current 38.211 have the following requirements:

A UE not capable of full-duplex communication is not expected to transmit in the uplink earlier than NRx-TxTc after the end of the last received downlink symbol in the same cell where NRx-Tx is given by Table 4.3.2-3.

A UE not capable of full-duplex communication is not expected to receive in the downlink earlier than NTx-RxTc after the end of the last transmitted uplink symbol in the same cell where NTx-Rx is given by Table 4.3.2-3.

Table 4.3.2-3: Transition time NRx-Tx and NTx-Rx

| Transition time | FR1 | FR2 |
| --- | --- | --- |
| NTx-Rx | 25600 | 13792 |
| NRx-Tx | 25600 | 13792 |

These requirements are from RAN4, and the detailed background analysis can be found in [2][3]. The final outcome is an LS [4]. It should be noted that the whole analysis is based on TDD system, and the current RAN1 spec is solely for TDD system (including BS and UE). Some of the key part were also listed below:

As analysed in [2], TDD system needs Timing advance of UL symbols to cover BS/UE Tx<->Rx transition time and provide a time cushion to avoid various interference between BSs. In [3], details of the transition time were analysed, one of the most important timeline illustration at BS and UE side is the following figure which is excerpted below:

0

1

2

3

4

11

12

13

BS Timeline

0

1

TAoffset

Slot

0

1

2

3

4

11

12

13

UE Timeline

0

1

Tprop

Tprop

Tprop+TAoffset

2

2

TRXTX

TTXRX

Figure 2.1: Format 34 Symbol Timeline

The key restriction is that for the UL-> DL, that is Tx->Rx from UE side, TAoffset is needed as minimum Gap, to enable BS do Rx->Tx and UE do Tx->Rx transition, the final requirements are based on this part

For DL->UL transition, the same requirements were used based on UL->DL.

In addition, for the value of TA offset, the current 38.133 has the following definition:

It should be noted that LTE FDD do not have TA offset since FDD system do not really need it since no Tx<->Rx transition is needed for both BS and UE. However, for NR, TA offset is applied for both FDD/TDD in case there is no co-existence issue. The number here is aligned.

HD-FDD in RedCap

In TR 38.875, there is the following definition for HD-FDD:

-    For Type A HD-FDD, a high proportion of the cost saving occurs because the duplexer can be replaced with a switch and a lowpass filter.

-    For Type B HD-FDD, uplink and downlink can share one local oscillator, therefore, some additional saving on RF transceiver can be obtained.

It should be noted that for HD-FDD, the BS part is still full duplex and there is no Tx<->Rx transitions and therefore is no gap between thereof.

Applicability Analysis

For a RedCap UE, type A HD-FDD could be no duplex filter and there would be Tx<->Rx interference in case of simultaneous Tx/Rx. So the interference scenario is similar to TDD, and there is a need for UE Tx<->Rx transition in case of UL->DL and DL->UL.

In addition, since the duplexer was changed to switch/lowpass filter, the architecture is also become quite similar to TDD case. There is separate LO for uplink and downlink for Type A. So, the transition time can also be regarded as similar to TDD.

Observation 1: HD-FDD UE also needs Tx<->Rx transition time to avoid self-interference which is similar to TDD.

Observation 2: HD-FDD UE architecture would be similar to TDD and the actual transition time is also similar.

Based on those observations, here is the following proposal:

Proposal: Reuse current Transition time NRx-Tx and NTx-Rx for HD-TDD is technically reasonable. It is proposed to reply RAN1 based on this understanding.

Conclusion

In this paper, the LS is discussed and following observation and proposal is provided.

Observation 1: HD-FDD UE also needs Tx<->Rx transition time to avoid self-interference which is similar to TDD.

Observation 2: HD-FDD UE architecture would be similar to TDD and the actual transition time is also similar.

Proposal: Reuse current Transition time NRx-Tx and NTx-Rx for HD-TDD is technically reasonable. It is proposed to reply RAN1 based on this understanding.

A draft reply LS is attached.

- References

[1] R1-2102146, LS on Half-duplex FDD switching time for RedCap UE, RAN1#104-e

[2] R4-1800865, On gNB Cell Frame Synchronization Accuracy Requirements, Qualcomm, RAN4#AH-1801

[3] R4-1802447, UE RX/TX Transition Times, Qualcomm, RAN4#86

[4] R4-1805766, LS on UE Rx/Tx Transition Times, Qualcomm, RAN4#86bis

- Annex

# 3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-21xxxxx

Electronic Meeting, April 12th -20th, 2021

Title:	[Draft] Reply LS on Half-duplex FDD switching time for RedCap UE

Response to:	(R1-2102146) LS on Half-duplex FDD switching time for RedCap UE

Release:	Rel-17

Work Item:	NR_redcap

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name: Sanjun Feng

E-mail Address: fengsanjun@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on minimum requirements for LS on Half-duplex FDD switching time for RedCap UE regarding the working assumption for RedCap UEs:

- For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

RAN4 discussed the LS and came to the conclusion:

This working assumption is technically reasonable from RAN4’s perspective.

2. Actions:

To RAN5:

ACTION: RAN4 respectfully asks RAN1 to take the above information into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #99-e                   19-27, May, 2021          E-meeting

TSG-RAN WG4 Meeting #100-e	23 - 27 Aug 2021         E-meeting