3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2104551

Electronic meeting, 12th – 20th April, 2021

Source:	Ericsson

Title:	Simulation results for NR-U PUCCH demodulation requirements

Agenda item:	5.1.4.4.3

Document for:	information

# Introduction

In RAN4#98-e meeting, companies got agreements on all PUCCH simulation assumptions [1]. This contribution delivers some simulation results based on our analysis and proposals [2].

# Simulation assumptions and results

### 2.1	PUCCH Format 0

Table 2.1.1 Simulation assumptions for PF0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits Note 1 | 1 |
| Number of symbol | 1 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note1 |
| Propagation conditions | TDLA30-10 low |
| Test metric | - SNR@Prob(ACK miss)≤10^(−2) Note 3 |
| Note 1: The UCI information only include HARQ-ACK and the bit pattern is [0]. Note 2: RBs 0, 10, 20,…,100 are allocated for 15kHz if agreed and RBs 0,5,10,…,50 are allocated for 30kHz. Note 3: The DTX to ACK probability shall not exceed 1% |  |

Table 2.1.2 Simulation results for PF0

| ePF0 |  | SNR @ ACK missed detection probability = 10^(-2) |
| --- | --- | --- |
| SCS = 15 kHz | ideal | -5.2 |
|  | impairment | -2.7 |
| SCS = 30 kHz | ideal | -5.1 |
|  | impairment | -2.6 |

### 2.2	PUCCH Format 1

Table 2.2.1 Simulation assumptions for PF1

| Parameter | Test |
| --- | --- |
| Number of information bits Note 1 | 2 |
| Number of symbols | 14 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note2 |
| Propagation conditions | TDLA30-10 low |
| Test metric | - SNR@Prob(ACK miss)≤10^(−2)) SNR@Prob(NACK→ACK)≤10^(−3)) Note3 |
| Note 1: The UCI information only contain HARQ-ACK and the bit pattern is [0 1]. Note 2: RBs 0, 10, 20,…,100 are allocated for 15kHz if agreed and RBs 0,5,10,…,50 are allocated for 30kHz. Note 3: The DTX to ACK probability shall not exceed 1% |  |

Table 2.2.2 Ideal simulation results for PF1

| ePF1 |  | SNR @ ACK missed detection probability = 10^(-2) | SNR @ NACK to ACK detection probability = 10^(-3) |
| --- | --- | --- | --- |
| SCS = 15 kHz | Ideal | -16.3 | -16.1 |
|  | Impairment | -13.8 | -13.6 |
| SCS = 30 kHz | Ideal | -16.4 | -16.1 |
|  | Impairment | -13.9 | -13.6 |

### 2.3	PUCCH Format 2

Table 2.3.1 Simulation assumptions for PF2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Intra-slot frequency hopping | N/A |
| Number of symbols | 1 |
| The number of UCI information bits Note 1 | 22 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 Note 2 |
| Propagation conditions | TDLA30-10 Low |
| OCC-Length-r16 | Not configured |
| Test metric | SNR @ ProbUCI block BLER=10-2 |
| Note 1: The UCI information only contain HARQ-ACK and CSI part 1, and the bit pattern is [0 1 0 1 … 0 1]. Note 2: RBs 0, 10, 20,…,100 are allocated for 15kHz and RBs 0,5,10,…,50 are allocated for 30kHz. |  |

Table 2.3.2 Ideal simulation results for PF2

| ePF2 |  | SNR @ ProbUCI block BLER=10-2 |
| --- | --- | --- |
| SCS = 15 kHz | Ideal | 1.3 |
|  | Impairment | 2.8 |
| SCS = 30 kHz | Ideal | 1.2 |
|  | Impairment | 2.7 |

### 2.4	PUCCH Format 3

Table 2.4.1 Simulation assumptions for PF3

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Additional DM-RS configuration | No additional DM-RS |
| Number of symbols | 4 |
| The number of UCI information bits Note 1 | 4 bits |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Antenna configuration | 1T2R |
| Number of interlaces | 1 |
| Interlace index | 0Note 2 |
| Propagation conditions | TDLA30-10 Low |
| Index of OCC | Not configure |
| Length of OCC | Not configure |
| Cyclic shift index for DMRS | 0 |
| Test metric | Option 1: SNR @ ProbUCI block BLER=10-2 Option 2: SNR@Prob(ACK miss)≤10^(−2) SNR@Prob(NACK→ACK)≤10^(−3)) Note3 |
| Note 1: The UCI information only contain HARQ-ACK and the bit pattern is [0 1 0 1]. Note 2: RBs 0, 10, 20,…,90 are allocated for 15kHz and RBs 0,5,10,…,45 are allocated for 30kHz. Note 3: The DTX to ACK probability shall not exceed 1% |  |

Table 2.4.2 Ideal simulation results for PF3

| ePF2 |  | SNR @ UCI block BLER = 10^(-3) | SNR @ ACK missed detection probability = 10^(-2) | SNR @ NACK to ACK detection probability = 10^(-3) |
| --- | --- | --- | --- | --- |
| SCS = 15 kHz | Ideal | -10.2 | -9 | -10.3 |
|  | Impairment | -7.7 | -6.5 | -7.8 |
| SCS = 30 kHz | Ideal | -10.2 | -9 | -10.2 |
|  | Impairment | -7.7 | -6.5 | -7.7 |

# Conclusion

In this contribution, we deliver our simulation results based on the WF [1] the last meeting.

# References

[1]	R4-2013989, “Way Forward on NR-U PUCCH demodulation requirements”, RAN4#98-e, Ericsson