3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2104552

Electronic meeting, 12th – 20th April, 2021

Source:	Ericsson

Title:	Discussion on remain issue of NR-U PRACH demodulation requirements

Agenda item:	5.1.4.4.4

Document for:	Discussion

# Introduction

In RAN4#98-e meeting, PRACH discussion had advanced dramatically, leaving the merely open issue being the decision on declaration options.  There are two options.

- Create new declaration field for extended PRACH sequences in 38.141-1 which includes format, SCS, and LRA

  - Option 1:

| [D.111] | PRACH format with LRA = 1151 and LRA = 571 and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both. |
| --- | --- | --- |

  - Option 2:

| [D.111] | PRACH format with LRA = 1151 and LRA = 571 and SCS | Declaration of the supported PRACH format(s) with LRA=1151 and LRA=571 as specified in TS 38.211 [17], i.e., format: A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format, as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both, as well as sequence length other than LRA =139, i.e. LRA =1151 for 15 kHz SCS, and LRA =571 for 30 kHz SCS |
| --- | --- | --- |

In this contribution, this issue will be discussed.

# Discussion

The main difference between two options is the declared PRACH format, option 1 only declare support A2, B4, C2 but option 2 declare support all short sequence.

In our understanding, it would be better to align the declaration with the defined requirements because the declared format can be tested in that case. We agreed to introduce only A2, B4, C2. For option 2, if a BS declare to only support format A1, A3, C0, there is no requirement for these formats. That will be a risk for such BS.

To make the declaration clearer, we would like to add small modification on option 1 as follows:

Proposal:

| [D.111] | PRACH format with LRA = 1151 for 15kHz SCS and LRA = 571 for 30kHz SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both. |
| --- | --- | --- |

# Conclusion

Proposal: Create new declaration field for extended PRACH sequences in 38.141-1 which includes format, SCS, and LRA.

| [D.111] | PRACH format with LRA = 1151 for 15kHz SCS and LRA = 571 for 30kHz SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both. |
| --- | --- | --- |

# References

[1]	R4-2103808, “WF on NR-U BS demod PRACH”, Nokia, Nokia Shanghai Bell, RAN4#98-e