3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2104553

Electronic meeting, 12th – 20th April, 2021

Source:	Ericsson

Title:	Simulation results for NR-U PRACH demodulation requirements

Agenda item:	5.1.4.4.4

Document for:	information

# Introduction

In RAN4#98-e meeting, companies got agreements on PRACH simulation assumptions [1]. This contribution delivers some simulation results based on our analysis and proposals [2].

# Simulation assumptions and results

Table 2.1.1 Simulation assumptions for large bandwidth PRACH

| PRACH | PRACH SCS | Time error tolerance |  |  |  |
| --- | --- | --- | --- | --- | --- |
| preamble | (kHz) | AWGN | TDLA30-10 | Lra | Ncs |
| A2, B4, C2 | 15 | 0.52 us | 0.67 us | 1151 | 164 |
|  | 30 | 0.26 us | 0.41 us | 571 | 190 |

Table 2.1.2 Ideal simulation results for PRACH with 15kHz SCS

| Number of |  | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Number of RX antennas | conditions and correlation matrix (Annex G) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0Hz | -24.2 | -26.8 | -24.2 |
|  |  | TDLA30-10 Low | 400 Hz | -17.1 |  | -17.1 |

Table 2.1.3 Impairment simulation results for PRACH with 15kHz SCS

| Number of |  | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Number of RX antennas | conditions and correlation matrix (Annex G) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0Hz | -21.7 | -24.3 | -21.7 |
|  |  | TDLA30-10 Low | 400 Hz | -14.6 |  | -14.6 |

Table 2.1.4 Ideal simulation results for PRACH with 30kHz SCS

| Number of |  | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Number of RX antennas | conditions and correlation matrix (Annex G) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0Hz | -21.3 | -25.5 | -21.3 |
|  |  | TDLA30-10 Low | 400 Hz | -14.6 | -17.1 | -14.6 |

Table 2.1.5 Impairment simulation results for PRACH with 30kHz SCS

| Number of |  | Propagation | Frequency | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TX antennas | Number of RX antennas | conditions and correlation matrix (Annex G) | offset | Burst format A2 | Burst format B4 | Burst format C2 |
| 1 | 2 | AWGN | 0Hz | -18.8 | -23.0 | -18.8 |
|  |  | TDLA30-10 Low | 400 Hz | -12.1 | -14.6 | -12.1 |

# Conclusion

In this contribution, we deliver our simulation results based on the WF [1] in the last meeting.

# References

[1]	R4-2103808, “Way forward on NR-U PRACH demodulation requirements”, RAN4#98-e, Nokia, Nokia Shanghai Bell