3GPP TSG-RAN WG4 Meeting #98bis-e		R4-2104557

Electronic meeting, 12th April – 20th April, 2021

Source:	Ericsson

Title:	Discussion on FR1 256QAM PUSCH demodulation requirement

Agenda item:	8.14.3.1

Document for:	discussion

# Introduction

In Rel-15, the PUSCH demodulation performance requirements for 256QAM are not specified. In this contribution, the simulation assumption for FR1 256QAM PUSCH demodulation requirements will be discussed.

# Discussion

256QAM is not mandatory for all base stations, therefore, the PUSCH demodulation performance requirements for 256QAM can be applied only for BS manufacturers declaring that 256QAM is supported for uplink.

Proposal 1: PUSCH demodulation performance requirements for 256QAM is applied for BS manufacturers declaring that 256QAM is supported for uplink.

To define the PUSCH demodulation performance requirements for 256QAM, it is reasonable to use the Rel-15 simulation parameters for 64QAM as the starting point.

The general parameters as follows are kept same with that for 64QAM,

- Channel bandwidths:

  - 15 kHz SCS: 5, 10, 20 MHz

  - 30 kHz SCS: 10, 20, 40, 100 MHz

- TDD UL-DL pattern:

  - 15 kHz SCS: 3D1S1U, S=10D:2G:2U

  - 30 kHz SCS: 7D1S2U, S=6D:4G:4U

- PUSCH mapping type: A and B

- Number of Tx antenna: 1

- Number of Rx antenna: 2, 4, 8

- Test metric: 70% of max. throughput

256QAM is targeted for high throughput, but DFT-S-OFDM is for ensuring the coverage typically. Therefore, DFT-S-OFDM is not necessary to be considered for 256QAM.

256QAM will operate in a scenario with high SNR, so the parameters related to the PUSCH transmission setting can be kept the same as for 64QAM, and more additional DMRS is not necessary. In that case, for 256QAM,

- DMRS:

  - 1-symbol DMRS

  - DMRS config. type 1

  - Front-loaded DMRS + 1 additional DMRS

- RB assignment: full bandwidth

- Max. number of HARQ transmissions: 4 with RV sequence {0, 2, 3, 1}

The main difference between 64QAM and 256QAM is the code rate/MCS setting. As discussed in LTE UL and NR DL, coderate 5/6 is typical case for 256QAM, so it can also be considered for 256QAM as the starting point, the corresponding MCS of which is #24. Thus for 256QAM

- MCS: #24 as the starting point

Proposal 2: To define the demodulation requirements for 256QAM on FR1 UL, simulation assumptions defined for that for 64QAM can be reused, except that a new corresponding MCS needs to be considered, e.g. MCS#24 in the Rel-15 256QAM-specific MCS table as the starting point.

# Conclusions

In this contribution, our proposal on the simulation assumption for specifying the PUSCH performance requirements for 256QAM is given as follows, and detailed parameters are listed in the table 1.

Proposal 1: PUSCH demodulation performance requirements for 256QAM is applied for BS manufacturers declaring that 256QAM is supported for uplink.

Proposal 2: To define the demodulation requirements for 256QAM on FR1 UL, simulation assumptions defined for that for 64QAM can be reused, except that a new corresponding MCS needs to be considered, e.g. MCS#24 in the Rel-15 256QAM-specific MCS table as the starting point.

Table 1: Parameters for PUSCH performance requirements for 256QAM

| Parameter |  | Value |
| --- | --- | --- |
| Bandwidth |  | 15kHz SCS: 5, 10, 20 MHz 30kHz SCS: 10, 20, 40, 100 MHz |
| CP |  | Normal |
| Transform precoding |  | Disabled |
| Duplex mode |  | FDD, TDD |
| Default TDD UL-DL pattern |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | 1-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| TD-RA | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| FD-RA | RB assignment | Full applicable test bandwidth |
|  | FH | Disabled |
| CBG-based PUSCH transmission |  | Disabled |
| MCS |  | #24 as the starting point |
| Number of Tx antennas |  | 1 |
| Number of Rx antennas |  | 2, 4, 8 |
| Channel |  | TDLA30-10 Low |
| Test metric |  | 70% of max. throughput |