3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2104624

# Electronic Meeting, Apr. 12-20, 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	NR-U PUCCH simulation results

Agenda item: 	5.1.4.4.3

Document for: 	Discussion

Introduction

This paper presents simulation results of PUCCH with enhanced formatted for operation on unlicensed bands [1].

Simulation assumptions

Table 1 Simulation assumptions for PUCCH format 0 analysis

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of symbol | 1 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 Low |
| Test metric | SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 Prob(Ack missed detection) ≤ 10-2 |

Table 2 Simulation assumptions for PUCCH format 1 analysis

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of symbols | 14 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| OCC index (timeDomainOCC) | 0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 Low |
| Test metric | SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 Prob(Ack missed detection) ≤ 10-2 SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 ProbPUCCH NACK→ACK bits ≤ 10-3 |

Table 3 Simulation assumptions for PUCCH format 2 analysis

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Intra-slot frequency hopping | N/A |
| Number of symbols | 1 |
| The number of UCI information bits | 22 bits |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 Low |
| OCC-Length-r16 | Not configured |
| Test metric | SNR @ ProbUCI block BLER=10-2 |

Table 4 Simulation assumptions for PUCCH format 3 analysis

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Additional DM-RS configuration | No additional DM-RS |
| Number of symbols and UCI information bits | 4 bits and 4 symbols |
| Channel bandwidth | 20MHz |
| SCS | 15kHz 30kHz |
| Antenna configuration | 1T2R |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLA30-10 Low |
| Index of OCC | n0 |
| Length of OCC | n1 |
| Cyclic shift index for DMRS | 0 |
| Test metric | SNR @ UCI block BLER=10-2 SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 Prob(Ack missed detection) ≤ 10-2 |

Simulation results

Table 5 Simulation results for interlaced PUCCH format 0

| Format | Propagation conditions | SCS | Test metric | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- |
| Format 0 | TDLA30-10 | 15 kHz | 1% ACK miss | -4.7 | -2.2 |
|  |  | 30 kHz | 1% ACK miss | -3.6 | -1.1 |

Table 6 Simulation results for interlaced PUCCH format 1

| Format | Propagation conditions | SCS | Test metric | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- |
| Format 1 | TDLA30-10 | 15 kHz | 1% ACK miss | -16.5 | -14 |
|  |  |  | 0.1% NACK to ACK | -15.5 | -13 |
|  |  | 30 kHz | 1% ACK miss | -15.6 | -13.1 |
|  |  |  | 0.1% NACK to ACK | -14.6 | -12.1 |

Table 7 Simulation results for interlaced PUCCH format 2

| Format | Propagation conditions | SCS | UCI bits | OCC length | Test metric | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 2 | TDLA30-10 | 15 kHz | 22 | 1 | 1% UCI BLER | 1.5 | 4 |
|  |  | 30 kHz | 22 | 1 | 1% UCI BLER | 2.2 | 4.7 |

Table 8 Simulation results for interlaced PUCCH format 3

| Format | Propagation conditions | SCS | UCI bits | OCC length | Test metric | Ideal | Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 3 | TDLA30-10 | 15 kHz | 4 | 1 | 1% ACK miss | -5.3 | -2.8 |
|  |  |  |  |  | 1% UCI BLER | -10.1 | -7.6 |
|  |  | 30 kHz | 4 | 1 | 1% ACK miss | -4.3 | -1.8 |
|  |  |  |  |  | 1% UCI BLER | -9.2 | -6.7 |

Conclusion

In this contribution we provided interlaced PUCCH simulation results for NR-U.

References

- R4-2103989, Way forward on PUCCH demodulation requirements, 3GPP TSG-RAN WG4 Meeting #98-e, Ericsson

- R4-2014942, PUCCH Demodulation performance requirements for operation in unlicensed bands, 3GPP TSG-RAN WG4 Meeting #97-e, Nokia

- R4-2100576, PUCCH Demodulation performance requirements for operation in unlicensed bands, 3GPP TSG-RAN WG4 Meeting #98-e, Nokia

- R4-2104623, PUCCH Demodulation performance requirements for operation in unlicensed bands, 3GPP TSG-RAN WG4 Meeting #98-bis-e, Nokia