3GPP TSG-RAN WG4 Meeting # 98-bis-e                	R4-2104625

Electronic Meeting, Apr. 12-20, 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	PRACH Demodulation performance requirements for operation in unlicensed bands

Agenda item: 	5.1.4.4.4

Document for: 	Discussion

Introduction

This paper extends the discussion with Nokia’s views on PRACH BS  demodulation requirements on [3] considering the outcome of RAN4 #98-e meeting on that topic [1]. The discussion on this paper is supported by preliminary simulation results and a test scenario is proposed to be used as baseline for discussion on RAN4 in [4].

Applicability rules for PRACH

Current PRACH applicability rules for Normal PRACH in 38.141-1 are [5]:

| D.103 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both. |
| --- | --- | --- |

The manufacturer declaration D.103 currently doesn’t include an option for the sequence length. A similar approach could be used for the NR-U extended sequences, which could be written in a generic manner that can be reused in case it is extended to other SCSs. The open options from the last meeting are:

- Create new declaration field for extended PRACH sequences in 38.141-1 which includes format, SCS, and LRA. - Option 1: [D.111]; PRACH format with LRA = 1151 and LRA = 571 and SCS; Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both.. - Option 2: [D.111]; PRACH format with LRA = 1151 and LRA = 571 and SCS; Declaration of the supported PRACH format(s) with LRA=1151 and LRA=571 as specified in TS 38.211 [17], i.e., format: A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format, as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both, as well as sequence length other than LRA =139, i.e. LRA =1151 for 15 kHz SCS, and LRA =571 for 30 kHz SCS.

It was agreed to test only formats A2, B4, C2, therefore, if a gNB declares only support for other formats there will be no respective performance requirement for the performance tests.

RAN4 to adopt text of Option 1 as part of the manufacturer declaration for PRACH with LRA=1151 and LRA=571 as:
        Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2.
        Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both.

Conclusion

In this contribution we provided Nokia’s views on the demodulation performance of PRACH for NR-U. Among the conclusions and observations derived from this discussion are the following:

- It was agreed to test only formats A2, B4, C2, therefore, if a gNB declares only support for other formats there will be no respective performance requirement for the performance tests.

- RAN4 to adopt text of Option 1 as part of the manufacturer declaration for PRACH with LRA=1151 and LRA=571 as:
        Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2.
        Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e., 15 kHz, 30 kHz or both.

References

- R4-2103808, Way forward on PRACH demodulation requirements, Nokia, 3GPP TSG-RAN WG4 Meeting #98-e

- R4-2103933, Email discussion summary for [97e][316] NR_unlic_Demod_BS, 3GPP TSG-RAN WG4 Meeting #98-e

- R4- 2100578, PRACH Demodulation performance requirements for operation in unlicensed bands, 3GPP TSG-RAN WG4 Meeting #98bis-e, Nokia

- R4- 210xxxx, NR-U PRACH simulation results, 3GPP TSG-RAN WG4 Meeting #98bis-e, Nokia

- TS 38.141-1, V16.6.0, NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing

- TS 38.141-2, V16.6.0, NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing