3GPP TSG-RAN WG4 Meeting #98-bis-e                                                         R4-2104644

Online, 12th April - 20th April, 2021

Source:	Vodafone

Title:	TP for TR 38.717-03-01: CA_n1-n20-n78

Agenda item:			7.9.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-01 to include CA_n1-n20-n78 according to the request in [1].

# 2. Reference

- RP-202720, Revised WID: Rel-17 NR inter-band CA for 3 bands DL with 1 band UL.

# 3. Text Proposal

<Start of Text Proposal>

## 6.X	CA_n1-n20-n78

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported bandwidths per CA band combination of band n1+n20+n78

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n1A-n20A-n78A | - | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.X.3	UE co-existence studies

Co-existence studies of CA_n1-n20-n78 with 1UL have been covered in the constituent fall-back modes.

### 6.X.4	∆TIB and ∆RIB values

For three DLs of Band n1, n20 and n78, the TIB,c and RIB,c values specified for the constituent fall-back modes are used below.

Table 6.X.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n20-n78 | n1 | 0.3 |
|  | n20 | 0.6 |
|  | n78 | 0.8 |

Table 6.X.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n20-n78 | n1 | 0 |
|  | n20 | 0 |
|  | n78 | 0.5 |

### 6.X.5	REFSENS requirements

There are no additional MSD requirements for this band combination.

<End of Text Proposal>