3GPP TSG-RAN WG4 Meeting #98-bis-e                                                         R4-2104726

Electronic Meeting, 12 - 20 Apr., 2021

Source: 	CATT

Title: 	Link level simulation results for 1024QAM for NR FR1

Agenda item:	8.15.1

Document for:	Approval

- Introduction

In RAN #90-e, the revised WID on Introduction of DL 1024QAM for NR FR1was approved. The main objectives for RAN4 in revised WID [1] are shown as follows:

- Specify corresponding UE and BS RF core requirements [RAN4] - UE and BS RF core requirements are specified for stationary wireless scenarios with up to 2 layer DL MIMO - The cell size(s) and type of stationary wireless scenarios for which UE and BS RF core requirements are defined will be studied and decided by RAN4.

In this contribution, we provide link simulation for 1024QAM for NR FR1.

- Discussion

Link level simulation is targeted as mainstream way to evaluate if 1024QAM for NR FR1 can achieve benefit by comparing to 256QAM.

Link level simulation assumptions

To evaluate the throughput difference between256QAM and 1024QAM, the link level simulation assumptions are listed as in table 2.1-1.

Table 2.1-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| CBW | 20 MHz |
| SCS | 15kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, Maximum Doppler frequency: 5Hz TDL-D 10ns delay spread, Maximum Doppler frequency: 5Hz |
| MCS | 256QAM: MCS 24 in TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH, and other MCSs are not precluded 1024QAM: MCS 24 in the following Table accroding to the agreement in RAN1 #104, and other MCSs are not precluded MCS Index; Modulation Order; Target code Rate R x [1024]; Spectral. IMCS; Qm; efficiency. 23; 10; 805.5; 7.8662. 24; 10; 853; 8.3301. 25; 10; 900.5; 8.7939. 26; 10; 948; 9.2578. |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels; follow PMI |
| Symbol type | CP-OFDM |
| HARQ | 8, None |
| RANK | 1 and 2 |
| BS antenna configuration | 1 and 2 |
| UE antenna configuration | 4 |
| Antenna correlation (Tx and Rx) | Low correlation |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| txEVM | 2%, 2.5%, 3% |
| rxEVM | 2%, 3%, 4% |

Link level simulation results

The throughput simulation result for txEVM 2%, 2.5% and 3% in TDL-A are shown in Figure A-1, Figure A-2, and Figure A-3 in Annex respectively.  And the throughput simulation result for txEVM 2%, 2.5% and 3% in TDL-A are shown in Figure A-4, Figure A-5, and Figure A-6 in Annex respectively.

Table 2.1-2 Summary of crossover SNR between 256QAM and 1024QAM in TDL-A and TDL-D

| Channel | txEVM rxEVM | 2% | 3% | 4% |
| --- | --- | --- | --- | --- |
| TDL-A, 1T4R | 2% | 25.3dB | 26.1dB | 27.6dB |
| TDL-A, 1T4R | 2.5% | 26.4dB | 27.5dB | 29.7dB |
| TDL-A, 1T4R | 3% | 28.4dB | 30.2dB | 35.7dB |
| TDL-A, 1T4R | 2% | 24.7dB | 25.6dB | 26.7dB |
| TDL-A, 1T4R | 2.5% | 25.9dB | 26.8dB | 28.6dB |
| TDL-A, 1T4R | 3% | 27.7dB | 29.3dB | 33.2dB |

We summarize the crossover SNR between 256QAM and 1024QAM as shown in Table 2.1-2. From the Table 2.1-2, it is observed that

  - The crossover SNR with 3%/4% TX/RX EVM in TDL-A is ~35.7dB.

  - As TX/RX EVM decreases, crossover SNR also decreases

  - For the same TX/RX EVM, the crossover SNR in TDL-D is lower than that in TDL-A.

Table 2.1-3 Summary of throughput gain of 1024QAM compared to 256QAM (percentage@40dB)

in TDL-A and TDL-D

| Channel | txEVM rxEVM | 2% | 3% | 4% |
| --- | --- | --- | --- | --- |
| TDL-A, 1T4R | 2% | 26.6% | 26.1% | 21.7% |
| TDL-A, 1T4R | 2.5% | 26.4% | 24.8% | 16.9% |
| TDL-A, 1T4R | 3% | 25.7% | 19.8% | 3.2% |
| TDL-D, 1T4R | 2% | 26.4% | 25.4% | 21.3% |
| TDL-D, 1T4R | 2.5% | 26.1% | 24.1% | 17.1% |
| TDL-D, 1T4R | 3% | 24.9% | 19.9% | 6.9% |

The throughput gain of 1024QAM compared to 256QAM is shown in Table 2.1-3. From table 2.1-3, the following is observed,

  - As TX/RX EVM decreases, throughput gain of 1024QAM compared to 256QAM increases, if TX/RX EVM decreases to 3%/3%, the throughput gain of 1024QAM compared to 256QAM in TDL-A is increased by ~19.8%.

  - For the same TX/RX EVM, the throughput gain of 1024QAM compared to 256QAM in TDL-D is larger than that in TDL-A.

It should be mentioned that we only provided simulation results for Rank 1. Situation might be worse if we consider Rank 2 due to interference between the 2 streams.

Proposal 1: It is proposed to approve the simulation assumptions in Table 2.1-1 for further EVM evaluations.

- Conclusions

This contribution provides link level simulation assumptions and Rank 1 simulation results for NR FR1 1024QAM. The following observations and proposals are concluded.

Observation 1: Regarding crossover SNR between 1024QAM and 256QAM for Rank 1, the crossover SNR is shown as in table 2.1-2.

  - The crossover SNR with 3%/4% TX/RX EVM in TDL-A is ~35.7dB.

  - As TX/RX EVM decreases, crossover SNR also decreases

  - For the same TX/RX EVM, the crossover SNR in TDL-D is lower than that in TDL-A.

Observation 2: Regarding EVM for Rank 1 between 1024QAM and 256QAM, the performance gain is shown as in table 2.1-3.

  - As TX/RX EVM decreases, throughput gain of 1024QAM compared to 256QAM increases, if TX/RX EVM decreases to 3%/3%, the throughput gain of 1024QAM compared to 256QAM in TDL-A is increased by ~19.8%.

  - For the same TX/RX EVM, the throughput gain of 1024QAM compared to 256QAM in TDL-D is larger than that in TDL-A.

Proposal: It is proposed to approve the simulation assumptions in Table 2.1-1 for further EVM evaluations.

- References

- RP-202886,Revised WID on Introduction of DL 1024QAM for NR FR1,Nokia,RAN#90-e

- Annex

Figure A-1 throughput simulation result for txEVM 2% in TDL-A

Figure A-2 throughput simulation result for txEVM 2.5% in TDL-A

Figure A-3 throughput simulation result for txEVM 3% in TDL-A

Figure A-4 throughput simulation result for txEVM 2% in TDL-D

Figure A-5 throughput simulation result for txEVM 2.5% in TDL-D

Figure A-6 throughput simulation result for txEVM 3% in TDL-D