3GPP TSG-RAN WG4 Meeting # 98-bis-e                                         R4-2104757

Electronic Meeting, Apr. 12-20, 2021

Title: 	Update simulation results for RLM/BFD relaxation

Source: 	CATT

Agenda item:	8.9.2

Document for:	Discussion

Introduction

In this contribution, we update our simulation results on RLM/BFD relaxation factor.

Simulation results

The simulation uses the simulation assumption in [1] and the definition in [1]:

The CDF curves of Delta SINR comparison between R15 an R17.

- ΔSINR = MAX (ABS [(estimated SINR at Rel-17– estimated SINR at Rel-15) CDF=X ] ), ABS [(estimated SINR at Rel-17 – estimated SINR at REl-15) CDF=Y ]) [dB]

  - (X,Y) =(5%, 95%) or (1%, 99%)

  - Note: Other options to (X,Y) are not precluded. Companies are encouraged to bring CDF curves of ΔSINR in the next meeting.

The update simulation results of ΔSINR are shown as below:

| SSB 30kHzSCS |  |  | 5% | 95% | ΔSINR |
| --- | --- | --- | --- | --- | --- |
| 3km/h DRX20ms | 10 samples | K=2 | -1.1688 | 1.0719 | 1.1688 |
|  |  | K=4 | -1.4022 | 1.22 | 1.4022 |
|  | 5 samples | K2 | -1.7818 | 1.6801 | 1.7818 |
|  |  | K=4 | -2.0712 | 2.0652 | 2.0712 |
| 3km/h DRX40ms | 10 samples | K=2 | -1.9036 | 1.8027 | 1.9036 |
|  |  | K=4 | -2.3289 | 2.0235 | 2.3289 |
|  | 5 samples | K=2 | -2.7551 | 2.7065 | 2.7551 |
|  |  | K=4 | -3.2464 | 3.1907 | 3.2464 |

| CSI-RS 30kHzSCS |  |  | 5% | 95% | ΔSINR |
| --- | --- | --- | --- | --- | --- |
| 3km/h DRX20ms | 20 samples | K=2 | -1.4988 | 1.2755 | 1.4988 |
|  |  | K=4 | -1.7519 | 1.5574 | 1.7519 |
|  | 10 samples | K2 | -1.8871 | 1.7636 | 1.8871 |
|  |  | K=4 | -2.4913 | 2.5452 | 2.5452 |
| 3km/h DRX40ms | 20 samples | K=2 | -1.1910 | 1.4713 | 1.4713 |
|  |  | K=4 | -2.0848 | 1.5757 | 2.0848 |
|  | 10 samples | K=2 | -1.7734 | 1.7959 | 1.7959 |
|  |  | K=4 | -2.2174 | 1.9061 | 2.2174 |

From the simulation results it can be observed: The performance degradation due to DRX is not serious for 20ms and 40ms.

References

[1] R4-2104066, “Updated evaluation assumptions for R17 RLM/BFD relaxation”, vivo, MediaTek