3GPP TSG-RAN WG4 Meeting # 98-bis-e 		                       R4-2104777

Electronic Meeting, Apr. 12-20, 2021

Title: 	Discussion on TDM operation between SL and Uu

Source: 	CATT

Agenda item:	8.10.5.2

Document for:	Approval

- Introduction

In RAN4#98e meeting, the operating scenarios for partial usage of SL operation in band n79 were discussed, including TDM and FDM between SL and Uu. For TDM operation, the discussion on the time mask for SL and Uu switching was postponed as it was premature before the operating scenario is clear. This contribution will provide our views on TDM operation between SL and Uu, including UE RF architecture and the time mask for SL and Uu switching.

- Discussion

As per the WF [1], the agreements and remaining issue regarding TDM between SL and Uu in licensed band are as follows:

Whether to introduce TDM operation between SL and Uu - Candidate options: - Option 1: RAN4 allow TDM operation between spectrally partially used PC5 SL and Uu UL/DL operation in a licensed TDD band (different carriers). - Option 2: RAN4 allow TDM operation without FDM for SL and Uu in a licensed TDD band (same carrier). - Recommended WF: - Both Option 1 and option 2 are feasible in TDD licensed band, e.g. n79. Time mask for SL and Uu switching - Candidate options: - Option 1: Consider the TDM timing mask for partially used SL operation with NR Uu in paper R4-2102346. - Option 2: Postpone until the operation mode and coexistence scenario are clear. TDM timing mask for partially used SL operation to be specified, FFS on TDM timing mask detail. - Recommended WF: - Option 2 can be agreed. UE RF architecture for TDM - Companies are encourage to share views on UE RF architecture for TDM if possible. - Option 1: One single RF chain can be used for TDM operation between SL and Uu in TDD licensed band. - Option 2: A single RF chain or other architectures can be used for TDM operation between SL and Uu in TDD licensed band. - Recommended WF: - Single RF chain is considered as baseline for TDM (without additional FDM). Other RF architecture is not precluded.

UE RF architecture for TDM

Based the latest agreement, single RF chain is considered as a baseline for TDM operation with the same carrier. For TDM operation with different carriers, it is also required to decide the UE RF architecture to specify the time mask for SL and Uu switching. Considering the large frequency range of band n79, different carriers for SL and Uu are possible to be operated with a large frequency distance in between. So it is proposed to consider both single RF chain and separate RF chain for TDM operation with different carriers.

Proposal 1: To consider both single RF chain and separate RF chain for TDM operation with different carriers.

Time mask for SL and Uu switching

The length of switching time between NR Uu and NR SL is associated with UE RF architecture or carrier frequency location. In general, large-distance carrier frequency switching and single RF chain leads to larger switching time. Based on the agreements in the last meeting, RAN4 allow TDM operation with the same carrier and different carriers. It is required to study the respective time mask for SL and Uu switching based on the specific TDM scenario, i.e. same carrier and different carriers.

For TDM operation with the same carrier, the latest agreement is to utilize the single RF chain for SL and Uu. In such a case, whether to have dual PA capability has a negligible impact on the switching time between SL and Uu as both RATs operate at the same carrier. One important issue needing to be clarified is whether the same carrier here means the same carrier frequency and same channel bandwidth. If different channel bandwidths with the same carrier frequency between SL and Uu are allowed, the time occupied by different channel bandwidths configuration during the switching process should be taken into account. It is not preferred to specify zero switching period in case of different channel bandwidth switching.

Observation 1: One important issue needing to be clarified is whether the same carrier here means the same carrier frequency and same channel bandwidth. If different channel bandwidths with the same carrier frequency between SL and Uu are allowed, the time occupied by different channel bandwidths configuration during the switching process should be taken into account.

Indeed, if SL and Uu operate at the same carrier frequency with the same CBW, it is expected that the switching period should be 0. Specifically, in case Uu switches to SL, the transient period (10us+10us) can be evenly located on both RAT side as shown in Figure 1. In case SL switches to Uu, the transient period (20us) can be wholly located on SL side as the last symbol is used for guard period as shown in Figure 2. So it is proposed to consider Figure 1 and Figure 2 for SL and Uu switching with the same carrier.

Figure 1: Time mask for NR Uu switching to NR SL

Figure 2: Time mask for NR SL switching to NR Uu

Proposal 2: To consider the time mask in Figure 1 and Figure 2 for SL and Uu switching with the same carrier.

To specify the time mask for TDM operation with different carriers, the first issue needing to be decided is UE RF architecture, i.e. single RF chain or separate RF chain. As discussed above, it is proposed to consider both single RF chain and separate RF chain for TDM operation between SL and Uu with different carriers. The different UE RF architectures lead to different switching period between SL and Uu. More specifically, in case separate RF chain is used, it is expected the switching period should be 0. If single RF chain is used, it is required to consider the switching period used for different carriers switching without dual PA capability.

Regarding LTE SL and NR SL switching in ITS band, the switching period position was intensively discussed through a long period and the potential conclusion tends to be stable. For LTE SL and NR SL switching in ITS band, the overwhelming view is that the switching period position should be decided based on the RAT priority, i.e. the switching period is always located on the RAT that has a lower priority. When it comes to NR SL and NR Uu switching in the licensed band, the same principle can be applied. Based on TS 38.213, RAN1 has defined the prioritization mechanism for SL and Uu. To conclude, it is proposed to locate the switching period based on the prioritization for SL and Uu, i.e. the switching period is located on the RAT that has a lower priority.

- if the UL transmission is for a PUSCH or for a PUCCH with priority index 1, - if sl-PriorityThreshold-UL-URLLC is provided - the SL transmission or reception has higher priority than the UL transmission if the priority value of the SL transmission(s) is smaller than sl-PriorityThreshold-UL-URLLC; otherwise, the UL transmission has higher priority than the SL transmission or reception - else - the UL transmission has higher priority than the SL transmission or reception - else - the SL transmission or reception has higher priority than the UL transmission if the priority value of the SL transmission(s) or reception is smaller than sl-PriorityThreshold; otherwise, the UL transmission has higher priority than the SL transmission or reception

Proposal 3: To locate the switching period based on the prioritization for SL and Uu, i.e. the switching period is located on the RAT that has a lower priority.

Based on the prioritization between SL and Uu, the time masks for SL and Uu switching without dual PA capability are illustrated in Figure 3 and Figure 4, where the guard symbol of SL is also considered to contain switching period.

Figure 3: Time mask for NR SL (higher priority) switching to NR Uu without dual PA capability

Figure 4: Time mask for NR Uu (higher priority) switching to NR SL without dual PA capability

Proposal 4: To consider the time mask in Figure 3 and Figure 4 for SL and Uu switching with different carriers without dual PA capability.

- Conclusion

This contribution discusses TDM operation between SL and Uu, including UE RF architecture and the time mask for SL and Uu switching. The following observation and proposals are derived based on above discussion:

UE RF architecture for TDM

Proposal 1: To consider both single RF chain and separate RF chain for TDM operation with different carriers.

Time mask for SL and Uu switching

Observation 1: One important issue needing to be clarified is whether the same carrier here means the same carrier frequency and same channel bandwidth. If different channel bandwidths with the same carrier frequency between SL and Uu are allowed, the time occupied by different channel bandwidths configuration during the switching process should be taken into account.

Proposal 2: To consider the time mask in Figure 1 and Figure 2 for SL and Uu switching with the same carrier.

Proposal 3: To locate the switching period based on the prioritization for SL and Uu, i.e. the switching period is located on the RAT that has a lower priority.

Proposal 4: To consider the time mask in Figure 3 and Figure 4 for SL and Uu switching with different carriers without dual PA capability.

- Reference

- R4-2103244, WF on TDM operation for SL and Uu in licensed band, CATT, RAN4#98e