3GPP TSG RAN WG4 Meeting #98-bis-e	R4-2104917

Electronic Meeting, 12 - 20 Apr., 2021

Agenda item:	7.9.2

Source:	SoftBank Corp.

Title:	TP update for TR 38.717-03-01: NR CA_n28-n77-n79

Document for:	Approval

1	Introduction

TP for 3DL/1UL NR CA of CA_n28-n77-n79 was approved in RAN4#98-e[1]. This TP is to update it to add n77(2A).

2.  Reference

[1]  R4-2101188  TP for CA_n28-n77-n79 for TR 38.717-03-01, NTT DOCOMO INC.

Annex.

[Unchanged Parts Skipped]

6.2		CA_n28-n77-n79

6.2.1	Operating bands for CA

Table 6.2.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n77-n79 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.2.2	Channel bandwidths per operating band for CA

Table 6.2.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n28A-n77A-n79A4 | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n28A-n77(2A)-n79A4 | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| NOTE 3: The SCS of each channel bandwidth for NR band refer to Table 5.3.5-1. NOTE 4: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers for TDD combinations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 when operating in single carrier mode.

6.2.3	Co-existence studies

Co-existence studies can be omitted because harmonic interference have been already studied for 2DL/1UL fallback combinations such as CA n28-n77, CA_n28-n79 and n77-n79.

6.2.4	∆TIB,c and ∆RIB,c values

For three DLs and one UL of Band n28, n77 and n79, the TIB,c and RIB,c values are shown in table 6.2.4-1 and table 6.2.4-2, respectively.

Table 6.2.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28-n77-n79 | n28 | 0.5 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.2.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28-n77-n79 | n28 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

6.2.5	REFSENS requirements

MSD studies can be omitted because harmonic interference have been already studied for 2DL/1UL fallback combinations such as CA n28-n77, CA_n28-n79 and n77-n79.

[Unchanged Parts Skipped]