3GPP TSG-RAN WG4 Meeting #98-bis-e

R4-2105045

Electronic Meeting, April. 12-April. 20, 2021

Agenda Item:   8.8.2

Source:        Samsung

Title:           Simulation assumptions for FR1 coexistence study

Document for:  Approval

# Introduction

This document proposed parameters, assumptions and layouts that to be used in co-existence study between NTN and TN for FR1 exemplary band (2 GHz). And this document will focus on the scenarios of NTN interfering TN.

# Discussions

### Scenarios 2 GHz

In the case of coexistence study of NTN interfering TN, there are two general scenarios should be taken into account,

- NTN DL to TN DL

- NTN UL to TN UL

For the specific NTN frequency ranges, additional cross link interference may need to be further considered, such as for NTN S-band on 1980-2010/2110-2170MHz, NTN UL to TN DL scenario should be evaluated considering the case that TN operated in adjacent band (n34/2010-2025MHz).

In WID[1], one of the principle in the objective of the WI is “Handheld devices in FR1 are supported (e.g. Power class 3)”. And a general agreement in RAN4 is also reached to apply UE parameters refer to TR 38.821 for FR1 coexistence study, and it is noted that NTN UE is assumed has same Tx characteristics as TN UE. In NTN network, as complementary coverage to the terrestrial IMT system, the density of NTN UE is much lower than TN UE. In that case, it seems impossible for NTN UE to create more interference to TN BS compared with interference from TN UE to TN BS. Considering 3GPP had conducted sufficient studies between two TN networks in FR1, we believe that, for the above mentioned general interference scenarios of DL-DL and UL-UL, no more simulation is needed to be developed for NTN UL to TN UL scenario and the ACLR of TN UE can be considered as baseline to be reused for NTN handheld UE, which is also beneficial to obtain large scale economic for NTN industry. Note that cross link interference may need to be further evaluated to verify the feasibility to reuse the ACLR.

Proposal 1: Deprioritize the inference scenario of NTN UL (aggressor) to TN UL (victim) for coexistence study in Rel-17

### Simulation assumptions for NTN coexistence study

In this section, simulation assumptions including technical and deployment characteristics, network layout, propagation model as well as evaluation metrics etc. were provided for discussion and proposed to be applied in NTN 2 GHz coexistence study.

Proposal 2: Simulation assumptions in section 2.2 are provided for NTN 2 GHz coexistence study.

### NTN Technical Characteristics

The following tables, refer to TR 38.821 Table 6.1.1.1-1 and 6.1.1.1-2, representing set-1 of satellite parameters and UE parameters are considered as the baseline for FR1 coexistence study.

Table 2.2.1-1: Satellite parameters for system level simulator calibration (Set-1)

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | 2 GHz (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| Satellite EIRP density |  | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 250 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | 2 GHz (i.e. 2 GHz) | 22 m | 2 m | 2 m |
| G/T |  | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 2.2.1-2: Other simulation parameters for NTN DL

| Parameters | NTN | Remark |
| --- | --- | --- |
| Carrier frequency | 2GHz |  |
| Channel bandwidth | 30MHz | TR 38.821 |
| The number of active UE (DL) (Note 1) | 1 |  |
| Traffic model | Full buffer |  |
| DL power control | NO |  |

The following table is for UE characteristics for System Level Simulations

Table 2.2.1-3: UE characteristics for system level simulations

| Characteristics | Handheld |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |

The satellite antenna pattern is referring to TR 38.811 Section 6.4.1, as follows.

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered

where J1(x) is the Bessel function of the first kind and first order with argument x, a is the radius of the antenna's circular aperture, k = 2f/c is the wave number, f is the frequency of operation, c is the speed of light in a vacuum and θ is the angle measured from the bore sight of the antenna's main beam. Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

The normalized gain pattern for a = 10 c/f (aperture radius of 10 wavelengths) is shown in Figure 2.2-1.

Figure 2.2-1: Satellite antenna gain pattern for aperture radius 10 wavelengths, a=10 c/f

### NTN Beam Layouts

The following tables, modified from TR 38.821 Table 6.1.1.1-4 and Table 6.1.1.1-5, are proposed for the assumptions and layouts of NTN beams for a single satellite system level simulation. The proposed modifications are highlighted.

Table 2.2.2-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |
| --- | --- |
| Number of beams | Baseline: NTN DL to TN DL: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |
| UV plane illustration (extracted from [19]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS = sqrt(3) x sin(HPBW/2 [rad]) |
| Central beam bore sight direction definition | Baseline: Central beam center is considered at nadir point |

Table 2.2.2-2: Other parameters for NTN Beam layout and assumptions

| Frequency re-use factor | Option 1: 1 Option 2: 3 |
| --- | --- |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UE distribution | Uniform distribution in all the Voronoi cell area associated to each beam |
| UE density | FFS, [x] UEs/km2 |
| UE configuration | 2 GHz: Handheld (optional for scenario A) |
| UE orientation | Handheld: Random |
| Handover Margin | 0 dB |
| UE attachment | RSRP |

### Earth-Space Large-Scale Propagation Model

The large-scale propagation model for NTN satellite-UE transmission is referred to TR 38.811 Section 6.6.

#### LOS probability

Line-Of-Sight (LOS) probability depends on UE environment and elevation angle, and is obtained from Table 2.2.3.1-1. Reference elevation angles are considered from 10° to 90° with a 10° step. For an UE-to-satellite or UE-to-HAPS link, the LOS probability is taken from the nearest reference elevation angle.

Table 2.2.3.1-1 LOS probability

| Elevation | Dense urban scenario | Urban scenario | Suburban and Rural scenarios |
| --- | --- | --- | --- |
| 10° | 28.2% | 24.6% | 78.2% |
| 20° | 33.1% | 38.6% | 86.9% |
| 30° | 39.8% | 49.3% | 91.9% |
| 40° | 46.8% | 61.3% | 92.9% |
| 50° | 53.7% | 72.6% | 93.5% |
| 60° | 61.2% | 80.5% | 94.0% |
| 70° | 73.8% | 91.9% | 94.9% |
| 80° | 82.0% | 96.8% | 95.2% |
| 90° | 98.1% | 99.2% | 99.8% |

#### Path loss and Shadow fading

The signal path between a satellite or HAPS transmitter and an NTN terminal undergoes several stages of propagation and attenuation. The path loss (PL) is composed of components as follows:

,	(2.2.3.2-1)

where

is the total path loss in dB,

is the basic path loss in dB,

is the attenuation due to atmospheric gasses in dB,

is the attenuation due to either ionospheric or tropospheric scintillation in dB,

is building entry loss in dB.

This section specifies the basic path loss model () which accounts for the signal's free space propagation, clutter loss, and shadow fading. Attenuations due to building entry loss, atmospheric gasses and scintillation are described in Sections 2.4.3, 2.4.4 and 2.4.6, respectively.

The free space path loss (FSPL) in dB for a separation distance d in meter and frequency fc in GHz is given by

(2.2.3.2-2)

For a ground terminal, the distance d (a.k.a. slant range), as shown in Figure 2.4.2-1, can be determined by the satellite/HAPS altitude  and elevation angle α by

,	(2.2.3.2-3)

where  denotes Earth radius.

Figure 2.4.2-1: Slant range d between a satellite and a ground terminal

Clutter loss (CL) models the attenuation of signal power caused by surrounding buildings and objects on the ground. It depends on the elevation angle α, the carrier frequency fc, and the environment. Shadow fading (SF) is modeled by a log-normal distribution, which when expressed in decibel unit, is a zero-mean normal distribution with a standard deviation , i.e., .

The basic path loss in dB unit is modeled as

,	(2.2.3.2-4)

where FSPLd, fc is the free space path loss, is clutter loss, and  is shadow fading loss represented by a random number generated by the normal distribution, i.e.,  ~SF~N0, σSF2. When the UE is in LOS condition, clutter loss is negligible and should be set to 0 dB in the basic path loss model.

The values of σSF and  are given in tables 2.2.3.2-1 to 2.2.3.2-3 at reference elevation angles for different scenarios. The UE in a particular scenario should take the values corresponding to the reference angle nearest to its elevation angle α.

Table 2.2.3.2-1: Shadow fading and clutter loss for dense urban scenario (2 GHz2 GHz)

| Elevation | 2 GHz |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS |  |
|  | σSF (dB) | (dB) | (dB) |
| 10° | 3.5 | 15.5 | 34.3 |
| 20° | 3.4 | 13.9 | 30.9 |
| 30° | 2.9 | 12.4 | 29.0 |
| 40° | 3.0 | 11.7 | 27.7 |
| 50° | 3.1 | 10.6 | 26.8 |
| 60° | 2.7 | 10.5 | 26.2 |
| 70° | 2.5 | 10.1 | 25.8 |
| 80° | 2.3 | 9.2 | 25.5 |
| 90° | 1.2 | 9.2 | 25.5 |

Table 2.2.3.2-2: Shadow fading and clutter loss for urban scenario (2 GHz2 GHz)

| Elevation | 2 GHz |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS |  |
|  | σSF (dB) | (dB) | (dB) |
| 10° | 4 | 6 | 34.3 |
| 20° | 4 | 6 | 30.9 |
| 30° | 4 | 6 | 29.0 |
| 40° | 4 | 6 | 27.7 |
| 50° | 4 | 6 | 26.8 |
| 60° | 4 | 6 | 26.2 |
| 70° | 4 | 6 | 25.8 |
| 80° | 4 | 6 | 25.5 |
| 90° | 4 | 6 | 25.5 |

Table 2.2.3.2-3: Shadow fading and clutter loss for suburban and rural scenarios (2 GHz2 GHz)

| Elevation | 2 GHz |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS |  |
|  | σSF(dB) | σSF(dB) | (dB) |
| 10° | 1.79 | 8.93 | 19.52 |
| 20° | 1.14 | 9.08 | 18.17 |
| 30° | 1.14 | 8.78 | 18.42 |
| 40° | 0.92 | 10.25 | 18.28 |
| 50° | 1.42 | 10.56 | 18.63 |
| 60° | 1.56 | 10.74 | 17.68 |
| 70° | 0.85 | 10.17 | 16.50 |
| 80° | 0.72 | 11.52 | 16.30 |
| 90° | 0.72 | 11.52 | 16.30 |

#### O2I penetration loss

According to Table 2.2.2 -2, the UE distribution assumed to be 100% outdoor.

Thus, it is proposed that the O2I penetration loss of NTN DL propagation link for this study in 2 GHz2 GHz will not use.

#### Atmospheric absorption

Referring to TR 38.811, section 6.6.4, attenuation by atmospheric gases which is entirely caused by absorption depends mainly on frequency, elevation angle, altitude above sea level and water vapour density (absolute humidity). At frequencies below 10 GHz, it may normally be neglected.

Thus, it is proposed that the Atmospheric absorption loss for this study in 2 GHz is assumed to be 0 dB.

#### Rain and cloud attenuation

Referring to TR 38.811, section 6.6.5, rain and cloud attenuation is considered as negligible for frequencies below 6 GHz.

Thus, it is proposed that the rain and cloud attenuation for this study in 2 GHz is assumed to be 0 dB.

#### Scintillation

Referring to TR 38.821 Table 6.1.1.1-5 Note 2, “For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude).”, it is proposed that the ionospheric scintillation loss for this study in 2 GHz is assumed to be 0 dB.

### TN Technical Characteristics

Considering 3GPP TR 38.803, TR 37.842, TR 36.942, TR 38.901, LS to ITU-R WP5D RP-200559 and ITU-R Report M.2101, M.2292, M.[IMT_Parameters], it is proposed to use following parameters for 2 GHz NR system level simulation.

Table 2.2.4-1: Technical characteristics of TN (2 GHz)

|  | NR | Remarks |
| --- | --- | --- |
| Carrier frequency in GHz | 2 |  |
| Size of each nominal channel BW in MHz | 20 |  |
| Network layout | 19-sites 57 sectors with wrap-around |  |
| System loading and activity | Full buffer 100% |  |
| Network location | TN as victim: Randomly generated in NTN central beam |  |
| DL subcarrier spacing | 15kHz |  |
| UL | OFDMA |  |
| DL power control | No |  |
| UL power control | 36.942 section 5.1.1.6 (set=1) |  |
| Frequency reuse | 1 |  |
| Number of scheduled UE per cell (DL) | 1 |  |
| Number of scheduled UE per cell (UL) | 3 |  |
| UE TX power in dBm | [-40] to 23 | TR 36.942 / 38.803 -40dBm depends on the channel bandwidth used for NR UE. |
| UE antenna gain in dBi | 0 | Omni-directional |
| Cell selection margin in dB | 3 | TR 36.942 / 38.803 |
| BS noise figure in dB | 5 | TR 36.942 |
| UE noise figure in dB | 9 | TR 36.942 |
| BS-UE path-loss model | TR 38.901 | TR 38.803 not applied in 2 GHz. |
| UE distribution | Uniform |  |
| Evaluation metrics | Throughput loss | TR 38.803 SINR loss is used for NB-IoT coexistence study. |

Table 2.2.4-2: Deployment-related parameters of TN (2 GHz)

|  | Urban Macro | Suburban Macro | Rural Macro | Remarks |
| --- | --- | --- | --- | --- |
| Cell radius in meters | 500 | 1000 | 5000 | ITU-R Report M.2292 |
| BS Antenna height in meters | 25 | 30 | 30 |  |
| Base Station Antenna Characteristics |  |  |  |  |
| Antenna Pattern | TR 37.842 Section 5.3.3 |  |  | TR 37.842 |
| Element Gain in dBi | 6.4 | 7.1 | 7.1 | 3GPP LS to ITU-R WP5D RP-200559 and ITU-R WP5D [IMT_Parameters] |
| H and V 3dB beamwidth of single element in degree | 90º for H 65º for V | 90º for H 54º for V | 90º for H 54º for V |  |
| H and V front-to-back ratio in dB | 30 for both H/V | 30 for both H/V | 30 for both H/V |  |
| Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º |  |
| Antenna array configuration (Row × Column) | 8 x 8 elements | 8 x 8 elements | 8 x 8 elements |  |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H, 0.7 of wavelength for V | 0.5 of wavelength for H, 0.9 of wavelength for V | 0.5 of wavelength for H, 0.9 of wavelength for V |  |
| Conducted power per antenna element in dBm | 25 | 25 | 25 |  |
| Mechanical downtilt in degree | 10 | 6 | 3 |  |
| UE Parameters |  |  |  |  |
| UE Outdoor/indoor | 100% Outdoor |  |  | Because NTN Satellite to TN UE O2I is hard to calibrate, we propose to only consider outdoor TN UE cases in this study. |
| UE height in meter | 1.5 | 1.5 | 1.5 | 3GPP LS to ITU-R WP5D RP-200559 and ITU-R WP5D [IMT_Parameters] |
| Minimum BS-UE distance in meter | 35 | 35 | 35 |  |

#### Base Station Antenna Array Model (2 GHz)

It is proposed adopt the element pattern and composite antenna pattern from TR 37.842 section 5.3.3, with the parameters assumed in Table 2.2.4-2 above.

##### Array element pattern

Table2.2.4.1.1-1: Array element pattern for antenna array model

| Horizontal Radiation Pattern |  |
| --- | --- |
| Horizontal half-power bandwidth of single array element | To be found in Table 2.2.4-2. |
| Front-to-back ratio | Am, SLAv to be found in Table 2.2.4-2. |
| Vertical Radiation Pattern |  |
| Vertical half-power bandwidth of single array element | To be found in Table 2.2.4-2. |
| Array element radiation pattern |  |
| Element Gain without antenna losses | To be found in Table 2.2.4-2. |

##### Composite antenna pattern

Table2.2.4.1.2-1: Composite antenna pattern for UE specific beamforming

| Configuration | Multiple columns (NVxNH elements) |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the super position vector is given by: the weighting is given by: |
| Antenna array configuration (Row×Column) | To be found in Table 2.2.4-2. |
| Horizontal radiating element spacing d/ λ | To be found in Table 2.2.4-2. |
| Vertical radiating element spacing d/ λ | To be found in Table 2.2.4-2. |
| Down-tilt angle | To be found in Table 2.2.4-2. |

### Uplink Power Control Scheme

It is proposed to adopt TR 36.942, Section 9.1 as the power control scheme used for TN 2 GHz study:

where Pmax = 23dBm, Rmin = -64dB if UE minimum power is -40dBm, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario.

### Earth-Earth Large-Scale Propagation Model

It is proposed to adopt TR 38.901 Section 7.4 as the reference for the propagation model used for 2 GHz in this study.

#### Pathloss

The pathloss models are summarized in Table 2.2.5.1-1 and the distance definitions are indicated in Figure 2.2.5.1-1 and Figure 2.2.5.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 2.2.5.1-1.

| Figure 2.2.5.1-1: Definition of d2D and d3D for outdoor UTs | Figure 2.2.5.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UTs. |
| --- | --- |

Note that

(7.4-1)

Table 2.2.5.1-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 6 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| RMa | LOS | , see note 5 |  | h = avg. building height W = avg. street width The applicability ranges: |
|  | NLOS | for |  |  |
| UMa | LOS | , see note 1 |  |  |
|  | NLOS | for |  | Explanations: see note 3 |
|  |  | Optional |  |  |
| UMi - Street Canyon | LOS | , see note 1 |  |  |
|  | NLOS | for |  | Explanations: see note 4 |
|  |  | Optional |  |  |
| InH - Office | LOS |  |  |  |
|  | NLOS |  |  |  |
|  |  | Optional |  |  |
| InF | LOS | PLLOS=31.84+21.50log10d3D+19.00log10fc | σSF=4.3 | 1≤d3D≤600 m |
|  | NLOS | InF-SL: PL=33+25.5log10d3D+20log10fc PLNLOS=max⁡(PL,PLLOS) | σSF=5.7 |  |
|  |  | InF-DL: PL=18.6+35.7log10d3D+20log10fc PLNLOS=max⁡(PL,PLLOS,PLInF-SL) | σSF=7.2 |  |
|  |  | InF-SH: PL=32.4+23.0log10d3D+20log10fc PLNLOS=max⁡(PL,PLLOS) | σSF=5.9 |  |
|  |  | InF-DH: PL=33.63+21.9log10d3D+20log10fc PLNLOS=max⁡(PL,PLLOS) | σSF=4.0 |  |
| Note 1: Breakpoint distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and hE is the effective environment height. For UMi hE = 1.0m. For UMa hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. With C(d2D, hUT) given by , where . Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. Note 2: The applicable frequency range of the PL formula in this table is 0.5 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

#### LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 2.2.5.2-1.

Table 2.2.5.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| RMa |  |
| UMi - Street canyon |  |
| UMa | where |
| Indoor - Mixed office |  |
| Indoor - Open office |  |
| InF-SL InF-SH InF-DL InF-DH | PrLOS,subsced2D=exp-d2Dksubsce where ksubsce=-dclutterln1-rfor InF-SL and InF-DL-dclutterln1-r∙hBS-hUThc-hUTfor InF-SH and InF-DH The parameters dclutter, r, and hc are defined in Table 7.2-4 of TR 38.901. |
| InF-HH | PrLOS=1 |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

### TN current ACLR and ACS (2 GHz)

Table 2.2.5.3-1 ACLR/ACS for TN (2GHz)

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 45 dB |
|  | ACS | 45 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) |
|  | ACS | 33 |

### TN SINR Performance Evaluation Metrics

It is proposed to adopt TR 38.803, Section 5.2.7 as the SINR-Throughput performance matrics for this study in 2 GHz.

The following equations approximate the throughput over a channel with a given SINR (dB), when using link adaptation:

where:

S(SINR)		Shannon bound, S(SINR) =log2(1 + 10SINR/10) [bps/Hz];

Attenuation factor, representing implementation losses;

SINRMIN		Minimum SINR of the code set, dB;

SINRMAX 	Maximum SINR of the code set, dB.

The parameters α, SINRMIN and SINRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in Table 2.2.6-1 represent a baseline case, which assumes:

–	1:1 antenna configurations;

–	AWGN channel model;

–	Link Adaptation (see Table 2.2.6-1 for details of the highest and lowest rate codes);

–	No HARQ.

Table 2.2.6-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

### Co-existence Simulation Methodology (for NTN DL to TN DL on 2 GHz)

#### NTN DL to TN DL Co-existence Layout

It is proposed that the TN, as victim system, is generated inside the central beam of the NTN, as aggressor system.

Due to the assumption of using omni-directional antenna for both NTN and TN UE in 2 GHz, mathematically, it is sufficient to study the impact of TN inside the NTN central beam, and to skip the cases of TN in other NTN beams.

Figure 2.9.1-1: TN and NTN Co-ex Layout

#### Coordinate System

Referring to TR 38.811 Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN beams direction and location on the earth surface. It means the NTN beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z), with , below figure and table shows the illustration and example of such coordinate system.

Figure 2.3.2-1: Illustration of the coordinate system (TR 38.811 Section 6.3)

Table 2.3.2-1: Example of the NTN Beam coordinates on ground (TR 38.811, Annex A)

| Parameter | Values |
| --- | --- |
| Scenarios | NGSO satellite at 1500km, suburban environment |
| Satellite position | (7871,0,0) |
| Number of beams | 4 |
| Beam layout (as seen from satellite) |  |
| Beam centre | B1: (6371,0,0), elevation = 90° B2: (6347, 481,278), elevation = 65° B3: (6347, 481,-278), elevation = 65° B4: (5242,3622,0), elevation = 45° |

#### Methodology of System Level Simulation

It is proposed to adopt following simulation steps by considering both implementation complexity and statistical nature.

Step 1: Aggressor (NTN) and victim (TN) network locations are generated.

-	NTN central beam is at satellite nadir, surrounded with 6 co-frequency beams.

-	TN (19-cell with wrap around) center is randomly generated within the NTN central beam on earth surface.

Step 2: UE generation and association.

-	NTN UE is randomly generated within the TN area depending on the NTN UE density.

-	TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

Step 3: Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

Step 4: Throughput is computed in the victim systems without considering ACI as below:

-	ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

Step 5: Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

Step 6: RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

Proposal 3: Simulation methodology in section 2.3 is proposed to be applied for coexistence simulation of NTN DL to TN DL interference scenario.

# Conclusion

In this contribution, several key issues were reviewed on NTN coexistence study for 2 GHz including scenarios, parameters as well as simulation methodology, and the proposals were also captured as way forward for following calibration and coexistence simulation.

Proposal 1: Deprioritize the inference scenario of NTN UL (aggressor) to TN UL (victim) for coexistence study in Rel-17.

Proposal 2: Simulation assumptions in section 2.2 are proposed to be applied for NTN 2 GHz coexistence study.

Proposal 3: Simulation methodology in section 2.3 is proposed to be applied for coexistence simulation of NTN DL to TN DL interference scenario.

# Reference

[1]. RP-210908, Solutions for NR to support non-terrestrial networks (NTN), Thales