3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2105315

Electronic Meeting, 12 April – 20 April 2021

Source:	Ericsson

Title:	TP for TR 37.717-21-11: Including band combinations DC_28-40_n78

Agenda item:			7.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-21-11 to add DC_28A-40A_n78A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_28-40_n78

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 28-40_n78 | CA_28-40 | n78 |  |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_28A-40A_n78A | DC_28A_n78A DC_40A_n78A | CA_28A-40A | n78A |
| DC_28A-40C_n78A | DC_28A_n78A DC_40A_n78A | CA_28A-40C | n78A |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_28-40_n78, the TIB,c and RIB,c values are reused from DC_28_n40-n78 and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28-40_n78 | 28 | 0.5 |
|  | 40 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28-40_n78 | 28 | 0.2 |
|  | 40 | 0.4 |
|  | n78 | 0.5 |

### 5.1.x.4	REFSENS requirements

UL 28_n78 might cause IMD3 into 40 DL. MSD value for 40 is chosen as in DC_28A_n1A-n78A.

UL 40_n78 might cause IMD3 into the sidelobe of 28 DL. MSD value for 28 is chosen as in DC_20A-40A_n78A.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A-40A_n78A DC_28A-40C_n78A | 28 | N/A | 5 | 25 | 800.5 | 11 | IMD3 |
|  | 40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | N/A | N/A |
|  | 40 | 2320 | 5 | 25 | 2320 | 15.7 | IMD3 |
|  | n78 | 3750 | 10 | 50 | 3750 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-202613, Dual Connectivity (DC) of 2 bands LTE inter-band CA  (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon