3GPP TSG-RAN WG4 Meeting # 98-bis-e                                                       R4-2106382

Electronic Meeting, Apr. 12-20, 2021

Source:	ZTE Corporation

Title:	TP for TR 38.717-02-01 :CA_n34A-n40A

Agenda Item:	7.2.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on 2DL CA band combination of NR band n34+ NR Band n40 for TR38.717-02-01[1] to capture the MSD values.

- Reference

- TR38.717-02-01, Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) v0.2.0

- Text Proposal

----- Start of TP -----

## 6.x			CA_n34-n40

- 6.x.1		Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n34+n40

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n34-n40x | n34 | 2010 MHz | – | 2025 MHz | 2010MHz | – | 2025 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| Note x: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |  |  |  |  |  |  |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported bandwidths per CA band combination of band n34+n40

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n34A-n40A | CA_n34A-n40A | n34 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n40 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 |  |  |  |

#### 6.x.1.3		Co-existence studies

Table 6.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n34-n40.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 |  |  |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 |  |  |

Based on above table, there is no harmonic issue for the band combination of n34 and n40.

Table 6.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 |  |  |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n34 and n40.

#### 6.x.1.4		∆TIB and ∆RIB values

For CA_n34-n40, it is proposed to use the same TIB,c and RIB values of CA_n39-n40, which are given in the tables below.

Table 6.x.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n34-n40 | n34 | 0 |
|  | n40 | 0 |

Table 6.x.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n34-n40 | n34 | 0.3 |
|  | n40 | 0.3 |

- 6.x.1.5		REFSEN requirements

According to the analysis in subclause 6.x.1.3, there are no harmonic issues for this combination. Hence it is no need to define the MSD values caused by harmonic issues for NR CA n34+n40 band combination.

- 6.x.2		Specific for 2 bands UL CA

      - 6.x.2.1	Maximum output power for inter-band CA

Table 6.24.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n34A-n40A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

- 6.x.2.2		UE co-existence studies

Due to both bands n34 and n40 are TDD bands, and only non-simultaneous Rx/Tx is supported for band n34+n40 inter-band NR CA band combinations. Therefore no IMDs issues for NR CA n34+n40 band combination.

Table 6.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.x.2.1-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n34-n40 | E-UTRA Band 1, 3, 7, 8, 20, 22, 26, 28, 31, 32, 33, 38, 39, 41, 42, 43, 44, 45, 50, 51, 65, 67, 69, 72, 73, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz.. |  |  |  |  |  |  |  |

- 6.x.2.3		REFSENS requirements

No MSD issues for NR CA n34+n40 band combination.

----- End of TP -----