3GPP TSG-RAN4 Meeting #98-bis-e	R4-2105371

E-meeting, 12  – 20 April, 2021

Source: 	Ligado Networks

Title: 	WF on work items LTE_B24_mod and NR_band_n24

Document for: 	Approval

# Agreements

Agreement #1: Revise Table 6.5.3.3.27-1 in TS 38.101-1 (Rel-17) and Table 6.6.3.3.35-1 in 36.101 (Rel-10 to Rel-17) as shown below.  Submit CR in RAN4#99e for TS 38.101-1 (Rel. 17) and TS 36.101 (Rel-10 to Rel-17).

Table 6.5.3.3.27-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit in EIRP1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | EIRP of Discrete emissions of less than 700 Hz bandwidth, Averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | EIRP and Averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1627.5 | -70+ 57/17.5 (f-1610) | 1MHz |  |
| 1627.5 | -37 | 4kHz |  |
| 1638.5 ≤f ≤ 1645.5 | -28 | 4kHz |  |
| 1657.5 ≤f ≤ 1660.5 | -28 | 4kHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using 0 dBi antenna. not declared by the UE manufacturer, a 0 dBi antenna gain shall be used. |  |  |  |

Agreement #2: Modify the Band 24/n24 NS value of 52 to 56 after confirming that it is not used by other bands. Submit CR in RAN4#99e for TS 38.101-1 (Rel. 17) and TS 36.101 (Rel-10 to Rel-17).

Agreement #3: Remove the following note from Clause 6.2.3.30 of TS 38.101-1 (Rel-17):  “Note that the additional emission requirements in Table 6.5.3.3.27 are specified in EIRP.  The above A-MPR values to meet these additional emission requirements have been estimated using a 0 dBi antenna gain.  The A-MPR may have to be adjusted if the supported antenna gain Gant declared by the UE manufacturer is different from 0 dBi.”

Remove identical Note from Table 6.2.4-34a of 36.101 (Rel-10 to Rel-17).

Submit CR in RAN4#99-e for TS 38.101-1 (Rel. 17) and TS 36.101 (Rel-10 to Rel-17).

Agreement #4: A-MPR Table Update

Agreement on AMPR values towards left side of channel. Remaining brackets to be verified and finalized at the RAN4#99-e.

For 5 MHz channel centered on frequencies (FC) = 1630.0, 1630.3 MHz, A-MPR is defined as

if RBstart <= ceil{3/SCS/15 kHz)}and LCRB <= ceil{17/SCS/15 kHz)},

then

the A-MPR = 14 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart <= ceil{3/(SCS/15 kHz)} and LCRB > ceil{17/(SCS/15 kHz)},

then

the A-MPR = 6 dB,

else,

if RBstart <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 4 dB.

For 5 MHz channel centered on frequencies (Fc) = 1635.0, 1649.0, 1654.0 MHz, no A-MPR is needed.

For Channel 10 MHz with center frequency of 1632.5 MHz, A-MPR is defined as

if RBstart < ceil{3/(SCS/15 kHz)} and LCRB <= ceil{8/(SCS/15 kHz)},

then

the A-MPR = 12 dB for SCS = 15 kHz and AMPR = 8 dB for SCS >= 30 kHz,

else,

if RBstart < ceil{9/(SCS/15 kHz)}, and LCRB > ceil{8/(SCS/15 kHz)},

then

the A-MPR = 8 dB,

else,

if RBstart <= ceil{18/(SCS/15 kHz)},

then

the A-MPR = 6 dB

else,

if RBstart >= floor {[40/(SCS/15 kHz)]} and LCRB <= ceil{[7/SCS/15 kHz)]},

then

A-MPR = [5] dB

else,

if RBstart >= floor {[40/(SCS/15 kHz)]} and LCRB > ceil{[7/SCS/15 kHz)]},

A-MPR = [3] dB

else,

if RBstart >= floor {[35/(SCS/15 kHz)]} and LCRB <= ceil{[7/SCS/15 kHz)]},

then

A-MPR = [4] dB

else

if RBstart >= floor {[35/(SCS/15 kHz)]} and LCRB > ceil{[7/SCS/15 kHz)]},

then

A-MPR = [2] dB

# References

[1] R4-210XXXX, Email discussion summary for [98-bis-e][116] NR_LTE_band_n24