3GPP TSG RAN WG4 Meeting #98-bis-e		R4-2105710

Electronic Meeting, April 12th -20th, 2021

Agenda item:       5.1.3.1

Source: 	Qualcomm

Title: 	WF on CCA models for NR-U RRM performance requirements

Document for:     Approval

# 1	Introduction

This document contains the CCA models related discussions and agreements as discussed during RAN4#98-bis-e in the email thread [98-bis-e][202] NR_unlic_RRM_2 for NR-U RRM performance requirements. The summary containing the complete discussion on that topic is found on the document [1].

Background information: in RAN4#97-e meeting, NR-U RRM performance requirements we first discussed, with the agreements captured in the WF [2]. Agreements on CCA models for NR-U RRM performance requirements from RAN#98-e meeting were captured in the WF [3].

The following colour code is used to highlight the agreements in this meeting:

Agreements in the GTW session

Agreements in the 1st round

Agreements in the 2nd round

FFS items and candidate options

# 2	DL CCA models during RRM tests

CCA DL success probability for semi-static and dynamic channel access configurations (Issue 2-3-2 and 2-3-3)

Candidate options:

Option 1:

- For LBE: P1=0.75, P2=0.5,

- For FBE: P = 0.9

- FFS if the probabilities shall apply only for the low Es/Iot (e.g., Es/Iot<-6 dB).

Option 2:

- For LBE: P1=0.75, P2=0.75,

- For FBE: P = 0.95

- FFS if the probabilities shall apply only for the low Es/Iot (e.g., Es/Iot<-6 dB).

How to avoid exceeding Lmax in RRM tests (Issue 2-3-4)

- Test environment should not have test runs that are rendered useless due to exceeded LBT failures

- Test equipment should make sure that Lmax is not exceeded during a test by monitoring the number of CCA failures and preventing additional CCA failures from happening after Lmax is reached.

DRX CCA model (Issue 2-3-5)

Current DL CCA model applies only for non-DRX

- Model that applies for DRX is FFS

# 3	UL CCA model during RRM tests

UL CCA model (Issue 2-4-1)

- Adopt a baseline UL CCA model as below:

  - TCCA ms  prior to each UL transmission burst in the test:

    - The test equipment (TE) generates a uniform random variable p from the range [0, 1].

    - If p<PCCA_UL, the TE transmits an [OCNG noise pattern] with an energy level X within the UE BW scheduled/configured for the UL transmission for at-least TCCA ms.

      - TCCA is the channel sensing period depending on LBT category being used by the UE

      - PCCA_UL is the probability of a successful UL CCA

      - Energy level X is FFS and is higher than the LBT detection threshold

  - The TE keeps a count of the number of UL CCA failures it causes.

  - The TE monitors the UL resource for the desired UL signal.

  - Based on when and/or whether the TE receives the desired UL signal, it deems the test case to pass/fail

  - Note 1: applicability of OCNG noise pattern is FFS

UL CCA success probability (Issue 2-4-2)

- PCCA_UL is the probability of a successful UL CCA

  - To be determined along with the test case specification

  - To model consistent UL CCA failure, PCCA_UL takes a low value, e.g. 0%

  - To model no UL CCA failure, PCCA_UL takes a high value, e.g. 100%

  - A typical/default value is TBD for PCCA_UL in other test cases, e.g. 75%

Configuration of UL CCA Failure Detection Recovery (Issue 2-4-3)

- Configure UL CCA failure recovery only as part of the following RRM test case:

  - NR-U – NR-U PCell UL active BWP switch based on persistent UL LBT failure

Additional delay in acquiring PRACH resource due to UL LBT failures (Issue 2-4-4)

- RAN4 to test additional delay in acquiring PRACH resource due to UL LBT failures in the following requirement:

  - Handover to a target cell using CCA

Test case list to include UL CCA failures (Issue 2-4-5)

- Include UL CCA failure in Random Access test cases

- Keep UL CCA failures on already endorsed TC: UL BWP switching due to consistent UL failures

- Include UL CCA failure in one of these options

  - Option 2a: SCell activation

    - Additional delay in transmission of CSI reporting due to CCA failure

  - Option 2b: Event triggered measurement reporting delay

    - Additional delay due to UL LBT failure not defined

    - FFS: Assume it similar to above-mentioned SCell activation case

  - Option 2c: MAC CE based TCI state switch delay

    - Delay in sending HARQ feedback transmissions

  - Option 2d: Specifying one test case with UL CCA failure for each of the options 2a, 2b and 2c above.

Note: Option 2c depends on the decision on whether to include tests for Active TCI state switching

# References

- R4-2105804, “Email discussion summary for [98-bis-e][202] NR_unlic_RRM_2”, Nokia, Nokia Shanghai Bell, RAN4#98-bis-e.

- R4-2017089, “WF on NR-U RRM performance,” Nokia, Nokia Shanghai Bell, RAN4#97-e.

- R4- 2104088, “WF on CCA models for NR-U RRM performance requirements,” Qualcomm, RAN4#98-bis-e.