3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2105846

Electronic Meeting, 12 – 20 April, 2021

Agenda item:			8.5.2.1 & 8.5.2.3

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [98-bis-e][219] NR_MG_Part_2

Document for:	Information

# Introduction

The scope of this email discussion is UE RRM requirements for NR positioning from the following agenda items:

- AI 8.5.2.1 Pre-configured MG pattern

- AI 8.5.2.3 Network Controlled Small Gap

In providing comments, companies are encouraged to:

- Be concise

- Provide comments on all topics/sub-topics of interest

- Ensure that comments are inserted in the latest version of the document by checking the folder before uploading

- Use “Track changes” to help identify added comments/changes

# Topic #1: Pre-configured MG pattern(s)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104635 | vivo | Proposal 1: for the pre-configured MG configuration, use R15/16 legacy MGs configuration mechanism Proposal 2: the pre-configured MG is deactivated only when all MOs can be measured without gap. Otherwise the pre-configured MG should be used. Proposal 3: for a BWP, under a measurement object set, the activated/deactivated status of pre-configured gap for that BWP can be determined by the network based on the following rule: the pre-configured MG is deactivated only when all MOs can be measured without gap. Otherwise the pre-configured MG should be used. Proposal 4: Using legacy R15/R16 as the base, extra information which includes the activated/deactivated status of pre-configured gap for all BWP can be included into the measurement configurations. Proposal 5: the pre-configured MG activation/deactivation is autonomously/implicitly triggered by a BWP switching. |
| R4-2104687 | Xiaomi | Proposal 1: It is assumed the pre-configured MG is considered as activated by default for the measurement on MO(s) required pre-configured MG in the current active BWP. Proposal 2: Pre-configured MGs shall be configured per-UE or per-FR based on UE capability. Proposal 3: Pre-configured MG shall be the RRC configured MG in Rel-15, which can be activated or deactivated dynamically according to active BWP switching. Proposal 4: The activation of the pre-configured MG depends on not only the active BWP, but also the MOs configuration. Proposal 5: If UE is indicated the activation/deactivation information for the measurement on each MO requiring the pre-configured MG in each configured BWP, the pre-configured MG can be activated/deactivated autonomously triggered by DCI/Timer based BWP switching. Proposal 6: No need to define separate activation/deactivation delay requirement for pre-configured MG activation/deactivation. Proposal 7: If there is one or more transitions between gap-based and gapless measurement during one measurement period, the relaxed measurement requirement shall be applied. |
| R4-2104750 | CATT | Proposal 1: Pre-configured MGs are configured per UE or per FR which are same as these of legacy MGs. Proposal 2: Activated pre-configured gap can over-ride the current RRC configured MG. Proposal 3: The pre-configured MG is activated for the following cases - Gap is not configured before BWP switch and is requested by at least one MO after BWP switch - Gap (RRC configured or activated pre-configured gap) is configured before BWP switch but a different gap pattern is requested after BWP switch Proposal 4: The pre-configured MG is deactivated for the following case - Pre-configured MG is used before BWP switch and no gap is needed after BWP switch - Pre-configured MG is used before BWP switch but a different gap pattern is requested after BWP switch Proposal 5: No RRC signaling is defined to activate or deactivate the pre-configured MG. Proposal 6: Study CA and with BWP switch on single CC cases only. Proposal 7: No separated activation/deactivation delay is needed for the pre-configured MG. Proposal 8: The measurement period for the measurement with pre-configured MG can reuse the current requirement with measurement gap in R16. Proposal 9: UE can be scheduled during the pre-configured gaps while meeting existing scheduling restriction requirements defined in TS 38.133 if the pre-configured MG is not activated. Proposal 10: The existing gap patterns (0~25) in Rel16 can be reused for the pre-configured MG. Proposal 11: Current per-UE/per-FR applicability can be reused for pre-configured MG. |
| R4-2104854 | Apple | Proposal 1: No specific procedure for activation after the RRC configuration is needed. Proposal 2: Pre-configured MGs are configured per BWP. Different MG patterns (or no MG) can be pre-configured for different BWP. Proposal 3: Pre-configured MG in one active BWP can over-ride current RRC configured MG, i.e. as long as there is pre-configured MG pattern (including no MG) associated with the active BWP, UE shall follow pre-configured MG pattern and ignore the MG pattern (if any) configured in MO configuration. Proposal 4: pre-configured MGs can be activated/deactivated autonomously/implicitly triggered by DCI/Timer based BWP switching. Proposal 5: RAN4 shall focus on BWP switching on single CC case at this stage. Proposal 6: some transition time (T) shall be included in the pre-configured MG activation/deactivation time. T should be in several milliseconds level. Proposal 7: If the gap patterns which can be used as the pre-configured gap are reused from Rel16, the same applicability(per-UE/per-FR) shall follow the rules defined in Rel16 also. |
| R4-2104921 | ZTE Corporation | Proposal 1: Introducing the dynamic MG on/off mechanism can be effective to deal with the dynamic active BWP switching via DCI or timer. Proposal 2: The transition time should be further determined between the dynamic MG on/off. Proposal 3: Dynamic MG on/off only applies for intra-frequency. Proposal 4: For NR-CA case, focus on intra-frequency measurement, dynamic MG off can be enabled only if all active serving cells can execute measurement without gap. |
| R4-2104936 | CMCC | Observation 1: according to the contribution in the last meeting, except the activation/deactivation of MG, the change of MG pattern following BWP are also considered by some companies. Proposal 1: since the use cases of pre-configured MG will have impact on the mechanisms of activation/deactivation of MG, it is better for RAN4 to reach consensus on the use case of pre-configured MG firstly, i.e. except the activation/deactivation of MG, whether MG change following BWP switch is considered or not. Proposal 2: if the use case of MG pattern change following BWP switch is not considered, this autonomously/implicitly triggered way is a good way to move forward. But considering that whether MG is needed or not is also related with the MOs, it is better to specify a clear rule on the activation/deactivation of MG so that NW and UE share the same understanding on the usage of pre-configured MG. Proposal 3: If the use case of MG pattern change following BWP switch is considered, since the selection of MG pattern depends on the SMTC (or CSI-RS) of measurement objects measured in MG and different UE may have different implementation, the autonomously/implicitly triggered way may be not suitable. The better way is to activate/deactivate the pre-configured MG by the network indication. RRC (re)configuration of pre-configured MGs Proposal 4: Pre-configured MGs are configured per UE or per FR, which are same as these of legacy MGs. Proposal 5: it is necessary to discuss whether multiple pre-configured MGs can be configured by RRC or only single pre-configured MG can be configured by RRC. Even though multiple concurrent and independent MG patterns are not considered for pre-configured MGs, it is suggest RAN4 to discuss whether consider the case that RRC configure multiple pre-configured MGs but only one pre-configured MG is activated during a period of time. Pre-configured MG for BWP switching on multiple CCs Proposal 6: Study CA and with BWP switch on single CC cases firstly, after RAN4 has reached consensus on the mechanism of pre-configured MG, we can have further discussion for the case with BWP switching on multiple CCs. |
| R4-2104983 | NEC | Proposal 1: Pre-configured MGs are configured per BWP. Proposal 2: Pre-configured MG in one active BWP can over-ride current RRC configured MG until active BWP switch to a new BWP without per-configured MG pattern. Proposal 3: RAN4 to agree that pre-configured MG is activated/deactivated autonomously/implicitly with BWP switching. Proposal 4: RAN4 to agree that delay of MG (de)activation is same as that of BWP switching. |
| R4-2106446 | Intel Corporation | Observation 1: It is feasible that the pre-configured MG can be configured by the similar way as the legacy MGs (e.g. “MeasGapConfig” in Rel16), which define MG associated with MOs themselves. Observation 2: There are much higher standardazation works loading increased if the pre-configured MG is based on per-BWP. Proposal 1: Pre-configured MGs are configured per UE or per FR which are same as these for legacy MGs. Observation 3: BWP switching is one of necessary conditions for pre-configured MG’s activation. Observation 4: Both UE and NW have the same understanding on the needs on the measurement gap for the measurements after BWP switching. Proposal 2: It is feasible and efficient with autonomously/implicitly activation for preconfigured MG triggered by DCI/Timer based BWP switching. Observation 5. The gap for PRS measurement shall be configured immediately when UE is requested to measure PRS. That is the gap for PRS measurement is independent with BWP switching. Observation 6. The pre-configured gap can be helpful to reduce MG configuration delay for CSI-RS measurement significantly. Proposal 3: The preconfigured MGs can be applied to SSB, CSI-RS measurement only in Rel17. Observation 7. When BWP switching on the multiple CCs, the criteria to justify whether the pre-configured gap is needed is same as that of when BWP switching on single CC. Proposal 4: RAN4 can consider the application scenarios of the pre-configured gap for single CC BWP switching as a start point. Observation 8: The activation delay for pre-configured MGs shall include the BWP switching delay at least. Proposal 5: RAN4 needs NOT to define the separated activation delay requirements for the pre-configured MG activation unless the BWP switching time is shorter than “gap transition time”. Proposal 6a: The RAN4 minimum requirements for intra-frequency SSB measurement can follow that of intra-frequency SSB measurement requirements with gap specified in 9.2.6 of TS38.133 [3]. Proposal 6b: The RAN4 minimum requirements for intra-frequency SSB measurement and CSI-RS measurement with pre-configured MG can follow that of intra-frequency SSB measurement requirements with gap specified in 9.2.6 of TS38.133 [3] and inter-frequency CSI-RS measurement requirements specified in 9.10.3 of TS38.133 [3] respectively. Proposal 7: The same scheduling restriction in Rel16 [4] when preconfigured MG configured can be appliable. Observation 9: The same RF switching time when considering pre-configured gap pattern as the legacy gap patterns in NR [3] can be reused. Observation 10: MGL of the pre-configured gap patterns can also rely on the measurement type (e.g. SSB or CSI-RS). Observation 11: The shorter MGL can be considered because of the perfect synchronization of measured BWP. Proposal 8 : The existing gap patterns in Rel16 [3] can be reused for the pre-configured MG depending on the configuration of the targeted measurements reference signal. Observation 12: It is feasible to include the pre-configured gap as one of instance of multiple concurrent gap pattern if UE supported. The more concerted discussion jointly with them can be continued in 2nd later state of this WI. Observation 13: There is no restrictions on the total number of preconfigured gaps. Observation 14: The serving gNB need not activate all pre-configured gaps but part of them depending on the measurement configuration and bwp-InactivityTimer [4]. |
| R4-2106535 | OPPO | Observation 1: Pre-configured MGs can be activated/deactivated autonomously/implicitly triggered by DCI/Timer based BWP switching. Observation 2: UE behavior should be clarified after deactivation of pre-configured MG and switching to a new BWP without any per-configured gap. Observation 3: The relation of per-configured MG and concurrent/independent gap need to be clarified before decision on the applicability. Proposal 1: Set pre-configured MG inactive as default after the RRC configuration, with no additional signaling. Proposal 2: After deactivation of pre-configured MG and switching to a new BWP without any per-configured gap, - option 1: UE shall perform measurement without gap until switching away from this BWP. - option 2: Immediate activation of default gap. UE shall perform measurement with default gap. - Option 3: UE shall perform measurement with legacy per UE or per FR gaps. Proposal 3: The applicability of per FR gap should apply for per-configured MG. Proposal 4: Pre-configured MGs are configured per BWP, and used as per FR gaps. Proposal 5: pre-configured MG in one active BWP with its own applicability can over-ride current RRC configured MG until active BWP switch to a new BWP without per-configured MG pattern. Proposal 6: Per-configured MG pattern should comply with current requirements of NR MG pattern 0~23. Proposal 7: All mandatory gaps for UE should be able to be supported by pre-configured BWP MG configuration. Proposal 8：Limit the available pre-configured MG patterns within the most commonly used MG patterns. Proposal 9: Per BWP MG configuration become valid once this BWP become active, with no extra impact on current BWP switch delay. |
| R4-2107027 | Huawei, HiSilicon | Proposal 1: RAN4 to support ON/OFF of the pre-configured MG. Changing of MG configuration following a BWP switching is not considered. Proposal 2: Pre-configured MGs are configured per UE or per FR which are same as these of legacy MGs. Proposal 3: Enable/disable of pre-configured MG should be explicitly configured by the NW. Proposal 4: FFS if activation and deactivation of pre-configured MG is implicitly triggered by BWP switch or explicitly configured by NW. Proposal 5: RAN4 should discuss how NW and UE should determine the status of the pre-configured MG following RRC/MAC procedures such as configuration of pre-configured MG, (re)configuration of MO, (re)configuration of serving cells and SCell activation and deactivation. Proposal 6: RAN4 to discuss if pre-configured MG is supported in CA or not, considering activation and deactivation of pre-configured MG is to be done. Proposal 7: RAN4 to discuss the activation and deactivation delay after deciding on the activation and deactivation mechanism. Proposal 8: The transition requirements defined in clause 9.1.6 apply also with pre-configured MG. Proposal 9: RAN4 not to define limit on the maximum number of transitions (between MG based and MG-less measurement) during the measurement period. Proposal 10: Existing scheduling restriction for RRM measurement without MG applies when pre-configured MG is deactivated. Proposal 11: All existing MG patterns #0~25 in Rel-16 are applicable for the pre-configured MG. Proposal 12: Both per-UE and per-FR MG are applicable for pre-configured MG. Proposal 13: When per-FR gap is configured, activation/deactivation of pre-configured MG are independent for each FR. |
| R4-2107151 | Ericsson | P-MGP status upon RRC configuration: - Observation # 1: Default or fixed pre-configured measurement gap pattern (P-MGP) status (activated or deactivated) is not efficient. - Proposal # 1: P-MGP shall not have a fixed status (activated or deactivated) upon RRC configuration. - Proposal # 2: - Option 1: Upon RRC configuration the P-MGP is considered to be activated by UE and gNB if the SSB is not within the BW of the active BWP; otherwise (if the SSB is within the BW of the active BWP) then the P-MGP is considered to be deactivated by UE and gNB. - Option 2: Network signals the P-MGP status (activated or deactivated) in the RRC message configuring the P-MGP. P-MGP configuration procedure: - Observation # 2: A MGP does not have any relation to the BWP. - Observation # 3: P-MGP is used when the UE may not always need gaps for measurement on certain carrier. - Observation # 4: The UE configured with P-MGP may later be configured to measure on carriers (e.g. inter-RAT) which always need gaps for the measurement. - Proposal # 3: P-MGP is configured as per UE or per FR depending on UE capability. - Proposal # 4: Any of the Rel-15 MGP can be configured as the P-MGP, which can be activated or deactivated (ON or OFF). - Proposal # 5: - Option 1: The already configured P-MGP is transformed into legacy MGP (with same MGL/MGRP) if the UE is configured to measure on any carrier (e.g. inter-RAT) which always need gaps for performing the measurement. - Option 2: Network can transform an already configured P-MGP into legacy MGP with same MGL/MGRP or vice versa without deconfiguring the P-MGP. P-MGP activation/deactivation procedure: - Observation # 5: The UE needs gaps to measure SSBs when the measured SSB is not fully within the BW of the active BWP. Otherwise the UE can measure the SSBs without gaps. This change between gap based and gapless measurement is triggered by active BWP switching. - Proposal # 6: P-MGP is autonomously/implicitly triggered by DCI/Timer based BWP switching. - Proposal # 7: The P-MGP is considered as activated by the UE and gNB if the BW of the active BWP (after the switching) does not fully contain the BW of the SSB(s) to measure. - Proposal # 8: The P-MGP is considered as deactivated by the UE and gNB if the BW of the active BWP (after the switching) fully contains the BW of the SSB(s) to measure. Scenarios for P-MGP: - Proposal # 9: Study CA and with BWP switch on single/multiple CC cases. - Proposal # 10: When the UE is configured with CA then the P-MGP is considered as activated by the UE and gNB if the following condition is met for at least one CC within the CA: - BW of active BWP (after the switching) does not fully contain the BW of the SSB(s) to measure on a CC. - Proposal # 11: When the UE is configured with CA then the P-MGP is considered as deactivated by the UE and gNB if the following condition is met for all the CCs within the CA: - BW of active BWP (after the switching) fully contains the BW of the SSB(s) to measure on a CC. Activation/deactivation delay requirements: - Observation # 6: Transition time for switching between gap-based measurement (P-MGP’s activated state) and gapless measurement (P-MGP’s deactivated state) is needed by the UE to adapt to the new measurement procedure after the active BWP switching e.g. since measurement sampling may be different in the two procedures - Observation # 7: Transition time for switching between gap-based measurement (P-MGP’s activated state) and gapless measurement (P-MGP’s deactivated state) is needed the gNB to adapt to scheduling after the active BWP switching e.g. complete on going scheduling in gaps or start scheduling in gaps. - Proposal # 12: Transition time (T) shall be included in the pre-configured MG activation/deactivation time. Measurement period requirements: - Observation # 8: The delay requirements in section 9.1.6, TS 38.133 for transitions from measurements performed outside gaps to measurements performed within gaps or vice versa does not account for any time to configure the measurement gaps e.g. when switching from gapless to gap-based measurement. - Observation # 9: The requirements in section 9.1.6, TS 38.133 apply when the UE is already configured with gaps for certain measurement but after the BWP switching the UE starts using the same gaps also for measurement which was being performed without gaps before the BWP switching. - Proposal # 13: The total measurement period Tmeasure, total) to account for transition between activated and deactivated P-MGP during the measurement needs to be specified. - Proposal # 14: Total measurement period (Tmeasure, total) can be expressed in terms of basic measurement period (Tmeasure, basic) and aggregated time consumed due to total number of transitions between gapless measurement procedure and gap-based measurement procedure during the ongoing measurement. - Proposal # 15: In proposal 8, Tmeasure, basic can be expressed as: Tmeasure, basic = MAX(TBWP, TG); where: - TBWP= It is the measurement period when the measurement is fully performed without measurement gap - TG= It is the measurement period when the measurement is fully performed with measurement gap. Number of transitions during measurement period: - Observation # 10: Too frequently switching between gapless measurement procedure and gap-based measurement procedure may lead to measurement instability and may also not give gNB enough opportunity to adapt scheduling. - Proposal # 16: The maximum number of transitions (N1,max) allowed for switching from gapless measurement procedure to gap-based measurement procedure during the ongoing measurement is defined e.g. N1,max = [2-4]. - Proposal # 17: The maximum number of transitions (N2,max) allowed for switching from gap-based measurement procedure to gapless measurement procedure during the ongoing measurement is defined e.g. N1,max = [2-4]. - Proposal # 18: If an active BWP switch occurs while UE is performing a measurement with gaps for the last at least X subframes and the new active BWP after the active BWP switching fully contains the measured SSB only then the UE shall continue the ongoing measurement without measurement gaps; otherwise the UE shall continue the ongong measurement using pre-configured gaps. Scheduling restriction requirements: - Proposal # 19: If the UE is measuring without pre-configured gaps and no other frequency layer which needs gaps is configured then the UE can be scheduled during the pre-configured gaps while meeting existing scheduling restriction requirements defined in TS 38.133. Gap patterns for P-MGP: - Observation # 11: The MGP # 24 and # 25 are used when the UE is configured with positioning measurements, which always need gaps. - Proposal # 20: The existing gap patterns (0~23) in Rel16 can be reused for the pre-configured MGP. - Proposal # 21: If the gap patterns which can be used as the pre-configured gap are reused from Rel-16 then the same applicability (per-UE/per-FR) shall also follow the rules defined in Rel-16. |
| R4-2107175 | Nokia, Nokia Shanghai Bell | We observed: - In NR Rel-15, RRC signalling for configuring and activating measurement gaps has been used due to signalling robustness. - Errors in measurement gap configuration can have significant negative UE and system impact. - It is important that changes in the measurement gaps are signalled in a robust way. Following proposals are made, which we see necessary to be considered in the discussion to ensure that a practically usable feature will be developed: - The design of general procedures for use of pre-configured MG should take into account single MGP and multiple concurrent MGPs as well as implicit NCSG. RAN4 need to account for robustness of the measurement gap changes when evaluating and agreeing on activation/deactivation of MG pattern(s) without using RRC signalling. Current RRC signalling message for configuring pre-configured MG pattern (i.e. RRC reconfiguration command) shall be used and is extended to include one or more MGPs per BWP used for per-UE or per-FR gap support. When pre-configured MG patterns are configured via RRC signalling, they are not activated at the same time of configuration. RAN4 to consider the robustness of the mechanisms for activation and deactivation of MG pattern when recommending the method to be used. MGP change delay shall be evaluated based on realistic latencies. No additional separate delay is needed for activating a preconfigured MGP. No separate additional delay is needed for deactivating a preconfigured MGP. Robustness shall be evaluated including the final signal loss probability. Analyse and evaluate, under realistic assumption, the possible impact on ongoing cell detection from a change in MGP. Analyse and evaluate, under realistic assumption, the possible impact on the latency of ongoing measurements from a change in MGP. RAN 4 to define procedures for pre-configured MG pattern support, which provide flexibility and full network control related to MG pattern usage. RAN4 to focus on single CC for the work on pre-defined MG patterns. RAN4 to consider MG patterns #0-#25 for use as pre-configured MG pattern. RAN4 to specify the same per-UE / Per-FR applicability for pre-configured MG patterns as for legacy MG patterns in Rel-16. RAN4 not to specify maximum number of transitions between gapless and gap-based measurement procedures for ongoing measurements. Current scheduling restrictions in TS 38.133 apply in case the UE does not require pre-configured or legacy measurement gaps. |
| R4-2107347 | Qualcomm CDMA Technologies | Observation1: It is not preferred to have UE and network implicitly follow the same set of rules for creating the gap instances. Observation2: Whether a MG shall be ON or OFF for a BWP can be under the full jurisdiction of the network for robustness and avoiding redundant UE processing. The decision can be a binary indication and explicitly configured to a UE per BWP with minimized extra signalling cost. Observation3: pre-configured MG via network controlled fast gap can be either a default state for the legacy gap per BWP; or, a separately configured MG and default state per BWP. Proposal1: Fast gap is completely controlled by the network and the decision can be explicitly configured to the UE as a binary indication per BWP. Proposal1.1: RRC signalling support is needed to indicate whether a legacy MG is ON or OFF by default for a given BWP. Proposal1.2: if proposals 1 and 1.1 are agreed, RAN4 to send LS to RAN2 for defining the signaling and configuration/reconfiguration procedures for the network controlled fast gaps (NCFG) to realize pre-configured MG. Proposal2: RAN4 to discuss if network controlled fast gap applies to CA case. Proposal2.1: If considered, the same MG shall be applied to all the carriers per FR or per UE consistently according to agreed rules. Proposal3: RAN4 to discuss realistic time margins to activate and deactivate the pre-configured MG during BWP switch. Proposal3.1: MG deactivation delay of [Y]ms is defined from the DCI of BWP switch is received to the eraliest time instant when MG can be aborted. Proposal3.2: MG activation delay of [X]ms is defined from the completion of target BWP activation to the earliest time instant when a MG can be entered. Proposal3.3: No requirements shall be defined for the measurements within the gaps during [Y] and/or [X]ms. Proposal4: RAN4 may firstly discuss whether a separate MG configuration is provided in the NCFG configuration by the network. Proposal4.1: If NCFG contains a separate MG configuration, its relationship with the legacy MG shall be clarified. |
| R4-2104582 | MediaTek inc. | Observation 1: Both NW and UE shall have the same understanding on MG ON/OFF in each gap occasion. Observation 2: When multiple BWP switch, it’s too complex to decide the MG ON/OFF and how to indicate the signalling to UE if RAN4 agrees to use explicit configuration. Observation 3: Too limited application scenarios for pre-configured MG in multiple carriers. Observation 4: When MG ON/OFF is dynamically switch, both Kp and CSSF will be updated for measurement requirements. Observation 5: From UE’s perspective, MG ON/OFF dynamically switching will bring more challenges to UE scheduling and algorithm design. Proposal 1: The pre-configured MGs is the legacy RRC configured MG which is a per-UE or per-FR gap and may be ON/OFF after BWP switch Proposal 2: After the MG RRC configuration, no additional specific MAC/DCI procedure for activation is needed. Proposal 3: Pre-configured MG can be implicit ON/OFF because both NW and UE should have the same understanding on BWP switch and MO configurations. Proposal 4: RAN4 to focus on single carrier to define the basic procedure and requirement for pre-configured MG. Proposal 5: When considering multiple BWP switch, the overall procedure for pre-configured MG is too complex. RAN4 to deprioritize the multiple CC scenarios and focus on single carrier in the 1st stage. Proposal 6: Due to no dynamic coordination between MN and SN, the pre-configured MG mechanism is not applied in DC scenario. Proposal 7: The pre-configured delay includes BWP switch delay with additional validation time. The validation time is 20ms. Proposal 8: RAN4 defines requirement with at most one transition during one measurement period. No requirements for more than one transition in one measurement period. Proposal 9: No new measurement delay and accuracy requirement for pre-configured MG needed to be defined. The intra-frequency measurement period and minimum transition requirement in Rel-15 can be reused. Proposal 10: If the UE supports Rel-17 pre-configured gap capability and OFF the gap after BWP switch when no other frequency layer which needs gap is configured, then the UE can be scheduled during the gaps while meeting existing scheduling restriction requirements defined in TS 38.133. |

## Open issues summary and companies views’ collection for 1st round

[Moderator Notes: In the last meeting, the basic procedures to use the preconfigured MG for UE measurement was agreed as:

- Further study the following procedures for pre-configured MGs:

  - 1. (Re)Configuration of the pre-configured MG

    - FFS if specific procedure for activation after the RRC configuration is needed

  - 2. Activation the pre-configured MG following a DCI or timer-based BWP switch

  - 3. Deactivation the pre-configured MG following a DCI or timer-based BWP switch

  - Note 1: The conditions and details of each procedure are FFS

  - Note 2: MG activation in this context means that both NW and UE assume that the pre-configured MG will be used for measurements.

  - Note 3: MG deactivation in this context means that both NW and UE assume that the pre-configured MG will not be used for measurements and UE should be able to receive scheduled data

Therefore, in this meeting we can focus on the issues of using scenario and configuration/activation procedures of pre-configured MG below.]

### Sub-topic 1-0 Definition and using scenarios for Pre-configured measurement gap

#### Issue 1-0-1 Which types of measurements can the pre-configured MG be used for?

- Option 1a (Ericsson): SSB measurements.

- Option 1b (Intel): SSB, CSI-RS measurementOption 2 (Nokia): Pre-configured MG patterns should be usable according to the existing MG applicability rules, i.e. for all types of measurements that use a legacy MG pattern in Rel-15 or Rel-16.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | We support option 2. In R15 and R16, MG can be used for SSB based L3 measurement, CSI-RS based L3 measurement and positioning related measurement including RSTD, UE Rx-Tx and PRS-RSRP. In principle pre-configured MG patterns can be used for all types of measurements that use a legacy MG pattern. |
| CMCC | There is no doubt that SSB is considered for pre-configured MG. For CSI-RS and PRS, more discussion is needed. In Rel-16 CSI-RS, only intra-f without MG and inter-f with MG is considered. Since pre-configured MG is to activate/deactivate MG with BWP switching, we are not sure whether it can be applied to CSI-RS measurement. For PRS, it is assumed to be measured in measurement gap, also not sure whether the pre-configured MG can be applied to PRS measurement (at least deactivation of pre-configured MG is not applicable for PRS. For activation, it seems that the pre-configured MG is always activated when there is PRS measurement). |
| MTK | Support Option 1. For CSI-RS, we do not have the rule and requirements for intra-frequency measurement with gap in Rel-16. For PRS, all measurements are gap-assisted, we fail to see the point to discuss pre-configured gap for it. |
| Qualcomm | Option2 is supported. |
| vivo | Option 1 is ok. |
| OPPO | Option 2 is fine, as pre-configured MG patterns can be used as a legacy MG pattern.for all types of measurements. |
| Ericsson | We support option 1a. It is better to focus on SSB based measurements in the first phase. The pre-configured gaps are not relevant for PRS based measurements. |
| Xiaomi | Option 1a can be considered as the baseline, for CSI-RS measurement, similar view as CMCC, as it was only defined the CSI-RS inter-frequency measurement requirement with MG, there is no benefit to apply pre-configured MG for CSI-RS measurement without the inter-frequency measurement without MG requirement. |
| ZTE | Support Option 1. For CSI-RS, intra-frequency measurement with gap is not considered in Rel-16, so we are not sure the rule and requirements for such case. For PRS, only measurement with gap allowed, so there is no need to consider intra-frequency measurement with gap. |
| CATT | Support option 2. |
| Nokia | We support option 2. |
| Intel | Support Option 1b/1a For PRS measurement, it is not feasible to utilize such pre-configured MG because of PRS configuration is controlled by LMF instead of gNB. |
| LG Electronics | Support Option 1a. |
| Huawei | We support option 2. If a measurement such as positioning measurement, which always requires MG, is configured, then the pre-configured MG should just remain activated. |
| NEC | We support option 1a. We do not see the need for PRS based measurements to be preconfigured as they always require MG. |

#### Issue 1-0-2 Whether the pre-configured MG for the BWP switching on multiple CCs shall be considered?

- Option 1 (Ericsson, Qualcomm, CMCC): Yes.

- Option 1a (ZTE): For NR-CA case, focus on intra-frequency measurement.

- Option 2 (Apple, Intel, Nokia, CATT, , MTK, Huawei): Focus on the application scenarios of the pre-configured gap for single CC BWP switching

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | We prefer option 2, i.e. starting from single CC first. When we have solid design for single CC case, then we can further check if it is possible to extend to multiple CC scenario. Some other aspects have to be decided first before we come to multiple CC cases, such as whether per-CC gap is to be introduced in this WI, how the pre-configured MG pattern would be enabled/disabled and so on. |
| CMCC | We prefer option 1 to support CA. And our suggestion is to study single CC case firstly, after RAN4 has reached consensus on the mechanism of pre-configured MG, we can have further discussion for the case with BWP switching on multiple CCs. |
| MTK | We support Option 2 as the starting point. Then up to RAN4’s progress, we can later check if we still have time to work on multiple CCs. |
| Qualcomm | Option1 is supported. When to start discussing is up to RAN4 agreement, e.g. in the 2nd stage? |
| vivo | Prefer option 2. |
| OPPO | Support option 2, to start from single CC BWP switching. |
| Ericsson | We support option 1. But for the sake of progress we can also compromise on option 2 (single CC). In phase 2 we can consider the case of multiple CCs. |
| Xiaomi | Support option 2, prefer to focus on single CC BWP switching in current stage. |
| ZTE | We prefer option 1a. Firstly, NR CA is an important type of deployment scenario. If not considering NR CA, many application scenarios for pre-configured MG are lost. Secondly, the MG configuration is per UE or per FR, not per CC, so it is natural for the CA scenario to support pre-configured MG. Why only focus on intra-frequency measurement, because whether measurement without gap can be implemented for inter-frequency requires not only active BWP totally contains SSB but also UE to support interFrequencyMeas-Nogap-r16 capability. Not each UE has such capability, so for the sake of simplicity, we suggest only focus on intra-frequency. |
| CATT | Support option 2. Prefer to discuss based on single CC BWP switch first. |
| Nokia | We support option 2. Single CC BWP switch should be first investigated. |
| Intel | Our intention is to focus the single CC in the earlier stage. Then we can optimize them for the multiple CC cases. |
| LG Electronics | Support Option 2. |
| Huawei | Support option 2. |
| NEC | Support option 2 |

#### Issue 1-0-3 Whether the pre-configured gaps shall be considered as a part of multiple concurrent gap patterns framework

- Option 1: (Qualcomm, Intel)

  - It is feasible to include the pre-configured gap as one of instance of multiple concurrent gap pattern if UE supported. But RAN4 can focus on the separated pre-configured MG firstly.

- Option 1a(CMCC): RRC configure multiple pre-configured MGs but only one pre-configured MG is activated during a period of time.

- Option 2 (Nokia): The design of general procedures for use of pre-configured MG should take into account single MGP and multiple concurrent MGPs.

Recommended WF:

- RAN4 can focus on the separated pre-configured MG firstly.

| Company | Comments |
| --- | --- |
| Apple | We prefer option 1. As for option 1a, according to R4-2104936 we think by saying “RRC configure multiple pre-configured MGs” CMCC means NW can configure different MG pattern for different BWP. Since there is only one active BWP, there should be only one pre-configured is activated during a period of time. If our understanding is correct, then option 1a should not be categorized into this issue. Regarding option 2, in principle it is also fine for us to keep that in mind. However, if we really want to support pre-configured MG + multiple concurrent MG, we should also keep this mind when designing multiple concurrent MG procedure. |
| CMCC | We would like to provide some clarification for option 1a. We are OK to handle pre-configured MG and multiple concurrent gaps separately, and focus on the pre-configured MG firstly. Option 1a is about how many MG patterns can be (re)configured by RRC: whether multiple pre-configured MGs can be configured or only single pre-configured MG can be configured. Even multiple gaps are configured by RRC, option 1a is different from multiple concurrent gap patterns. Multiple concurrent gap patterns means that multiple gaps are configured by RRC and multiple gaps are activated. While option 1a is the scenario that multiple gaps are configured by RRC but only one gap is activated. We would like to know companies’ view on this scenario. |
| MTK | Support Option 1. Actually we do not think RAN4 needs to revisit this issue again. RAN4 already had the agreement in the approved WF R4-2104096 that - Before RAN4#100b (Q4’21), RAN4 focuses on the functionality and principles needed to support parallel MG patterns without considering pre-configured gap and NCSG. On CMCC’s proposal, we kind of prefer to keep the scope simple in the beginning, i.e., single configured gap and single activated gap. |
| Qualcomm | Option1 is supported. Agree with MTK to follow the previous WF agreement. |
| vivo | Agree with “But RAN4 can focus on the separated pre-configured MG firstly |
| OPPO | Option 1 is fine. |
| Ericsson | Our view is closest to option 1. In the first phase we should consider a scenario where only one pre-configured MG pattern is considered. Multiple concurrent MG patterns should be considered in the 2nd phase. |
| Xiaomi | Support option1 |
| ZTE | Support option 1. |
| CATT | Support option 1. For option 1a, after the clarification, our understanding is that multiple gap patterns can be pre-configured to UE but only one can be activated. We are fine with this. |
| Nokia | We support option 2, i.e. the design of preconfigured MG patterns should allow extension to multiple concurrent MG patterns. This should not only be progressed in 2nd phase. Thus, we need to keep this in mind also in 1st phase. |
| Intel | We support Option 1. For Option 1a, the multiple concurrent MG means there are more than one MGs(including the legacy MG and pre-configured MGs) being activated. |
| LG Electronics | Support Option 1. |
| Huawei | Support the Recommended WF. It was already agreed in last meeting that this would be considered in 2nd stage of the WI. |
| NEC | Share similar view as Huawei. Agree with recommended WF. |

### Sub-topic 1-1 Configuration for Pre-configured measurement gap

#### Issue 1-1-1: Whether can pre-configured MGs be configured per UE/FR or per BWP

[Moderator Notes:]

- Option 1a (Apple, NEC). Pre-configured MGs are configured per BWP. Different MG patterns (or no MG) can be pre-configured for different BWP.

- Option 1b(OPPO) Pre-configured MGs are configured per BWP, and used as per FR gaps.

- Option 2 (Ericsson, Intel, Huawei, Xiaomi, CATT, MTK). Pre-configured MGs are configured per UE or per FR

- Option 2a (Nokia): Pre-configured MGs are configured per-UE or per-FR. Pre-configured MGs additionally can be configured per BWP.

Recommended WF:  Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | We support combination of 1a and 2a. i.e. different MG patterns (or no MG) can be pre-configured for different BWP. Once the pre-configured MG is enabled, the “MG type” should be same as legacy, either per-UE or per-FR. We are not sure if all the proponents of option 2 disagree with per-BWP configured MG. We would like to point out that both UE and NW can benefit from per-BWP configured MG pattern. For instance, as we know one of the main motivations of BWP switching is to allow power saving. When there is no huge data demand, UE can fall back to the “casual” BWP with e.g. less MIMO layers for power saving. When huge data packages come, NW would switch the UE back to the “busy” BWP. It would be beneficial to give NW flexibility to configure MGP with small MG overhead on “busy” BWP to focus on throughput but configure MGP with larger MG overhead to focus on mobility on “casual” BWP. This is just an example. More and more use case can be found in future. On the other hand, option 2 can be covered by option 1a or 2a. NW can configure the same MGP for all the BWP if NW believes in certain scenario different MGP for different BWP is not necessary. |
| CMCC | Option 2. The motivation of pre-configured MGs is to dynamically activate/deactivate the MG following the BWP switch, which seems no impact on the configuration of MG. Based on this understanding, pre-configured MGs can be configured per UE or per FR which are same as these of legacy MGs. |
| MTK | Support Option 2 As some companies already mentioned, - The benefit to change gap pattern for different BWP is not clear. The number of inter-frequency layers and inter-RAT layers is unlikely to be different after BWP switch. - Per-BWP gap is problematic under multiple CC case. We do not believe the BWP status on only one single CC is sufficient to change the MG pattern. To progress, RAN4 should target to complete the baseline mechanism before considering more advanced variations in order to deliver the feature to the market on time. If companies also agree that Option 2 is covered by option 1a or 2a, why not to start from Option 2? |
| Qualcomm | Option2 is supported for flexibility of configuration and usage. |
| OPPO | Support option 1b, and can compromise to option 2a. Whether a UE can be (pre-)configured with per UE gap and per FR gap simultaneously should be clarified firstly. This issue should be also solved together with applicability of concurrent gaps. |
| Ericsson | Support option 2. Other options are complex and involve unnecessary overheads but without any real benefit. When SSB will be within an active BWP of any of the configured BWP then the pre-configured MG pattern will be used. It does not matter what is the characteristics of the BWP. It is even better to use the same MG pattern to make the measurements more consistent and delay is more predictable. |
| Xiaomi | Support option 2, from our perspective, the pre-configured MG is the legacy MG which it can be dynamically ON/OFF with the BWP switching. So, the pre-configured MG should be configured either per-UE or per-FR. |
| ZTE | Support option 2. We agree with CMCC, the motivation of pre-configured MG is to dynamically activate/deactivate the MG following the BWP switch, so no impact on the configuration of MG. |
| CATT | Support option 2. The pre-configured MG is also legacy MG and should be configured per-UE or per-FR. |
| Nokia | We support option 2a. The per-BWP configuration can be used to configure different MG patterns depending on e.g. the traffic load, as pointed out by Apple. Configuring all BWPs with same MGP according option 2 does not provide sufficient flexibility for the network. |
| Intel | We support Option 2. One of import question for Option 1a/1b is that how the per-BWP MG associated with the target measurement object. Several fundamental parameters defined in MG pattern is the measurement length and MGRP, which shall be depended on the targeted MO only instead of UE's active BWP. For an example, if the MGs for UE’s BWP are different as Option 1a, are the MGL and MGRP for different BWPs different? But technically the measurement object are same for UE (the measurement object changed after BWP switching will not considered so far). On the other hand, the benefits of power saving can be achieved by the way with per-UE/per-FR MG which are pre-configured before BWP switching but not enabled until BWP switching or other conditions satisfied. |
| LG Electronics | Support Option 2. |
| Huawei | Support option 2. Per-BWP configured MG does not work when UE s configured with CA, and even for single CC case, it causes unnecessary complexity for NW and UE, as it unnecessarily relates measurement configuration to BWP configuration. |
| NEC | We support option 1a. Though there may be some signaling overhead, there are benefits like lesser reconfigurations and optimized MG configuration. |

#### Issue 1-1-2: How can pre-configured MGs be configured

- Option 1 (MTK, vivo, Intel, Xiaomi). Pre-configured MG can be configured by the similar way as the legacy MGs (Rel16)

- Option 1a (Nokia). Current RRC signalling message for configuring pre-configured MG pattern (i.e. RRC reconfiguration command) shall be used and is extended to include one or more MGPs per BWP used for per-UE or per-FR gap support.

- Option 2(Qualcomm). A binary indication is pre-configured for every BWP which specifies the default ON or OFF state for the legacy MG  or a separate/independent pre-configured MG by RRC.

- Option 3 (Huawei). Enable/disable of pre-configured MG should be explicitly configured by the NW

Recommended WF:  Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | Depending on conclusion of previous open issues. Option 1a is closer to our design. |
| CMCC | From configuration point of view, we are OK with option 1 to reuse the legacy way. For the activation/deactivation of the pre-configured MG, it is separated discussion and can be discussed in Sub-topic 1-2 |
| MTK | Support Option 1. We are open to Option 2 at least for single CC case. For CA case, more discussion is needed. We believe that Option 3 is talking about how to enable the feature, but not how to configure the gap. |
| Qualcomm | Option2 is supported. For Option1, it emphasizes the MG pattern to be pre-configured. Option2 additionally asks the binary indication shall also be pre-configured to UE. And we think Option2 is offered to be one solution for Option3. |
| vivo | For us option 1 is regarding how to configure pre-configured MG. Option 2 and 3 are on how to build the link between pre-configured gap and a particular BWP and could be discussed separately. OK with option 1 for pre-configure gap configuration. OK with option 2 as a detailed solution for the link between a BWP and a pre-configured gap. |
| OPPO | Option 1 is more feasible. For other options, additional signaling would be introduced which causes complexity of this feature. |
| Ericsson | We support option 1. |
| Xiaomi | Support option1, as the pre-configured MG is the legacy MG which it can be dynamically ON/OFF with the BWP switching. So, the pre-configured MG should be configured by the similar way as the legacy MG. |
| ZTE | Support option 1. Further more, pre-configured MG can be the traditional RRC configured MG and it may be dynamic ON/OFF with active BWP switching if UE supports this capability. |
| CATT | Option 1 can be a starting point. Our understanding is that the MG patterns can be pre-configured using the similar way as legacy gaps, but whether to add ON/OFF bit or other information for pre-configured MG need further discussion. |
| Nokia | We support option 1a. Current RRC signalling can be extended for configuring pre-configured MG patterns providing full network control. |
| Intel | Even there is some additional aspects introduced for pre-configured MG in the further (e.g. MG’s activation indications), the framework of configuration for preconfigured MG can be the same configuration as the legacy MG. That is the measurement gap shall be associated with MO as below. “MeasConfig ::= SEQUENCE { measGapConfig MeasGapConfig ]] }” |
| LG Electronics | Support Option 1. |
| Huawei | We support a combination of option 1 and option 3. The MG configuration, e.g. MG pattern, offset, MGTA, can be configured in the similar way as today. In addition, NW should explicitly enable or disable use of the pre-configured MG, so that NW and UE have common understanding about whether the configured MG is used as pre-configured MG or not. |
| NEC | Closer to option 1a. Exact design may be RAN2 aspect. |

#### Issue 1-1-3  Whether is the pre-configured MG activated or not after configuration completed?

- Option 1 (Xiaomi): Activated by default

- Option 2 (Nokia, Intel, OPPO) : Not activated by default until being activated.

- Option 3 (Ericsson):  Status of pre-configured MG is not fixed (no default status):

  - Option 3a: Whether pre-configured MG activated or not depends on whether reference signal to measure is within the active BWP or not.

  - Option 3b: Network signals the status (activated or deactivated) when pre-configured MG is configured.

Recommended WF:  Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | In our understanding the pre-configured MG should be configured associated with certain BWP. Once the BWP becomes active, corresponding pre-configured MG shall be activated. Otherwise, it is deactiavated. |
| CMCC | Option 3. This is related with the mechanism of activation/deactivation of the pre-configured MG. If the implicitly way is agreed, the pre-configured MG is activated or not depend on whether there is MO which is measured with MG. if the explicitly way is agreed, the pre-configured MG is activated based on the network configuration. |
| MTK | Support Option 3a. UE and network should check whether the SSBs of all intra-frequency layers are within UE’s active BWPs or not. And the use this to turn ON or OFF the pre-configure gap |
| Qualcomm | Assuming option2 in issue 1-1-2 is employed, the default state can always be changed via RRC reconfiguration when new measurement types are added. So Option1 can be supported and network still can signal the state via RRC. |
| vivo | Option 3a |
| OPPO | Option 2 is preferred. To be clarified, it is not activated by default until being activated along with its BWP being activated. Whether UE is required to use the MG can further depend on the relation of RS and BWP location. |
| Ericsson | We support option 3. Among them our preferred option is 3b. But we can also compromise on option 3a (implicit) i.e. pre-configured MG is deactivated if measured reference signal is within BWP or activated if it is outside the active BWP. The pre-configured MG pattern should NOT have default status at configuration: - Problem with option 1: It is too conservative to set it as activated e.g. UE does not need the MG pattern when SSB is within the BWP. Problem with option 2: UE cannot measure as pre-configured MG pattern cannot be used until the BWP is switched e.g. when SSB is outside the BWP. |
| Xiaomi | Option 1, from our understanding, the network configure the pre-configured MGs based on current active BWP, and the default status (ON/OFF status) of pre-configured MG should be consistent with the MG request for the current active BWP. Otherwise, additional ON/OFF operation is needed to activate or deactivate the pre-configured MG for the current active BWP. |
| ZTE | Support option 3a. The mechanism of pre-configured MG is to make the measurement adaptively with active BWP switching, so option 3a is the simplest and most direct method. |
| CATT | Fine with option 3 in generally. If the pre-configured MG is associated with the current active BWP, option 3a is reasonable. But this issue is related to another issue that whether multiple MGs can be pre-configured, if yes, how many MGs and which ones can be activated when they are pre-configured? |
| Nokia | Option 2, i.e. no implicit activation of the MG pattern common to BWPs, which restricts the number of usable MG patterns and does not provide full network control. |
| Intel | If no any indication on activation status, the default status shall be pre-defined (issue 1-2-3). Otherwise, NW and UE will be misunderstood each other. But Ericsson support not fixed status also propose no indication on this status in issue 1-2-3. They are a little bit contradicted each other. |
| LG Electronics | Support 3a. |
| Huawei | We support option 3. |
| NEC | Depends on what is the BWP of operation when the configuration is completed. |

#### Issue 1-1-4  Whether can MG configuration be changed after BWP switching?

[Moderator Notes:]

- Option 1 (Huawei, Intel) : No

- Option 2(CMCC): FFS

Recommended WF:  Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | Similar comments as mentioned in issue 1-1-1: We would like to point out that both UE and NW can benefit from per-BWP configured MG pattern. For instance, as we know one of the main motivations of BWP switching is to allow power saving. When there is no huge data demand, UE can fall back to the “casual” BWP with e.g. less MIMO layers for power saving. When huge data packages come, NW would switch the UE back to the “busy” BWP. It would be beneficial to give NW flexibility to configure MGP with small MG overhead on “busy” BWP to focus on throughput but configure MGP with larger MG overhead to focus on mobility on “casual” BWP. This is just an example. More and more use case can be found in future. On the other hand, NW can also configure the same MGP for all the BWP if NW believes in certain scenario different MGP for different BWP is not necessary. |
| CMCC | We think it is necessary for RAN4 to reach consensus on whether to consider MG change after BWP switching, since it will have impact on the mechanisms of activation/deactivation of MG. If the use case of MG pattern change following BWP switch (e.g. gap pattern 1 is used before BWP switching, gap pattern 2 is activated after BWP switching) is not considered, the autonomously/implicitly triggered way can be used. However, If the use case of MG pattern change following BWP switch is considered, since the selection of MG pattern depends on the SMTC of measurement objects measured in MG and different UE may have different implementation, the autonomously/implicitly triggered way may be not suitable. The better way is to activate/deactivate the pre-configured MG by the network indication. |
| MTK | Support Option 1. This discussion is the same as 1-1-1. |
| Qualcomm | Option1 is supported. We prefer to focus on creating the different ON/OFF status of pre-configured MG per BWP and stick to the original motivation of the WI. |
| Vivo | Option 1. |
| OPPO | Confused with this question. Some clarification may be needed. RRC-based BWP switching may happen together with RRC reconfiguration which may include MG configuration. If we limit to DCI/timer-based BWP switching, then RRC configuration including MG configuration should keep unchanged, But whether current MGs are still valid after BWP switching should be further discussed, i.e., in issue 1-1-5. |
| Ericsson | Support option 1. MG configuration should not be impacted due to BWP switching. |
| Xiaomi | Support option 1. Our thinking is that only the ON/OFF status can be changed before and after BWP switching. |
| ZTE | Support option 1. |
| CATT | We can understand the intention of this issue, but the wording need to be modified. The MG pattern can change after BWP switching, but this is not applicable for pre-configured MG. |
| Nokia | We don’t support option 1. The option of MG change after BWP switching can be enabled through the use of pre-configured MGP including signalling for activation/deactivation of the MGP. We agree to views from CMCC and Apple. The network has full control on the pre-configured MGP usage. |
| Intel | Support Option 1. We share same view as Qualcomm. The initial motivation of this WI is to resolve the problem when UE needs the gap for the measurement on the ongoing MOs after BWP switching. The scenario in which MG can be changed during BWP switching means that the measurement target (MO) will be changed as well. This extended our object of this WI. |
| LG Electronics | Support Option 1. |
| Huawei | We support option 1. The gain in supporting this use case (changing MG configuration following a BWP switch) is not as large as the ON/OFF of the pre-configured MG, and it will require additional specification efforts and implementation complexity for both NW and UE. |

#### Issue 1-1-5 Relation of pre-configured MG and with the current legacy MG

- Option 1. (OPPO, Apple, NEC): Pre-configured MG in one active BWP can over-ride current RRC configured.

- Option 1a. (CATT): Activated pre-configured MG in one active BWP can over-ride current RRC configured.

- Option 2 (Ericsson)

  - The already configured P-MGP is transformed into legacy MGP (with same MGL/MGRP) if the UE is configured to measure on any carrier (e.g. inter-RAT) which always need gaps for performing the measurement.

  - Network can transform an already configured P-MGP into legacy MGP with same MGL/MGRP or vice versa without deconfiguring the P-MGP.

- Option 3. (Qualcomm) FFS:

  - If NCFG(Network Controlled Fast Gap) contains a separate MG configuration, its relationship with the legacy MG shall be clarified.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | Support option 1. Principle mentioned in option 3 is also agreeable. The relationship needs to be clarified. BTW, @Qualcomm, by reading R4-2107347 we think the “NCFG” used here is same as pre-configured MG. is it correct understanding? |
| MTK | Option 2 is more closed to our understanding. Regarding Option 1 and 1a, we prefer to override only the ON/OFF status, rather than the gap pattern and offset. |
| Qualcomm | We feel companies could firstly discuss if we want a separate MG pattern for pre-configure gap. Then we can discuss whether this separate MG pattern overrides the legacy MG if it’s activated. Since in the 2nd stage, the multiple concurrent gap would come in the picture, it may be flexible to allow pre-configured MG to be orthogonal from the legacy MG for flexibility. So we support Option3 for now. @CMCC, thanks for the question! We introduce the notion of NCSG to include two properties of pre-configured MG. 1) A gap pattern 2) a set of binary indications for every target BWP. |
| Vivo | We prefer option 2 |
| OPPO | Option 1 is preferred. For NCSG, it is another issue to be clarified. |
| Ericsson | We support option 2. There might be a situation where the NW configures the UE with MO (e.g. inte-RAT) that always need gaps but the UE is already configured with pre-configured MG. In this case there should be possibility that the current pre-configured gap remains activated so that the UE can also use it for measurements on the MO that needs gaps all the time. In our view in first phase we should consider the scenario where the UE is configured with one only pattern. Therefore, when pre-configured MG is activated or deactivated there should not be any other (legacy pattern). Therefore scenarios in option 1 and 1a can be discussed in the 2nd phase. |
| Xiaomi | The further clarification of “over-ride” is needed, according to our understanding, only the ON/OFF status can be changed before and after BWP switching.. |
| ZTE | Support option 2. |
| CATT | Prefer option 1a. The difference between 1a and option 1 is that only the activated pre-configured gap can over-ride the current RRC configured gap. This issue is related to issue 1-1-4, if the gap configuration cannot change after BWP switching, this issue is not needed to be discussed. |
| Nokia | Preconfigured MG patterns reuse the structure of legacy MGPs (MGRP, MGL, gap offset, MGTA), but are operated differently through pre-configuration and activation / deactivation. Pre-configured MG patterns can be de-configured by configuration of a legacy MG pattern or by re-configuration of the pre-configured MG pattern. |
| Intel | So far we think this shall be FFS with the multiple concurrent MG together. For an example, if UE support the concurrent MG, this pre-configured MG activated can be considered as one of instance within these concurrent MGs with other legacy MG instance. Under such case, they are independent. |
| LG Electronics | Prefer Option 2. |
| Huawei | Suggest to discuss this issue after issue 1-1-1 and 1-1-2 are concluded. Technically, we understand that the pre-configured MG is same as the current legacy MG. |
| NEC | Support option 1 |

### Sub-topic 1-2 Pre-configured MG activation/deactivation

[Moderator notes: the agreements in the last meeting are:

- FFS on in which cases the pre-configured MGs shall be activated

  - Option 1. Depending on both BWP switching and Mos configured

- FFS on how pre-configured MGs can be activated/deactivated:

  - Option 1 Autonomously/implicitly triggered by DCI/Timer based BWP switching.

  - Option 1a A per-UE or per-FR MG is (de)activated following a BWP switch as follows:

    - If MG is not required by any of the configured Mos, the MG is deactivated

    - If MG is required by one or more of the configured Mos, the MG is activated

  - Option 2 Either network centralized or UE centralized rules will work.

  - Option 3 :RAN4 need to account robustness of the gap changes when evaluating and agreeing on activation/deactivation of MG pattern(s).

- FFS on evaluation on MG activation/deactivation mechanism

- FFS on Signaling to activate (enable) the pre-configured MGs

]

#### Issue 1-2-1  In which cases shall pre-configured MGs be activated/deactivated?

[Moderator notes: in order to clearly define the preconfigured MG activation/deactivation rules,  the side conditions for pre-configured MG activation/deactivation can be summarized as below.]

- Option 1a (Ericsson):

  - Using pre-configured gaps:

    - If new active BWP after the active BWP switching does not fully contain the measured SSB then the UE continues the measurement using pre-configured measurement gap.

  - Stop using pre-configured gaps:

    - If new active BWP after the active BWP switching fully contains the measured SSB then the UE continue the measurement without measurement gaps.

- Option 1b (Intel). BWP switching is one of necessary conditions for pre-configured MG’s activation

- Option 1c(Xiaomi) depends on not only the active BWP, but also the Mos configuration.

- Option 1d (CATT)

  - The pre-configured MG is activated for the following cases

    - Gap is not configured before BWP switch and is requested by at least one MO after BWP switch

    - Gap (RRC configured or activated pre-configured gap) is configured before BWP switch but a different gap pattern is requested after BWP switch

  - The pre-configured MG is deactivated for the following case

    - Pre-configured MG is used before BWP switch and no gap is needed after BWP switch

    - Pre-configured MG is used before BWP switch but a different gap pattern is requested after BWP switch

- Option 2(vivo) The pre-configured MG is deactivated only when all Mos can be measured without gap. Otherwise the pre-configured MG should be used.

- Option 3 (Huawei): RAN4 should discuss how NW and UE should determine the status of the pre-configured MG following RRC/MAC procedures such as configuration of pre-configured MG, (re)configuration of MO, (re)configuration of serving cells and Scell activation and deactivation.

- Option 4 (Qualcomm) Under control by the NW with explicit NW indicated ON/OFF bits as pre-configured by RRC and thus RAN4 doesnot need to discuss and specify the rules in the core requirements.

[Moderator notes: this issue is focus on the activation/deactivation necessary conditions or criteria only. Which entity (NW or UE ) shall control the activation/deactivation will be discussed in issue1-2-2. ]
Recommend WF: Could the proposal below be agreeable?

- The side conditions to activate the Pre-configurate MG are:

- BWP switching

- MO configured can perform gapless measurement before BWP switching but not after BWP switching

| Company | Comments |
| --- | --- |
| Apple | We suggest that we leave it (In which cases shall pre-configured MGs be activated/deactivated) to network control. UE can just follow NW indication (could be BWP switching command) to control the MG. This can simply the UE implementation. Otherwise, we may need to define many criteria in our spec. especially in future if we have more and more types of “inter-frequency measurement w/o gap”, such as inter-frequency CSI-RS L3 measurement w/o gap, then we need to revisit the criteria whatever agreed in this WI. |
| MTK | Option 1 is more closed to our understanding. Some minor revision is suggested as follows: - Using pre-configured gaps: - If one new active BWP after the active BWP switching does not fully contain the measured SSB for its intra-frequency layer then the UE conducts the measurement using pre-configured measurement gap. - Stop using pre-configured gaps: - If all new active BWPs after the active BWP switching fully contains the measured SSB its intra-frequency layer then the UE conducts the measurement without measurement gaps. |
| Qualcomm | We support Option4 and agree with Apple on the reasoning to avoid specifying the rules in 38.133. |
| vivo | We are ok with option 1a and option 2. To our understanding option 2 is roughly the same as MTK’s proposal above and has similar understanding as option 1a. We also agree at least part of Q4 that it should be network controlled. |
| OPPO | Support option 1a, 1b and 2. For option 4, prefer no explicit NW indication. |
| Ericsson | We support option 1a. Options 1b and 1c have one thing in common that BWP switching is the trigger to activate or deactivate the gaps. But we need clearly defined condition for activating or deactivating gaps to ensure both UE and BS are well aligned. |
| Xiaomi | We think whether to activate the pre-configured MG or not depends on the frequency domain relationship between the current active BWP and the MOs to be measured. And we support MTK’s revised version. |
| ZTE | We agree with MTK and vivo, option 2 is very similar as MTK’s revised option 1. We support option 2 and revised option 1. |
| CATT | Support option 1d. option 1a only consider the SSB based measurement. We have no conclusion that pre-configured MG is only used for SSB measurement. We are also fine with the way in option 4 to leave it to NW control. |
| Nokia | We principally agree with option 4, that is preconfigured MG pattern is (de-)activated by the network, We don’t agree with ON/OFF signalling as this unnecessarily restricts the number of preconfigured MG pattern to 1 and more than 1 MG pattern should be configurable per-BWP. |
| Intel | We support the principle of Option 1a/1b/1c/1d. The option 3,4 which are most likely for the detailed mechanism of activation/de-activation can be discussed after we reach the consensus on these conditions. |
| LG Electronics | Support Option 1a/1b/1c. |
| Huawei | We understand two thing are mixed in this issue, one is the conditions for activation and deactivation, and the other one is the criteria. On the conditions, we support option 3. Based on our understanding, besides BWP switch, a number of RRC/MAC procedures may change the need for MG for UE measurement, so they need to be considered as conditions for activation and deactivation of pre-configured MG. On the criteria, we support option 1a and option 2, but how this criteria are to be used depends on outcome of issue 1-2-2. |

#### Issue 1-2-2 Pre-configured MG activation/deactivation mechanism

- Option 1 (Apple, CATT, MTK,  Intel, OPPO, Xiaomi, Ericsson, CMCC, NEC) Autonomously/implicitly triggered by BWP switching  DCI/Timer.

- Option1a (Qualcomm) Autonomously triggered by BWP switching DCI/timer but explicitly directed according to the pre-configured ON/OFF state/bit for the target BWP

- Option 2 (Huawei) FFS if activation and deactivation of pre-configured MG is implicitly triggered by BWP switch or explicitly configured by NW.

- Option 2a (CMCC): If the use case of MG pattern change following BWP switch is considered, to activate/deactivate the pre-configured MG by the network indication

- Option 3 (Nokia): RAN4 need to account for robustness of the measurement gap changes when evaluating and agreeing on activation/deactivation of MG pattern(s) without using RRC signaling.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | For a given BWP, whether MG is needed depends on whether there is any MO that can only be measured with MG. The situation won’t be changed unless the MO is updated. However, MO can only be updated via RRC. Therefore it seems no need to further activate/deactivate the MG if no BWP switching. Based on that we can link the MG status (activated/deactivated) with the status of the BWP (active or not). |
| CMCC | If only the able/disable of the gaps is considered and the MG change following BWP switching is not considered, we are OK with autonomously/implicitly, but considering that whether MG is needed or not is also related with the Mos, it is better to specify a clear rule on the activation/deactivation of MG so that NW and UE share the same understanding on the usage of pre-configured MG. Except able/disable of the gaps, MG change following BWP switching is also considered, the autonomously/implicitly triggered way may be not suitable, since the selection of MG pattern depends on the SMTC (or CSI-RS) of measurement objects measured in MG and different UE may have different implementation. For this case, the better way is to activate/deactivate the pre-configured MG by the network indication. |
| MTK | Support Option 1. Autonomously/implicitly mechanism is preferred. Also share similar view with Apple that the change of the ON/OFF status of the gap does not always be triggered by BWP switch. It can also be triggered by adding/releasing a new MO or adding/releasing/activate/deactivate a new cell. |
| Qualcomm | Both Options 1a and 2a are supported. In our proposal, Option1a means the explicit configuration of the binary indication via RRC and UE could then autonomously activate or deactivate the pattern upon BWP switch. |
| vivo | Option 1 and 1a are ok. |
| OPPO | Option 1 |
| Ericsson | Support option 1. |
| Xiaomi | Support option 1 |
| ZTE | Support option 1. |
| CATT | support option 1. |
| Nokia | We support options 2a and 3. |
| Intel | Firstly we shall be noted that the RRC signaling to trigger MG activation will introduce the larger latency (>10ms) to activate the pre-configured MG. This also reverts the object of this WI. In our understanding, one of motivation to use the explicit singling to trigger the activation is to avoid the misalignment on the gap activation between NW and UE side due to BWP swithching DCI missed by UE. However, DCI decoding by UE shall be enough reliable (e.g. >99% BLER for PDCCH). On the other hand, UE wrong understanding on pre-configured MG activation/deactivation may not lead the big issue as that by NW. Therefore, we thought the implicit activation is feasible and more efficient. For Option 1b, in our views, if the ON/OFF bit is pre-defined when pre-configured MG being configured, such signaling need not to be conveyed to UE and feedbacked to NW. We can FFS on this possibility in issue 1-2-3. |
| LG Electronics | Support Option 1. |
| Huawei | We support option 2 and option 3. Our main concern with implicit mechanism is the robustness, since NW and UE may have misalignment regarding whether a DCI based BWP switch is successful or not. |
| NEC | Support option 1 |

#### Issue 1-2-3  Whether the status of activation/deactivation (e.g. ON/OFF) is needed?

- Option 1 (Ericsson, CATT) : No.

  - e.g. P-MGP shall not have a fixed status (activated or deactivated) upon RRC configuration

- Option 1a (OPPO):

  - Set pre-configured MG inactive as default after the RRC configuration, with no additional signalling.

Option 2 (Qualcomm, ZTE): Yes

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | Similar comment as we provided under issue 1-2-2, i.e. option 1. |
| MTK | Support Option 1. This is the same discussion as 1-1-3, right? |
| Qualcomm | Option2, this is what we prefer to have an explicit configuration of the ON/OFF bits(activation/deactivation) per BWP. |
| OPPO | Prefer option 1a. No additional signaling is preferred. Once P-MGP being activated after BWP switching, whether UE is required to use the MG can further depend on the relation of RS and BWP location. |
| Ericsson | We support option 1. But in our view this issue is related to activation/deactivation status at the RRC configuration of the pre-configured MG pattern based on the explanation in both options (issue 1-1-3). |
| ZTE | We are confused about the relationship between this issue and issue 1-2-2. Based on issue 1-2-2, why we need to determine this issue? |
| Nokia | We agree to option 2. As mentioned above, we see a need for signalling activation / deactivation of the pre-configured MG pattern, since at time of RRC configuration no activation/deactivation status is provided. |
| Intel | We are open for this issue if such ON/OFF need not to be exchanged between UE and NW when pre-configured MG being activated. E.g. UE need only check the rules for these pre-configured MG activation which was notified to UE before BWP switching. |
| LG Electronics | Support Option 1. |
| Huawei | The issue is not fully clear to us, and we suggest to discuss it after issues in sub-topic 1-1 are concluded. |
| NEC | We support option 1 |

#### Issue 1-2-4 Evaluation on MG activation/deactivation mechanism

- Option 1 (Nokia)

  - RAN4 need to account robustness of the gap changes when evaluating and agreeing on activation/deactivation of MG pattern(s).

- Option1a (Qualcomm): RRC pre-configured ON/OFF bit per BWP for indicating the default state of activation/de-activation

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | Similar comment as above under issue 1-2-2 and 1-2-3. We propose to link the status of MG pattern with status of BWP. |
| MTK | This is part of the discussion in the activation/deactivation mechanism. We believe that BWP switch already provide to some extend the robustness because ACK/NACK feedback will be provided to network. This feedback information can be used both DCI-based and Timer-based BWP switch. |
| Qualcomm | Option1a is supported. To elaborate, the ON/OFF bit for (de)activation is a sheer indication from the network to UE for every candidate BWP. So it is not expected to change unless the network reconfigures it. Since it is network centric and conveyed through RRC, the robustness is assured. Also, these ON/OFF bits are pre-configured together with the MG pattern, so whenever UE switches its BWP, it is expected to map the BWP index and find out what’s the configured bit by NW for enabling or disabling the pre-configured gap accordingly. Thus, no additional signaling like RRC/MAC-CE/DCI is needed. |
| Ericsson | We should first resolve issue 1-2-2: implicit versus explicit MG activation/deactivation mechanism. Implicit activation with well-defined rules will be robust enough for both UE and gNB. |
| ZTE | Once gNB and UE have the same understanding of active BWP switching, the robustness of the mechanism of activation/deactivation of pre-configured MG can be guaranteed. So we do not believe it is needed to do such evaluation. |
| Nokia | We support option 1. |
| Intel | Such evaluation on robustness of activation mechanism was also mentioned in our comments for issue 1-2-2.( the DCI BWP switching is quite reliable.) |
| Huawei | We understand the issue is part of issue 1-2-2. |

### Sub-topic 1-3 RRM requirements

#### Issue 1-3-1 Activation/Deactivation Delay

- Option 1 (Intel, OPPO, Nokia, Xiaomi, CATT,NEC): NOT to define the separated activation delay requirements for the pre-configured MG activation unless the BWP switching time is shorter than “gap transition time”.

- Option 2. (Ericsson, Qualcomm, vivo, Apple): some transition time (T) shall be included in the pre-configured MG activation/deactivation time.

- Option 2a (Qualcomm)

  - MG deactivation delay of [Y]ms is defined from the DCI of BWP switch is received to the arliest time instant when MG can be aborted.

  - MG activation delay of [X]ms is defined from the completion of target BWP activation to the earliest time instant when a MG can be entered.

  - No requirements shall be defined for the measurements within the gaps during [Y] and/or [X]ms

- Option 2b (MTK) The pre-configured delay includes BWP switch delay with additional validation time. The validation time is 20ms.

- Option 3 (Huawei): FFS pending on the conclusion of activation/deactivation procedure.

- Option 4 (Nokia): MGP change delay shall be evaluated based on realistic latencies.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | UE needs some preparation time to enable/disable MG pattern. In our understanding it should be in several milliseconds. We would expect longer time compared to BWP switching delay, especially for type 1 UE (0.75ms delay requirement). |
| MTK | Support Option 2b or Option 3. If UE is requested to switch BWP twice during 10ms, it makes no sense to ask UE to re-calculate its measurement scheduling twice. There could be even no SMTC or measurement gap occasions during this 10ms. |
| Qualcomm | Option2 can be supported and details can be further discussed. |
| Vivo | Support option 2 |
| OPPO | Option 1 is fine. Regarding MTK’s comments, in our understanding, the end point of MG activation is the time ready for the MG to be used, but not being used right now for UE, i.e., due to non-overlapping with SMTC. |
| Ericsson | Support option 2. Options 2, 2a and 2b are similar in principle. We need some time for both UE and gNB to adapt sampling and scheduling etc. The extra time needed can be further discussed. |
| Xiaomi | Option 1 is preferred, as the status of pre-configured MG is changed with BWP switching, the prepared time of pre-configured MG and BWP switching procedure can be done in parallel. |
| ZTE | Support Option 2. |
| CATT | Support option 1 if the pre-configured gap is activated implicitly by BWP switching. But we are fine to further study after we have conclusion on the activation procedure. |
| Nokia | We support option 4. |
| Intel | Actually we are also fine to FFS this delay time after we reach the consensus on the activation /deactivation procedure in issue 1-2-2 |
| LG Electronics | Same view with Intel. |
| Huawei | Support option 3. It is noted that the delay can depend on whether implicit or explicit activation and deactivation is to be used. Technically, we can support option 2, that some additional time on top of BWP switch delay could be considered. |
| NEC | We support option 1 |

#### Issue 1-3-2 Interruption requirements

- Option 1. (Apple, Intel,): No

Recommended WF: Companies to check Option 1 is agreeable.

- No interruption requirements will be introduced due to pre-configured MG activation/deactivation.

| Company | Comments |
| --- | --- |
| MTK | Option 1 seems agreeable, but it should be clarified that the interruption due to BWP switching (as we introduced in Rel-15) should still be allowed. |
| Qualcomm | FFS |
| vivo | Ok with option 1 |
| Ericsson | Agree with the WF |
| Xiaomi | Fine with option 1 |
| ZTE | FFS |
| CATT | Fine with option 1. |
| Nokia | We agree to option 1. |
| Intel | The recommended WF is agreeable, |
| LG Electronics | FFS |
| Huawei | This is already agreed in last meeting. |

#### Issue 1-3-3 Measurement period

- Option 1. (Ericsson):

  - The total measurement period Tmeasure, total) to account for transition between activated and deactivated P-MGP during the measurement needs to be specified

  - Total measurement period (Tmeasure, total) can be expressed in terms of basic measurement period (Tmeasure, basic) and aggregated time consumed due to total number of transitions between gapless measurement procedure and gap-based measurement procedure during the ongoing measurement.

  - Tmeasure, basic can be expressed as: Tmeasure, basic = MAX(TBWP, TG); where:

    - TBWP= It is the measurement period when the measurement is fully performed without measurement gap

    - TG= It is the measurement period when the measurement is fully performed with measurement gap.

- Option 2a (Intel)

  - The RAN4 minimum requirements for intra-frequency SSB measurement can follow that of intra-frequency SSB measurement requirements with gap specified in 9.2.6 of TS38.133 [3].

  - The RAN4 minimum requirements for intra-frequency SSB measurement and CSI-RS measurement with pre-configured MG can follow that of intra-frequency SSB measurement requirements with gap specified in 9.2.6 of TS38.133 [3] and inter-frequency CSI-RS measurement requirements specified in 9.10.3 of TS38.133 [3] respectively.

- Option 2b (Huawei)

  - With one or more MG (de)activation in the measurement period, the transition requirements in clause 9.1.6 apply.

- Option 2c (CATT) The measurement period for the measurement with pre-configured MG can reuse the current requirement with measurement gap in R16

- Option 2d (MTK) No new measurement delay and accuracy requirement for pre-configured MG needed to be defined. The intra-frequency measurement period and minimum transition requirement in Rel-15 can be reused.

- Option 3 (Xiaomi)

  - If there is one or more active BWP switching during one measurement period, the relaxed measurement requirement corresponding to the measurement without gap and the measurement with gap shall be applied.

- Option 4 (Nokia):

  - Analyse and evaluate, under realistic assumption, the possible impact on cell detection from a change in MGP.

  - Analyse and evaluate, under realistic assumption, the possible impact on the measurement period from a change in MGP.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | In general we agree with option 2 that existing transition requirements defined in clause 9.1.6 can apply. Except the case wherein measurement occasion collides with activation/deactivation delay as mentioned in issue 1-3-1. For this case some extra delay can be expected since UE may miss some of the measurement occasion. |
| MTK | The discussion is mixing the requirement for measurement period and requirements for transition. Firstly, if there is no transition happened, we believed that the measurement period should not be changed. With this common understanding, we can discuss whether and how to handle the requirement for transition. |
| Qualcomm | We support to capture that BWP transitions can cause longer measurement period when there are collisions of BWP switch and a target measurement object. So we prefer FFS for this issue. |
| Ericsson | Support option 1. This issue is related to issue 1-3-1: the measurement time should include Activation/Deactivation Delay. The existing transition requirements defined in clause 9.1.6 consider the case when the UE is already configured with legacy measurement gap and UE might be using the gap for certain MO. But UE starts using the gaps for the other carrier/MO that was being done without gaps or vice versa. Therefore, the scenario with pre-configured MG is different where the UE need some time for transition/activation. |
| Xiaomi | Option 3, If there is one or more active BWP switching during one measurement period, the relaxed measurement requirement corresponding to the measurement without gap and the measurement with gap shall be applied. |
| CATT | Support option 2c. When the pre-configured MG is activated, it is the same as legacy MG, the measurement period should reuse the current requirement that measured within gaps. But we are fine to further discuss since it seems some options are discussing the measurement period under one or more BWP switching. |
| Nokia | We support option 4. |
| Intel | In our view, UE start a new measurement procedure with the activated pre-configured from the 1st pre-configurd MG available. The measurement period requirement need not to include the time with legacy MG and pre-configured MG for simplicity because UE performance with the legacy MG can be guarantee by the existing measurement period requirements. But we are fine to FFS on this. |
| Huawei | Support option 2b, as same scenario of switching between MG based and MG-less measurement is already addressed in Rel-15. The new aspect is that in Rel-15 the MG is always ON once configured, while the pre-configured MG can be activated and deactivated. This mainly impacts the availability of resources for data scheduling, but from measurement perspective there is no difference compared to Rel-15. |

#### Issue 1-3-4 Transitions between gapless and gap-based measurement procedures during ongoing measurements

- Option 1(Ericsson):

  - RAN4 is to investigate the maximum number of transitions (N1,max) allowed for switching from gapless measurement procedure to gap-based measurement procedure during the ongoing measurement.

  - RAN4 is to investigate the maximum number of transitions (N2,max) allowed for switching from gap-based measurement procedure to gapless measurement procedure during the ongoing measurement

- Option 2 (Huawei, Intel, MTK, Nokia). Not needed.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | Seems all candidate options in Issue 1-3-3 can cover multiple transitions scenario. Thus we don’t see the necessity to investigate the maximum number of transitions. |
| MTK | We do not see the need to define the max number of transitions as a limitation to network. Instead, we can discuss in which case we define the UE requirement. |
| Qualcomm | Option2 is supported. In our understanding, the pre-configured MG is controlled by the network. The number of transitions is an expected consequence due to BWP switch but can be predicted by the NW. |
| vivo | Prefer option 2. It is not clear based on which principle the target (maximum number) can be derived. |
| Ericsson | Support option 1. Each transition needs some time/delay (see issue 1-3-1). But if max number of transitions is not defined then we should add delay to account for all transitions e.g. - N*activation/deactivation delay during the measurement period. Where N = number of transitions for switching between gapless and gap-based measurements during the measurement period |
| ZTE | FFS Can the supporter of Option 1 further clarify why limit the maximum number of transmissions? |
| Nokia | We support option 2. |
| Intel | In our view, UE start a new measurement procedure with the activated pre-configured from the 1st pre-configurd MG available. The measurement period requirement need not to include the time with legacy MG and pre-configured MG for simplicity because UE performance with the legacy MG can be guarantee by the existing measurement period requirements. But we are fine to FFS on this. |
| Huawei | Support option 2. From UE side, this is unnecessary as long as transition requriements are well defined. From NW side, this will create unnecessary restrictions. |
| NEC | Prefer option 2. May be more clarification is needed on option 1. |

#### Issue 1-3-5 Whether there is scheduling restriction during pre-configured MGs when not used.

- Option 1 (Ericsson,  Nokia, MTK): If the UE is measuring without pre-configured gaps and no other frequency layer which needs gaps is configured then the UE can be scheduled during the pre-configured gaps while meeting existing scheduling restriction requirements defined in TS 38.133.

- Option 1a(Huawei) Existing scheduling restriction for RRM measurement without MG applies when pre-configured MG is deactivated.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | In our understanding it is straightforward that if the pre-configured MG is not used then UE shall conduct measurement without gap. Thus existing scheduling restriction defined for measurement without gap (including both intra and inter-f) can be reused. |
| MTK | Support both Options. |
| Qualcomm | Option1a is supported. |
| vivo | Support either option 1 or 1a |
| Ericsson | We support option 1. But wording in option 1a is also ok. |
| ZTE | Support both Option 1 and 1a. |
| CATT | Fine with option 1a. |
| Nokia | We support option 1. |
| Intel | Both options are fine for us because they are quite same. |
| Huawei | Support option 1 and 1a, and we understand they are same as each other. |

#### Issue 1-3-6 UE behavior after deactivation of pre-configured MG

- Option 1 (OPPO): After deactivation of pre-configured MG and switching to a new BWP without any per-configured gap,

  - option 1: UE shall perform measurement without gap until switching away from this BWP.

  - option 2: Immediate activation of default gap. UE shall perform measurement with default gap.

  - option 3: UE shall perform measurement with legacy per UE or per FR gaps.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | It depends on how we design the configuration and activation procedure. In our view, UE only needs to check the MG configuration for the active BWP. When active BWP changes, UE will change MG pattern (including no MG) accordingly. |
| MTK | It is up to the conclusion of previous discussions. Suggest to postpone this issue at this moment. |
| Qualcomm | This is related to the relationship of pre-configured MG pattern v.s. legacy MG pattern. Can be FFS. |
| OPPO | Support option 1. The UE behavior should be clarified. |
| Ericsson | This issue is related to issue 1-1-1. If there is per UE or per FR pre-configured MG then this scenario does not exist. |
| ZTE | FFS, until some basic issues determined. |
| Nokia | We agree with Ericsson. |
| Intel | Same as the issue 1-1-5. Can be FFS. |
| Huawei | The issue depends on sub-topic 1-1, so suggest to defer the discussion after we have conclusion on issues in sub-topic 1-1. |
| NEC | We think more design details are needed. It can be FFS for now |

### Sub-topic 1-4 MG pattern configurations

#### Issue 1-4-1 Number of pre-configured MG patterns

- Option 1a. (Apple): No. Different MG pattern can be preconfigured for different BWP.

- Option 1b (Intel)

  - There are no restrictions on the total number of preconfigured gaps.

  - The serving gNB need not activate all pre-configured gaps but part of them depending on the measurement configuration and bwp-InactivityTimer.

- Option 1c (OPPO). Limit the available pre-configured MG patterns within the most commonly used MG patterns

- Option2 (Qualcomm) Consider only one pre-configured MG pattern for different BWPs as the baseline. So RAN4 could focus on the mechanisms to enable/disable it per BWP.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | Support option 1. From both UE and NW implementation perspective we see no harm to allow different MG pattern for different BWP. |
| CMCC | This is related with the use case of pre-configured MG. Except able/disable the pre-configured MG， whether the MG change following BWP switching (e.g. gap pattern 1 is used before BWP switching, gap pattern 2 is activated after BWP switching) is considered or not. If MG change following BWP switching is considered, multiple MG patterns to be pre-configured is necessary. If MG change following BWP switching is not considered, only able/disable the gap, it seems no need to pre-configure multiple MG patterns. |
| MTK | Support Option 2. This issue is also related to some previous discussions on whether to allow per-BWP gap configuration. |
| Qualcomm | Option2 is supported. Should a different MG pattern is desired, an intermediate step is to pursue via multiple concurrent GAPs in another parallel WI or in the 2nd stage. Also balance between complexity and efficiency is preferred. Note allowing change in MG pattern upon BWP switch entails potentially increasing signal overhead, requirements, and test cases for RAN4 discussions. |
| OPPO | Support Option 1 and 1c. Different MG can be preconfigured for different BWP, but patterns can be limited in a small group with most commonly used MGRP/MGL for UE. |
| Ericsson | This is also related to basic issue 1-1-1. We do not see the need to configure multiple pre-configure MG patterns. There is only one MG pattern per UE or per FR applicable for all BWPs. |
| Xiaomi | Option 2 is preferred |
| ZTE | Support Option 2. Considering the configuration of pre-configured MG can be per UE or per FR, so there is no need to configure multiple pre-configured MGs. |
| CATT | support option 1b. Pre-configured multiple gap patterns should be supported. |
| Nokia | We support option 1b. Same view as CATT. |
| Intel | This issue is also needed to coupled with the concurrent MGs discussion. So, in current stage we can focus on the single pre-configured MG. |
| LG Electronics | Support Option 1c. The number of P-MGPs needs to de decided considering impact on serving cell. |
| Huawei | The issue depends on sub-topic 1-1, so suggest to defer the discussion after we have conclusion on issues in sub-topic 1-1. |
| NEC | Can be FFS for now |

#### Issue 1-4-2 MG patterns used for the pre-configured MG mechanism

- Option 1a. (Intel, OPPO, Ericsson): The existing gap patterns (0~23) in Rel16 can be reused for the pre-configured MG, which can be activated or deactivated when being configurated.

- Option 1b. (Huawei, Nokia, CATT): All existing MG patterns #0~25 in Rel-16 are applicable for the pre-configured MG

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | As long as the supported of each pre-configured MG pattern can be independently indicated (similar with legacy), all the existing MG patterns defined in R16 are applicable for this feature. |
| CMCC | This issue is about whether to consider PRS for pre-configured MG. It is assumed that PRS is always measured in measurement gap, not sure whether the pre-configured MG can be applied to PRS measurement (at least deactivation of pre-configured MG is not applicable for PRS. For activation, it seems that the pre-configured MG is always activated when there is PRS measurement). If pre-configured MG is not applied for PRS, gap pattern 0~23 in Rel16 can be reused for the pre-configured MG. If PRS is considered for pre-configured MG, patterns #0~25 are applicable. |
| MTK | Support Option 1a. Positioning gap is not supposed to be discussed here. Rel-16 UE behavior is to only measure PRS in measurement gap. In other words, UE does not needs to transmit or receive data during PRS measurement. The concept of introducing pre-configured gap actually conflicts with the reference UE behavior on PRS measurement. |
| Qualcomm | Option1a can be supported. |
| Vivo | Support 1a |
| OPPO | Option 1a is preferred. |
| Ericsson | We support option 1a. MG patterns # 24 and # 25 are used when at least PRS measurements are configured. This means MG patterns # 24 and # 25 cannot be deactivated and are always activated. So they are not feasible for using as pre-configured MG. |
| Xiaomi | Support option 1a |
| ZTE | Support Option 1a. |
| CATT | Support option 1b. It is also related to the previous issue that whether gap pattern change should be considered. |
| Nokia | We support option 1b. |
| Intel | Regarding to the discussion on issue 1-0-1, we don’t think gap#24&25 mainly used for PRS are needed for pre-configured MG. |
| LG Electronics | Support Option 1a. |
| Huawei | Support option 1b. There is no particular need to restrict on which MG patterns can be used for pre-configured MG, but this can be up to NW implementation. Excluding MG pattern #24 and #25 is not future proof. |
| NEC | Support option 1a |

#### Issue 1-4-3 Per-UE/Per-FR pre-configured MG pattern applicability

- Option 1 (Intel, Ericsson, Apple, Nokia, CATT)

  - If the gap patterns which can be used as the pre-configured gap are reused from Rel16, the same applicability(per-UE/per-FR) shall follow the rules defined in Rel16 also.

- Option 1b(OPPO)

  - The applicability of per FR gap should apply for per-configured MG.

- Option 1c (Huawei)

  - Both per-UE and per-FR MG are applicable for pre-configured MG.

  - When per-FR gap is configured, activation/deactivation of pre-configured MG are independent for each FR.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | Support option 1. 1b and 1c are also quite similar. |
| CMCC | We are OK with option 1. |
| MTK | Support all Options. Actually they can be merged in one. |
| Qualcomm | Option1 is supported. |
| Vivo | Ok with option 1 |
| OPPO | OK to merge all the options. |
| Ericsson | Support option 1 |
| Xiaomi | Fine with all the options |
| ZTE | Support all Options, and they can be merged into one or only Option 1 exists. |
| CATT | Support option 1. |
| Nokia | We support option 1. |
| Intel | In principle we support Option 1. Option 1b is the subset of Option 1. |
| LG Electronics | Support Option 1. |
| Huawei | Support option 1c, and can also support option 1. |

### Sub-topic 1-5 RAN2 Singling

#### Issue 1-5-1 Signaling to enable the pre-configured MGs

- Option 1(Qualcomm) RAN4 to send LS to RAN2 for defining the signaling and configuration/reconfiguration procedures for the network controlled fast gaps (NCFG) to realize pre-configured MG.

- [Moderator Notes: subject to the conclusion of  Issue 1-1-2, 1-2-2. E.g. If the autonomous activation is feasible, such signaling is not needed]

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | LS can be sent once we have solid conclusion in RAN4. Is NCFG same as pre-configured MG? if so, we prefer to use existing term to simplify discussion. |
| MTK | Agree with Option 1 in principle. But maybe not this meeting. |
| Qualcomm | @Apple, yes, thanks! We are fine to postpone the issue to the future meeting when RAN4 has consolidated the needs in the signaling. In our view, the signaling is meant to be RRC level. And it shall provide following information as we mention in previous comments. To re-iterate, - 1. A pre-configure MG pattern which can be separate from the legacy MG, which is the same across BWPs 2. A binary indication for default state of the pattern per BWP |
| vivo | Same view as MTK. In principle it is ok however it is unlikely for this meeting. |
| OPPO | Pending on the discussion on other issues. |
| Ericsson | RAN4 should first agree on principles/mechanisms of configuration, activation/deactivation, supported patterns etc., for pre-configured MGs. Then RAN4 can send LS to RAN2. Detailed signaling should be left for RAN2 to decide. |
| Xiaomi | Similar view as Ericsson, it is too early to discuss the LS. |
| ZTE | Agree with MTK, Option 1 is needed but not this meeting. |
| Nokia | We agree that an LS is to be sent, once RAN4 has further progressed the procedures for pre-configured MG patterns, hence likely not in this meeting. |
| Intel | Agree this working procedure. |
| LG Electronics | It is not urgent issue. |
| Huawei | Agree with the comments from moderator, and signaling can be discussed after RAN4 agrees on the mechanism. |
| NEC | Share similar view as Ericsson. |

## CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Issue#1-0-1 | Which types of measurements can the pre-configured MG be used for? Tentative agreements: None. Candidate options: - Option 1a (Ericsson, CMCC,MTK, Intel, vivo, Xiaomi, ZTE, LG, NEC): SSB measurements. - Option 1b (Intel): SSB, CSI-RS measurement - Option 2 (Nokia, Apple, Qualcomm, OPPO, Huawei, CATT): Pre-configured MG patterns should be usable according to the existing MG applicability rules, i.e. for all types of measurements that use a legacy MG pattern in Rel-15 or Rel-16. Recommendations for 2nd round: Regarding to the feasibility of pre-configured MG for all measurement types, such pre-configured MG being activated can be feasible. Also, for SSB measurement, it is undoubtable the pre-configured MG is feasible and necessary as explained in WID. The main controversial issues are on necessity of pre-configured MG for other two types of measurement. Therefore, we can further discuss the following questions: - Issue#1-0-1-1Whether is the pre-configured MG needed for PRS measurement? - Option 1: Yes - Option 2: No - Issue#1-0-1-2 Whether is the pre-configured MG needed for CSI-RS L3 measurement? - Option 1: Yes - Option 2: No |
| Issue#1-0-2 | Whether the pre-configured MG for the BWP switching on multiple CCs shall be considered? Tentative agreements: GTW agreements: - Focus on the scenarios with the pre-configured MG for single CC BWP switching scenario. - FFS whether to define pre-configured MG for multiple CC BWP switching scenario for NR-CA case Candidate options: - Option 1 (Ericsson, Qualcomm, CMCC): Yes. - Option 1a (ZTE): For NR-CA case, focus on intra-frequency measurement. - Option 2 (Apple, Intel, Nokia, CATT, Ericsson, MTK, Huawei, Xiaomi,vivo, LG, NEC): Focus on the application scenarios of the pre-configured gap for single CC BWP switching Recommendations for 2nd round: Regarding to majority companies view, could we agree the following proposals: - In early stage of this WI focus on the application scenarios of the pre-configured gap for single CC BWP switching. - The application scenarios of the pre-configured gap for multiple CC BWP switching in NR CA can be FFS in 2nd stage of this WI |
| Issue#1-0-3 | Whether the pre-configured gaps shall be considered as a part of multiple concurrent gap patterns framework? Tentative agreements: None. Candidate options: - Option 1: (Qualcomm, Intel, CMCC, MTK,vivo, Ericsson, Xiaomi,CATT, LG, Huawei, NEC) - RAN4 can focus on the separated pre-configured MG firstly. - Option 2 (Nokia): The design of general procedures for use of pre-configured MG should take into account single MGP and multiple concurrent MGPs Recommendations for 2nd round: Regarding to majority views, could we agree the following proposal: - RAN4 can focus on the separated pre-configured MG firstly. |
| Issue#1-1-1 | Whether can pre-configured MGs be configured per UE/FR or per BWP Tentative agreements: GTW agreements: - Pre-configured MG(s) are configured per UE or per FR - FFS if pre-configured MGs can be additionally configured per BWP GTW Tentative agreement (for further discussion) - A single pre-configured MG is considered for the case of non-concurrent MG scenarios. - FFS if more pre-configured MGs shall be considered for the multiple concurrent MG scenarios Candidate options: - Option 1a (Apple, NEC). Pre-configured MGs are configured per BWP. Different MG patterns (or no MG) can be pre-configured for different BWP. - Option 1b(OPPO) Pre-configured MGs are configured per BWP, and used as per FR gaps. - Option 2 (Ericsson, Intel, Huawei, Xiaomi, CATT, MTK, CMCC, QC, ZTE, LG). Pre-configured MGs are configured per UE or per FR - Option 2a (Nokia, Apple, OPPO): Pre-configured MGs are configured per-UE or per-FR. Pre-configured MGs additionally can be configured per BWP - Option2b(Qualcomm) Pre-configured MGs are configured per-UE or per-FR. The (de)activation indication is pre-configured per BWP. Recommendations for 2nd round: Can be FFS. [Moderator notes: the proponents of Option 2a can clarify the exact means of the per-BWP configurated pre-configurated MG? For an example, if the common MG parameters (e.g. MGRP, MGL) are not same when UE measuring the same layer but within the different BWP. ] |
| Issue#1-1-2 | Issue 1-2-2 How can pre-configured MGs be configured Tentative agreements: Candidate options: - Option 1 (MTK, vivo, Intel, Xiaomi, CMCC, vivo, OPPO, Ericsson, ZTE, CATT, LG). Pre-configured MG can be configured by the similar way as the legacy MGs (Rel16) - Option 1a (Nokia, Apple). Current RRC signalling message for configuring pre-configured MG pattern (i.e. RRC reconfiguration command) shall be used and is extended to include one or more MGPs per BWP used for per-UE or per-FR gap support. - Option 2(Qualcomm, MTK). A binary indication is pre-configured for every BWP which specifies the default ON or OFF state for the legacy MG or a separate/independent pre-configured MG by RRC. - Option 3 (Huawei). Enable/disable of pre-configured MG should be explicitly configured by the NW Recommendations for 2nd round: Could the following proposal be agreeable? - The common configuration parameters of pre-configured MG (e.g. MGRP, MGL, etc) which are same as these of Rel16 legacy MG can be configured by the similar way as the Rel16 legacy MGs - FFS on the specific configuration parameters for the new aspects of pre-configured MG to be introduce |
| Issue#1-1-3 | Whether is the pre-configured MG activated or not after configuration completed? Tentative agreements: None Candidate options: - Option 1 (Xiaomi, Qualcomm): Activated by default - Option 2 (Nokia, Intel, OPPO) : Not activated by default until being activated. - Option 3 (Ericsson, CMCC, CATT, Huawei): Status of pre-configured MG is not fixed (no default status): - Option 3a(MTK,vivo,ZTE, LG ): Whether pre-configured MG activated or not depends on whether reference signal to measure is within the active BWP or not. - Option 3b: Network signals the status (activated or deactivated) when pre-configured MG is configured Recommendations for 2nd round: Can be FFS |
| Issue#1-1-4 | Whether can MG configuration be changed after BWP switching Tentative agreements: None Candidate options: - Option 1 (Huawei, Intel, MTK, Qualcomm, vivo, Ericsson, Xiaomi, ZTE, ) : No - Option 2(CMCC, Nokia, CATT): FFS Recommendations for 2nd round: Can be FFS |
| Issue 1-1-5 | Relation of pre-configured MG and with the current legacy MG Tentative agreements: None Candidate options: - Option 1. (OPPO, Apple, NEC): Pre-configured MG in one active BWP can over-ride current RRC configured. - Option 1a. (CATT): Activated pre-configured MG in one active BWP can over-ride current RRC configured. - Option 2 (Ericsson, MTK, vivo, ZTE, LG) - The already configured P-MGP is transformed into legacy MGP (with same MGL/MGRP) if the UE is configured to measure on any carrier (e.g. inter-RAT) which always need gaps for performing the measurement. - Network can transform an already configured P-MGP into legacy MGP with same MGL/MGRP or vice versa without deconfiguring the P-MGP. - Option 3. (Qualcomm, Xiaomi, Intel, Huawei) FFS Recommendations for 2nd round: Can be FFS |
| Issue 1-2-1 | In which cases shall pre-configured MGs be activated/deactivated? Tentative agreements: None Candidate options: - Option 1a (Ericsson, vivo,OPPO, Intel): - Using pre-configured gaps: - If new active BWP after the active BWP switching does not fully contain the measured SSB then the UE continues the measurement using pre-configured measurement gap. - Stop using pre-configured gaps: - If new active BWP after the active BWP switching fully contains the measured SSB then the UE continue the measurement without measurement gaps. - Option 1d (MTK, Xiaomi, ZTE, Intel) - Using pre-configured gaps: - If one new active BWP after the active BWP switching does not fully contain the measured SSB for its intra-frequency layer then the UE conducts the measurement using pre-configured measurement gap. - Stop using pre-configured gaps: - If all new active BWPs after the active BWP switching fully contains the measured SSB its intra-frequency layer then the UE conducts the measurement without measurement gaps. - Option 1b (Intel, OPPO). BWP switching is one of necessary conditions for pre-configured MG’s activation - Option 1c(Xiaomi, Intel) depends on not only the active BWP, but also the Mos configuration. - Option 1d (CATT) - The pre-configured MG is activated for the following cases - Gap is not configured before BWP switch and is requested by at least one MO after BWP switch - Gap (RRC configured or activated pre-configured gap) is configured before BWP switch but a different gap pattern is requested after BWP switch - The pre-configured MG is deactivated for the following case - Pre-configured MG is used before BWP switch and no gap is needed after BWP switch - Pre-configured MG is used before BWP switch but a different gap pattern is requested after BWP switch - Option 2(vivo, OPPO) The pre-configured MG is deactivated only when all Mos can be measured without gap. Otherwise the pre-configured MG should be used. - Option 3 (Huawei): RAN4 should discuss how NW and UE should determine the status of the pre-configured MG following RRC/MAC procedures such as configuration of pre-configured MG, (re)configuration of MO, (re)configuration of serving cells and Scell activation and deactivation. - Option 4 (Qualcomm) Under control by the NW with explicit NW indicated ON/OFF bits as pre-configured by RRC and thus RAN4 doesnot need to discuss and specify the rules in the core requirements. Recommendations for 2nd round: Could the proposal below be agreeable? - The sufficient conditions to activate/de-activate the Pre-configurate MG are: - BWP switching - MO configured can perform gapless measurement before BWP switching but not after BWP switching - Other conditions are FFS - NW can fully control whether the pre-configured MG will be activated/deactivated. |
| Issue 1-2-2 | Pre-configured MG activation/deactivation mechanism Tentative agreements: None Candidate options: - Option 1 (Apple, CATT, MTK, Intel, OPPO, Xiaomi, Ericsson, CMCC, NEC, vivo, LG) Autonomously/implicitly triggered by BWP switching DCI/Timer. - Option1a (Qualcomm, ZTE, Intel) Autonomously triggered by BWP switching DCI/timer but explicitly directed according to the pre-configured ON/OFF state/bit for the target BWP - Option 2 (Huawei) FFS if activation and deactivation of pre-configured MG is implicitly triggered by BWP switch or explicitly configured by NW. - Option 2a (CMCC, Nokia): If the use case of MG pattern change following BWP switch is considered, to activate/deactivate the pre-configured MG by the network indication - Option 3 (Nokia): RAN4 need to account for robustness of the measurement gap changes when evaluating and agreeing on activation/deactivation of MG pattern(s) without using RRC signaling. Recommendations for 2nd round: Can be FFS |
| Issue 1-2-3 | Whether the status of activation/deactivation (e.g. ON/OFF) is needed? [Moderator notes: this issue is couple into issue 1-2-2. ] Tentative agreements: None. Candidate options: Recommendations for 2nd round: No further discussion. |
| Issue 1-2-4 | Evaluation on MG activation/deactivation mechanism Tentative agreements: None Candidate options: - Option 1 (Nokia) - RAN4 need to account robustness of the gap changes when evaluating and agreeing on activation/deactivation of MG pattern(s). - Option1a (Qualcomm): RRC pre-configured ON/OFF bit per BWP for indicating the default state of activation/de-activation Recommendations for 2nd round: Can be FFS up to issue issue 1-2-2 |
| Issue#1-3-3 | Activation/Deactivation Delay Tentative agreements: None Candidate options: - Option 1 (Intel, OPPO, Nokia, Xiaomi, CATT,NEC): NOT to define the separated activation delay requirements for the pre-configured MG activation unless the BWP switching time is shorter than “gap transition time”. - Option 2. (Ericsson, Qualcomm, vivo, Apple): some transition time (T) shall be included in the pre-configured MG activation/deactivation time. - Option 2a (Qualcomm) - MG deactivation delay of [Y]ms is defined from the DCI of BWP switch is received to the arliest time instant when MG can be aborted. - MG activation delay of [X]ms is defined from the completion of target BWP activation to the earliest time instant when a MG can be entered. - No requirements shall be defined for the measurements within the gaps during [Y] and/or [X]ms - Option 2b (MTK) The pre-configured delay includes BWP switch delay with additional validation time. The validation time is 20ms. - Option 3 (Huawei, MTK): FFS pending on the conclusion of activation/deactivation procedure. Option 4 (Nokia): MGP change delay shall be evaluated based on realistic latencies Recommendations for 2nd round: Can be FFS up to issue 1-2-2 |
| Issue#1-3-2 | Interruption requirements Tentative agreements: Candidate options: Recommendations for 2nd round: No further discussion needed agreed in the last meeting.. |
| Issue#1-3-3 | Measurement period Tentative agreements: None. Candidate options: - Option 1. (Ericsson, Apple): - The total measurement period Tmeasure, total) to account for transition between activated and deactivated P-MGP during the measurement needs to be specified - Total measurement period (Tmeasure, total) can be expressed in terms of basic measurement period (Tmeasure, basic) and aggregated time consumed due to total number of transitions between gapless measurement procedure and gap-based measurement procedure during the ongoing measurement. - Tmeasure, basic can be expressed as: Tmeasure, basic = MAX(TBWP, TG); where: - TBWP= It is the measurement period when the measurement is fully performed without measurement gap - TG= It is the measurement period when the measurement is fully performed with measurement gap. - Option 2a (Intel) - The RAN4 minimum requirements for intra-frequency SSB measurement can follow that of intra-frequency SSB measurement requirements with gap specified in 9.2.6 of TS38.133 [3]. - The RAN4 minimum requirements for intra-frequency SSB measurement and CSI-RS measurement with pre-configured MG can follow that of intra-frequency SSB measurement requirements with gap specified in 9.2.6 of TS38.133 [3] and inter-frequency CSI-RS measurement requirements specified in 9.10.3 of TS38.133 [3] respectively. - Option 2b (Huawei) - With one or more MG (de)activation in the measurement period, the transition requirements in clause 9.1.6 apply. - Option 2c (CATT) The measurement period for the measurement with pre-configured MG can reuse the current requirement with measurement gap in R16 - Option 2d (MTK) No new measurement delay and accuracy requirement for pre-configured MG needed to be defined. The intra-frequency measurement period and minimum transition requirement in Rel-15 can be reused. - Option 3 (Xiaomi) - If there is one or more active BWP switching during one measurement period, the relaxed measurement requirement corresponding to the measurement without gap and the measurement with gap shall be applied. - Option 4 (Nokia): - Analyse and evaluate, under realistic assumption, the possible impact on cell detection from a change in MGP. - Analyse and evaluate, under realistic assumption, the possible impact on the measurement period from a change in MGP. Recommendations for 2nd round: No further discussion. Can be FFS in the future meetings. |
| Issue#1-3-4 | Transitions between gapless and gap-based measurement procedures during ongoing measurements Tentative agreements: Candidate options: - Option 1(Ericsson): - RAN4 is to investigate the maximum number of transitions (N1,max) allowed for switching from gapless measurement procedure to gap-based measurement procedure during the ongoing measurement. - RAN4 is to investigate the maximum number of transitions (N2,max) allowed for switching from gap-based measurement procedure to gapless measurement procedure during the ongoing measurement - Option 2 (Huawei, Intel, MTK, Nokia, NEC, vivo). Not needed Recommendations for 2nd round: No further discussion. Can be FFS in the future meetings. |
| Issue#1-3-5 | Whether there is scheduling restriction during pre-configured MGs when not used Tentative agreements: Candidate options: - Option 1 (Ericsson, Nokia, MTK, vivo): If the UE is measuring without pre-configured gaps and no other frequency layer which needs gaps is configured then the UE can be scheduled during the pre-configured gaps while meeting existing scheduling restriction requirements defined in TS 38.133. - Option 1a(Huawei, MTK, Qualcomm, vivo, CATT) Existing scheduling restriction for RRM measurement without MG applies when pre-configured MG is deactivated. Recommendations for 2nd round: Can be FFS in the next meeting. |
| Issue 1-3-6 | UE behavior after deactivation of pre-configured MG Tentative agreements: None Candidate options: - Option 1 (OPPO): After deactivation of pre-configured MG and switching to a new BWP without any per-configured gap, - option 1: UE shall perform measurement without gap until switching away from this BWP. - option 2: Immediate activation of default gap. UE shall perform measurement with default gap. - option 3: UE shall perform measurement with legacy per UE or per FR gaps. Recommendations for 2nd round: No further discussion. Can be FFS in the future meetings. |
| Issue#1-4-1 | Number of pre-configured MG patterns Tentative agreements:None Candidate options: - Option 1a. (Apple): No. Different MG pattern can be preconfigured for different BWP. - Option 1b (Intel, CATT, Nokia) - There are no restrictions on the total number of preconfigured gaps. - The serving gNB need not activate all pre-configured gaps but part of them depending on the measurement configuration and bwp-InactivityTimer. - Option 1c (OPPO, LG). Limit the available pre-configured MG patterns within the most commonly used MG patterns - Option2 (Qualcomm, MTK) Consider only one pre-configured MG pattern for different BWPs as the baseline. So RAN4 could focus on the mechanisms to enable/disable it per BWP. Recommendations for 2nd round: Can be FFS up to issue 1-1-1 |
| Issue#1-4-2 | MG patterns used for the pre-configured MG mechanism Tentative agreements: None Candidate options: - Option 1a. (Intel, OPPO, Ericsson, MTK, Qualcomm, vivo, OPPO, Xiaomi, ZTE, LG): The existing gap patterns (0~23) in Rel16 can be reused for the pre-configured MG, which can be activated or deactivated when being configurated. - Option 1b. (Huawei, Nokia, CATT, Apple): All existing MG patterns #0~25 in Rel-16 are applicable for the pre-configured MG Recommendations for 2nd round: Can be FFS up to issue 1-0-1 |
| Issue#1-4-3 | Per-UE/Per-FR pre-configured MG pattern applicability agreements: : If the gap patterns which can be used as the pre-configured gap are reused from Rel16, the same applicability(per-UE/per-FR) shall follow the rules defined in Rel16 also. Candidate options: Recommendations for 2nd round: No need further discussion. |
| Issue#1-5-1 | Signaling to enable the pre-configured MGs Tentative agreements: RAN4 can send LS to RAN2 for defining the signaling and configuration/reconfiguration procedures when principles/mechanisms of configuration, activation/deactivation, supported patterns etc., for pre-configured MGs are agreed in RAN4. Candidate options: Recommendations for 2nd round: No need further discussion |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

[Moderator notes: all 2nd round discussions are moved to a dedicated Email thread for WF on the Email reflector.

]

#### Issue#1-0-1-1Whether is the pre-configured MG needed for PRS measurement?

  - Option 1: Yes

  - Option 2: No

| Company | Comments |
| --- | --- |

#### Issue#1-0-1-2 Whether is the pre-configured MG needed for CSI-RS L3 measurement?

  - Option 1: Yes

  - Option 2: No

| Company | Comments |
| --- | --- |

#### Issue 1-1-2: How can pre-configured MGs be configured

Recommended WF:

- The common configuration parameters of pre-configured MG (e.g. MGRP, MGL, etc) which are same as these of Rel16 legacy MG can be configured by the similar way as the Rel16 legacy MGs

- FFS on the specific configuration parameters for the new aspects of pre-configured MG to be introduce

| Company | Comments |
| --- | --- |

#### Issue 1-2-1  In which cases shall pre-configured MGs be activated/deactivated?

Recommended WF:

- The conditions to activate/de-activate the Pre-configurate MG are:

  - BWP switching

  - MO configured can perform gapless measurement before BWP switching but not after BWP switching

  - Other conditions are FFS

- NW can fully control whether the pre-configured MG will be activated/deactivated

| Company | Comments |
| --- | --- |

#### Issue 1-2-2 Pre-configured MG activation/deactivation mechanism

Recommended WF:

- FFS on how pre-configured MGs can be activated/deactivated:

  - Option 1 Autonomously/implicitly triggered by DCI/Timer based BWP switching.

  - Option1a Autonomously triggered by BWP switching DCI/timer but explicitly directed according to the pre-configured ON/OFF state/bit for the target BWP

  - Option 2: If the use case of MG pattern change following BWP switch is considered, to activate/deactivate the pre-configured MG by the network indication

| Company | Comments |
| --- | --- |

#### Issue 1-4-1 Number of pre-configured MG patterns

Recommended WF:

- A single pre-configured MG is considered for the case of non-concurrent MG scenarios.

- FFS if more pre-configured MGs shall be considered for the multiple concurrent MG scenarios

| Company | Comments |
| --- | --- |

## Summary on 2nd round

No further agreement was reached in the 2nd round.

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2105791 (WF) | Agreeable |

# Topic #2: Network Controlled Small Gap (NCSG)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104634 | vivo | Proposal 1: legacy gap pattern is reused for NCSG with the assumption that MGL= VIL1+ML+VIL2 (ML>0) and VIRP = MGRP of legacy MG. When VIL1+ Vil2≥MGL, the NCSG should not be defined. Proposal 2: The VIL1 and VIL2 are defined as the interrupted slots, i.e., using victim cell’s slot as the unit. Proposal 3: For synchronous case, the value of VIL1, expressed by victim cell’s number of slots, is max{ RF switching time (0.5ms or 0.25ms), victim cell slot duration}/victim cell slot duration Proposal 4: Based on VIL1 value for synchronous case, the value of VIL2 for synchronous case is the same as the value of VIL1 for DL and 1 extra slot may be added for UL. For asynchronous case, in general 1 slot should be added to both VIL1 and VIL2. Proposal 5: Do not design NCSG for legacy NR measurement gap when the MGL of that legacy measurement gap is less than a particular threshold, for example 4ms for FR1 (3.5ms for FR2) Proposal 6: Do not design NCSG for legacy NR measurement gap when the percentage of (VIL1+VIL2)/ML is larger than a particular threshold, for example 2. Proposal 7: From UE power consumption point of view, it is suggested that NCSG for legacy NR measurement gap with MGRP = 20ms are not defined. Proposal 8: Design explicit configuration for NCSG firstly; implicit configuration for NCSG is FFS |
| R4-2104584 | MediaTek inc | Observation 1: UE is not expected to perform data reception/transmission on serving cell together with the inter-frequency measurements during NCSG in an intra-band or inter-band with CBM scenario, but it’s possible for inter-band with IBM. Proposal 1: Do not consider per-CC scenario for NCSG in Rel-17. Proposal 2: The use cases for NCSG in NR are as follow. - Enable measurement on target frequency layers with interruption controlled on activated CC - Reduce interruption rate due to deactivated Scell measurement Proposal 3: Intra-frequency measurements with MG, inter-frequency measurements with MG or inter-RAT measurements may use NCSG instead of MG when UE supports the corresponding band combination and has additional RF chains for measurements. Proposal 4: Rel-17 NCSG capability is reported on top of existing RAN2 ‘NeedForGap’ signalling structure with a new component ‘NCSG’. Proposal 5: Introduce absolute RRT (RF retuning time) to replace VIL in NCSG pattern definition. RRT is 0.5ms in FR1 and 0.25ms in FR2, occurring in the beginning and the end of NCSG. Proposal 6: VIL is the interrupted slots on active serving cells and is defined by following R15 measurement interruption rule which depends on both - Victim cell SCS and Aggressor reference cell RRT, and - Sync and Async. Deployment Proposal 7: The NR gap patterns #0~23 can be used to NCSG pattern, and gap patterns #24 and #25 won’t apply to NCSG. Proposal 8: When UE supports NCSG, the supported gap pattern index shall be the same as its reported legacy MG pattern capability. Proposal 9: Existing gap applicability in Rel-16 for NR-only measurements and mandatory gap patterns Proposal 10: UE is not expected to measure more than one inter-freq/inter-RAT layers in parallel even if UE reports the support of NCSG to both corresponding bands. Proposal 11: NW needs to be informed that the inter-frequency measurements with NCSG is CBM or IBM with serving cells in FR2. Proposal 12: RAN4 shall start the NCSG discussion based on explicitly configuration to replace legacy gap. Proposal 13: In the 2nd stage, consider NCSG to be concurrently configured with legacy gap. Proposal 14: Introduce CSSF within NCSG for NCSG measurements. UE is only required to measure one frequency layer in each NCSG occasion. Proposal 15: No additional NCSG capability for per-UE and per-FR differentiation is needed. - When UE only supports per-UE gap, the NCSG is per-UE. - When UE supports per-FR gap, the NCSG interruption is constrained in the FR. Proposal 16: RAN4 shall discuss the NCSG measurement mode issue similar as legacy MG The existing measurement mode requirements (effective MGRP, data scheduling depends on gap configuration) can be the baseline |
| R4-2104752 | CATT | Proposal 1: NCSG can be defined for intra/inter-band CA and synchronous/asynchronous DC. Proposal 2: No need to define separate NCSG for synchronous and asynchronous DC. Proposal 3: For the UE capable of per-UE gap, NCSG can be configured when legacy gap is not configured. For the UE capable of per-FR gap, NCSG can be configured when no legacy gap is configured or the legacy gap is configured in the different FR. Proposal 4: Similarly as LTE, define NCSG for part of legacy gap pattern with long MGL. And both per-UE and per-FR NCSG can be considered as UE capability. Proposal 5: The VIL should be defined as the number of interrupted slots. Proposal 6: NCSG is configured explicitly by a separate ignallin. Proposal 7: Support of NCSG can be UE capability. Proposal 8: The interruption requirement for NCSG in LTE can be a starting point. Proposal 9: During ML, the measurement that does not request NCSG or MG can be performed, but whether the measurement is performed in Pcell or Scell requested NCSG is UE implementation. Proposal 10: Measurement requirement for NCSG is defined based on the assumption that only one layer can be monitored in one NCSG occasion. Proposal 11: The signaling ‘NeedForGap’ can be a starting point and be extended to add indication for NCSG. Proposal 12: The NCSG pattern configuration should also be introduced in higher layer signaling after the patterns are defined. Proposal 13: Both the NCSG configuration and signaling indication should be discussed and decided in RAN2. |
| R4-2104856 | Apple | Proposal 1: RAN4 can consider two use cases for NCSG: 1) UE can use spare RF chain(s) for inter-frequency measurement. 2) UE can enlarge channel bandwidth to perform measurement on contiguous carriers. Proposal 2: per-CC is not considered at this stage. Proposal 3: if the support of NCSG and the support of legacy MG patterns can be independently indicated, then all the existing MG patterns can be extended to be NCSG. Proposal 4: VIL should be explicitly defined as the number of interrupted slots, similar as existing Table 9.1.2-4 in TS38.133. Proposal 5: both per-FR and per-UE NCSG can be supported. For UE supporting per-FR gap, VIL is allowed only on the serving cell in the same FR wherein there is NCSG operation. Otherwise, VIL is allowed on all serving cells. Proposal 6: If existing MG pattern table 9.1.2-1 is to be reused, then implicit configuration of NCSG can work, i.e. network would still configure MG pattern as legacy and UE may introduce VILs at the start and stop of a configured MG while ML is the same as MGL and VIRP is the same as MGRP. Proposal 7: if NCSG patterns are to be defined in a new table, then explicit indicator of NCSG would be needed. The principle is that both UE and network shall have same understanding of VIL, ML and VIRP. Proposal 8: existing per-UE and per-FR capability can apply to NCSG. Proposal 9: NCSG in FR2 should be deprioritized in current stage. |
| R4-2104934 | CMCC | Use cases of NR NCSG Proposal 1: NCSG can be used for intra-frequency measurements with MG, inter-frequency measurements with MG, inter-RAT measurements, deactivated SCC measurement when UE have spare RF chains during the measurements. NCSG pattern Proposal 2: it is proposed to support both per-UE NCSG patterns and per-FR NCSG patterns. Proposal 3: it is proposed to use NR gap patterns #0~23 to define NCSG pattern. Proposal 4: it is proposed to define VIL and interruption separately with following consideration: - If VIL is defined as the equivalent time of the interrupted slots, and different NCSG pattern for synchronous and asynchronous operation is adopted, there may be too many NSCG patterns. - If VIL is defined as the equivalent time of the interrupted slots, and same NCSG pattern for synchronous and asynchronous operation is designed, the benefit of introducing NCSG in synchronous scenario will be sacrificed. Proposal 5: it is proposed that VIL equals to absolute RF retuning time defined in Rel-15, which is 0.5ms for FR1 and 0.25ms for FR2. Proposal 6: if VIL is agreed to be equal to RF tuning/retuning time, it is proposed that same NSCG pattern is applied for synchronous and asynchronous operation. And the interruption can be defined separately for synchronous and asynchronous. Interruption Proposal 7: the interruption is proposed as following: SCS; Synchronous; Asynchronous. interruption length before measurement; interruption length after measurement; interruption length before measurement; interruption length after measurement. 15KHz SCS; 1 slot; 1 slot; 2 slots; 2 slots. 30KHz SCS; 2 slots; 2 slots; 3 slots; 3 slots. 60KHz SCS; 3 slots; 3 slots; 4 slots; 4 slots. 120KHz SCS; 6 slots; 6 slots; 7 slots; 7 slots. Configuration of NR NCSG Proposal 8: - When UE is not configured with measurement gap, NCSG can be explicitly provided to UE. - When UE is configured with per-UE measurement gap, NCSG should not be configured. - When UE is configured with FR1 measurement gap, FR2 NCSG pattern can be configured. - When UE is configured with FR2 measurement gap, FR1 NCSG pattern can be configured. Relation between NCSG and ‘NeedForGap’ Observation 1: In LTE, for ‘NeedForGap’, no interruption is specified. Observation 2: NCSG and ‘NeedForGap’ are different feature. ‘NeedForGap’ is the case that measurement is performed without gap and without interruption, but NCSG means measurement with network controlled small interruption. Proposal 9:considering that NCSG and ‘NeedForGap’ are different， it is not preferred to reuse Rel-16 ‘NeedForGap’ signalling for NCSG. |
| R4-2106448 | Intel Corporation | Proposal 1: The main using scenario of NCSG in NR can be focus on: - Eliminate/reduce interruption rate due to the transition of the target measurement frequency or BWP Proposal 2: No per-CC NCSG will be considered in Rel17. Proposal 3: Reuse part of the legacy MG patterns in [2] as the new NCSG patterns in NR. Proposal 4: In order to minimize RAN4 and other RAN group’s standardization efforts, RAN4 can define NR NCSG patterns based on existing NR legacy MG patterns in [2]. Proposal 5: The explicit configuration of NCSG in NR is preferred. Proposal 6: NCSG pattern should be configured based on MG configuration considering per FR1 or FR2 gap Proposal 7: The interruption requirements during measurements on SCC defined in TS38.133 and TS36.133 shall be revisited because of NCSG is used. Proposal 8: The “NeeForGap” signaling structure can be reused for NR NCSG as a start point. |
| R4-2106537 | OPPO | Observation 1: It is feasible to consider 2 sets of VIL1/VIL2/ML vs. one VIRP. Observation 2: RAN4 should firstly decide whether to implicit or explicit configure of NCSG. Proposal 1: Prefer to reuse part of the legacy MG patterns as reference for NR NCSG patterns, with long MGL, e.g., 6ms for FR1 or 5.5ms FR2. Proposal 2: NCSG pattern should be configured based on MG configuration considering per UE and FR gap. Proposal 3: At most 2 NCSG patterns should be associated with one MG pattern. Proposal 4: As defined in Table 1, NCSG Pattern ID 0~7 are for measurement configured with per FR1 gap, and pattern ID 8~15 for measurement configured with per FR2 gap. Proposal 5: NW explicitly configures the NCSG for VIL1/VIL2/ML/VIRP and offset. Proposal 6: NCSG pattern should follow the applicability of corresponding MG pattern. Proposal 7: NR NCSG signaling is introduced newly, independent from Rel-16 “Needforgap” signaling, e.g., NCSG, No-NCSG. Table 1: NCSG configuration for NR NCSG Pattern Id; (VIL1, ms); (ML, ms); (VIL2, ms); (VIRP, ms). 0; 1; 4; DL: 1 UL: 2; 40. 1; 1; 4; DL: 1 UL: 2; 80. 2; 2; 3; 2; 40. 3; 2; 3; 2; 80. 4; 1; 4; DL: 1 UL: 2; 20. 5; 1; 4; DL: 1 UL: 2; 160. 6; 2; 3; 2; 20. 7; 2; 3; 2; 160. 8; 1; 3.5; DL: 1 UL: 2; 40. 9; 1; 3.5; DL: 1 UL: 2; 80. 10; 2; 2.5; 2; 40. 11; 2; 2.5; 2; 80. 12; 1; 3.5; DL: 1 UL: 2; 20. 13; 1; 3.5; DL: 1 UL: 2; 160. 14; 2; 2.5; 2; 20. 15; 2; 2.5; 2; 160. |
| R4-2107029 | Huawei, HiSilicon | Proposal 1: RAN4 to support the following use cases of NCSG: - To eliminate the interruption due to RRM measurement on deactivated SCC and interruption due to RRM/CSI measurement on SCC in dormancy, and - To perform inter-frequency/RAT measurement when UE is capable of such measurement with NCSG Proposal 2: Per CC MG is not supported. Proposal 3: RAN4 to define NCSG patterns corresponding to MG pattern #0~23. Proposal 4a: RAN4 to define VIL as an absolute value for RF re-tuning, which is common for all cases including sync/async, UL/DL and with/without MGTA. Proposal 4b: RAN4 to define the number of interrupted slots due to VIL dependent on sync/async and with/without MGTA. Proposal 4c: RAN4 to further discuss if the number of interrupted slots due to VIL for UL. Proposal 5: Both per UE and per FR NCSG are supported, and related UE capability can be FFS. Proposal 6a: Use of NCSG should be explicitly configured by the NW. Proposal 6b: The NCSG pattern can be implicitly determined by the configured MG pattern Proposal 7: RAN4 to further discuss the condition, capability and impacts to measurement requirements for UE to use NCSG to control interruptions due to measurement on deactivated SCC or Scell in dormancy. Proposal 8: Further discuss two options for measurement during ML - Option 1: UE only performs measurement that requires NCSG - Option 2: UE performs both measurement that requires NCSG and measurement that does not require NCSG or MG Proposal 9: Extend the Rel-16 NeedForGap ignallin for UE to indicate - Need for normal MG, or - No need for normal MG but NCSG, or - No need for either normal MG or NCSG to perform a measurement for a serving cell (intra-frequency) or a frequency band (inter-frequency). |
| R4-2107152 | Ericsson | Scenarios for NCSG patterns: - Proposal # 1: If NCSG is configured then the interruptions on Pcell, PSCell or activated Scell(s) due to measurements on Pcell, PSCell, activated Scell, deactivated Scell, Scell with dormant BWP or unused RF chain shall not occur outside the visible interruption length before measurement (VIL1) and the visible interruption length after measurement (VIL2). - Proposal # 2: If NCSG is pre-configured then after switching from non-dormant BWP to dormant BWP on a Scell, then interruptions on Pcell, PSCell or activated Scell(s) due to measurements on the Scell with dormant BWP shall not occur outside the visible interruption length before measurement (VIL1) and the visible interruption length after measurement (VIL2). - Proposal # 3: NCSG should be supported for both synchronous and asynchronous operations. - Proposal # 4: Separate NCSG patterns for synchronous and asynchronous operations are defined. NCSG patterns: - Proposal # 5: Define NCSG patterns for synchronous and asynchronous operations corresponding to legacy gap patterns with ID # 0 to ID #23 (defined in Table 9.1.2-1, TS 38.133). - Proposal # 6: VIL1 and VIL2 for FR1 and FR2 are defined as follows: VIL1; VIL2. Sync; Async; Sync; Async. 1 ms; 2 ms; 1 ms; 2 ms. 0.75 ms; 0.75 ms. - Proposal # 7: ML for synchronous MR-DC (MLsync) and ML for ignallings MR-DC (MLasync) can be derived as follows: MLsync = Nslots_sync Tslot - (VIL1+VIL2) [ms] MLasync = Nslots_async Tslot - (VIL1+VIL2) [ms] Where: - Nslots_sync = Total number of interrupted slots on serving cells defined in Table 9.1.2-4, TS 38.133. - Nslots_async = Total number of interrupted slots on serving cells defined in Table 9.1.2-4a, TS 38.133. - Tslot = NR slot length in ms NCSG patterns: - Proposal # 8: Network explicitly configures the NCSG patterns including all parameters related to the NCSG pattern (VIL1/VIL2/ML/VIRP and gap offset). Impact on RRM requirements due to NCSG: - Proposal # 9: Existing interruption requirements for Scell activation/deactivation can serve as starting point for the study of VIL requirements. - Proposal # 10: Per UE and per FR NCSG for RRM measurement needs the specific UE capability. Measurement applicability: - Proposal # 11: In the first phase of the WI consider only the scenario in which the UE is configured only with NCSG pattern. - Proposal # 12: When configured to perform measurements only on serving carriers using NCSG then: - no scaling factor (CSSF) is needed to scale their measurement periods - their measurement period scales with/depends on VIRP - Proposal # 13: When configured to perform measurements only on non-serving carriers using NCSG then: - scaling factor (CSSF) is needed to scale their measurement periods - their measurement period further scales with/depends on VIRP - Proposal # 14: When configured to perform measurements on serving and non-serving carriers using NCSG then: - gap sharing parameter (X) is used for splitting NCSG gaps between serving and non-serving carriers. - scaling factor (CSSF) including X is needed to scale their measurement periods - their measurement period further scales with/depends on VIRP - Proposal # 15: When NCSG is configured then during the ML the existing scheduling restriction requirements defined in TS 38.133 shall also apply. Measurement applicability: - Proposal # 16: Decision of the specification section(s) for specifying the NCSG patterns is deferred until reasonable level of agreements on NCSG concept and patterns are agreed by RAN4. Signaling aspects: - Proposal # 17: Let RAN2 decide NCSG ignalling details and any relation between NCSG and ‘NeedForGap’ based on RAN4 technical input on NCSG pattern design. |
| R4-2107176 | Nokia, Nokia Shanghai Bell | - Use cases for NCSG in NR encompass explicit and implicit configuration, as well as monitoring multiple deactivated Scells. NCSG support for synchronous and asynchronous network operation will be specified with separate NCSG patterns. Related to VIL requirements, basing them on absolute RF tuning times in FR1 and FR2 will provide improvement in data throughput on the serving carrier as number of interrupted slots can be substantially reduced in some cases compared to basing them on Rel-15 interruption requirements. Related to reuse of legacy gap patterns for NCSG usage: - Gap patterns #24 and #25 are excluded. - Gap patterns #20 to #23 are excluded. - Gap patterns #2, #3 and #10, #11 are excluded. Both implicit and explicit NCSG configuration is considered. NCSG may be configured simultaneously with legacy MG pattern. Related to NCSG applicability and UE capability support: if UE supports NCSG, it is mandated to support per-UE gap patterns #0, #1, #4-9 for NCSG. In case UE supports NCSG and per-FR gap patterns, it is mandated to support also gap patterns #12-19 in FR2. The selection of the appropriate signalling structure for informing the network on UE’s capability support for NCSG can be discussed at a later point in time, when the NCSG pattern design as well as NCSG applicability and UE capability support are finalized. RAN4 to specify NCSG support in separate (new) subclause in clause 9.1 of TS 38.133. |
| R4-2107318 | Qualcomm CDMA Technologies | Observation1: In NRDC, per UE NCSG may be configured to replace per UE MG on both CGs but it can also be benefitial to allow one CG with per UE MG and another CG with per UE NCSG under certain condition. Proposal1: in NRDC, following NCSG configurations shall be allowed for MCG and SCG respectively, i.e. FR1-NCSG+FR1/2-NCSG, per UE NCSG only, per UE MG+per UE NCSG*. Proposal1.1: Introduce ue-ncsg, fr1-ncsg and fr2-ncsg to NR RRC 38.331. Proposal1.2: Include refServCellIndicator in the NR NCSG configuration. Proposal2: Both per UE NR NCSG and per FR NR NCSG can be supported and configured in ENDC/NEDC. Proposal2.1: To support per FR NR NCSG for ENDC/NEDC, introduce fr1-Ncsg as a Boolean indication in LTE 36.331. Observation2: In ENDC, LTE NCSG configuration may determine the NR NCSG type to either be per UE or per FR. Observation3: per UE NCSG can be configured for FR1+FR2 NRCA. Observation4: the design of NCSG can follow two steps, 1) have VIL1+ML+VIL2 equal to MGL of legacy MG 2) choose VIL1 and VIL2 s.t. ML=MGL-VIL1-VIL2 Proposal3: Donot introduce NCSG pattern corresponding to the legacy patterns #24 and 25. Observation5: UE operations during VIL can be more than performing the RF retuning task due to booting /activating the spare RF chain and re-arranging the baseband processors etc.. Proposal4: Choice of VIL are based on the interruption time of transition bw/ non-DRX to DRX for 15Khz for FR1 and 60Khz for FR2 i.e. 1ms for per UE or per FR1 NCSG and 0.75ms for per FR2 NCSG. Proposal4.1: Number of interrupted slots is derived by assuming the proposal4 for VIL time and the slot duration for a given numerology, i.e. N=⌈VIL/slot_dur⌉ for sync condition and N=⌈VIL/slot_dur⌉ +1 for async condition Proposal5: per UE NCSG doesnot consider the patterns of legacy MGs with IDs 2-3. (0-1 are recommended) Proposal5.1: per FR1 NCSG doesnot consider the patterns of legacy MGs with IDs 2-3 and 6-11. (0/1/4/5 are recommended) Proposal5.2: per FR2 NCSG doesnot consider the patterns of legacy MGs with IDs 20-23. (12-19 are recommended) Proposal6: Expanding the NR IE NeedForGapsNR::gapIndication to include the enumerated values {ncsg, nogap-noNcsg} for clarity. |

## Open issues summary and Companies views’ collection for 1st round

NCSG of this WI are as follows:

  - Network Controlled Small Gap (NCSG) specification [RAN4, RAN2]

    - RRM requirements for NCSG [RAN4]

      - Requirements for Visible Interruption Length (VIL) for different numerologies in FR1 and FR2

      - Specification of NCSG patterns, Measurement Length (ML), and Visible Interruption Repetition Period (VIRP)

      - Requirements for DL reception and UL transmission during ML, before start VIL and after end VIL

      - Measurement requirements with NCSG

    - Specification of applicability of NCSG patterns [RAN4]

    - Procedures and signaling for NCSG patterns [RAN2]

### Sub-topic 2-1 Scenarios and use cases

#### Issue 2-1-1 NCSG scenarios

- Option 1a (Intel): RAN4 can prioritize the following NR NCSG using scenario Eliminate/reduce interruption rate due to the transition of the target measurement frequency or BWP

- Option 1b (Ericsson):

  - Case 1: If NCSG is configured then the interruptions on Pcell, PSCell or activated Scell(s) due to measurements on Pcell, PSCell, activated Scell, deactivated Scell, Scell with dormant BWP or unused RF chain shall not occur outside the visible interruption length before measurement (VIL1) and the visible interruption length after measurement (VIL2).

  - Case 2: If NCSG is pre-configured then after switching from non-dormant BWP to dormant BWP on a Scell, then interruptions on Pcell, PSCell or activated Scell(s) due to measurements on the Scell with dormant BWP shall not occur outside the visible interruption length before measurement (VIL1) and the visible interruption length after measurement (VIL2).

- Option 1c (Huawei): RAN4 to support the following use cases of NCSG:

  - To eliminate the interruption due to RRM measurement on deactivated SCC and interruption due to RRM/CSI measurement on SCC in dormancy, and

  - To perform inter-frequency/RAT measurement when UE is capable of such measurement with NCSG

- Option 1d(Nokia)

  - Monitoring multiple deactivated Scells

- Option 2 (Apple) : RAN4 can consider two use cases for NCSG:

  - 1) UE can use spare RF chain(s) for inter-frequency measurement.

  - 2) UE can enlarge channel bandwidth to perform measurement on contiguous carriers.

- Option 3(CATT).

  - NCSG can be defined for intra/inter-band CA and synchronous/asynchronous DC.

  - For the UE capable of per-UE gap, NCSG can be configured when legacy gap is not configured. For the UE capable of per-FR gap, NCSG can be configured when no legacy gap is configured or the legacy gap is configured in the different FR.

- Option 4a (MTK)

  - Intra-frequency measurements with MG,

  - inter-frequency measurements with MG or inter-RAT measurements may use NCSG instead of MG when UE supports the corresponding band combination and has additional RF chains for measurements.

- Option 4b(CMCC) NCSG can be used for

  - intra-frequency measurements with MG,

  - inter-frequency measurements with MG,

  - inter-RAT measurements,

  - deactivated SCC measurement when UE have spare RF chains during the measurements.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | In short we think NCSG can be used for all measurement defined in R15/16 which requires measurement gap. Some companies mentioned using NCSG for measurement on deactivated SCC. We would like to further study how significant benefit we can get from that. The advantage of NCSG is that UE can still do data with serving cell during ML. Note that during VIL UE is not expected to be able to keep Rx/Tx with serving cell. This has already been reflected in current interruption requirements: ##### 8.2.2.2.3 Interruptions during measurements on deactivated SCC Interruptions on Pcell due to measurements when an Scell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [2] is 640 ms or longer. The UE is only allowed to cause interruptions immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 8.2.2.2.2-1 if the Pcell is not in the same band as the deactivated Scell. Each interruption shall not exceed requirement in Table 8.2.2.2.2-2 if the Pcell is in the same band as the deactivated Scell.. |
| CMCC | Option 4b. In LTE, NCSG is applied to inter-frequency measurements, inter-RAT measurements and deactivated SCC measurement when UE have spare RF chains during the measurements, we prefer to follow the same principle for NR NCSG. |
| MTK | Many of the Options do not contradict to one another. We are fine to take either Option 4a or 4b as a starting point. |
| Qualcomm | Option4b is supported. |
| OPPO | Option 4a and 4b is fine. Based on these, NCSG should be configured based on MG configuration. |
| Ericsson | We support option 1b. The main aim of NCSG is to ensure that there are no autonomous interruptions due to measurements. This is to avoid losing data when gNB schedules the UE but does not know when the UE will retune its receiver. |
| Intel | Technically speaking, if all legacy MG patterns (in Rel16) can be reused for NCSG, naturally NCSG can be used for all gap-based measurement scenarios in NR. However, Option 1a, 1b,1c by which the possible using scenarios were limited is to avoid too much new NGSG patterns needed and optimized. Therefore, we suggest that we can focus on the more specific scenario (e.g. to avoid the interruption due to measured the deactivated SCell). Please also noted that this issue will impact our discussion on the NCSG pattern design (issue 2-2-1) |
| Huawei | We can support option 4b which is more comprehensive than option 1c we proposed, i.e. intra-frequency measurement with MG can be also measured with NCSG. |
| Nokia | In our view, options 1b, 1c and 1d are quite similar and thus can be merged. We see this group of scenarios as relevant for NCSG. |

#### Issue 2-1-2 Per-CC gap usage

- Option 1 (MTK, Apple, Intel, Huawei, Nokia) : No

Recommended WF: Agree Option 1.

| Company | Comments |
| --- | --- |
| Apple | Support option 1. |
| MTK | Support Option 1 |
| Qualcomm | Support option1 |
| vivo | Ok with option 1 |
| OPPO | Option 1 is fine. |
| Ericsson | Support option 1 |
| ZTE | Support Option 1. |
| CATT | Fine with option 1. |
| Intel | Support option 1 and recommended WF |
| LG Electronics | Support Option 1. |
| Huawei | Support option 1. |
| Nokia | We support option 1 and the recommended WF, |

#### Issue 2-1-3 NCSG for synchronous/asynchronous operation

- Option 1. (Nokia, Intel, Ericsson, CATT): NCSG patterns shall be defined for both synchronous and asynchronous operations

Recommended WF: Agree Option 1.

| Company | Comments |
| --- | --- |
| Apple | Support option 1. |
| CMCC | We are OK with option 1. |
| MTK | No. We prefer the NCSG pattern to be AGNOSTIC to synchronous and asynchronous operations. The visible interruption can be sync/async dependent. |
| Qualcomm | Option1 is supported. We interpret option1 as that NCSG shall be allowed/designed for sync and async operations for use cases. |
| vivo | Ok with option 1 |
| OPPO | Option 1 is fine. |
| Ericsson | Support option 1 |
| ZTE | Support Option 1. |
| CATT | Support option 1. |
| Intel | Option 1 |
| LG Electronics | Support Option1. |
| Huawei | Option 1 needs to be clarified. We agree that NCSG pattern should be defined for both, but it does not means RAN4 needs to be define separate NCSG patterns for sync an asyc scenarios. |
| Nokia | We support option 1 and the recommended WF, |

### Sub-topic 2-2 NCSG pattern

#### Issue 2-2-1 General NCSG pattern design principle

- Option 1a (CATT) Similarly as LTE, define NCSG for part of legacy gap pattern with long MGL.

- Option 1b (Apple): if the support of NCSG and the support of legacy MG patterns can be independently indicated, then all the existing MG patterns can be extended to be NCSG.

- Option 2 (Intel, vivo, OPPO) reuse part of the legacy MG patterns in [2] as the new NCSG patterns in NR.

- Option 2a(vivo):

  - Do not design NCSG for legacy NR measurement gap when the MGL of that legacy measurement gap is less than a particular threshold, for example 4ms for FR1 (3.5ms for FR2)

  - Do not design NCSG for legacy NR measurement gap when the percentage of (VIL1+VIL2)/ML is larger than a particular threshold, for example 2.

  - NCSG for legacy NR measurement gap with MGRP = 20ms are not defined.

- Option 2b (OPPO): Prefer to reuse part of the legacy MG patterns as reference for NR NCSG patterns, with long MGL, e.g., 6ms for FR1 or 5.5ms FR2.

- Option 2c (Qualcomm)

  - gap patterns #24 and #25 won’t apply to NCSG.

  - per UE NCSG doesnot consider the patterns of legacy MGs with IDs 2-3. (0-1 are recommended)

  - per FR1 NCSG doesnot consider the patterns of legacy MGs with IDs 2-3 and 6-11. (0/1/4/5 are recommended)

  - per FR2 NCSG doesnot consider the patterns of legacy MGs with IDs 20-23. (12-19 are recommended)

- Option 2d(Nokia)

  - Gap patterns #24 and #25 are excluded.

  - Gap patterns #20 to #23 are excluded.

  - Gap patterns #2, #3 and #10, #11 are excluded

- Option 3 (Ericsson, Huawei, CMCC, MTK) Define NCSG patterns corresponding to legacy gap patterns with ID # 0 to ID #23 (defined in Table 9.1.2-1, TS 38.133).

- Option 3a(Ericsson) Separate NCSG patterns for synchronous and asynchronous operations are defined

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | As mentioned in option 1b, if the support of NCSG and the support of legacy MG patterns can be independently indicated, then all the existing MG patterns can be extended to be NCSG. It would be great that UE has flexibility to support any NCSG pattern and network has chance to enable any pattern that is supported by the UE. |
| CMCC | Option 3. Gap pattern #24 and #25 are introduced in positioning WI, which is used to enable the longer PRS measurement. The motivation to introduce Gap pattern #24 and #25 seems contradict with the motivation of introducing NCSG. From this point of view, Gap pattern #24 and #25 can be precluded from NCSG. As for gap pattern 0~23, considering that except the mandatory gap patterns, other gap patterns are optionally supported. If we only define NCSG patterns for the selected gap patterns, it is possible that for the supported gap pattern, there is no corresponding NCSG pattern and the benefit of NCSG cannot be experienced. Based on above consideration, it is proposed to use NR gap patterns #0~23 for NCSG. |
| MTK | We support Option 3, but also open to any down selection. |
| Qualcomm | Option 2 is supported overall. |
| Vivo | On top level, option 2 is supported. |
| OPPO | Option is supported in principle. FFS how to merge options. |
| Ericsson | Option 3 and option 3a. We need to define pattern for sync and async. This means one sync and one async for each MG pattern are needed. |
| CATT | Support option 1a. For option 2, why the legacy MG pattern is reused as NCSG pattern? They are in different structure. |
| Intel | The main concern for NCSG pattern is which legacy gap patterns can be reused. This is also up to the main using scenario of NCSG. The more designs can be FFS. The other general principle agreed in the last meeting is “ legacy gap pattern can be reused for ncsg, we can assume that MGL= VIL1+ML+VIL2, MGRP =VIRP.” For some legacy pattern with very short ML, if the new NCSG pattern reuse it, the available ML for NCSG will be too short to contain any measurement reference signal (e.g. SSB). Therefore, we may not reuse these legacy pattern for NCSG. |
| Huawei | Support option 3. It can be up to NW to decide which NCSG pattern to use. |
| Nokia | We support option 2d and option 2c, which is very similar. MGP’s with short ML should be discarded. For LTE we have only MGL= 6 ms. |

#### Issue 2-2-2 Visible Interruption Length (VIL)

- Option 1 (Apple, CATT, vivo): VIL should be explicitly defined as the number of interrupted slots, similar as existing Table 9.1.2-4 in TS38.133.

- Option 1a (vivo)

  - For synchronous case, the value of VIL1, expressed by victim cell’s number of slots, is max{ RF switching time (0.5ms or 0.25ms), victim cell slot duration}/victim cell slot duration

  - Based on VIL1 value for synchronous case, the value of VIL2 for synchronous case is the same as the value of VIL1 for DL and 1 extra slot may be added for UL. For asynchronous case, in general 1 slot should be added to both VIL1 and VIL2.

- Option 2 (Qualcomm):

  - Choice of VIL are based on the interruption time of transition bw/ non-DRX to DRX for 15Khz for FR1 and 60Khz for FR2 i.e. 1ms for per UE or per FR1 NCSG and 0.75ms for per FR2 NCSG.

  - Number of interrupted slots is derived by assuming the proposal4 for VIL time and the slot duration for a given numerology, i.e. N=⌈VIL/slot_dur⌉ for sync condition and N=⌈VIL/slot_dur⌉ +1 for async condition

- Option 3 (Nokia):

  - Related to VIL requirements, basing them on absolute RF tuning times in FR1 and FR2.

- Option 3a (Ericsson):

  - VIL1 and VIL2 for FR1 and FR2 are defined as follows:

| VIL1 |  | VIL2 |  |
| --- | --- | --- | --- |
| Sync | Async | Sync | Async |
| 1 ms | 2 ms | 1 ms | 2 ms |
| 0.75 ms |  | 0.75 ms |  |

- Option 3b(CMCC): define VIL and interruption separately with following consideration:

  - If VIL is defined as the equivalent time of the interrupted slots, and different NCSG pattern for synchronous and asynchronous operation is adopted, there may be too many NSCG patterns.

  - If VIL is defined as the equivalent time of the interrupted slots, and same NCSG pattern for synchronous and asynchronous operation is designed, the benefit of introducing NCSG in synchronous scenario will be sacrificed.

  - same NSCG pattern is applied for synchronous and asynchronous operation.

- Option 3c(Huawei):

  - RAN4 to define VIL as an absolute value for RF re-tuning, which is common for all cases including sync/async, UL/DL and with/without MGTA.

  - RAN4 to define the number of interrupted slots due to VIL dependent on sync/async and with/without MGTA.

  - RAN4 to further discuss if the number of interrupted slots due to VIL for UL.

- Option 4 (MTK)

  - Introduce absolute RRT (RF retuning time) to replace VIL in NCSG pattern definition. RRT is 0.5ms in FR1 and 0.25ms in FR2, occurring in the beginning and the end of NCSG.

  - VIL is the interrupted slots on active serving cells and is defined by following R15 measurement interruption rule which depends on both

    - Victim cell SCS and Aggressor reference cell RRT, and

    - Sync and Async. Deployment

- Option 5 (OPPO)

  - NW explicitly configures the NCSG for VIL1/VIL2/ML/VIRP and offset

- Table 1: NCSG configuration for NR

| NCSG Pattern Id | Visible interruption length before measurement (VIL1, ms) | Measurement Length during which there is no gap (ML, ms) | Visible interruption length after measurement (VIL2, ms) | Visible interruption Repetition Period (VIRP, ms) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 4 | DL: 1 UL: 2 | 40 |
| 1 | 1 | 4 | DL: 1 UL: 2 | 80 |
| 2 | 2 | 3 | 2 | 40 |
| 3 | 2 | 3 | 2 | 80 |
| 4 | 1 | 4 | DL: 1 UL: 2 | 20 |
| 5 | 1 | 4 | DL: 1 UL: 2 | 160 |
| 6 | 2 | 3 | 2 | 20 |
| 7 | 2 | 3 | 2 | 160 |
| 8 | 1 | 3.5 | DL: 1 UL: 2 | 40 |
| 9 | 1 | 3.5 | DL: 1 UL: 2 | 80 |
| 10 | 2 | 2.5 | 2 | 40 |
| 11 | 2 | 2.5 | 2 | 80 |
| 12 | 1 | 3.5 | DL: 1 UL: 2 | 20 |
| 13 | 1 | 3.5 | DL: 1 UL: 2 | 160 |
| 14 | 2 | 2.5 | 2 | 20 |
| 15 | 2 | 2.5 | 2 | 160 |

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | We propose option 1 as principle. To make progress step by step, maybe we can follow Option 3c. |
| CMCC | We prefer to specify the NCSG pattern and interruption separately. In detail, VIL only includes the RTT, interruption is not reflected in VIL and is specified separately. The reason we suggest to specify the NCSG pattern and interruption separately is that: • If VIL is defined as the equivalent time of the interrupted slots, and different NCSG pattern for synchronous and asynchronous operation is adopted, there may be too many NSCG patterns. • If VIL is defined as the equivalent time of the interrupted slots, and same NCSG pattern for synchronous and asynchronous operation is designed, the benefit of introducing NCSG in synchronous scenario will be sacrificed. |
| MTK | Support Options 3b, 3c and 4. It is important to separate the concepts of - The time UE needs to re-tune or switch ON/OF its RF - The interruption caused by UE’s RF change. So that we can avoid to define too many NCSG patterns for different SCS and sync/async cases. |
| Qualcomm | Option2 and option3a are both supported. We also feel it is not necessary to introduce the concept of RF tuning time(RTT) but RAN4 does need to agree what is the required time during which UE may undertake more than just RF tuning operations. Further number of interrupted slots can be derived based on the agreed VIL time accordingly. For the VIL time, we see two options mainly, FR1 0.5ms v.s. 1ms and FR2 0.25 v.s. 0.75ms. We shall firstly discuss if we agree to one or both options. |
| vivo | Support 1 and 1a, option 3a is also ok. |
| OPPO | Pending on issue 2-2-1. Support option 5, For FR1, VIL1 or VIL2 can be 1ms or 2ms, For FR2, we can compromise to option 3a. |
| Ericsson | We support option 3a. In our view there are two main principles: define separate VIL for sync and async cases. Secondly to limit VIL values define VIL one for FR1 and another one for FR2 i.e. VIL is not function of SCS. |
| CATT | Support option 1. |
| Intel | We prefer Option 3a/3b/3c. That is VIL can be defined by the absolute time depending on the RF retuning time. The interruption introduced by RF operations on the serving cells can be defined by slots. For Option 4, with such clarification above, RTT is same as VIL. The interruption slots can be decoupled from the gap pattern issues. |
| Huawei | Support option 3c. This is aligned how MG pattern is defined in the spec, e.g. MGL is defined as absolute value like 6ms, and the number of interrupted slots can be depending on the SCS of the victim serving cell, sync/async scenario and MGTA. |
| Nokia | We support option 3. However, options 1a, 3, 3c and 4 are all based on using absolute RF retuning times. |

#### Issue 2-2-3 Measurement Length (ML)

- Option 1 (vivio): legacy gap pattern is reused for NCSG with the assumption that MGL= VIL1+ML+VIL2 (ML>0) and VIRP = MGRP of legacy MG..

- Option 1a (Intel): ML can be defined based on NR legacy gap patterns [2, TS38.133]  but some of them could be excluded (e.g. #24, 25).

- Option 2 (Ericsson): ML for synchronous MR-DC (MLsync) and ML for synchronous MR-DC (Mlasync) can be derived as follows:

MLsync =  Nslots_sync Tslot  - (VIL1+VIL2)               [ms]

MLasync =  Nslots_async Tslot  - (VIL1+VIL2)              [ms]

Where:

  - Nslots_sync = Total number of interrupted slots on serving cells defined in Table 9.1.2-4, TS 38.133.

  - Nslots_async = Total number of interrupted slots on serving cells defined in Table 9.1.2-4a, TS 38.133.

  - Tslot  = 	NR slot length in ms

- Option 2b(Qualcomm):  have VIL1+ML+VIL2 equal to MGL of legacy MG in time 2) choose VIL1 and VIL2 s.t. ML=MGL-VIL1-VIL2

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | In principle it should be MGL= VIL1+ML+VIL2. We can come back to ML after we have conclusion on VIL. |
| MTK | In principle MGL= VIL1+ML+VIL2 (or we replace VIL by RRT instead). Suggest to conclude on previous issues first |
| Qualcomm | If longer VIL time than RF retuning time is considered, it may be needed to ensure ML is preserved to be the same as (MGL-2*RF tuning time). Otherwise, longer VIL interruption can reduce the achievable ML length. |
| Vivo | Ok with option 1 |
| OPPO | Agree to come back after conclusion on VIL1/2. |
| Ericsson | We support option 2. But in principle all are similar. |
| CATT | Option 1 has been concluded in last meeting: - ML: If legacy gap pattern can be reused for NCSG, we can assume that MGL= VIL1+ML+VIL2 - VIRP: If legacy gap pattern can be reused for NCSG, VIRP = MGRP of legacy MG |
| Intel | The general principle below was agreed in last meeting. - “ML: If legacy gap pattern can be reused for NCSG, we can assume that MGL= VIL1+ML+VIL2 “ So, it is straightforward to conduct ML based on the legacy MG pattern. That is Option 1/1a/2b can fine for us. |
| Huawei | It was already agreed in last meeting that If legacy gap pattern can be reused for NCSG, we can assume that MGL= VIL1+ML+VIL2, so ML should be defined as MGL-VIL1-VIL2. |
| Nokia | We agree with other companies, that this was agreed in last meeting. Thus, we support option 1. |

### Sub-topic 2-3 Configuration of NCSG

#### Issue 2-3-1 Per-UE/Per-FR NCSG applicability

- Option 1(Qualcomm, CATT, OPPO, Huawei) both per-FR and per-UE NCSG can be supported

- Option 1a(Apple) both per-FR and per-UE NCSG can be supported

  - For UE supporting per-FR gap, VIL is allowed only on the serving cell in the same FR wherein there is NCSG operation. Otherwise, VIL is allowed on all serving cells

- Option 1b(OPPO): NCSG pattern should follow the applicability of corresponding MG pattern

- Option 2(Nokia): Related to NCSG applicability and UE capability support: if UE supports NCSG, it is mandated to support per-UE gap patterns #0, #1, #4-9 for NCSG. In case UE supports NCSG and per-FR gap patterns, it is mandated to support also gap patterns #12-19 in FR2.

- Option 2a (Intel): based on MG configuration considering per FR1 or FR2 gap

- Option 3 (MTK):  Existing gap applicability in Rel-16 for NR-only measurements and mandatory gap patterns

Recommended WF: Further discussion needed. Collect companies’ views

| Company | Comments |
| --- | --- |
| Apple | Support option 1 and 1a. |
| CMCC | We are OK with option 1. |
| MTK | Support Option 3. Option 1/1a/1b is also fine to us. But one thing needs to be clarified is whether we allow inter-RAT EUTRAN measurement via NCSG. |
| Qualcomm | Option1 is supported |
| vivo | Ok with option 1 and 1a |
| OPPO | Support Option 1, NCSG should be considered as a specific MG. |
| Ericsson | Support option 1. |
| ZTE | Support Option 1. |
| CATT | Support option 1. |
| Intel | We are fine for Option 1 |
| LG Electronics | Support Option 1 |
| Huawei | Support option 1. On option 1a, we understand whether VIL applies to all serving cells or serving cells in one FR depends on whether the NCSG is configured as per-UE or per-FR. |
| Nokia | We support option 2 and also option 1. |

#### Issue 2-3-2 Implicit or explicit configuration of NCSG

- Option 1 (Nokia): Both implicit and explicit NCSG configuration is considered

- Option 1a (Apple):

  - If existing MG pattern table 9.1.2-1 is to be reused, then implicit configuration of NCSG can work, i.e. network would still configure MG pattern as legacy and UE may introduce VILs at the start and stop of a configured MG while ML is the same as MGL and VIRP is the same as MGRP.

  - if NCSG patterns are to be defined in a new table, then explicit indicator of NCSG would be needed. The principle is that both UE and network shall have same understanding of VIL, ML and VIRP.

- Option 2 (MTK, Intel, Qualcomm, Ericsson, Huawei, vivo, CMCC, OPPO) Only NW explicit configuration

- Option 2a(Qualcmm):

  - Expanding the NR IE NeedForGapsNR::gapIndication

  - Introduce ue-ncsg, fr1-ncsg and fr2-ncsg to NR RRC 38.331

  - To support per FR NR NCSG for ENDC/NEDC, introduce fr1-Ncsg as a Boolean indication in LTE 36.331.

  - Include refServCellIndicator in the NR NCSG configuration.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | The principle is both UE and NW shall have same understanding on whether and which NCSG pattern is being used. So we give option 1a, i.e. depending on how the NCSG patterns would be introduced. |
| CMCC | Option 2. In LTE NCSG, both explicit configuration and implicit configuration are supported, but implicit configuration is for per-CC gap usage. If per-CC gap usage is agreed not to be considered for NR, then only explicit way can be used. |
| MTK | Support Option 2. Explicit configuration helps to avoid mis-understanding between network and UE. For an examples, if legacy gap is not configured, should UE interpret it as “NCSG is enabled” or “both legacy gap and NCSG are disabled”? |
| Qualcomm | Option2 is supported. Option2a is what we think is needed for introducing the configuration signaling. But we are fine to leave it for further discussion. For this WI, note WID allows introducing new signaling. |
| Vivo | Support option 2 |
| OPPO | Support option 2 |
| Ericsson | We support option 2. |
| CATT | Support option 2. |
| Intel | Support Option2. For Option 2a, we supposed that it is one of potential explicit NCSG indication. But the exact signaling is up to RAN2 discussion. |
| LG Electronics | Support Option 2. |
| Huawei | Support option 2. We understand one way to make the configuration is that the MG pattern is configured same as today via MeasGapConfig, and NW could further configure if the pattern should be used as legacy MG pattern (with full MGL) or as NCSG pattern (with VIL and ML). |
| Nokia | We see benefit in supporting implicit and explicit configuration of NCSG, but we can compromise to option 2. |

### Sub-topic 2-4 Measurement requirements impacts

#### Issue 2-4-1 Interruption requirements

- Option 1 (Intel): The interruption requirements in TS38.133 and TS36.133 shall be revisited

- Option 2(Ericsson, CATT, Qualcomm): Existing interruption requirements for SCell activation/deactivation can serve as starting point for the study of VIL requirements

- Option 3(CMCC): the interruption is proposed as following:

| SCS | Synchronous |  | Asynchronous |  |
| --- | --- | --- | --- | --- |
|  | interruption length before measurement | interruption length after measurement | interruption length before measurement | interruption length after measurement |
| 15KHz SCS | 1 slot | 1 slot | 2 slots | 2 slots |
| 30KHz SCS | 2 slots | 2 slots | 3 slots | 3 slots |
| 60KHz SCS | 3 slots | 3 slots | 4 slots | 4 slots |
| 120KHz SCS | 6 slots | 6 slots | 7 slots | 7 slots |

- Option 4(Huawei): RAN4 to further discuss the condition, capability and impacts to measurement requirements for UE to use NCSG to control interruptions due to measurement on deactivated SCC or Scell in dormancy

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | Similar with issue 2-2-2. |
| CMCC | As mentioned in 2-2-2, We prefer to specify the NCSG pattern and interruption separately. In detail, VIL only includes the RTT, interruption is not reflected in VIL and is specified separately. As for the interruption length for NCSG, we are OK to use the existing interruption requirements for Scell activation/deactivation as baseline. And interruption table in option 3 is specified based on the existing interruption requirements for Scell activation/deactivation. |
| MTK | Let’s conclude 2-2-2 first. |
| Qualcomm | Option2 and we are fine to consider Option3 as it would be in line with derived number of interrupted slots based on 1ms for FR1 and 0.75ms for FR2. @CMCC, please note we think more slots of interruption may be needed for 60/120Khz if the serving cell is on FR1 or LTE, i.e. 4 and 8 slots respectively. Thanks, |
| vivo | Wait for the conclusion of 2-2-2 |
| OPPO | FFS |
| Ericsson | Support option 2. |
| CATT | Support option 2. |
| Intel | All options are compatible. In conclusion, the interruption in case of NCSG shall be FFS. And how to define interruption requirements is also dependent with the using scenario of NCSG. If we apply NCSG for the interruption elimination scenario, the interruption may be not allowed. |
| LG Electronics | Prefer Option 2 as starting point. |
| Huawei | It seems the issue is a bit mixed. Option 4 from our side is addressing the interruption requirements for measurement on deactivated SCC or SCell in dormancy. We understand some other options are addressing the interruption for VIL which are more relevant to issue 2-2-2. |
| Nokia | We should first get consensus for issue 2-2-2. |

#### Issue 2-4-2 Measurement mode

- Option 1 (MTK): RAN4 shall discuss the NCSG measurement mode issue similar as legacy MG.

  - The existing measurement mode requirements (effective MGRP, data scheduling depends on gap configuration) can be the baseline.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| MTK | We think this is what RAN4 anyway has to discuss, just like what we did in Rel-15 for legacy gap. Perhaps the discussion can happen later, but we would like to put this issue in the radar in order to make the whole work complete. |
| Qualcomm | Fine to keep it FFS. |
| Ericsson | Keep FFS for time being |
| Intel | Yes. The other aspects for measurement requirements can be FFS up to the basic using scenarios and patterns |
| Huawei | We need more time to study on this issue. |
| Nokia | This can be studied later when the NCSG design is progressed. |

### Sub-topic 2-5 Capability support

#### Issue 2-5-1 Per-UE or Per-FR capability support

- Option 1. (Apple, Ericsson, Qualcomm, CATT, , Nokia): both per UE and per FR NCSG for RRM measurement needs the specific UE capability.

- Option 2 (MTK):  No additional NCSG capability for per-UE and per-FR differentiation is needed.

  - When UE only supports per-UE gap, the NCSG is per-UE.

  - When UE supports per-FR gap, the NCSG interruption is constrained in the FR.

- Option 3 (Huawei): related UE capability can be FFS

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | Actually we propose existing per-UE and per-FR capability can apply to NCSG in our contribution. We are open for further discussion. |
| CMCC | More discussion is needed. |
| MTK | Support Option 2. We suggest to leverage existing UE capability as much as possible in order to avoid any discussion on the inconsistency between legacy and new UE capabilities. |
| Qualcomm | We can compromise to Option3 after companies agree on the scenarios to support. Option1 has the benefit of independently claiming the support of perUE v.s. perFR for NCSG without having to depend on the legacy UE capability. |
| Vivo | Prefer option 3 |
| OPPO | Option 2 is fine. |
| Ericsson | Support option 1 but also open for discussion. |
| CATT | Fine with option 3. |
| Intel | Option 2 is fine for us. But it can be FFS also. |
| LG Electronics | Prefer Option 3 |
| Huawei | We can support option 1. Option 2 means that a UE supporting per-FR MG should also support per-FR NCSG. It is a bit strong assumption in our view. |
| Nokia | We support option 1. |

### Sub-topic 2-6 Applicability

#### Issue 2-6-1 Measurement requirements applicability

- Option 1 (Huawei) Further discuss the options for the two cases for measurement during ML

  - Case 1: UE only performs measurement that requires NCSG

  - Case 2: UE performs both measurement that requires NCSG and measurement that does not require NCSG or MG

- Option 2 (Apple) : measurement in NCSG can be treated as measurement within gap when defining RRM requirements, such that the existing CSSF design can be reused.

- Option 3(Ericsson)

  - In the first phase of the WI consider only the scenario in which the UE is configured only with NCSG pattern.

  - When configured to perform measurements only on serving carriers using NCSG then:

    - no scaling factor (CSSF) is needed to scale their measurement periods

    - their measurement period scales with/depends on VIRP

  - When configured to perform measurements only on non-serving carriers using NCSG then:

    - scaling factor (CSSF) is needed to scale their measurement periods

    - their measurement period further scales with/depends on VIRP

  - When configured to perform measurements on serving and non-serving carriers using NCSG then:

    - gap sharing parameter (X) is used for splitting NCSG gaps between serving and non-serving carriers.

    - scaling factor (CSSF) including X is needed to scale their measurement periods

    - their measurement period further scales with/depends on VIRP

- Option 4 (CATT, MTK) : Measurement requirement for NCSG is defined based on the assumption that only one layer can be monitored in one NCSG occasion

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | To simplify CSSF design, we would like to treat NCSG same as legacy MG when calculating CSSF. One thing needs to be highlighted is that even though UE can support NCSG it doesn’t mean UE can measure all serving cells simultaneously. Limitation of baseband resource still needs to be considered. |
| MTK | Support Option 4 or Case 1 in Option 1 The intention is to make the specification simple. |
| Qualcomm | Option4 can be supported. |
| Vivo | Option 4 is ok |
| OPPO | Option 4 is fine |
| Ericsson | Option 3. It is not possible to generalize. We need to consider different measurement scenarios. For example if the UE measures on only SCCs with deactivated Scells then the UE can do measurements on SCCs in parallel. But if UE is measuring on CCs using spared RF chain then the measurement period needs to be scaled based on CSSF because the UE cannot measure all of them in parallel. Therefore, NCSG cannot simply be treated as legacy MG pattern for all measurement scenarios. |
| CATT | Support option 4. |
| Intel | Option 4 can simply discussion if NCSG shall be support more general cases. So, we would like FFS on this up to the fundamental using scenario are agreed. |
| Huawei | We can support option 2/4, which is same as Case 1 in option 1. |
| Nokia | Option 4 can serve as baseline. |

#### Issue 2-6-2 RF combination limitation

- Option 1. (MTK): UE is not expected to measure 2 inter-freq/RAT layers in parallel even if UE reports the support of NCSG to both corresponding bands.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | No need to explicitly highlight this. It will be reflected in CSSF design. |
| MTK | This is principle that RAN4 needs to agree first before specifying requirements for CSSF. There are 2 issues within this proposal - Whether to allow inter-RAT EUTRAN measurement during NCSG - Whether to mandate UE to measure more than 1 frequency layers during NCSG |
| Qualcomm | Option1 can be agreeable. |
| OPPO | Option 1 is fine. |
| Ericsson | Option 1 is ok. We need CSSF. See our option 3 in issue 2-6-1. |
| CATT | Whether this is already reflected in option 4 of issue 2-6-1? |
| Intel | Can be FFS up to the fundamental using scenarios agreed. |
| LG Electronics | Support Option 1. |
| Huawei | We can support option 1, but this can be already addressed by issue 2-6-1. |
| Nokia | We can support option 1. |

#### Issue 2-6-3 Rx beam limitation

- Option 1. (MTK): NW needs to be informed that the inter-frequency measurements with NCSG is CBM or IBM with serving cells in FR2.

- Option 2(Apple) NCSG in FR2 should be deprioritized in current stage.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | In FR2, even though UE can have some spare RF chain but it may not be able to measure neighbor cell while keeping data transmission with serving cell since different beams are expected for serving cell and neighbor cell. For UE with IBM, it is possible that UE uses spare RF chain to measure target cell in another band while keeping connection with serving cell. However, the CSSF design covers both intra-band inter-frequency and inter-band inter-frequency. It would be quite complicated to differentiate RRM requirements for inter-band inter-frequency and intra-band inter-frequency. Thus we propose not to consider NCSG in FR2 at current stage. |
| MTK | Support Option 1. Since this is the 1st time we bring this issue, we are also fine to FFS. But we would like to highlight that this is something RAN4 needs to resolve in order to complete this work. |
| Qualcomm | Option1 can be supported. |
| Vivo | Option 2 is preferred |
| OPPO | FFS for FR2 Rx beam limitation. |
| Ericsson | Our understanding is that in FR2: - In case of CBM there will be scheduling restriction on serving cell when measuring on inter-frequency using spare RF chain. - In case of IBM there will be no scheduling restriction on serving cell when measuring on inter-frequency using spare RF chain. |
| Intel | Can be FFS up to the fundamental using scenarios agreed. |
| LG Electronics | FFS for FR2 |
| Huawei | We need more time to check, e.g. how IBM/CBM capability is reported now. |
| Nokia | Should be left FFS. |

#### Issue 2-6-4 Scheduling and measurement restriction

- Option 1 (Ericsson): When NCSG is configured then during the ML the existing scheduling restriction requirements defined in TS 38.133 shall also apply,

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | We think the common understanding is that UE shall be able to follow DL/UL scheduling during ML. exact wording can be discussed in CR phase. |
| MTK | Support Option 1. |
| Qualcomm | Option1 is supported. |
| Vivo | Ok with option 1 |
| OPPO | Option 1 is fine. |
| Ericsson | Support option 1. |
| Intel | In principle Option 1 is fine for us. |
| LG Electronics | Support Option 1. |
| Huawei | Support option 1 in general, but the issue raised in option 1 of issue 2-6-3 should be also considered. |
| Nokia | We support option 1. |

### Sub-topic 2-7 Relation with ‘NeedForGap’

#### Issue 2-7-1 How to consider the relation between NCSG and ‘NeedForGap’?

- Option 1 (Intel, Apple): The “NeefForGap” signaling structure can be reused for NR NCSG as a start point

- Option 1a (MTK):  Rel-17 NCSG capability is reported on top of existing RAN2 ‘NeedForGap’ ignaling structure with a new component ‘NCSG’.

- Option 1b (Huawei): Extend the Rel-16 NeedForGap ignaling for UE to indicate

  - Need for normal MG, or

  - No need for normal MG but NCSG, or

  - No need for either normal MG or NCSG

- Option 2 (CMCC):  Don’t reuse Rel-16 ‘NeedForGap’ ignaling for NCSG

- Option 3(OPPO): NR NCSG signaling is introduced newly, independent from Rel-16 “Needforgap” signaling, e.g., NCSG, No-NCSG.

- Option 4 (Ericsson, CATT): Let RAN2 decide NCSG signaling details and any relation between NCSG and ‘NeedForGap’ based on RAN4 technical input on NCSG pattern design

- Option 5 (Nokia) FFS when the NCSG pattern design as well 	as NCSG applicability and UE capability support are finalized.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Apple | In current specification there is no interruption requirement defined for NeedForGap. It is unclear that whether UE would cause interruption during measurement of target cell if UE indicates support of ‘NeedForGap’ for that BC due to RF retuning. NCSG requirements can make ‘NeedForGap’ more complete. |
| CMCC | Firstly, it is necessary to discuss whether NCSG and ‘NeedForGap’ is same or not. In our view, they are different feature. ‘NeedForGap’ is the case that measurement is performed without gap and without interruption (In LTE, no interruption is specified for ‘NeedForGap’). But NCSG means measurement with network controlled small interruption. Since NCSG and ‘NeedForGap’ are different feature, it is not preferred to reuse Rel-16 ‘NeedForGap’ signalling. |
| MTK | NCSG and NeedForGap are 2 different features, but they can share the same reporting framework. The intention is to aggregate all gap-less related UE capability together in one report. This helps to avoid any further discussion on the inconsistency between 2 features. For examples, - What does it mean if UE reports ‘gap’ for NeedForGap and supports NCSG - What does it mean if UE reports ‘no gap’ for NeedForGap and supports NCSG - What does it mean if UE reports ‘gap’ for NeedForGap and does not support NCSG - What does it mean if UE reports ‘no gap’ for NeedForGap and does not support NCSG |
| Qualcomm | Option1b is supported by following LTE convention. In our understanding, we need gap, ncsg and no-gap-no-ncsg. Ncsg means supporting gapless measurement with interruptions. No-gap-no-ncsg means supporting gapless measurement without any interruptions. Option1b can be recommended to RAN2 and we are fine with Option4 that it’s up to RAN2 to decide the final design and details. |
| OPPO | Regarding no reserved Bits in ‘Needforgap’, we prefer option 3 for independent NCSG configuration. |
| Ericsson | Support option 4. This is not urgent issue and need to involve RAN2. |
| CATT | Support option 4. We are fine with the understanding in option 1b, but we still think the signaling should be decided in RAN2. |
| Intel | In principle we support Option 1/1a/1b. RAN4 can input some general principle and framework of NCSG signaling. We are also fine Option 4. The exact signaling design is up to RAN2. |
| LG Electronics | Support Option 4. |
| Huawei | We support option 1b, but we are also fine with option 4. |
| Nokia | We support option 4 and option 5. |

### Sub-topic 2-8 Specification structure

#### Issue 2-8-1 Which clause can be used to include NCSG pattern in 38.133

- Option 1 (Nokia): RAN4 to specify NCSG support in separate (new) subclause in clause 9.1 of TS 38.133.

- Option 2(Ericsson): Decision of the specification section(s) for specifying the NCSG patterns is deferred until reasonable level of agreements on NCSG concept and patterns are agreed by RAN4.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| MTK | Option 2 is fine to us |
| Ericsson | Support option 2. Not very urgent. |
| CATT | Fine with option 2. |
| Intel | Support Option 2. |
| LG Electronics | Support Option 2 |
| Huawei | Support option 2. |
| Nokia | We support option 1, but we are ok to defer the decision, once NCSG pattern design has matured. |

### CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Issue#2-1-1 | NCSG Scenarios Tentative agreements: None. Candidate options: - Option 1a/1b/1d (Intel, Ericsson, Nokia): Eliminate/reduce interruption rate due to measurements on Pcell, PSCell, activated Scell, deactivated Scell, Scell with dormant BWP or unused RF chain. - Option 2 (Apple) : RAN4 can consider two use cases for NCSG: - 1) UE can use spare RF chain(s) for inter-frequency measurement. - 2) UE can enlarge channel bandwidth to perform measurement on contiguous carriers. - Option 3(CATT). - NCSG can be defined for intra/inter-band CA and synchronous/asynchronous DC. - For the UE capable of per-UE gap, NCSG can be configured when legacy gap is not configured. For the UE capable of per-FR gap, NCSG can be configured when no legacy gap is configured or the legacy gap is configured in the different FR. - Option 4a (MTK) - Intra-frequency measurements with MG, - inter-frequency measurements with MG or inter-RAT measurements may use NCSG instead of MG when UE supports the corresponding band combination and has additional RF chains for measurements. - Option 4b(CMCC, Huawei) NCSG can be used for - intra-frequency measurements with MG, - inter-frequency measurements with MG, - inter-RAT measurements, - deactivated SCC measurement when UE have spare RF chains during the measurements Recommendations for 2nd round: Can be FFS |
| Issue#2-1-2 | Per-cc usage Tentative agreements: No Per-cc gap usage in Rel17. Candidate options: Recommendations for 2nd round: No need further discussion |
| Issue#2-1-3 | NCSG for synchronous/asynchronous operation Tentative agreements: - NCSG patterns shall be defined for both synchronous and asynchronous operations - FFS on RAN4 needs to be define separate NCSG patterns for sync an asyc scenarios. Candidate options: Recommendations for 2nd round: No need further discussion in this meeting |
| Issue#2-2-1 | General NCSG design principle Tentative agreements:None Candidate options: - Option 1a (CATT) Similarly as LTE, define NCSG for part of legacy gap pattern with long MGL. - Option 1b (Apple): if the support of NCSG and the support of legacy MG patterns can be independently indicated, then all the existing MG patterns can be extended to be NCSG. - Option 2 (Intel, vivo, OPPO) reuse part of the legacy MG patterns in [2] as the new NCSG patterns in NR. - Option 2a(vivo): - Do not design NCSG for legacy NR measurement gap when the MGL of that legacy measurement gap is less than a particular threshold, for example 4ms for FR1 (3.5ms for FR2) - Do not design NCSG for legacy NR measurement gap when the percentage of (VIL1+VIL2)/ML is larger than a particular threshold, for example 2. - NCSG for legacy NR measurement gap with MGRP = 20ms are not defined. - Option 2b (OPPO): Prefer to reuse part of the legacy MG patterns as reference for NR NCSG patterns, with long MGL, e.g., 6ms for FR1 or 5.5ms FR2. - Option 2c (Qualcomm) - gap patterns #24 and #25 won’t apply to NCSG. - per UE NCSG doesnot consider the patterns of legacy MGs with IDs 2-3. (0-1 are recommended) - per FR1 NCSG doesnot consider the patterns of legacy MGs with IDs 2-3 and 6-11. (0/1/4/5 are recommended) - per FR2 NCSG doesnot consider the patterns of legacy MGs with IDs 20-23. (12-19 are recommended) - Option 2d(Nokia) - Gap patterns #24 and #25 are excluded. - Gap patterns #20 to #23 are excluded. - Gap patterns #2, #3 and #10, #11 are excluded - Option 3 (Ericsson, Huawei, CMCC, MTK) Define NCSG patterns corresponding to legacy gap patterns with ID # 0 to ID #23 (defined in Table 9.1.2-1, TS 38.133). - Option 3a(Ericsson) Separate NCSG patterns for synchronous and asynchronous operations are defined Recommendations for 2nd round : Can be FFS up to issue 2-1-1 |
| Issue#2-2-2 | Visible Interruption Length (VIL) Tentative agreements: None Candidate options: Recommendations for 2nd round: as there are too diverse views we can focus on the following questions in 2nd round. - Whether VIL can be defined by the absolute time depending on the RF retuning time or not? - If yes, the options below can be FFS - Option1: VIL time as interruption time of measuring inactive SCC e.g. 1ms for FR1 - Option2: VIL time as RF retuning time, e.g. 0.5ms for FR1 |
| Issue#2-2-3 | Measurement length (ML) Tentative agreements: Candidate options: ML can be derived from “MGL-VIL1-VIL2” If legacy gap pattern can be reused for NCSG Recommendations for 2nd round: Can be FFS. |
| Issue#2-3-1 | Per-UE/Per-FR NCSG applicability Tentative agreements: Both per-FR and per-UE NCSG can be supported in Rel17. Candidate options: Recommendations for 2nd round: No further discussion |
| Issue#2-3-2 | Implicit or explicit configuration of NCSG Tentative agreements: Only NW explicit configuration for NCSG is considered in Rel17. Candidate options: Recommendations for 2nd round: No further discussion |
| Issue#2-4-1 | Interruption requirements Tentative agreements: None. Candidate options: - Option 1 (Intel): The interruption requirements in TS38.133 and TS36.133 shall be revisited - Option 2(Ericsson, CATT, Qualcomm): Existing interruption requirements for SCell activation/deactivation can serve as starting point for the study of VIL requirements - Option 3(CMCC): the interruption is proposed as following: SCS; Synchronous; Asynchronous. interruption length before measurement; interruption length after measurement; interruption length before measurement; interruption length after measurement. 15KHz SCS; 1 slot; 1 slot; 2 slots; 2 slots. 30KHz SCS; 2 slots; 2 slots; 3 slots; 3 slots. 60KHz SCS; 3 slots; 3 slots; 4 slots; 4 slots. 120KHz SCS; 6 slots; 6 slots; 7 slots; 7 slots. - Option 4(Huawei): RAN4 to further discuss the condition, capability and impacts to measurement requirements for UE to use NCSG to control interruptions due to measurement on deactivated SCC or Scell in dormancy Recommendations for 2nd round: No further discussion. Can be FFS in the future meetings |
| Issue#2-4-2 | Measurement mode Tentative agreements: None Candidate options: - Option 1 (MTK): RAN4 shall discuss the NCSG measurement mode issue similar as legacy MG. - The existing measurement mode requirements (effective MGRP, data scheduling depends on gap configuration) can be the baseline. Recommendations for 2nd round: No further discussion. Can be FFS in the future meetings |
| Issue#2-5-1 | Per-UE or Per-FR capability support Tentative agreements:None Candidate options: - Option 1. (Apple, Ericsson, Qualcomm, Huawei , Nokia): both per UE and per FR NCSG for RRM measurement needs the specific UE capability. - Option 2 (MTK, OPPO, Intel): No additional NCSG capability for per-UE and per-FR differentiation is needed. - When UE only supports per-UE gap, the NCSG is per-UE. - When UE supports per-FR gap, the NCSG interruption is constrained in the FR. - Option 3 (Huawei, Qualcomm, vivo, CATT, LGE): related UE capability can be FFS Recommendations for 2nd round: No further discussion. Can be FFS in the future meetings |
| Issue#2-6-1 | Measurement requirements applicability Tentative agreements: None Candidate options: - Option 1 (Huawei) Further discuss the options for the two cases for measurement during ML - Case 1: UE only performs measurement that requires NCSG - Case 2: UE performs both measurement that requires NCSG and measurement that does not require NCSG or MG - Option 2 (Apple) : measurement in NCSG can be treated as measurement within gap when defining RRM requirements, such that the existing CSSF design can be reused. - Option 3(Ericsson) - In the first phase of the WI consider only the scenario in which the UE is configured only with NCSG pattern. - When configured to perform measurements only on serving carriers using NCSG then: - no scaling factor (CSSF) is needed to scale their measurement periods - their measurement period scales with/depends on VIRP - When configured to perform measurements only on non-serving carriers using NCSG then: - scaling factor (CSSF) is needed to scale their measurement periods - their measurement period further scales with/depends on VIRP - When configured to perform measurements on serving and non-serving carriers using NCSG then: - gap sharing parameter (X) is used for splitting NCSG gaps between serving and non-serving carriers. - scaling factor (CSSF) including X is needed to scale their measurement periods - their measurement period further scales with/depends on VIRP Option 4 (CATT, MTK) : Measurement requirement for NCSG is defined based on the assumption that only one layer can be monitored in one NCSG occasion Recommendations for 2nd round: No further discussion. Can be FFS in the future meetings |
| Issue#2-6-2 | RF combination limitation Tentative agreements: None Candidate options: Option 1. (MTK, Qualcomm, OPPO, Huawei, Nokia): UE is not expected to measure 2 inter-freq/RAT layers in parallel even if UE reports the support of NCSG to both corresponding bands Recommendations for 2nd round : No further discussion. Can be FFS in the future meetings |
| Issue#2-6-3 | Rx beam limitation Tentative agreements: Candidate options: - Option 1. (MTK): NW needs to be informed that the inter-frequency measurements with NCSG is CBM or IBM with serving cells in FR2. - Option 2(Apple) NCSG in FR2 should be deprioritized in current stage. Recommendations for 2nd round: No further discussion. Can be FFS in the future meetings |
| Issue#2-6-4 | scheduling and measurement restriction Tentative agreements: Candidate options: - Option 1 (Ericsson, MTK, Qualcomm, vivo,OPPO, Intel, LG, Nokia): When NCSG is configured then during the ML the existing scheduling restriction requirements defined in TS 38.133 shall also apply, Recommendations for 2nd round: No further discussion. Can be FFS in the future meetings |
| Issue#2-7-1 | How to consider the relation between NCSG and ‘NeedForGap’? Tentative agreements: Candidate options: - Option 1 (Intel, Apple): The “NeefForGap” signaling structure can be reused for NR NCSG as a start point - Option 1a (MTK, Intel): Rel-17 NCSG capability is reported on top of existing RAN2 ‘NeedForGap’ ignaling structure with a new component ‘NCSG’. - Option 1b (Huawei, Intel): Extend the Rel-16 NeedForGap ignaling for UE to indicate - Need for normal MG, or - No need for normal MG but NCSG, or - No need for either normal MG or NCSG - Option 2 (CMCC): Don’t reuse Rel-16 ‘NeedForGap’ ignaling for NCSG - Option 3(OPPO): NR NCSG signaling is introduced newly, independent from Rel-16 “Needforgap” signaling, e.g., NCSG, No-NCSG. - Option 4 (Ericsson, CATT, Intel): Let RAN2 decide NCSG signaling details and any relation between NCSG and ‘NeedForGap’ based on RAN4 technical input on NCSG pattern design - Option 5 (Nokia) FFS when the NCSG pattern design as well as NCSG applicability and UE capability support are finalized. Recommendations for 2nd round : No further discussion. Can be FFS in the future meetings. |
| Issue#2-8-1 | Which clause can be used to include NCSG pattern in 38.133 Tentative agreements: Decision of the specification section(s) for specifying the NCSG patterns is deferred until reasonable level of agreements on NCSG concept and patterns are agreed by RAN4 Candidate options: Recommendations for 2nd round: No further discussion. |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

[Moderator notes: all 2nd round discussions are moved to a dedicated Email thread for WF on the Email reflector.

]

#### Issue 2-1-1: NCSG Scenarios

Recommended WF:

- The use cases of NR NCSG can be

  - Eliminate/reduce interruption rate due to measurements on Pcell, PSCell, activated Scell, deactivated Scell, Scell with dormant BWP or unused RF chain

  - FFS on

    - intra-frequency measurements with MG,

    - inter-frequency measurements with MG,

    - inter-RAT measurements,

| Company | Comments |
| --- | --- |
| Company A |  |

#### Issue 2-3-1  General NCSG design principle

Recommended WF:

- The general NCSG design principle :

  - Option 1. All 26 NCSG patterns in Rel16

  - Option 2 Reuse subset of the legacy MG patterns in [TS38.133 v16.6.0] as the new NCSG patterns in NR.

    - FFS on which subsets shall be reused for the new NCSG

| Company | Comments |
| --- | --- |
| Company A |  |

#### Issue 2-3-3  Measurement length (ML)

Recommended WF:

- ML can be derived from “MGL-VIL1-VIL2” If legacy gap pattern can be reused for NCSG

| Company | Comments |
| --- | --- |
| Company A |  |

## Summary on 2nd round

| CR/TP/LS/WF number | T-doc status update recommendation |
| --- | --- |
| R4-2105792 (WF) | Agreeable |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on R17 NR MG enhancements – Pre-configured MG | Intel |  |
| WF on R17 NR MG enhancements – NCSG | Intel |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2105791 | WF on R17 NR MG enhancements – Pre-configured MG | Intel | Agreeable |  |
| R4-2105792 | WF on R17 NR MG enhancements – NCSG | Intel | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents