3GPP TSG-RAN WG4 Meeting # 98-bis-e 												     R4-2106137

Electronic Meeting, 12th – 20th April, 2021

Agenda item:			5.3.1

Source:	Moderator (CATT)

Title:	Email discussion summary for [98-bis-e][303] NR_IAB_RF_Maintenance

Document for:	Information

Introduction

This email thread is to discuss the issues for R16 NR IAB RF maintenance. The main open issue is IAB-MT EVM procedure and there’re several other maintenance CRs submitted.

The targets of the email discussion for 1st round and 2nd round are as follows,

- 1st round: Discuss the open issues for IAB-MT EVM procedure and try to reach agreements. Collect comments for the maintenance CRs.

- 2nd round: continue the discussion and agree the CRs.

Topic #1: IAB-MT EVM measurement procedure

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2104781 | CATT | Observation 1: BS EVM equalizer approach may be reused by IAB-MT for both FR1 and FR2. Observation 2: Rewriting B.7 and C.7 is clearer. |
| R4-2106667 | Nokia, Nokia Shanghai Bell | Observation 1: Both UE and BS ways of estimation of Tx chain amplitude and frequency response could be allowed. Proposal: Enable re-use of BS test equipment for EVM testing. |
| R4-2107046 | Keysight Technologies UK Ltd | Observation 1 - EVM measurement capability available today in test equipment are for Uplink signal with using definition from UE (TS38.521) conformance test specification, not opposite. - Some of proposed measurement capability in previous RAN4 meeting doesn’t exist today and may not be available at the time needed, which is to measure Uplink signal EVM using definition from BS (TS38.141) conformance specification Observation 2 - Using existing test setup, it is simply to choose which Uplink or Downlink signal to make EVM measurement on measurement equipment, if use of existing EVM measurement capability is agreed. Which is to use existing Uplink signal EVM measurement for IAB-MT EVM measurement. Observation 3 - It doesn’t make sense for TE vender to add IAB-MT specific EVM measurement, while there are already EVM measurement capability available for Uplink signal and Downlink signal following existing conformance test specification respectively. And there are not much of measured result difference expected. Observation 4 - While WF [2] show no observation (on using BS TS38.104 definition) for some of items for annex text, there are some dependency with equalizer definition and other items for IAB-MT. Proposal 1 - Use of existing equalizer definition from TS38.521 (Uplink signal EVM measurement) for IAB-MT EVM measurement Proposal 2 - For specification annex text, re-use some of text from TS38.521 annex for IAB-MT rather from TS38.104, which is, “E.3 Signal Processing” in place of B.5.2/C.5.2 “window length” and B.6/C.6 “Estimation of Tx chain amplitude and frequency response parameters” |
| R4-2107225 | Ericsson | Observation-1: The UE equalizer coefficient is calculated with both data and reference data and average per time slot and per allocated subcarrier. Observation-2: The BS equalizer coefficient is the same zero-forcing equalizer as UE equalizer. The derivation of the equalizer is different, BS only use reference signal and optimize the phase and amplitude response selectively and UE use both data and reference signal and use the least square estimation. Proposal-1: Apply both BS EVM procedure or UE EVM procedure with CP-OFDM only to IAB-MT EVM procedure. Proposal-2: Use the declaration D.106 to support the optional configuration of the PTRS signal in test model. |

Open issues summary

Sub-topic 1-1

Issue 1-1: Which EVM equalizer generation method should be used by IAB-MT, BS or UE?

- Proposals

  - Option 1: BS method (CATT)

  - Option 2: UE method (Keysight)

  - Option 3: Both BS and UE method should be allowed (Nokia, Ericsson)

- Recommended WF

  - To be discussed

1st round comments:

| Company | Comments |
| --- | --- |
| Ericsson | Option 3, For proponent of option 2, what is the EVM impact of PT-RS signal if only the UE EVM procedure is used. |
| Qualcomm | Option 2. The method simplest to implement for the test equipment should be chosen. Based on the inputs, this seems to be option 2. Option 3 would also be fine if TE vendors can confirm it is easy to implement |
| ZTE | Fine with option 3 |
| Huawei | Option 3 seems to allow for option 1 and 2 so should satisfy the test equipment concerns. If the result is the same as indicyted then option 3 is best. |
| Nokia, Nokia Shanghai Bell | Option 3 is our preference, as there is technically minor difference between the UE and BS measurement procedures. We are also ok with option 1. |
| CATT | If option 3 is the conclusion, it seems TE vendors’ concern is not solved because TE vendors need to implement both BS and UE method. |
| Keysight | Option 2. Yes, opt2 is easier, and we strongly recommend RAN4 to choose this. From TE vender point of view, there is no measurement instrument with EVM functionality for BS EVM method on Uplink signal. It doesn’t make sense to make this topic as flexible, actually confusing. And Agree with CATT’s comment, with Option 3, TE venders might be needed to do both method for for IAB-MT for very similar result expected. It doesn’t make sense for us. For Ericsson, UE method has defined already for Uplink without PT-RS but looks at data for equalization, and already been used for UE EVM measurement, shouldn’t be any problem. |

------------------GTW Discussion-----------------:

Keysight: For uplink EVM measurement, TE use UE method, no implementation on BS method for uplink. Also no much difference from performance aspect with two methods.

E///: For FR2 test method, PTRS configured and this impact EVM equalizer which impact EVM procedure. How to ensure PTRS used?

Nokia: We already have agreed WF stating BS approach used except averaging EVM. We prefer to enable either both or only BS approach.

Keysight: There is different among BS and UE for EVM equalizer. TE vendors need to implement considering the difference. NO issue in the market with UE method.

E///:  Not only EVM measurement procedure, also test model. The changes probably always can’t be avoided.

Keysight: Why we need to use BS approach for uplink EVM measurement? This will be a specific IAB-MT test specific implementation with additional effort from TE vendors.

E///: We have BS specific procedure i.e. PTRS, we want to reuse BS approach as much as possible.

Tentative Agreement: Both BS and UE method should be allowed in the specification.

Issue 1-2: How to adapt “Averaged EVM” clause to IAB-MT

- Proposals

  - Option 1: Copy the whole content from TS 38.141 and make the necessary modifications as the approach in CR R4-2104782.

  - Option 2: Refer BS spec TS 38.141 as the approach in CR R4-2107226.

- Recommended WF

  - To be discussed

1st round comments:

| Company | Comments |
| --- | --- |
| Ericsson | This connect to the Issue 1-1 and it may be too early to discuss the CR. If we keep the previous WF, the discussion point is that if it is more future prove to general reference way with possible modification of the “changing downlink signal to uplink signal”. |
| Qualcomm | If option 2 is chosen in Issue 1-2, shouldn’t text from 38.521 be reused? |
| ZTE | Both options are fine for us, maybe option1 has better clarifications on its details, however it should also keep consistent with Issue 1-1; |
| Huawei | If modifications/clarifications are required then maybe option 1 is better but its ok to reference as long as meaning is not lost. |
| Nokia, Nokia Shanghai Bell | We prefer option 1. As the downlink and uplink imply opposite behaviours for IAB-MT and BS, it is cleanest to write the text out to avoid any possible misunderstanding when it comes to details in the formulas. Additionally, if this is the only aspect to be modified, the scope of the modifications could be documented in the TR. |
| CATT | Option 1 and agree the TP should align with Issue 1-2. |

Issue 1-3: Proposal 2 in R4-2107046

- Proposals

  - Option 1: For specification annex text, re-use some of text from TS38.521 annex for IAB-MT rather from TS38.104, which is, “E.3 Signal Processing” in place of B.5.2/C.5.2  “window length” and B.6/C.6 “Estimation of Tx chain amplitude and frequency response parameters”

  - Option 2: Keep the agreements in last meeting for the parts other than equalizer.

- Recommended WF

  - To be discussed

1st round comments:

| Company | Comments |
| --- | --- |
| Ericsson | This connect to the Issue 1-1. When the UE EVM procedure is used, could the whole section can be referred or only selected section indicated in Option 1? |
| ZTE | Please see the comments on Issue 1-1. |
| Huawei | Obviously depends on agreement n 1-1, but if option 3 is used then both methods need to be reused or referenced. |
| Nokia, Nokia Shanghai Bell | We prefer option 2. |
| CATT | We can discuss together with Issue 1-1. |
| Keysight | This topic is related with issue 1-1. Depending discussion on Issue 1-1, then decide what to do with text. |

Issue 1-4: Proposal 2 in R4-2107225

Proposal-2: Use the declaration D.106 to support the optional configuration of the PTRS signal in test model.

Moderator: TP R4-2106318 proposed the whole set of declarations, so moderator suggests discussing this issue in thread [305].

- Recommended WF

  - It’ll be discussed together with other declarations in R4-2106318 in thread [305].

1st round comments:

| Company | Comments |
| --- | --- |
| Ericsson | There is no PTRS signal declaration in [305], then this may be the only place to discuss though it is not related to maintenance. There is concern of the optional meaning of PTRS signal and this is our opinion to explain it. |
| Qualcomm | Option 1 is preferable since it will simplify implementation. |
| ZTE | Fine with that. |
| Nokia, Nokia Shanghai Bell | We are fine to discuss this in 305, but we add our view here also. D.106 is demodulation performance related declaration aimed towards declaring the capabilities of PUSCH receiver. It is not good to mix neither RF declarations and demod declarations nor PUSCH transmitter and receiver declarations. For BS a declaration about PT-RS on transmitter side is not used so it is still unclear to us why declaration would be needed for IAB-MT? |
| CATT | Sorry for the misunderstanding as a moderator. Let’s discuss it in this thread if it’s needed. PTRS for IAB-MT is optional, so declaration may be needed? Don’t understand the comments from Nokia. In R4-2106318, D.106 applies to both IAB-DU and IAB-MT. For IAB-MT, my understanding is that it should be transmitter’s declarations, not DEMOD declarations. Although PTRS support is related to RF test, but it’s physical layer signals not just pure RF implementations. Still think the number should align with R4-2106318? |

Companies views’ collection for 1st round

Open issues

Please comment in the tables below the issues in 1.2.

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2104782, Draft CR for TS 38.174: IAB-MT EVM measurement, CATT | Ericsson: This connect to the issue 1-1 and need to wait the outcome of the it. the discussion point is that if it is more future prove to general reference way with possible modification of the “changing downlink signal to uplink signal”. |
|  | Nokia, Nokia Shanghai Bell: The part for FR1 includes FDD part though we do not have any FDD band. This would need to be revised in case also UE approach is supported, based on outcome of sub-topic 1-1 |
| R4-2104783, Draft CR for TR 38.809: IAB-MT EVM measurement CATT | Ericsson: This connect to the issue 1-1 and need to wait the outcome of the it. |
|  | Nokia, Nokia Shanghai Bell: ok as such but may need to be revised based on outcome of sub-topic 1-1 |
| R4-2107226, CR on Tx signal quality requirements, Ericsson | Huawei: Clearly linked to CR R4-2104782 and topic 1-1, wait for outcome of topic 1-1 (and merge with CATT CR) |
|  | Nokia, Nokia Shanghai Bell: As commented in issue 1-2, it is better to write out the sections needing any changes e.g. changes between UL and DL to avoid any misunderstandings with the detailed formulas. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1: Which EVM equalizer generation method should be used by IAB-MT, BS or UE? | Based on the GTW discussion, the tentative agreement is as following, Tentative agreements: Both BS and UE method should be allowed in the specification. Recommendations for 2nd round: Confirm the above tentative agreements. |
| Issue 1-2: How to adapt “Averaged EVM” clause to IAB-MT | Based on the email discussion, some companies prefer writing some companies prefer referring. In order to avoid misunderstanding of the spec, moderator suggests the following tentative agreements: Tentative agreements: For IAB-MT EVM measurement procedure, use two different approaches for BS method and UE method as followings, BS approach: Copy the whole content from TS 38.141 and make the necessary modifications. The spec drafting is discussed in the TP. UE approach: Refer TS 38.521. Recommendations for 2nd round: Confirm the above tentative agreements. |
| Issue 1-3: Proposal 2 in R4-2107046 | Based on the discussion in Issue 1-1, moderator’s understanding is that the whole UE procedure can be reused by IAB-MT as a choice although Issue 1-1 is only about EVM equalizer. Tentative agreements: Refer TS 38.521 for IAB-MT EVM measurement when UE procedure is used. The spec drafting is discussed in the TP. Recommendations for 2nd round: Confirm the above tentative agreements. |
| Issue 1-4: Proposal 2 in R4-2107225 | Based on the email discussion, PTRS declaration for IAB-MT is needed, the exact number can be discussed in the TP for TS 38.176-2. Tentative agreements: PTRS declaration for IAB-MT is needed, the exact number can be discussed in the TP for TS 38.176-2. Recommendations for 2nd round: Confirm the above tentative agreements. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2104782, Draft CR for TS 38.174: IAB-MT EVM measurement, CATT | To be revised. |
| R4-2104783, Draft CR for TR 38.809: IAB-MT EVM measurement CATT | To be revised. |
| R4-2107226, CR on Tx signal quality requirements, Ericsson | To be merged with the revision of R4-2104782. |

Discussion on 2nd round (if applicable)

| Issues | Comments |
| --- | --- |
| Issue 1-1: Which EVM equalizer generation method should be used by IAB-MT, BS or UE? | Tentative agreements: Both BS and UE method should be allowed in the specification. Comments in 2nd round: Ericsson: Ok with tentative agreements. Keysight: OK with tentative agreement, still recommend starting with UE method to start with EVM measurement simply because it is available today. It’s up to each TE vender to implement BS method on Uplink EVM. |
| Issue 1-2: How to adapt “Averaged EVM” clause to IAB-MT | Tentative agreements: For IAB-MT EVM measurement procedure, use two different approaches for BS method and UE method as followings, BS approach: Copy the whole content from TS 38.141 and make the necessary modifications. The spec drafting is discussed in the TP. UE approach: Refer TS 38.521. Comments in 2nd round: Ericsson: Ok with tentative agreements. Keysight: OK with tentative agreement, like to have more time to careful check on relation with other part of text, like to come back on next meeting on reviewing TP from this meeting. |
| Issue 1-3: Proposal 2 in R4-2107046 | Tentative agreements: Refer TS 38.521 for IAB-MT EVM measurement when UE procedure is used. The spec drafting is discussed in the TP. Comments in 2nd round: Ericsson: Ok with tentative agreements. Keysight: OK with tentative agreement, like to have more time to careful check on TP for allowing both UE procedure and BS procedure, but not mixed together. Like to come back on next meeting. |
| Issue 1-4: Proposal 2 in R4-2107225 | Tentative agreements: PTRS declaration for IAB-MT is needed, the exact number can be discussed in the TP for TS 38.176-2. Comments in 2nd round: Ericsson: Ok with tentative agreements. Company B: |

Comments for the CR:

| CR/TP number | Comments |
| --- | --- |
| R4-2106040 Revision of R4-2104782, Draft CR for TS 38.174: IAB-MT EVM measurement, CATT | Ericsson:ok |
|  | Nokia: It seems we cannot directly refer to TS 38.104 or TS 38.521-1 for MT EVM measurement, as we need to cover both MT transmit in DL slots; the least effort would be to add a statement saying ‘the procedure should apply to DL slots’ in case MT transmit in DL slots. |
|  | CATT: Thanks for the comments. But we’re not sure the comments on how to handle IAB-MT transmits on DL slots. My understanding is that IAB-MT is allowed to transmit on DL slots, but IAB-MT still should support the EVM measurement on UL slots which is covered in current version. The extra measurement is not needed in the core spec. For the adaption from BS procedure, it also mainly includes from DL to UL. From that aspect, both procedures align each other. So I didn’t revise 4782 further. Nokia reply to CATT: Regarding comment on DL slots, in clause 9.7.3.4 of 38.174 we have: #### 9.7.3.4 Minimum requirement for Local Area IAB-MT type 2-O The OTA ACLR limit is specified in table 9.7.3.4-1. The OTA ACLR absolute limit is specified in table 9.7.3.4-2. The OTA ACLR (CACLR) absolute limit in table 9.7.3.4-2 or 9.7.3.4-5 or the ACLR (CACLR) limit in table 9.7.3.4-1, 9.7.3.4-3 or 9.7.3.4-4, whichever is less stringent, shall apply. Requirements specified for Local Area IAB-DU type 2-O in clause 9.7.3.3 shall apply to Local Area IAB-MT type 2-O during transmission in DL timeslot. That is why we made the comments, that should add some statement saying ‘the procedure should apply to DL slots’ in case MT transmit in DL slots |
| R4-2106041 Revision of R4-2104783, Draft CR for TR 38.809: IAB-MT EVM measurement, CATT | Ericsson: “ to have some flexibility” seems not a technical reason to allow two EVM testing method. PTRS signal not considered in UE EVM procedure and thus BS EVM will be used, on top of that , the two EVM method not deviate from performance perspective, maybe simple way is to remove the “flexibility” wording. |
|  | CATT: Thanks, it’s updated. |

Summary for 2nd round

| Issues | Status summary |
| --- | --- |
| Issue 1-1: Which EVM equalizer generation method should be used by IAB-MT, BS or UE? | Agreements: Both BS and UE method should be allowed in the specification. |
| Issue 1-2: How to adapt “Averaged EVM” clause to IAB-MT | Agreements: For IAB-MT EVM measurement procedure, use two different approaches for BS method and UE method as followings, BS approach: Copy the whole content from TS 38.141 and make the necessary modifications. The spec drafting is discussed in the TP. UE approach: Refer TS 38.521. |
| Issue 1-3: Proposal 2 in R4-2107046 | Agreements: Refer TS 38.521 for IAB-MT EVM measurement when UE procedure is used. The spec drafting is discussed in the TP. |
| Issue 1-4: Proposal 2 in R4-2107225 | Agreements: PTRS declaration for IAB-MT is needed, the exact number can be discussed in the TP for TS 38.176-2. |

There’s no WF for the above agreements. They’re captured in the chairman’s meeting minutes.

Topic #2: Maintenance CRs

Companies views’ collection for 1st round

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2104784, Draft CR for TS 38.174: Correction of IAB-DU class definition, CATT | Ericsson: adding MT on multiple Band is ok, but not the class definition as it is purposely to state the connect to BS deployment scenario |
|  | Qualcomm: agree with Ericsson |
|  | Huawei: For the class definitions I don’t agree, the IAB-DU does not communicate with UE’s it communicates with IAB-MT as such the IAB_DU to UE distance does not really make sense. The scenarios for an IAB-DU wide area are used in a wide area deployment of BS and hence it’s the BS to UE distance that (partially) defines that scenario. Agree the current version seems wrong and open to clarification but I don’t think the suggested modification is correct. |
|  | Nokia, Nokia Shanghai Bell: Other changes ok, but there seems to be one additional newline added towards the end of symbols section. This newline should not be implemented. |
|  | CATT: Let me think the class definition more. The other changes can be resubmitted in next meeting. |
| R4-2106306, Draft CR to TS 38.174 – corrections to general and transmitter part, Nokia, Nokia Shanghai Bell | Ericsson: CR should be issued on TS 38.174 version 16.2.0, donot know what is the difference between them. |
|  | Qualcomm: ok |
|  | Samsung: The “Note 2” for term of “IAB-DU channel bandwidth “ should be updated for IAB as well. Under sub-clause “9.5.2.1” it's suggested to update the as Aggregated IAB-DU Channel Bandwidth or Aggregated IAB-MT Channel Bandwidth to align order with other requirement. |
| R4-2106307, Draft CR to TS 38.174 – corrections to receiver part, Nokia, Nokia Shanghai Bell | Ericsson: CR should be issued on TS 38.174 version 16.2.0, donot know what is the difference between them. |
|  | Qualcomm:ok |
|  | Huawei: the IAB-MT channel Bandwidth correction in clause 7.5.6 is in the previous CR (6306) as well, it’s probably not confusing so its ok, if CR is updated though worth clearing up. |
| R4-2107227, CR on Tx Power related requirements, Ericsson | Qualcomm: why is this change needed? the dynamic range also has to do with how many RBs are scheduled, not just power control. |
|  | Huawei: power control is the next clause so this could be confusing, the opening paragraph is stating the name and its description, this addition seems unnecessary. |
|  | Nokia, Nokia Shanghai Bell: Not OK. Dynamic range should be separated from power control in core spec as wide area IAB-MT has no power control requirement. Seems likely that there is related discussion in performance part also. |
| R4-2107228, CR on Rx Charateristic other related requirements, Ericsson | Qualcomm: changes are ok, seems to make changes to the same sections as R4-2106307. |
|  | Company B |

Summary for 1st round

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2104784, Draft CR for TS 38.174: Correction of IAB-DU class definition, CATT | Not pursued |
| R4-2106306, Draft CR to TS 38.174 – corrections to general and transmitter part, Nokia, Nokia Shanghai Bell | to be revised |
| R4-2106307, Draft CR to TS 38.174 – corrections to receiver part, Nokia, Nokia Shanghai Bell | to be revised |
| R4-2107227, CR on Tx Power related requirements, Ericsson | Not pursued |
| R4-2107228, CR on Rx Charateristic other related requirements, Ericsson | agreeable |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

| CR/TP number | Comments |
| --- | --- |
| R4-2106042 Revision of R4-2106306, Draft CR to TS 38.174 – corrections to general and transmitter part, Nokia, Nokia Shanghai Bell | Nokia to Ericsson comment: 16.2.0 version was used, however that time was not officially released 16.2.0, thus 16.1.0 on the cover page (but should not be differences) |
|  | Company B |
| R4-2106043 Revision of R4-2106307, Draft CR to TS 38.174 – corrections to receiver part, Nokia, Nokia Shanghai Bell | Nokia: similar comment as above. |
|  | Company B |

Recommendations for Tdocs

1st round

New tdocs

No new tdocs for this thread. Please Mr. chair capture the agreements in the chairman notes when the discussion is closed. Thanks.

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2104781 | Discussion on open issues of IAB-MT EVM measurement | CATT | Noted |  |
| R4-2104782 | Draft CR for TS 38.174: IAB-MT EVM measurement | CATT | Revised |  |
| R4-2104783 | Draft CR for TR 38.809: IAB-MT EVM measurement | CATT | Revised |  |
| R4-2104784 | Draft CR for TS 38.174: Correction of IAB-DU class definition | CATT | Not pursued |  |
| R4-2106306 | Draft CR to TS 38.174 – corrections to general and transmitter part | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2106307 | Draft CR to TS 38.174 – corrections to receiver part | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2106667 | EVM measurement procedure considerations | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2107046 | IAB-MT conformance Test about EVM Equalizer | Keysight Technologies UK Ltd | Noted |  |
| R4-2107225 | IAB-MT EVM procedure | Ericsson | Noted |  |
| R4-2107226 | CR on Tx signal quality requirements | Ericsson | Merged to the revision of R4-2104782. |  |
| R4-2107227 | CR on Tx Power related requirements | Ericsson | Not pursued |  |
| R4-2107228 | CR on Rx Charateristic other related requirements | Ericsson | agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

The agreements in 1.6 will be captured in the chairman’s meeting minutes.

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2106040 | Draft CR for TS 38.174: IAB-MT EVM measurement | CATT | Not Pursued |  |
| R4-2106041 | Draft CR for TR 38.809: IAB-MT EVM measurement | CATT | Agreeable |  |
| R4-2106042 | Draft CR to TS 38.174 – corrections to general and transmitter part | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2106043 | Draft CR to TS 38.174 – corrections to receiver part | Nokia, Nokia Shanghai Bell | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents