3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106360

# Electronic Meeting, Apr. 12-20, 2021

Agenda Item:	8.8.4.2

Source:	MediaTek Inc.

Title:	Discussion on timing requirements in NTN

Document for:		Discussion

Introduction

In this paper, NTN RRM requirements related to UE Pre-compensation in time domain are discussed. In a RAN1 LS to RAN4 in [1], it was stated

As part of the WI NR-NTN-solution, under agenda item 8.4.2, RAN1 discussed the NTN UL synchronization requirements in terms of time alignment and frequency error for:

•	Initial access (i.e. PRACH transmission)

•	UL transmissions in RRC CONNECTED State.

RAN1 identified the following questions that need clarification from RAN4:

Question 1: What are the NTN UL time synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Question 2: What are the NTN UL frequency synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Note-1: The questions above are related to the requirements for signal reception at gNB side.

Note-2: From RAN1 side we would like to have the requirements such that they are including the ”total accumulated error” in both time and frequency domain.

Simulation result of UE Pre-compensation accuracy with serving satellite PV ephemeris and orbital ephemeris

This section provides simulation result of UE Pre-compensation accuracy with GNSS-acquired device location and serving satellite ephemeris, including PV ephemeris and orbital ephemeris. The details of this method are provided in ANNEX A and ANNEX B (based on gravity) in [2].

Note for both ephemeris formats, it is not necessary to include the epoch time if it is implicitly known as a reference time linked to DL subframe where NTN SIB is broadcast.

The orbital parameters can be transformed to state vector position and velocity at epoch time t0 followed by propagation of satellite position and velocity to time t; or the orbital parameters can be propagated to time t followed by transform to state vectors position and velocity at time t. This is a choice of implementation in the UE that depends on accuracy and complexity of the propagation methods for UE pre-compensation for UL synchronization.

The simulation result for LEO-600 km Set 1 is provided in the Figure 1, and also in Table 1 and Table 2 for PV ephemeris and orbital ephemeris, respectively. With prediction time of 10 s ahead for pre-compensation, the delay error of UE pre-compensation to counter the delay experienced on the service link is accurate within 0.012 us (i.e., < 3% of the ±0.39us Te budget) for position and velocity ephemeris format; and accurate within 0.012 us (3% of ±0.39us Te budget) for the orbital format.

Observation 1: UL timing error contributed by UE pre-compensate satellite delay can be within 3% error budget of ±Te, with the prediction time up to 10 s ahead for pre-compensation.

Figure 2: Prediction of delay error and frequency error over service link

Table 1: Simulations of accuracy of propagation based on PV ephemeris

| Prediction time ahead for pre-compensation | Delay error (us) | percentage of the Te error budget ± 0.39 us |
| --- | --- | --- |
| 10s | 0.012 us | <3% |
| 30s | 0.037 us | 9% |

Table 2: Simulations of accuracy of propagation based on orbital ephemeris

| Prediction time ahead for pre-compensation | Delay error (us) | percentage of the Te error budget ± 0.39 us |
| --- | --- | --- |
| 10s | 0.012 us | < 3 % |
| 30s | 0.040 us | 10 % |

NTN UE UL timing requirements

UE specific TA estimation accuracy requirement

  - RAN4 is to define the requirements for UE specific TA estimation accuracy

  - FFS on how to capture the UE specific TA estimation accuracy

    - Option 1: the UE specific TA estimation accuracy is counted into the UE transmit timing error requirement

    - Option 2: the UE specific TA estimation accuracy is counted into the timing advance adjustment accuracy requirement

    - Option 3: the UE specific TA estimation accuracy is defined as a separate accuracy requirement

    - Other option is not precluded

This is to address the open issue on NTN UE UL timing requirements in the WF on NTN RRM timing related requirements [1]. Three options have been discussed as below.

RAN1 Agreements on “UL time synchronization”

An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris..

An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

- FFS: Operation of closed loop and open loop TA control.

For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN.

- FFS: Details of the combination of open and closed loop TA control

One the WF on NTN RRM timing related requirements, RAN1 has already agreed in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE the following, as shown below in the status report in RAN 90e [3] and RAN 91e [4].

Observation 2: RAN1 already agreed on an NR NTN UE shall be capable of at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE states.

RAN1 is discussing the UL-DL subframe timing alignment as follows:

When common TA is indicated by the Network:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA and NTA,offset are defined as in Release-16.

- NTA,UE-specific  is UE self-estimated TA

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

UE autonomously predict and adjusts the TA based on ephemeris and GNSS-acquired UE position.

In option 1, the error in applying NTA,offset + NTA-UE specific + NTA,common for RACH preamble transmission in initial access is included in the requirement for the specified maximum transmission timing error ±Te = ± 0.39 μs (=12*64*Tc). As discussed in Section 2, based on the observation 1, the accuracy of the UE prediction and pre-compensation of satellite delay is sufficiently accurate within 3% error budget of ±Te, with the prediction time up to 10 s ahead for pre-compensation, and it will have limited impact on the requirement.

Observation 3:  the error in applying NTA,offset + NTA-UE specific+NTA,common should be accommodated in the requirement for the specified maximum transmission timing error ±Te = ± 0.39 μs (=12*64*Tc).

In Option 2, the requirement would only be applied to a signalled TA command. However, the delay drift over the RTD of the service link can be also accurately predicted and pre-compensated by the UE without the need for TA command signalling, therefore the UE-specific TA error should not be included in the timing advance adjustment accuracy requirement.

Observation 4:  The delay drift over the RTD of the service link can be accurately predicted and pre-compensated by the UE, therefore it can also be accommodated in the transmission timing error requirement (i.e. Te).

Proposal 1: TA command for the delay drift over the RTD of the service link is not needed.

In option 3, it is considered that a separate requirements can be defined for UE self-estimating accuracy of NTA. This is un-necessary change in the specifications based on UE prediction simulation results presented in Section 2. The core requirement for UE self-estimating accuracy of NTA as discussed in Options 1 can re-use the requirement on the UE transmit timing error (Te), as specified in 7.1.2 in TS 38.133.

- 7.1.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission..

The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA for PRACH is defined as 0.

Observation 5: A separate requirements to define the UE self-estimating accuracy of NTA will un-necessarily require specification effort. The core requirement for UE self-estimating accuracy of NTA as discussed in Options 1 can re-use the requirement on the UE transmit timing error (Te), as specified in 7.1.2 in TS 38.133, as discussed above.

In our view, the UE self-estimating accuracy of NTA for the service link can be defined as the way as the UE transmit timing error (Te), and it will apply for the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission.

Proposal 2: NTN UL time synchronization requirements for the service link can be defined as the way of the UE transmit timing error ±Te = ± 0.39 μs (=12*64*Tc), as specified in 7.1.2 in TS 38.133, for PRACH transmission and the first transmission in a DRX cycle for PUCCH, PUSCH and SRS

Conclusion

In this contribution, we summarize issues and discuss impact on specifications for solutions for UL time and frequency synchronization.

Observation 1: UL timing error contributed by UE pre-compensate satellite delay can be within 3% error budget of ±Te, with the prediction time up to 10 s ahead for pre-compensation.

Observation 2: RAN1 already agreed on an NR NTN UE shall be capable of at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE states.

Observation 3:  the error in applying NTA,offset + NTA-UE specific+NTA,common should be accommodated in the requirement for the specified maximum transmission timing error ±Te = ± 0.39 μs (=12*64*Tc).

Observation 4:  The delay drift over the RTD of the service link can be accurately predicted and pre-compensated by the UE, therefore it can also be accommodated in the transmission timing error requirement (i.e. Te).

Proposal 1: TA command for the delay drift over the RTD of the service link is not needed.

Observation 5: A separate requirements to define the UE self-estimating accuracy of NTA will un-necessarily require specification effort. The core requirement for UE self-estimating accuracy of NTA as discussed in Options 1 can re-use the requirement on the UE transmit timing error (Te), as specified in 7.1.2 in TS 38.133, as discussed above.

Proposal 2: NTN UL time synchronization requirements for the service link can be defined as the way of the UE transmit timing error ±Te = ± 0.39 μs (=12*64*Tc), as specified in 7.1.2 in TS 38.133, for PRACH transmission and the first transmission in a DRX cycle for PUCCH, PUSCH and SRS.

References

- R1-2102263, LS on NTN UL time and frequency synchronization requirements.

- R4-2100780, Discussion on UE Pre-compensation for UL synchronization for in NTN, Mediatek.

- RP-20257, NR NTN Status Report to RP#90e, December 2020

- RP-210128, NR NTN Status Report to RP#91e, March 2021