3GPP TSG-RAN WG4 Meeting # 98bis-e 	R4-2106458

# Electronic Meeting, 12th–20th, April, 2021

Agenda item:      5.6.1

Source:                Intel Corporation

Title:                    Discussion of remaining issues about RRM enhancement in Rel-16

Document for:    Discussion

# Introduction

In RAN4#97 meeting, there is one open issue for RRC based BWP switching[1]:

- Simultaneous case or simultaneous + partial - Sub1: For RRC based BWP switching - Postpone defining testcases with RRC based simultaneous BWP switch and clarify the scenario first.

In RAN4#98e meeting, there is reply LS from RAN2[2]. By the meanwhile, there is another open issue for UL spatial relation switch[3]:

Delay requirements for MAC–CE based UL spatial relation switch for PUCCH - Option 1: Refer to section 8.14 for additional delay due to PL-RS switch in UL spatial relation switch - Option 1a: Refer to section 8.14 for additional delay due to PL-RS switch in UL spatial relation switch for known PL-RS - Option 2: Longer delay is expected when PL-RS and DL-RS in the UL spatial relation are changed simultaneously

In this contribution, we will continue to discuss the remaining issues.

# RRC based BWP switching for single CC

The replies from RAN2[2] are as follows:

- On whether RRC reconfiguration can change any parameter of the already active BWP of an activated SCell or SpCell. Response: According to the RAN2 specification the NW can change any parameters of an already active BWP of an SPCell or an SCell with the exception that the modification of firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id for an SCell is not allowed (for both Rel-15 and Rel-16). The modification of firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id for an SCell is allowed by first releasing and subsequently adding the SCell, and this is not considered by RAN2 as a BWP switch. - On whether the RRC reconfiguration without modification of firstActiveDownlinkBWP-Id or firstActiveUplinkBWP-Id for an activated SCell or SpCell can trigger a BWP switch. Response: RAN2 confirms that an RRC message with a firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id that is different from the UE’s current BWP ID, results in a BWP switch for an SpCell. And in Rel-15 or in Rel-16, the BWP switching for SCell using RRC message is not be possible. According to RAN2 specifications, the reconfiguration of any parameters (with the exception of firstActiveDownlinkBWP-Id or firstActiveUplinkBWP-Id as mentioned earlier) of an already active BWP of an SPCell or an Scell is allowed for both Rel-15 and Rel-16. However, RAN2 has not specified whether this is a BWP switch or not, i.e. it would require further discussion on whether particular parameter change would cause BWP switch or not and RAN2 does not intend to discuss this. Furthermore, RAN2 would like to inform RAN4 that during the RRCResume/RRCSetup procedure for the PCell, the active BWP parameters change for the UE or the BWP can be switched.

For question 1, it’s clear that RRC reconfiguration didn’t include the change of firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id  for an SCell.

For question 2, RAN2 reply that in Rel-15 or in Rel-16, the BWP switching for SCell using RRC message is not be possible. For the change of other parameter, it’s not specified in RAN2 whether it’s a BWP switch or not and they will not continue to discuss it.

Observation 1: From the reply LS of RAN2, in Rel-15 or in Rel-16, the BWP switching for SCell using RRC message is not be possible.

Observation 2: From the reply LS of RAN2, for the reconfiguration of any parameters (with the exception of firstActiveDownlinkBWP-Id or firstActiveUplinkBWP-Id), RAN2 has not specified whether this is a BWP switch or not.

Since it’s not clearly specified that the reconfiguration of any parameters (with the exception of firstActiveDownlinkBWP-Id or firstActiveUplinkBWP-Id) is BWP switch in RAN2, we suggest to limit the current RRC based BWP switching applicability for SpCell only in RAN4.

Proposal 1: Clarify that requirement for RRC based BWP switching on a single CC apply for SpCell only.

The modification in the spec will be:

### 8.6.3 RRC based BWP switch delay The requirements in this clause apply to the case that the BWP switch is performed on a single CC with one or more than one BWP configuration(s) configured for SpCell only.

# RRC based BWP switching for multiple CCs

If RRC based BWP switching is not applicable for SCell, there is no scenario for RRC based simultaneous BWP switching on multiple CCs.

Proposal 2: There is no scenario for RRC based simultaneous BWP switching on multiple CCs. Don’t need to design test case for the scenario.

For non-simultaneous case, since the spec already outlines that the requirement apply for NR-DC(FR1+FR2) case, we can further clarify that the requirement apply if there is only one CC in either FR1 or FR2 cell group.

#### 8.6.3A.2 Non-simultaneous RRC based BWP switch delay on multiple CCs In non-simultaneous case, the RRC-based BWP switch on multiple CCs is triggered over partially overlapping time period n different Cell groups. The delay requirements in this clause apply only if: BWP switching on multiple CCs in different cell groups are triggered by separate RRC commands, and UE is operating in NR-DC (FR1+FR2), and UE is capable of per-FR gap, and BWP switch does not involve SCS change. There is only one CC in either PCell or PSCell.

The previous delay time for non-simultaneous RRC based BWP switch on multiple CC is:

TWaiting+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRC*(M-1)NR slot length

if M=1, then the delay time can be further modified to:

TWaiting+TRRCprocessingDelay+TBWPswitchDelayRRCNR slot length

We provide corresponding CR in[3].

Proposal 3: For non-simultaneous RRC based multiple BWP switching case, clarify that the requirement apply if there is only one CC in either PCell or PSCell.

Proposal 4: Delay time for non-simultaneous RRC based BWP switch on multiple CC will be updated to:

TWaiting+TRRCprocessingDelay+TBWPswitchDelayRRCNR slot length

# Delay requirement for UL spatial relation switch

In last meeting, there is one open issue for UL spatial relation switch[4]:

Delay requirements for MAC–CE based UL spatial relation switch for PUCCH - Option 1: Refer to section 8.14 for additional delay due to PL-RS switch in UL spatial relation switch - Option 1a: Refer to section 8.14 for additional delay due to PL-RS switch in UL spatial relation switch for known PL-RS - Option 2: Longer delay is expected when PL-RS and DL-RS in the UL spatial relation are changed simultaneously

Delay requirement for PL-RS with known condition is specified in 8.14:

#### 8.14.3 MAC-CE based pathloss reference signal switch delay The requirements in this clause apply for a UE to update a pathloss reference signal by MAC-CE for PUCCH, PUSCH, semi-persistent SRS and aperiodic SRS. If the target pathloss reference signal is known, upon receiving PDSCH carrying MAC-CE activation in slot n, UE shall be able to apply the target pathloss reference signal of the serving cell on which pathloss reference signal switch occurs no later than the slot n + THARQ+ 3ms + 5*Ttarget_PL-RS + 2msNR slot length. The UE shall be able to apply old pathloss reference signals until the slot n + THARQ+ 3Nslotsubframe,µ. Where - THARQ is the timing between pathloss reference MAC-CE activation command and acknowledgement as specified in TS 38.321 [7]. - Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB or NZP CSI-RS.

PL-RS and DL-RS are in the same TCI chain to ensure the correct calculation of Tx power. it’s better to consider the PL-RS delay when PL-RS and UL spatial relation switches. For UL spatial relation switch, except for THARQ+MAC CE processing time, there is no extra processing time. Don’t need to discuss simultaneous or sequential processing. Therefore, we suggest to consider the extra delay introduced by PL-RS processing, i.e. 5*Ttarget_PL-RS + 2ms.

Therefore, we support option 1a.

Proposal 5: For Delay requirements for MAC–CE based UL spatial relation switch for PUCCH, support option 1a.

# Conclusion

In this paper we provide our views on the remaining open issues related to BWP switching on multiple CCs. Our proposals are captured below:

Observation 1: From the reply LS of RAN2, in Rel-15 or in Rel-16, the BWP switching for SCell using RRC message is not be possible.

Observation 2: From the reply LS of RAN2, for the reconfiguration of any parameters (with the exception of firstActiveDownlinkBWP-Id or firstActiveUplinkBWP-Id), RAN2 has not specified whether this is a BWP switch or not.

Proposal 1: Clarify that requirement for RRC based BWP switching on a single CC apply for SpCell only.

Proposal 2: There is no scenario for RRC based simultaneous BWP switching on multiple CCs. Don’t need to design test case for the scenario.

Proposal 3: For non-simultaneous RRC based multiple BWP switching case, clarify that the requirement apply if there is only one CC in either PCell or PSCell.

Proposal 4: Delay time for non-simultaneous RRC based BWP switch on multiple CC will be updated to:

TWaiting+TRRCprocessingDelay+TBWPswitchDelayRRCNR slot length

Proposal 5: For Delay requirements for MAC–CE based UL spatial relation switch for PUCCH, support option 1a.

# Reference

[1] R4-2017362, WF on NR RRM enhancements - BWP switching on multiple CCs and UL spatial info switch, Intel

[2] R2-2102476, LS Reply on RRC based BWP switch

[3] R4-2106460, CR on RRC based BWP switching on multiple CCs , Intel

[4] R4-2103619, WF on R16 RRM enhancement part 1 – BWP switching, UL spatial relation switch, Intel