3GPP TSG-RAN WG4 Meeting # 98-bis-e 	R4-2106477

# Electronic Meeting, Apr. 12-20, 2021

Source:	Huawei, HiSilicon

Title:	MU for unlicensed band n46 and n96

Agenda item:	5.1.1.1

Document for:	Approval

1 Introduction

In previous meetings the conformance testing for NR-U has been discussed. One of the remaining issues is measurement uncertainties. There are two options from the discussion.

  - Option 1:  propose to extend NR upper frequency from 6GHz to 7.125GHz

  - Option 2: reuse LAA MU requirements for n46 and n96

In this contribution, we provide our consideration on the options.

2 Discussion

The MU defined 38.141-1 and 36.141 are summarized in following Table 2-1 and Table 2-2 for transmitter and receiver respectively.

Table 2-1: Maximum Test System uncertainty for transmitter tests

| Clause | Maximum Test System Uncertainty defined in 38.141-1 | Maximum Test System Uncertainty defined in 36.141 |
| --- | --- | --- |
| 6.2 Base Station output power | ±0.7 dB, f ≤ 3 GHz | ±0.7 dB, f ≤ 3.0GHz |
|  | ±1.0 dB, 3 GHz < f ≤ 6 GHz (Note) | ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6.0GHz. |
| 6.3 Output power dynamics | ± 0.4 dB | ± 0.4 dB |
| 6.4.1 Transmit OFF power | ±2.0 dB , f ≤ 3 GHz | ±2.0 dB, f ≤ 3.0GHz |
|  | ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) | ±2.5 dB, 3.0GHz < f ≤ 4.2GHz ±3 dB, 4.2GHz < f ≤ 6.0GHz |
| 6.4.2 Transmitter transient period | N/A |  |
| 6.5.2 Frequency error | ± 12 Hz | ± 12 Hz |
| 6.5.3 EVM | ± 1% | ± 1 % |
| 6.5.4 Time alignment error | ± 25ns | ± 25 ns |
| 6.6.2 Occupied bandwidth | 5 MHz, 10 MHz BS Channel BW: ±100 kHz | 5MHz, 10MHz Channel BW: 100kHz |
|  | 15 MHz, 20 MHz, 25 MHz, 30 MHz, 40 MHz, 50 MHz BS Channel BW: ±300 kHz | 15MHz, ≥20MHz: Channel BW: 300kHz |
|  | 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz BS Channel BW: ±600 kHz |  |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | ACLR/ CACLR | ACLR ±0.8 dB |
|  | BW ≤ 20MHz: ±0.8 dB | Absolute power ±2.0 dB, f ≤ 3.0GHz |
|  | BW > 20MHz: ±1.2 dB | Absolute power ±2.5 dB, 3.0GHz < f ≤ 4.2GHz |
|  |  | Absolute power ±3.0 dB, 4.2GHz < f ≤ 6.0GHz |
|  | Absolute power ±2.0 dB, f ≤ 3 GHz | CACLR±0.8 dB |
|  | Absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) | Absolute power ±2.0 dB, f ≤ 3.0GHz |
|  |  | Absolute power ±2.5 dB, 3.0GHz < f ≤ 4.2GHz |
|  | CACLR | Absolute power ±3.0 dB, 4.2GHz < f ≤ 6.0GHz |
|  | BW ≤ 20MHz: ±0.8 dB |  |
|  | BW > 20MHz: ±1.2 dB |  |
|  | CACLR absolute power ±2.0 dB , f ≤ 3 GHz |  |
|  | CACLR absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) |  |
| 6.6.4 Operating band unwanted emissions | ±1.5 dB, f ≤ 3 GHz | ±1.5 dB, f ≤ 3.0GHz |
|  | ±1.8 dB, 3 GHz < f ≤ 6 GHz (Note) | ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.2 dB, 4.2GHz < f ≤ 6.0GHz |
| 6.6.5.5.1.1 Transmitter spurious emissions, Mandatory Requirements | 9 kHz < f ≤ 4 GHz: ±2.0 dB |  |
|  | 4 GHz < f ≤ 19 GHz: ±4.0 dB |  |
|  | 19 GHz < f ≤ 26 GHz: ±4.5 dB |  |
| 6.6.5.5.1.2 Transmitter spurious emissions, Protection of BS receiver | ±3.0 dB | ±3.0 dB |
| 6.6.5.5.1.3 Transmitter spurious emissions, Additional spurious emission requirements | ±2.0 dB for > -60 dBm, f ≤ 3 GHz | ±2.0 dB for > -60dBm, f ≤ 3.0GHz |
|  | ±2.5 dB, 3 GHz < f ≤ 4.2 GHz | ±2.5 dB, 3.0GHz < f ≤ 4.2GHz |
|  | ±3.0 dB, 4.2 GHz < f ≤ 6 GHz | ±3.0 dB, 4.2GHz < f ≤ 6.0GHz |
|  | ±3.0 dB for ≤ -60 dBm, f ≤ 3 GHz | ±3.0 dB for ≤ -60dBm, f ≤ 3.0GHz |
|  | ±3.5 dB, 3 GHz < f ≤ 4.2 GHz | ±3.5 dB, 3.0GHz < f ≤ 4.2GHz |
|  | ±4.0 dB, 4.2 GHz < f ≤ 6 GHz | ±4.0 dB, 4.2GHz < f ≤ 6.0GHz |
| 6.6.5.2.4 Transmitter spurious emissions, Co-location | ±3.0 dB | ± 3.0 dB |
| 6.7 Transmitter intermodulation | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests in 6.6.3 (ACLR), 6.6.4 (OBUE) and 6.6.5 (spurious emissions) which have to be carried out in the presence of the interferer. | The value below applies only to the interference signal and is unrelated to the measurement uncertainty of the tests (6.6.2, 6.6.3 and 6.6.4) which shall be carried out in the presence of the interferer. |
| (interferer requirements) |  | . |
| This tolerance applies to the stimulus and not the measurements defined in 6.6.3, 6.6.4 and 6.6.5 | ±1.0 dB | ±1,0 dB |

Table 2-2: Maximum Test System uncertainty for receiver tests

| Clause | Maximum Test System Uncertainty defined in 38.141-1 | Maximum Test System Uncertainty defined in 36.141 |
| --- | --- | --- |
| 7.2 Reference sensitivity level | ±0.7 dB, f ≤ 3 GHz | ±0.7 dB, f ≤ 3.0GHz |
|  | ±1.0 dB, 3 GHz < f ≤ 4.2 GHz | ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |
|  | ±1.2 dB, 4.2 GHz < f ≤ 6 GHz | ±1.5 dB, 4.2GHz < f ≤ 6.0GHz |
| 7.3 Dynamic range | ±0.3 dB |  |
| 7.4.1 Adjacent channel selectivity | ±1.4 dB, f ≤ 3 GHz | ±1.4 dB, f ≤ 3.0GHz |
|  | ±1.8 dB, 3 GHz < f ≤ 4.2 GHz | ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |
|  | ±2.1 dB, 4.2 GHz < f ≤ 6 GHz (Note 2) | ±2.5 dB, 4.2GHz < f ≤ 6.0GHz |
| 7.4.2.4.2 In-band blocking (General blocking) | ±1.6 dB, f ≤ 3 GHz | ±1.6 dB, f ≤ 3.0GHz |
|  | ±2.0 dB, 3 GHz < f ≤ 4.2 GHz | ±2.0 dB, 3.0GHz < f ≤ 4.2GHz |
|  | ±2.2 dB, 4.2 GHz < f ≤ 6 GHz (Note 2) | ±2.7 dB, 4.2GHz < f ≤ 6.0GHz |
| 7.4.2.4.3 In-band blocking | ±1.4 dB, f ≤ 3 GHz | ±1.4 dB, f ≤ 3.0GHz |
| (Narrow band blocking) | ±1.8 dB, 3 GHz < f ≤ 4.2 GHz | ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |
|  | ±2.1 dB, 4.2 GHz < f ≤ 6 GHz (Note 2) | ±2.5 dB, 4.2GHz < f ≤ 6.0GHz |
| 7.5.5.1 Out-of-band blocking (General requirements) | fwanted ≤ 3GHz | fwanted ≤ 3GHz |
|  | 1MHz < finterferer ≤ 3 GHz: ±1.3 dB | 1MHz < finterferer ≤ 3 GHz: ±1.3 dB |
|  | 3.0GHz < finterferer ≤ 4.2 GHz: ±1.5 dB | 3.0GHz < finterferer ≤ 4.2 GHz: ±1.5 dB |
|  | 4.2GHz < finterferer ≤ 12.75 GHz: ±3.2 dB | 4.2GHz < finterferer ≤ 12.75 GHz: ±3.2 dB |
|  | 3GHz < fwanted ≤ 4.2GHz: | 3GHz < fwanted ≤ 4.2GHz: |
|  | 1MHz < finterferer ≤ 3 GHz: ±1.5 dB | 1MHz < finterferer ≤ 3 GHz: ±1.5 dB |
|  | 3.0GHz < finterferer ≤ 4.2 GHz: ±1.7 dB | 3.0GHz < finterferer ≤ 4.2 GHz: ±1.7 dB |
|  | 4.2GHz < finterferer ≤ 12.75 GHz: ±3.3 dB | 4.2GHz < finterferer ≤ 12.75 GHz: ±3.3 dB |
|  | 4.2GHz < fwanted ≤ 6.0GHz: | 4.2GHz < fwanted ≤ 6.0GHz: |
|  | 1MHz < finterferer ≤ 3 GHz: ±1.7 dB | 1MHz < finterferer ≤ 3 GHz: ±1.9 dB |
|  | 3.0GHz < finterferer ≤ 4.2 GHz: ±1.8 dB | 3.0GHz < finterferer ≤ 4.2 GHz: ±2.0 dB |
|  | 4.2GHz < finterferer ≤ 12.75 GHz: ±3.3 dB | 4.2GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |
| 7.5.5.2 Out-of-band blocking (Co-location requirements) | Co-location blocking, using CW interferer: |  |
|  | ±2.5 dB, f ≤ 3.0 GHz | ±2.5 dB, f ≤ 3.0GHz |
|  | ±2.6 dB, 3.0 GHz < f ≤ 4.2 GHz | ±2.6 dB, 3.0GHz < f ≤ 4.2GHz |
|  | ±2.7 dB, 4.2 GHz < f ≤ 6.0 GHz | ±2.9 dB, 4.2GHz < f ≤ 6.0GHz |
| 7.6 Receiver spurious emissions | 30 MHz ≤ f ≤ 4 GHz: ±2.0 dB | 30 MHz ≤ f ≤ 4 GHz:±2.0 dB |
|  | 4 GHz < f ≤ 19 GHz: ±4.0 dB | 4 GHz < f ≤ 19 GHz: ±4.0 dB |
|  | 19 GHz < f ≤ 26 GHz: ±4.5 dB |  |
| 7.7 Receiver intermodulation | ±1.8 dB, f ≤ 3.0 GHz | ±1.8 dB, f ≤ 3.0GHz |
|  | ±2.4 dB, 3.0 GHz < f ≤ 4.2 GHz | ±2.4 dB, 3.0GHz < f ≤ 4.2GHz |
|  | ±3.0 dB, 4.2 GHz < f ≤ 6.0 GHz (Note 2) | ±3.3 dB, 4.2GHz < f ≤ 6.0GHz |
| 7.8 In-channel selectivity | ±1.4 dB, f ≤ 3 GHz | ±1.4 dB, f ≤ 3.0GHz |
|  | ±1.8 dB, 3 GHz < f ≤ 4.2 GHz | ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |
|  | ±2.1 dB, 4.2 GHz < f ≤ 6 GHz (Note 2) | ±2.5 dB, 4.2GHz < f ≤ 6.0GHz |

The MU for 4.2GHz < f ≤ 6.0GHz in 36.141 is defined for LAA bands. And in current 38.141 the MU for the range applies for BS operates in licensed spectrum only. It can be found that the MU for licensed spectrum operation is tightened by 0.2 dB~0.5 dB. NR-U band n46 requirements is similar that of LAA band 46. It is foreseen the RF and also testing framework would be common for NR-U and LAA. Hence reusing the LAA measurement uncertainty may be benefit for the evolution to NR-U from LAA. The other aspect to be confirmed is the extension to 7.125 GHz will not further increase the MU which needs TE vendors’ inputs.

Proposal: It is proposed to reuse LAA MU requirements for n46 and n96

3 Conclusion

In this contribution, we provide discussion on NR-U measurement uncertainties for BS conformance testing.

Proposal: It is proposed to reuse LAA MU requirements for n46 and n96

Reference

- R4-2017464, WF on work plan and working split for NR-U BS conformance testing, ZTE