3GPP TSG-RAN WG4 Meeting #98-bis-e	R4- 2106504

Electronic Meeting, Apr. 12-20, 2021

Source: 	Intel Corporation

Title: 					RRM requirements for HST in FR2

Agenda Item: 			8.7.4.2

Document for:	Discussion

# 1	Introduction

In RAN4 #98e meeting the discussion on the UE RRM requirements for high speed train scenario in FR2 started. It was agreed that most of the decisions can be made only after the deployment scenario will be fixed. However, some initial conclusions on the applicability of existing requirements can already be made. The corresponding table was included into the WF [1] and is shown here for reference

Table 1-1: Rel-15/16 RRM requirements applicability (version from WF [1])

| RRM Req. Category | Sub-Category | Whether or not applicable to FR2 HST |
| --- | --- | --- |
| Idle/inactive state mobility | Cell selection/re-selection, measurement | FFS |
| Connected state mobility | Handover | FFS |
|  | Connection Mobility Control - RRC re-establishment | FFS |
|  | Connection Mobility Control - Random Access | No impact identified |
|  | Connection Mobility Control - RRC Release with Redirection | FFS |
| Timing | Autonomous timing adjustment | FFS |
|  | TX timing, timer, TA, Cell Phase Sync accuracy, MRTD/MTTD, deriveSSB-IndexFromCell tolerance | FFS |
| Signalling | RLM | FFS |
|  | Interruption | No impact identified |
|  | SCell Activation and Deactivation Delay | Not applicable to FR2 HST |
|  | UE UL carrier RRC reconfiguration delay | Not applicable to FR2 HST |
|  | Link Recovery | FFS |
|  | Active BWP switch delay | No impact identified |
|  | Active TCI state switching delay | FFS |
|  | PSCell Change | Not applicable to FR2 HST |
|  | Uplink spatial relation switch delay | FFS |
|  | UE-specific CBW change | No impact identified |
|  | Pathloss reference signal switching delay | No impact identified |
| Measurement Procedure | General measurement requirement | No impact identified |
|  | NR intra-frequency measurements | FFS |
|  | NR inter-frequency measurements | FFS |
|  | Inter-RAT measurement | FFS |
|  | L1-RSRP/L1-SINR Measurement | FFS |
|  | CSI-RS based L3 measurements | FFS |
|  | NR measurements with autonomous gaps | Not applicable to FR2 HST |

In this contribution we provide our views on the existing RRM requirements applicability for FR2 HST scenario and .

# 2	Discussion

Our views on the existing RRM requirements applicability for FR2 HST scenario is summarized in Table 2-1.

Table 2-1: Rel-15/16 RRM requirements applicability

| RRM Req. Category | Sub-Category | Intel’s view |
| --- | --- | --- |
| Idle/inactive state mobility | Cell selection/re-selection, measurement | Rel-15/16 requirements are not applicable. In FR2 HST the UE is in connected mode most part of the time. The ratio of idle/inactive mode is very low. The idle/inactive state related procedures can be performed during the periods when the CPE is static (e.g. when train stops on the station). In this case the corresponding applicability rule for the existing requirements should be added into the spec. New requirements should be introduced in case if above-mentioned case is not acceptable |
| Connected state mobility | Handover | Existing requirements work well |
|  | Connection Mobility Control - RRC re-establishment | - Requirements for Tidentify_intra_NR for known NR cell might be needed to introduce for FR2 case - The requirements tightening due to less RX beams in FR2 HST might be needed |
|  | Connection Mobility Control - Random Access | No impact identified |
|  | Connection Mobility Control - RRC Release with Redirection | Based on our understanding the UE redirection to another frequency is expected to be a very rare case for FR2 HST. However, the requirements (Tidentify-NR) tightening due to less RX beams in FR2 HST might be useful |
| Timing | Autonomous timing adjustment | FFS |
|  | TX timing, timer, TA, Cell Phase Sync accuracy, MRTD/MTTD, deriveSSB-IndexFromCell tolerance | No impact identified |
| Signalling | RLM | New requirements should be introduced |
|  | Interruption | No impact identified |
|  | SCell Activation and Deactivation Delay | Not applicable to FR2 HST |
|  | UE UL carrier RRC reconfiguration delay | Not applicable to FR2 HST |
|  | Link Recovery | New requirements should be introduced |
|  | Active BWP switch delay | No impact identified |
|  | Active TCI state switching delay | Consider only known TCI state |
|  | PSCell Change | Not applicable to FR2 HST |
|  | Uplink spatial relation switch delay | No impact identified |
|  | UE-specific CBW change | No impact identified |
|  | Pathloss reference signal switching delay | No impact identified |
| Measurement Procedure | General measurement requirement | No impact identified |
|  | NR intra-frequency measurements | New requirements should be introduced |
|  | NR inter-frequency measurements | Deprioritize |
|  | Inter-RAT measurement | Deprioritize |
|  | L1-RSRP/L1-SINR Measurement | New requirements should be introduced |
|  | CSI-RS based L3 measurements | Deprioritize |
|  | NR measurements with autonomous gaps | Deprioritize |

# 3	Conclusion

In this contribution, we presented our view on the applicability of the existing RRM requirements to HST in FR2 scenario.

Observation 1: Our views on the applicability of Rel-15/16 RRM requirements is summarized in Table 2-1

Table 2-1: Rel-15/16 RRM requirements applicability

| RRM Req. Category | Sub-Category | Intel’s view |
| --- | --- | --- |
| Idle/inactive state mobility | Cell selection/re-selection, measurement | Rel-15/16 requirements are not applicable. In FR2 HST the UE is in connected mode most part of the time. The ratio of idle/inactive mode is very low. The idle/inactive state related procedures can be performed during the periods when the CPE is static (e.g. when train stops on the station). In this case the corresponding applicability rule for the existing requirements should be added into the spec. New requirements should be introduced in case if above-mentioned case is not acceptable |
| Connected state mobility | Handover | Existing requirements work well |
|  | Connection Mobility Control - RRC re-establishment | - Requirements for Tidentify_intra_NR for known NR cell might be needed to introduce for FR2 case - The requirements tightening due to less RX beams might be needed |
|  | Connection Mobility Control - Random Access | No impact identified |
|  | Connection Mobility Control - RRC Release with Redirection | Based on our understanding the UE redirection to another frequency is expected to be a very rare case for FR2 HST. However, the requirements (Tidentify-NR) tightening due to less RX beams might be useful |
| Timing | Autonomous timing adjustment | FFS |
|  | TX timing, timer, TA, Cell Phase Sync accuracy, MRTD/MTTD, deriveSSB-IndexFromCell tolerance | No impact identified |
| Signalling | RLM | New requirements should be introduced |
|  | Interruption | No impact identified |
|  | SCell Activation and Deactivation Delay | Not applicable to FR2 HST |
|  | UE UL carrier RRC reconfiguration delay | Not applicable to FR2 HST |
|  | Link Recovery | New requirements should be introduced |
|  | Active BWP switch delay | No impact identified |
|  | Active TCI state switching delay | Consider only known TCI state |
|  | PSCell Change | Not applicable to FR2 HST |
|  | Uplink spatial relation switch delay | No impact identified |
|  | UE-specific CBW change | No impact identified |
|  | Pathloss reference signal switching delay | No impact identified |
| Measurement Procedure | General measurement requirement | No impact identified |
|  | NR intra-frequency measurements | New requirements should be introduced |
|  | NR inter-frequency measurements | Deprioritize |
|  | Inter-RAT measurement | Deprioritize |
|  | L1-RSRP/L1-SINR Measurement | New requirements should be introduced |
|  | CSI-RS based L3 measurements | Deprioritize |
|  | NR measurements with autonomous gaps | Deprioritize |

# 4	References

- R4-2103679 “WF on Rel-17 NR HST FR2 enhancements, RRM requirements”, Nokia, Nokia Shanghai Bell, Jan. 25th – Feb. 5th, 2021