3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106508

Electronic Meeting, Apr. 12-20, 2021

Source: 	Intel Corporation

Title: 					NR-U PUSCH simulation results

Agenda Item: 			5.1.4.4.2

Document for:	Discussion

# 1	Introduction

This paper presents simulation results considering the parameters agreed during the previous meetings [1].

# 2	Discussion

The simulation results are shown in the figures in Table 2-2 below and summarized in Table 2-3 and Table 2-4. The simulation assumptions are shown in Table 2-1

Table 2-1: PUSCH simulation assumptions

| PARAMETER | VALUE |
| --- | --- |
| Transform precoding | Disabled |
| Antenna configuration | 1T2R Low |
| SCS and BW | 15kHz: 20MHz; 30kHz: 20MHz |
| TDD pattern | 7D2S1U S=6D:4G:4U for 30kHz SCS 3D1S1U S=10D:2G:2U for 15kHz SCS |
| Interlace structure | First single interlace per slot |
| DM-RS type | type 1 with single-symbol |
| Number of DMRS symbols | dmrs-AdditionalPosition ‘pos1’ (1+1) |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | Type A and B |
| Frequency domain resource | Full applicable test bandwidth |
| MCS index | 20 |
| Carrier frequency (GHz) | 4 |
| Propagation condition | TDLA30-10 |
| Guard band | Not configured |
| PT-RS | Not configured |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| RV sequence | {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |

Table 2-2: PUSCH throughput statistics

Table 2-3: Summary of alignment PUSCH simulation results

| Bandwidth (MHz) |  | 20 |  |
| --- | --- | --- | --- |
| SCS (kHz) |  | 15 | 30 |
| SNR@70% of maxTP(dB) | DMRS Type A | 9.5 | 9.3 |
|  | DMRS Type B | 9.5 | 9.3 |

Table 2-4: Summary of impairments PUSCH simulation results

| Bandwidth (MHz) |  | 20 |  |
| --- | --- | --- | --- |
| SCS (kHz) |  | 15 | 30 |
| SNR@70% of maxTP(dB) | DMRS Type A | 12.0 | 11.8 |
|  | DMRS Type B | 12.0 | 11.8 |

# 3	Conclusion

In this contribution we provided PUSCH simulation results for NR-U.

# 4	References

- R4-2103988 “Way forward on NR-U BS demodulation requirements for general part and PUSCH” Huawei, HiSilicon, RAN4 #98-e, Feb 2021