3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2106526

Electronic Meeting, Apr. 12 – 20, 2021

Agenda item:	5.7.2.2.1

Source: 	OPPO

Title: 	Simulation results for CSI-RS RSRP accuracy requirements

Document for:	Discussion

Introduction

In RAN4 #98-e meeting, the impacts of negative and positive timing offset on the CSI-RS based RSRP measurement were discussed [1] and the simulation assumptions were updated accordingly [2]. This contribution will provide our simulation results for CSI-RS RSRP based on the following conditions:

- CSI-RS configuration: <X=1,D=3,N=1,NA,noCDM> with bandwidth=48RBs

- Number of samples: 5

- Carrier frequency: 4GHz for FR1 and 30GHz for FR2

- SNR configuration: 6dB for cell 1 and 1dB for cell 2 (the Es/Iot for cell 2 is -5.97dB)

- Propagation condition: AWGN for FR1 and TDL-A with 30ns delay spread for FR2

- Relative delay for cell 2: 0, ±CP/2, ±0.9*CP, ±CP

Discussion

Table 1-3 provide the delta CSI-RS RSRP of cell 2 for 15kHz, 30kHz and 120kHz subcarrier spacing, respectively. And our observations drawn from the simulation results are given below.

Observation 1: The absolute measurement error of CSI-RS RSRP with negative timing offset is worse than that of CSI-RS RSRP with positive timing offset.

Observation 2: The absolute measurement error of CSI-RS RSRP can be within ±2.0dB for timing offset within [-0.5*CP, +0.5*CP], and ±3.0dB for timing offset within [-CP, +CP].

Observation 3: The relative measurement error of CSI-RS RSRP can be within ±1.5dB for timing offset within [-0.5*CP, +0.5*CP], and ±2.0dB for timing offset within [-CP, +CP].

Table 1: SCS=15 kHz, AWGN channel @ FR1

| Relative delay | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | -1.431 | -0.183 | 0.827 | 1.248 | 1.01 | 1.248 |
| 0.5*CP | -1.519 | -0.232 | 0.73 | 1.287 | 0.962 | 1.287 |
| 0.9*CP | -1.748 | -0.437 | 0.492 | 1.311 | 0.929 | 1.311 |
| CP | -1.661 | -0.427 | 0.556 | 1.234 | 0.983 | 1.234 |
| -0.5*CP | -1.97 | -0.779 | 0.532 | 1.191 | 1.311 | 1.311 |
| -0.9*CP | -2.48 | -0.972 | -0.204 | 1.508 | 0.768 | 1.508 |
| -CP | -2.804 | -1.056 | 0.135 | 1.748 | 1.191 | 1.748 |

Table 2: SCS=30 kHz, AWGN channel @ FR1

| Relative delay | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | -1.411 | -0.187 | 0.819 | 1.224 | 1.006 | 1.224 |
| 0.5*CP | -1.516 | -0.218 | 0.742 | 1.298 | 0.96 | 1.298 |
| 0.9*CP | -1.693 | -0.406 | 0.548 | 1.287 | 0.954 | 1.287 |
| CP | -1.682 | -0.395 | 0.522 | 1.287 | 0.917 | 1.287 |
| -0.5*CP | -1.937 | -0.53 | 0.545 | 1.407 | 1.075 | 1.407 |
| -0.9*CP | -2.453 | -0.984 | 0.196 | 1.469 | 1.18 | 1.469 |
| -CP | -2.626 | -1.069 | 0.089 | 1.557 | 1.158 | 1.557 |

Table 3: SCS=120 kHz, TDL-A channel @ FR2

| Relative delay | 5% (dB) | 50% (dB) | 95% (dB) | 50%-5% | 95%-50% | Relative accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | -1.443 | -0.164 | 0.869 | 1.279 | 1.033 | 1.279 |
| 0.5*CP | -1.728 | -0.231 | 0.923 | 1.497 | 1.154 | 1.497 |
| 0.9*CP | -1.726 | -0.406 | 0.601 | 1.32 | 1.007 | 1.32 |
| CP | -1.787 | -0.471 | 0.5 | 1.316 | 0.971 | 1.316 |
| -0.5*CP | -1.725 | -0.37 | 0.61 | 1.355 | 0.98 | 1.355 |
| -0.9*CP | -2.248 | -0.696 | 0.498 | 1.552 | 1.194 | 1.552 |
| -CP | -2.133 | -0.798 | 0.36 | 1.335 | 1.158 | 1.335 |

Conclusion

This paper provides our simulation results for CSI-RS RSRP measurements and the following observations.

Observation 1: The absolute measurement error of CSI-RS RSRP with negative timing offset is worse than that of CSI-RS RSRP with positive timing offset.

Observation 2: The absolute measurement error of CSI-RS RSRP can be within ±2.0dB for timing offset within [-0.5*CP, +0.5*CP], and ±3.0dB for timing offset within [-CP, +CP].

Observation 3: The relative measurement error of CSI-RS RSRP can be within ±1.5dB for timing offset within [-0.5*CP, +0.5*CP], and ±2.0dB for timing offset within [-CP, +CP].

- References

- R4-2104089, WF on performance requirements of CSI-RS based L3 measurement, CATT, OPPO

- R4-2103725, “Updated link-level simulation assumptions for CSI-RS based L3 measurements”, CATT