3GPP TSG-RAN WG4 Meeting #98-bis-e		R4-2106567
Electronic Meeting, 12th Apr – 20th Apr, 2021

Agenda item:	8.1.3.3

Source:	OPPO

Title:	Consideration on Spatial Correlation with combined beam

Document for:	Approval

1	Introduction

In the last meeting of RAN4 #98-e, the gNB beam usage criteria for spatial correlation of FR1 MIMO OTA was discussed [1]. And the option of Beam specific approach was selected as baseline [2].

This paper presents our consideration on the impact of spatial correlation validation using beam specific/combined beam approaches.

2	Discussion

The conclusion of selecting beam specific approach as baseline is drawn based on the fact that the beams are connected to the input ports of channel emulator separately, not combined, during the actual FR1 MIMO OTA test. So channel model validation under beam specific approach matches the real system implementation.

Our consideration on this topic is from another point of view, i.e. DUT. Specifically, DUT while performing MIMO OTA test receives wireless signals from all the clusters with both of the beam energy combined. So, the key factor that influences the DUT MIMO performance is related to the energy from combined beams, not any one single beam. Based on this fact, we think it is more reasonable to verify the spatial correlation of channel model with combined beam approach, making the DUT verified under a standardized and unified wireless channel environment. This is the justification of supporting combined beam approach.

Observation 1: DUT while performing MIMO OTA test receives wireless signals from all the clusters with both of the beam energy combined.

From the channel model validation perspective, simulations in R4-2102613 have shown that the spatial correlation profile is totally different with that of any single beam, illustrated below cited from R4-2102613. So for DUT, correct spatial correlation with combined beams is essential.

Another question to be answered is that whether the combined beam approach is enough. For CDL-C UMa, simulation results with 16 measurement probes show that the three conditions of Beam 1, Beam 2 and combined beams coincide well with the results with infinite probes in the above figure. Because of low coefficient of the two gNB beams for CDL-C UMa, it can be concluded that the coincided spatial correlation with combined beam approach reveals both of the two beams coincided well, accomplished with the PDP validation done separately for each beam. So the beam specific approach is not necessary under this situation.

Observation 2: For CDL-C UMa, the coincided spatial correlation with combined beam approach reveals both of the two beams coincided well, accomplished with the PDP validation done separately for each beam.

Moreover, combined beam approach is quite time efficient since all the CE input ports are combined with one measurement.

Proposal: RAN4 should reconsider the baseline based on the combined beam approach to perform the spatial correlation validation.

3	Conclusion

Observation 1: DUT while performing MIMO OTA test receives wireless signals from all the clusters with both of the beam energy combined.

Observation 2: For CDL-C UMa, the coincided spatial correlation with combined beam approach reveals both of the two beams coincided well, accomplished with the PDP validation done separately for each beam.

Proposal: RAN4 should reconsider the baseline based on the combined beam approach to perform the spatial correlation validation.

References

- R4-2102613 gNB Beam Usage_v3

- R4-2103913 WF on NR MIMO OTA_v3