3GPP TSG-RAN WG4 Meeting#98-bis-e	 R4-2106582

E-meeting, Apr 12th – Apr 20th, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion about RLM/BFD measurement relaxation

Agenda item:	8.9.2

Document for:	Discussion

Introduction

In RAN4#98-e meeting, the discussion about UE power saving enhancements continued with further details and simulation results. A way forward was agreed in [1] to capture the outcome of the discussion. Evaluation assumptions for UE power saving and simulation level study were captured with updates in [2].

In this contribution we discuss the potential of RLM and BFD measurement relaxation from UE power saving perspective based on simulated power saving gain and system level performance results that are shown in [3]. Because of the latest plenary agreement, we restrict the discussion in this contribution to RLM and BFD relaxation without RRM measurement relaxation.

Clarification of the scenarios to be studied

In the last RAN4 meeting, the following was agreed in the WF [1] about the scenarios to be studied for UE power saving enhancements for NR:

Issue 2-2-1: Confirmation on beneficial Scenarios, from UE power saving gain perspective - Agreements - Further evaluate UE power saving gains for the following UE implementations: - UE meets Rel-15 RRM measurement period and accuracy requirements - Option 1: - UE uses all L1 samples for RRM measurements based on Rel-15 assumptions - Option 2: - How many L1 samples UE applies for RRM measurements is up to UE implementation (e.g. UE can use lower number of measurement samples for RRM measurements) - Further discuss how many samples to use for evaluations - Companies shall evaluate RRM measurements accuracy for the proposed number of samples. - FFS whether Option 2 can be considered for requirements definition - Further assess impact on PDCCH monitoring due to relax UE measurements for RLM/BFD

However, in the latest RAN plenary meeting, the work item scope was clarified in the following way:

RAN Plenary guidance to RAN4: For Rel-17 WI of UE power saving enhancements for NR, no specification impact to RRM measurement procedure requirements and measurement performance requirements is expected.

Because of this agreement, we think that Option 2 should be removed from the scenarios to be studied, and the work item should only include Option 1, which is for the case where only RLM and BFD measurements, not RRM measurements, are relaxed.

Remove Option 2 (How many L1 samples UE applies for RRM measurements is up to UE implementation) from the list of scenarios to be studied for the UE power saving enhancements WI.

System level analysis

In our system level simulations, we have studied delta SINR and the time the UE spends in outage with and without RLM and BFD measurement relaxation. The results are shown in [3].

Delta SINR

Related to our simulations, we have observed three different ways to calculate delta SINR. Details of the options can be found in our simulation result contribution in [3] with figures. Each option leads to slightly different results for delta SINR. For better alignment of these results, we think RAN4 should clarify which of the options should be used as the baseline.

There are multiple ways to calculate delta SINR, and the simulation results depend on the chosen scenario.

Time of outage

In addition to delta SINR, we have evaluated the time the UE spends in outage with different relaxation factors and compared this to the Rel-15 baseline. The time of outage is counted whenever SINR for hypothetical PDCCH BLER in Table 8.1.1-1 of TS 38.133 falls below the threshold.

Based on our simulation results shown in [3], we have observed that the average time of each outage increases with higher relaxation of RLM and BFD measurements due to the late detection of failure and initiating the recovery procedure. The total number of outage occurrences is decreased, which is due to relaxed measurements which results in missing detection of failure. The percentage of increase in time of outage is also shown below for convenience.

Table 1: Percentage of increase in ToO; (ToO_relaxed – ToO_baseline)/ToO_baseline.

| Speed/K | 2 | 4 | 8 |
| --- | --- | --- | --- |
| 3 km/h | 152% | 367% | 739% |
| 30 km/h | 121% | 359% | 800% |
| 60 km/h | 74% | 233% | 537% |

The time the UE spends in outage increases significantly when the relaxation factor for RLM and BFD measurements increases due to the late detection of failure and initiating the recovery procedure.

It should be taken into account that our simulations are done in moderate radio conditions, and if the conditions get worse, even more delays in failure detection may occur. Depending on the relaxation scenarios RAN4 agrees to work on, it is important to make sure that there will not be a major performance degradation on system level.

Negative system level impact due to RLM/BFD relaxation should be minimized e.g. by studying the time of outage with different relaxation factors.

Power saving analysis

In [3] we have shown our simulation results for RLM and BFD measurement relaxation by extending the evaluation period with factor K = 2, 4, and 8.

In FR1, it is observed that in our simulations, we do not obtain any power saving gain if only RLM and BFD measurements are relaxed. This is because the UE anyway performs RRM measurements during the SMTC window, which consume power. The same measurement is assumed to be used to evaluate also radio link and beam quality, so not doing such evaluations does not lead to power saving in our analysis.

Based on our simulations, there is no power saving gain in FR1 when only RLM and BFD measurements are relaxed by extending the evaluation period.

In FR2, we can observe based on our simulations, we obtain limited power saving gains (less than 3%), if only RLM and BFD measurements are relaxed. This is because the UE anyway performs RRM measurements during the SMTC window, which cannot be relaxed. As RRM measurements are much more frequent than RLM and BFD measurements in FR2 (thanks to the relaxation factor P applied to RLM and BFD), thereby it dominates the overall power consumption.

In FR2, power saving gain of less than 3 % can be achieved by relaxing RLM and BFD measurements by extending the evaluation period in our simulations.

Discussion about other details

Criteria for entering the relaxed RLM/BFD measurement mode

In the way forward [1], the following was agreed about the criteria to enter relaxed RLM/BFD measurement mode:

- Issue 2-3-1: Criteria which the UE is allowed to relax the RLM/BM requirements - At least take UE mobility into account as the relaxation criteria. - also take serving cell’s quality into account - FFS whether and how to take other aspects into account - Issue 2-3-3: How to consider serving cell’s quality as relaxation criteria - RAN4 to further discuss how to take serving cell’s quality into account for the relaxation criteria - FFS how to consider serving cell’s quality. E.g. Based on SINR or BLER. - FFS how to address different UE implantation issues. - FFS: When radio link quality > Qout + X (dB) for RLM and Qout,LR + Y (dB) for BFD relaxation - X and Y are FFS. - Issue 2-3-2: How to consider UE mobility as relaxation criteria - FFS the following options - Option 1: R16 low-mobility criterion should not be directly reused in R17 SINR-based criterion for RLM/BFD relaxation. - Option 2: R16 RRM relaxation criterion can be used as baseline for RLM/BFD relaxation. - Option 3: “low mobility criteria” should consider both UE velocity and the channel quality variation. - Option 4: Consider time associated with a given condition when determining UE mobility state. - Option 5: Low mobility scenario under which the UE is allowed to apply the RLM/BFDBM requirements is determined and configured to UE by the network - Other options are not precluded - Issue 2-3-4: Network or UE to determine if the relaxation criteria is fulfilled - Network to enable and disable this feature. - FFS Should the relaxation criteria be predefined or configurable? - FFS Should it be network or UE to determine the relaxation criteria is fulfilled or not?

It was agreed that both UE mobility and serving cell quality will be used as the criteria for defining whether the UE is allowed to enter relaxed measurement mode. Several options were listed for both criteria regarding how to take the criteria into account.

For the serving cell quality, we think SINR can be used as the quality measure, since RLM and BFD are SINR based measures. The exact metric needs further discussion.

Use SINR as the quality measure for serving cell quality. FFS the exact metric.

Regarding UE mobility, as we stated in the last meeting, we think it would be preferable to use a simple condition for entering relaxed mode (or in general when UE can be considered to be in state/condition where it can relax its RLM/BFD measurements). As an example, if the UE has been under coverage of specific beams for certain period of time, it could be considered by the network to be stationary/low mobility UE. In similar manner the UE based condition to enter low mobility state could be based on same mechanism. As an example, if UE’s active TCI state for PDCCH does not change for specific time period, it could be allowed to adapt measurements.

Correspondingly, for a moving UE in an area which allows relaxed UE measurement, timer based approach prevents that the UE does not apply reduced measurements for a period of time after entering a given cell. This means that only after a given time period after entering a new cell the UE would be allowed to apply relaxed measurements. Such approach would ensure robust mobility for moving UEs while allowing non-moving UEs to perform relaxed measurements.

If UE is under coverage of a specific cell or beam for certain amount of time or certain observed conditions do not change for a predefined time, the UE could be considered to be in stationary/low mobility state.

Consider time associated with a given condition when determining UE mobility state.

Criteria for reverting back to normal measurements

In the way forward [1], the following FFS point and options were agreed related to the condition that would lead to the UE reverting back to normal (non-relaxed) RLM/BFD measurements:

- Issue 2-5-1: Reverting to the normal RLM operation - The UE while performing relaxed RLM upon detecting certain number of out-of-sync indications or upon triggering T310 or upon observed link quality degradation or mobility state change reverts to the normal RLM operation (i.e. without relaxation). - FFS the following options - Option 1a: revert when the relaxation criterion is not met - Option 1b: revert when N310 starts to count, i.e. 1 out-of-sync indication. - Option 1c: revert when T310 is running, i.e. N310 out-of-sync indication. - Option 1d: revert when observed link quality degradation. - Option 1e: revert regarding observed mobility state change. - Other options are not precluded - Issue 2-5-2: Reverting to the normal BFD operation - Option 1: Reverting to the normal BFD operation upon detect 1 beam failure instance indication. - Option 2: The UE while performing relaxed BFD upon beam failure detection reverts to the normal BFD operation (i.e. without relaxation). - Option 3: There might be no benefit to configure conditions for UE reverting to normal BFD. - Option 4: Whether reverting to normal BFD operation aligns with whether reverting to normal RLM operation - Other options are not precluded.

Several options were listed in the last RAN4 meeting for the criteria for reverting back to normal RLM/BFD operation, and no agreement was made yet. It is important that negative system level performance is not allowed due to relaxed measurements, so we think that it should be made sure that the UE reverts back to normal measurements as soon as negative system level performance impact may occur.

If the UE is allowed to adapt its RLM/BFD measurements based on e.g. mobility state, and there is no or very little data activity, it may take a long time before network could identify that UE has ‘changed the mobility state’. If UE considers itself to be in low mobility state or not at cell edge and maintains reduced measurement activity, but has actually e.g. started to move, there could be negative implications to mobility (and also beam management) performance due to e.g. delays in measurements.

Robust, UE autonomous mechanism, is needed to determine when UE should change back to normal measurement activity if UE has adapted its activity based on e.g. ‘mobility’ state.

This “exit” condition should be defined at least for case when UE is autonomously determining the conditions when it can adapt the RLM/BFD measurements. If UE RLM/BFD measurement activity adaptation is supported, there should be robust mechanism enabling returning to normal measurement activity in order to avoid negative system impacts.

When a UE is operating with relaxed RLM/BFD requirements, it is important that this operation does not impact the long-term UE performance related to RLF/BFD, since this would potentially increase the UE power consumption through the recovery procedures and/or negatively impact network performance parameters. Hence, when performing relaxed RLM/BFD operation, it could be considered whether UE would use the normal configured values for e.g., RLM, or whether some parameters should be adapted/changed for the relaxed operation. That is, the UE should potentially apply different values for the RLM parameters. I.e. N310/N311 values could be different from the N310/N311 values that are used for normal RLM operation. By applying for example different N310/N311 values in relaxed RLM mode, it is possible to try to compensate for the negative implications to the system performance.

When operating in relaxed RLM/BFD mode, there could be alternate values for related parameters such has values for N310/N311.

In any case, like discussed above, there should be robust mechanisms to ‘exit’ from relaxed mode i.e. fallback to normal measurement operation. I.e. when the UE detects problem with radio link quality such as out-of-synch indication or beam failure instance indication, it should revert back to normal measurement operation to monitor the link to ensure timely link recovery mechanism initialization, if needed.

Observed link quality degradation should cause the UE to revert back to normal measurement operation.

Conclusion

In this contribution we have discussed the UE power saving enhancement topic further. We have made the following observations and proposals:

- Remove Option 2 (How many L1 samples UE applies for RRM measurements is up to UE implementation) from the list of scenarios to be studied for the UE power saving enhancements WI.

- There are multiple ways to calculate delta SINR, and the simulation results depend on the chosen scenario.

The time the UE spends in outage increases significantly when the relaxation factor for RLM and BFD measurements increases due to the late detection of failure and initiating the recovery procedure.

Negative system level impact due to RLM/BFD relaxation should be minimized e.g. by studying the time of outage with different relaxation factors.

Based on our simulations, there is no power saving gain in FR1 when only RLM and BFD measurements are relaxed by extending the evaluation period.

In FR2, power saving gain of less than 3 % can be achieved by relaxing RLM and BFD measurements by extending the evaluation period in our simulations.

Use SINR as the quality measure for serving cell quality. FFS the exact metric.

If UE is under coverage of a specific cell or beam for certain amount of time or certain observed conditions do not change for a predefined time, the UE could be considered to be in stationary/low mobility state.

Consider time associated with a given condition when determining UE mobility state.

Robust, UE autonomous mechanism, is needed to determine when UE should change back to normal measurement activity if UE has adapted its activity based on e.g. ‘mobility’ state.

When operating in relaxed RLM/BFD mode, there could be alternate values for related parameters such has values for N310/N311.

Observed link quality degradation should cause the UE to revert back to normal measurement operation.

References

[1] R4-2103670, WF on NR UE Power Saving Enhancements, MediaTek, Vivo, 3GPP TSG-RAN WG4 Meeting #98e, Electronic Meeting, Jan. 25-Feb. 5, 2021.

[2] R4-2104066, Updated evaluation assumptions for R17 RLM/BFD relaxation, vivo, MediaTek, 3GPP TSG-RAN WG4 Meeting #98-e, Electronic Meeting, an. 25 – Feb. 5, 2021.

[3] R4-2106581, Simulation results for RLM/BFD measurement relaxation, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 Meeting#98-bis-e, E-meeting, Apr 12th – Apr 20th, 2021.