3GPP TSG-RAN WG4 Meeting # 98-bis-e                	R4-2106591

E-meeting,12thApril  – 20th April, 2021

Agenda item:	8.12.6

Source: 	ZTE Corporation

Title: 	Discussion on switching delay for 52.6-71GHz

Document for:	Approval

Background

In the last RAN1#104e meeting, one LS on beam switching gap for 60GHz to RAN4 was approved to seek the feedback on the following aspect. Therefore in this contribution, we want to share more initial understandings on these aspects.

RAN1 would like to ask RAN4 on time required for gNBs and UEs operating in 52.6 GHz to 71 GHz to perform the following operations:

- Switching Tx beams

- Switching Rx beams

- Switching from DL to UL

- Switching from UL to DL

Discussion

## 2.1. beam switching delay for Tx and Rx beams

- Switching Tx beams

- Switching Rx beams

For switching Tx beams and Rx beams, it’s well known that those beam switching delay should mainly depend on response delay of analog beam shifter within mmWave antenna array and common RF components are used for Tx and Rx beam switching, therefore switching delay for Tx and Rx beams should be same.

Observation 1:  switching delay for Tx and Rx beams should be the same.

For the specific beam switching delay, it was documented in TR 38.817-02 that the worst case for analog phase shifter for 28GHz should be 100ns, for switched phase shifter, like GaAs switches react in the approximately 10ns. As far as we know that, response delay for analog phase shifter is not quite dependent on the operation frequency, in other words, the proposed value for beam switching delay in Rel-15 could also been applied for 60GHz in Rel-17.

For 60GHz WID, due to the introduction of 480 kHz and 960 kHz SCS, then the length of CP used to accommodate beam switching would be much shorter than that of 60 kHz and 120 kHz in Rel-15/Rel-16. Especially for 960kHz SCS, then analog phase shifter with reaction delay around 100ns or 10s of ns might be not approximate choice for the implementation anymore, then switched based phase shifter should be more reasonable one, however whether GaAs based phase shifter with apprixmately10ns reaction delay should be used as baseline for further analysis need more discussion in RAN4. From our initial understandings, switched phase shifter with apprximately10ns reaction delay should be the optimum performance achieved based on current state of the art, however from massive manufacturing’s perspective for the practical deployment, some implementation variation  (e.g. due to delay variation of PCB controlling line) should be also been considered for better ecosystem prosperity. Based on the above considerations, the feasible beam switching delay for 60GHz WID should be around 20ns.

Table 1. Summary for CP and data length for different SCS

| u | SCS | CP[ns] | data[ns] | Sampling Rate [MHz] |
| --- | --- | --- | --- | --- |
| 2 | 60 | 1171.875 | 16666.6 | 245.76 |
| 3 | 120 | 585.9375 | 8333.3 | 491.52 |
| 4 | 240 | 292.96875 | 4166.6 | 983.04 |
| 5 | 480 | 146.484375 | 2083.3 | 1966.08 |
| 6 | 960 | 73.2421875 | 1041.6 | 3932.16 |

Proposal 1:  the feasible beam switching delay for 60GHz should be around 20ns from manufacturing’s variations.

## 2.2. switching time between DL an UL

- Switching from DL to UL

- Switching from UL to DL

For the switching time from DL to UL and UL to DL, since those values are contributed by two major factors, ON-OFF/OFF-ON transient period and  cell synchronization error. For cell synchronization error as 3us, it’s unlikely to be changed for 60GHz WID since similar timing network implementation and extreme corner case as the existing FR2 network would be expected for 60GHz WID unless there are some other deployment scenarios intended for. For ON-OFF/OFF-ON transition period, there were some concerns [2] raised in the last meeting on its high overhead especially for 480kHz and 960kHz and some further improvement are encouraged to be investigated.

From the hardware feasibility perspective, with the increasing operation frequency for 60GHz, then settling time for PLL might be shorter compared with that of existing FR2. In addition, the transceiver and PA setup time and PA ramp up time for 60GHz should also been considered.

Proposal 2: for ON-OFF/OFF-ON transition time for 60GHz, settling time for PLL, transceiver and PA setup time and PA ramp up time should be considered for 60GHz from hardware feasibility perspective.

Proposal 3: for cell synchronization error for 60GHz WID, 3us should be reused.

In addition, as mentioned before,the GP overhead with the following TDD periodicity assumption as analyzed in Table 1 and Table 2 might be too high if the same ON-OFF transition time for the existing FR2 was reused for 60GHz. Alternatives to reduce the GP overhead could be either extend the TDD periodicity for 480kHz and 960kHz or reduce ON-OFF transition time from BS and UE side to shorten the GP time.

Observation 2: to reduce the GP overhead for 480kHz and 960kHz of 60GHz, alternatives could be either extend the TDD periodicity or reduce ON-OFF transition time from BS and UE side.

Table 1 cell coverage analysis

| Frequency range | Existing FR2 |  | 60GHz |  |
| --- | --- | --- | --- | --- |
| Numerology (kHz) | 60 | 120 | 480 | 960 |
| GP (symbol) | 1 | 2 | 8 | 16 |
| GP (us) | 16.67 | 16.67 | 16.67 | 16.67 |
| Sync error (us) | 3 | 3 | 3 | 3 |
| BS OFF-ON (us) | 3 | 3 | 3 | 3 |
| UE OFF-ON (us) | 5 | 5 | 5 | 5 |
| Tcell (us) | 1.3333 | 1.3333 | 1.3332 | 1.3328 |
| Cell coverage (m) | 399.99 | 399.99 | 399.96 | 399.84 |
| Periodicity of TDD (ms) | 0.5 | 0.25 | 0.0625 | 0.03125 |
| GP overhead (%) | 3.58 | 7.12 | 26.67 | 53.33 |

Table 2 BS to BS distance without interference

| Frequency range | Existing FR2 |  | 60GHz |  |
| --- | --- | --- | --- | --- |
| Numerology (kHz) | 60 | 120 | 480 | 960 |
| GP (symbol) | 1 | 2 | 8 | 16 |
| GP (us) | 16.67 | 16.67 | 16.67 | 16.67 |
| Sync error (us) | 3 | 3 | 3 | 3 |
| BS ON-OFF (us) | 3 | 3 | 3 | 3 |
| BS OFF-ON (us) | 3 | 3 | 3 | 3 |
| Tbs-bs (us) | 4.67 | 4.67 | 4.67 | 4.67 |
| Two BSs distance (m) | 1401 | 1401 | 1399.92 | 1399.68 |
| Periodicity of TDD (ms) | 0.5 | 0.25 | 0.0625 | 0.03125 |
| GP overhead (%) | 3.334 | 6.668 | 26.67 | 53.33 |

Conclusion

In this contribution, we want to share more initial understandings on these aspects.

Observation 1:  switching delay for Tx and Rx beams should be the same.

Proposal 1:  the feasible beam switching delay for 60GHz should be around  20ns from manufacturing’s variations.

Proposal 2: for ON-OFF/OFF-ON transition time for 60GHz, settling time for PLL, transceiver and PA setup time and PA ramp up time should be considered for 60GHz from hardware implementation perspective.

Proposal 3: for cell synchronization error for 60GHz WID, 3us should be reused.

Observation 2: to reduce the GP overhead for 480kHz and 960kHz of 60GHz, alternatives could be either extend the TDD periodicity or reduce ON-OFF transition time from BS and UE side.

Reference

- TR 38.817-02

[2] R4-2103260,TP to TR 38.808: Timing considerations for operation between 52.6 and 71 GHz, approved.

[3] R1-2102202, LS on beam switching gap for 60 GHz band, Intel Corporation, approved.

[4] R4-1805766, LS on UE Rx/Tx Transition Times, Qualcomm, approved.

[5] R4-1706675,TDD Timing budget for NR